3GPP TSG-RAN WG4 Meeting #110						                 					     R4-2403535

Athens, Greece, February 26 – March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4008 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on combination of HST and RRM relaxation - R18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST, NR_UE_pow_sav-Core |  |  |  |  |  | Date: |  |  | 2024-03-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 received an LS from RAN2 (R2-2311435) on combination of HST and RRM relaxation. After discussion, RAN4 agreed to clarify UE measurement behavior when both HST flag and low mobility relaxation are enabled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify UE measurement behavior when both HST flag and low mobility relaxation are enabled. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When both HST flag and low mobility relaxation are enabled, UE measurement behavior would still be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.3, 4.2.2.4, 4.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

#### 4.2.2.3	Measurements of intra-frequency NR cells

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS38.304 [1] within Tdetect,NR_Intra when that Treselection= 0 . An intra frequency cell is considered to be detectable according to the conditions defined in Annex B.1.2 for a corresponding Band.

The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra (see table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5) for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5 provided that:

when rangeToBestCell is not configured:

-	the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them.

-	the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2 if the current serving cell is among them.

When evaluating cells for reselection, the SSB side conditions apply to both serving and non-serving intra-frequency cells.

If Treselection timer has a non zero value and the intra-frequency cell is satisfied with the reselection criteria which are defined in TS38.304 [1], the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

For UE neither configured with highSpeedMeasFlag-r16 nor highSpeedMeasFlagFR2-r17, Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate, NR_intra are specified in Table 4.2.2.3-1. For UE configured with highSpeedMeasFlag-r16, Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate, NR_intra are specified in Table 4.2.2.3-2.

For a UE configured with highSpeedMeasFlag-r16 or highSpeedMeasFlagFR2-r17, and configured with relaxed measurement criterion (lowMobilityEvaluation or both lowMobilityEvaluation and cellEdgeEvaluation), it is up to UE implementation whether to apply the relaxed measurements requirements specified in clause 4.2.2.9.2 or high speed requirements specified in this clause provided that the relaxed measurement criteria in section 4.2.2.9.2 are fulfilled.

For FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17, Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate, NR_intra are specified in Table 4.2.2.3-3.

For UE not configured with eDRX_IDLE cycle, Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra are specified in Table 4.2.2.3-1, 4.2.2.3-2 and 4.2.2.3-3.

For UE configured with eDRX_IDLE cycle, Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra are specified in Table 4.2.2.3-4 and Table 4.2.2.3-5 for FR1 and FR2 respectively, where the requirements apply provided that the serving cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra when multiple PTWs are used.

The requirements in Table 4.2.2.3-2 apply only when the UE supports measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. For UE neither supporting either measurementEnhancement-r16 nor intraNR-MeasurementEnhancement-r16, the UE is not required to meet the requirements specified in Table 4.2.2.3-2.

Table 4.2.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2-1Note1 | FR2-2 Note2 |  |  |  |
| 0.32 | 1 | 8 | 12 | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
| 0.64 |  | 5 | 8 | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 6 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 5 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. Note 2: Applies for UE supporting FR2-2 power class 2&3. For UE supporting FR2-2 power class 1, N1 = 12 for all DRX cycle length. Note 3: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. |  |  |  |  |  |  |

Table 4.2.2.3-2: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for UE configured with highSpeedMeasFlag-r16 (Frequency range FR1)

| DRX cycle length [s] | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.56 x M2 (8 x M2) | 0.32 x M3 (1 x M3) | 0.96 x M4 (3 x M4) |
| 0.64 | 5.12 (8) | 0.64 (1) | 1.92 (3) |
| 1.28 | 8.96 (7) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: when SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. |  |  |  |

Table 4.2.2.3-3: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for UE configured with highSpeedMeasFlagFR2-r17 (Frequency range FR2)

| DRX cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- |
| 0.32 | N2Note2 | 2.56 x N1 x M2 (8 x N1 x M2) | 0.32 x N1 x M3 (1 x N1 x M3) | 0.96 x N1 x M4 (3 x N1 x M4) |
| 0.64 | 5 | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: N2 = 2 when [highSpeedMeasFlagFR2-r17] = [set1]; N2 = 6 when [highSpeedMeasFlagFR2-r17] = [set2]. |  |  |  |  |

Table 4.2.2.3-4: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,NR_Intra [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Intra [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Intra [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| 5.12 | - | - | 117.76 (23) | 5.12 (1) | 10.24 (2) |
| 10.24 | - | - | 235.52 (23) | 10.24 (1) | 20.48 (2) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥[1.28] ([1]) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 x M2 (1 x M2) | 0.64 x M2 (2 x M2) |
|  | 0.64 | ≥[1.28] ([1]) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥[2.56] ([2]) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥[5.12] ([4]) |  | 2.56 (1) | 5.12 (2) |
| Note 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. Note 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s. Otherwise, number of DRX cycles. Note 4: The lower bound of PTW length is derived based on Tevaluate,NR_Intra*DRX_cycle1.28*1.28. Note 5: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |  |

Table 4.2.2.3-5: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for UE configured with eDRX_IDLE cycle (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Intra [s] (number of DRX cycles or eDRX cycles Note 4) | Tmeasure,NR_Intra [s] (number of DRX cycles or eDRX cycles Note 4) | Tevaluate,NR_Intra [s] (number of DRX cycles or eDRX cycles Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| 5.12 | - | - | 3 | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 10.24 x N1 (2 x N1) |
| 10.24 | - | - | 3 | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 20.48 x N1 (2 x N1) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥5.12 (4) | 8 | eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1) | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥6.4 (5) | 5 |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 | ≥10.24 (8) | 4 |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥15.36 (12) | 3 |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |
| NOTE 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. NOTE 2: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 3: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 4: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s. Otherwise, number of DRX cycles. NOTE 5: The lower bound of PTW length is derived based on Tevaluate,NR_Intra*DRX_cycle1.28*1.28. NOTE 6: When eDRX=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX cycles. |  |  |  |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

#### 4.2.2.4	Measurements of inter-frequency NR cells

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this clause.

For a UE configured with highSpeedMeasFlagFR2-r17, and configured with relaxed measurement criterion (lowMobilityEvaluation or both lowMobilityEvaluation and cellEdgeEvaluation), it is up to UE implementation whether to apply the relaxed measurements requirements specified in clause 4.2.2.10.2 or high speed requirements specified in this clause provided that the relaxed measurement criteria in section 4.2.2.10.2 are fulfilled.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS38.304 [1] within Kcarrier * Tdetect,NR_Inter + Kcarrier_HST * Tdetect,NR_Inter_HST if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least 5 dB in FR1 or 6.5dB in FR2 for reselections based on ranking or 6dB in FR1 or 7.5dB in FR2 for SS-RSRP reselections based on absolute priorities or 4dB in FR1 and 4dB in FR2 for SS-RSRQ reselections based on absolute priorities.

The parameter Kcarrier is the number of NR inter-frequency carriers which are not configured with highSpeedMeasInterFreq-r17 for FR1 or [highSpeedMeasFlagFR2-r17] for FR2 indicated by the serving cell. The parameter Kcarrier_HST is the number of NR inter-frequency carriers which are configured with highSpeedMeasInterFreq-r17 for FR1 or [highSpeedMeasFlagFR2-r17] for FR2 indicated by the serving cell. The parameter Kcarrier for a UE configured with idle mode CA measurements (while T331 is running), is the combined number of NR inter-frequency carriers indicated by the serving cell and the number of NR inter-frequency carriers configured for idle mode CA measurements which are not configured with highSpeedMeasInterFreq-r17 for FR1 or [highSpeedMeasFlagFR2-r17] for FR2. The parameter Kcarrier_HST for a UE configured with idle mode CA measurements (while T331 is running), is the combined number of NR inter-frequency carriers indicated by the serving cell and the number of NR inter-frequency carriers configured for idle mode CA measurements which are configured with highSpeedMeasInterFreq-r17 for FR1 or [highSpeedMeasFlagFR2-r17] for FR2.

Note:	combined total number means that if a carrier is an inter-frequency carrier indicated by the serving cell for mobility and additionally a carrier configured for idle mode CA measurements, it only counts as one carrier.

An inter-frequency cell is considered to be detectable according to the conditions defined in Annex B.1.3 for a corresponding Band.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE is not configured with eDRX_IDLE cycle shall measure SS-RSRP or SS-RSRQ at least every Kcarrier * Tmeasure,NR_Inter + Kcarrier_HST * Tmeasure,NR_Inter_HST (see table 4.2.2.4-1, table table 4.2.2.4-2 if UE declares support of idle mode inter-frequency measurement enhancement when configured with highSpeedMeasInterFreq-r17 for FR1, and table 4.2.2.4-2a if UE supporting [measurementEnhancementCAInterFreqFR2-r18] when configured with [highSpeedMeasFlagFR2-r17] for FR2, otherwise see table 4.2.2.4-1 only) for identified lower or equal priority inter-frequency cells. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE is not configured with eDRX_IDLE cycle shall filter SS-RSRP or SS-RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter/2 for carriers which are not configured with highSpeedMeasInterFreq-r17 for FR1 or [highSpeedMeasFlagFR2-r17] for FR2 or Tmeasure,NR_Inter_HST /2 for carriers which are configured with highSpeedMeasInterFreq-r17 forFR1 and [highSpeedMeasFlagFR2-r17] for FR2..

The UE is configured with eDRX_IDLE cycle shall measure SS-RSRP or SS-RSRQ at least every Kcarrier * Tmeasure,NR_Inter (see table 4.2.2.4-3 and table 4.2.2.4-4) for identified lower or equal priority inter-frequency cells. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE is configured with eDRX_IDLE cycle shall filter SS-RSRP or SS-RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE is not configured with eDRX_IDLE cycle shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 38.304 [1] within Kcarrier * Tevaluate,NR_Inter + Kcarrier_HST * Tevaluate,NR_Inter_HST when Treselection = 0 as specified in table 4.2.2.4-1, table 4.2.2.4-2 if UE declares support of idle mode inter-frequency measurement enhancement when configured with highSpeedMeasInterFreq-r17 for FR1, and table 4.2.2.4-2a if UE supporting [measurementEnhancementCAInterFreqFR2-r18] when configured with [highSpeedMeasFlagFR2-r17] for FR2, otherwise see table 4.2.2.4-1 only, provided that the reselection criteria is met by

-	the condition when performing equal priority reselection and

-	when rangeToBestCell is not configured:

-	the cell is at least 5dB better ranked in FR1 or 6.5dB better ranked in FR2 or.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them

-	the cell is at least 5dB better ranked in FR1 or 6.5dB better ranked in FR2 if the current serving cell is among them. or

-	6dB in FR1 or 7.5dB in FR2 for SS-RSRP reselections based on absolute priorities or

-	4dB in FR1 or 4dB in FR2 for SS-RSRQ reselections based on absolute priorities.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE is configured with eDRX_IDLE cycle shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 38.304 [1] within Kcarrier * Tevaluate,NR_Inter when Treselection = 0 as specified in table 4.2.2.4-3 and table 4.2.2.4-4, provided that the reselection criteria is met by

-	the condition when performing equal priority reselection and

-	when rangeToBestCell is not configured:

-	the cell is at least 5dB better ranked in FR1 or 6.5dB better ranked in FR2 or.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them

-	the cell is at least 5dB better ranked in FR1 or 6.5dB better ranked in FR2 if the current serving cell is among them. or

-	6dB in FR1 or 7.5dB in FR2 for SS-RSRP reselections based on absolute priorities or

-	4dB in FR1 or 4dB in FR2 for SS-RSRQ reselections based on absolute priorities.

When evaluating cells for reselection, the SSB side conditions apply to both serving and inter-frequency cells.

If Treselection timer has a non zero value and the inter-frequency cell is satisfied with the reselection criteria, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in Table 4.2.2.4-1 and Table 4.2.2.4-2 under the following conditions:

-	TSMTC_intra = TSMTC_inter = 160 ms; where

-	TSMTC_intra is the periodicity of the SMTC configured for the intra-frequency carrier if no identified intra-frequency cell is in the PCI list of smtc2-LP on this intra-frequency carrier; TSMTC_intra is the periodicity of the smtc2-LP configured for the intra-frequency carrier if at least one identified intra-frequency cell is in the PCI list of smtc2-LP on this intra-frequency carrier. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed for TSMTC_intra. If the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected.

-	TSMTC_inter is the actual SMTC periodicity used by the inter-frequency cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the inter-frequency carrier is assumed for TSMTC_inter. If the actual SSB transmission periodicity is greater than the SMTC configured for the inter-frequency carrier, longer Tdetect, NR_inter is expected.

-	SMTC occasions configured for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the SMTC occasions configured for the intra-frequency carrier, and

-	SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the paging occasion in TS38.304 [1].

For UE not configured with eDRX_IDLE cycle, Tdetect,NR_Inter, Tmeasure,NR_ Inter and Tevaluate,NR_ Inter are specified in Table 4.2.2.4.1-1 and 4.2.2.4.1-2.

For UE configured with eDRX_IDLE cycle, Tdetect,NR_ Inter, Tmeasure,NR_ Inter and Tevaluate,NR_ Inter are specified in Table 4.2.2.4-3 and Table 4.2.2.4-4 for FR1 and FR2 respectively. The requirements apply provided that the serving cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,NR_ Inter, Tmeasure,NR_ Inter and Tevaluate,NR_ Inter when multiple PTWs are used.

Table 4.2.2.4-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter

| DRX cycle length [s] | Scaling Factor (N1) |  |  | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2-1Note1 | FR2-2 Note2 |  |  |  |
| 0.32 | 1 | 8 | 12 | 11.52 x N1 x 1.5 (36 x N1 x 1.5) | 1.28 x N1 x 1.5 (4 x N1 x 1.5) | 5.12 x N1 x 1.5 (16 x N1 x 1.5) |
| 0.64 |  | 5 | 8 | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 6 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 5 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. Note 2: Applies for UE supporting FR2-2 power class 2&3. For UE supporting FR2-2 power class 1, N1 = 12 for all DRX cycle length. |  |  |  |  |  |  |

Table 4.2.2.4-2: Tdetect,NR_Inter_HST, Tmeasure,NR_Inter_HST and Tevaluate,NR_Inter_HST for FR1 configured with highSpeedMeasInterFreq-r17 note2

| DRX cycle length [s] | Tdetect,NR_Inter_HST [s] (number of DRX cycles) | Tmeasure,NR_Inter_HST [s] (number of DRX cycles) | Tevaluate,NR_Inter_HST [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | [3.2 x M2 (10 x M2)] Note 1 | [0.32 x M3 ([1] x M3)] Note 1 | 0.96 x M4 (3 x M4) Note 1 |
| 0.64 | [6.4 (10)] | [0.64 (1)] | 1.92 (3) |
| 1.28 | [10.24 (8)] | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: The support of HST Idle mode inter-frequency measurement enhancement is optional without capability signalling. Apply for UE declarating supports idle mode inter-frequency measurement enhancement for HST, otherwise Table 4.2.2.4-1 shall be used. |  |  |  |

Table 4.2.2.4-2a: Tdetect,NR_Inter_HST, Tmeasure, NR_Inter_HST and Tevaluate, NR_Inter_HST for FR2 configured with  [highSpeedMeasFlagFR2-r17]note2

| DRX cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Inter_HST [s] (number of DRX cycles) | Tmeasure, NR_Inter_HST [s] (number of DRX cycles) | Tevaluate, NR_Inter_HST [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- |
| 0.32 | N1Note3 | 2.56 x N1 x M2 (8 x N1 x M2) Note1 | 0.32 x N1 x M3 (1 x N1 x M3) Note1 | 0.96 x N1 x M4 (3 x N1 x M4) Note1 |
| 0.64 | 5 | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 Note 2: Apply for UE declarating supports [measurementEnhancementCAInterFreqFR2-r18], otherwise Table 4.2.2.4-1 shall be used. Note 3: N1 = 2 when highSpeedMeasFlagFR2-r17= set1, N1 = 6 when highSpeedMeasFlagFR2-r17= set2. |  |  |  |  |

Table 4.2.2.4-3: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter for UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,NR_Inter [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Inter [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| 5.12 | - | - | 117.76 (23) | 5.12 (1) | 10.24 (2) |
| 10.24 | - | - | 235.52 (23) | 10.24 (1) | 20.48 (2) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥1.28 (1) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 x 1.5 (1 x 1.5) | 0.64 x 1.5 (2 x 1.5) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s. Otherwise, number of DRX cycles. NOTE 4: The lower bound of PTW length is derived based on Tevaluate,NR_Inter*DRX_cycle1.28*1.28. |  |  |  |  |  |

Table 4.2.2.4-4: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter for UE configured with eDRX_IDLE cycle (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Inter [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter [s] (number of DRX cycles or eDRX cycles Note 3) |  | Tevaluate,NR_Inter [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 3 | 58.88 x N1 (23 x N1) |  | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| 5.12 | - | - | 3 | 117.76 x N1 (23 x N1) |  | 5.12 x N1 (1 x N1) | 10.24 x N1 (2 x N1) |
| 10.24 | - | - | 3 | 235.52 x N1 (23 x N1) |  | 10.24 x N1 (1 x N1) | 20.48 x N1 (2 x N1) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥5.12 (4) | 8 | eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1) |  | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥6.4 (5) | 5 |  |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 | ≥10.24 (8) | 4 |  |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥15.36 (12) | 3 |  |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |
| NOTE 1: Applies for UE supporting FR2-1 power class 2&3&4. For UE supporting FR2-1 power class 1 or 5, N1 = 8 for all DRX cycle length. NOTE 2: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 3: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 4: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s. Otherwise, number of DRX cycles. NOTE 5: The lower bound of PTW length is derived based on Tevaluate,NR_Inter*DRX_cycle1.28*1.28. NOTE 6: When eDRX=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX cycles. |  |  |  |  |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

#### 4.2.2.5	Measurements of inter-RAT E-UTRAN cells

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT E-UTRAN layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT E-UTRAN layers shall be the same as that defined below for lower priority RATs.

The requirements in this clause apply for inter-RAT E-UTRAN FDD measurements and E-UTRA TDD measurements. When the measurement rules indicate that inter-RAT E-UTRAN cells are to be measured, the UE shall measure RSRP and RSRQ of detected E-UTRA cells in the neighbour frequency list at the minimum measurement rate specified in this clause.

The parameter NEUTRA_carrier is the total number of configured E-UTRA carriers indicated to meet non high speed requirements in the neighbour frequency list. The parameter NEUTRA_carrier_HST is the total number of configured E-UTRA carriers indicated to meet high speed requirements in the neighbour frequency list. If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, an inter-RAT E-UTRAN layer is indicated to meet high speed requirements if highSpeedMeasFlag-r16 is configured and the carrier to be measured is configured with highSpeedEUTRACarrier-r16 and UE supports the enhanced inter-RAT E-UTRAN measurement requirements. If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, UE is required to meet non high speed requirements no matter whether highSpeedMeasFlag-r16 or highSpeedEUTRACarrier-r16 is configured or not.

For a UE configured with highSpeedMeasFlag-r16 or highSpeedMeasFlagFR2-r17, and configured with relaxed measurement criterion (lowMobilityEvaluation or both lowMobilityEvaluation and cellEdgeEvaluation), it is up to UE implementation whether to apply the relaxed measurements requirements specified in clause 4.2.2.11.2 or high speed requirements specified in this clause provided that the relaxed measurement criteria in section 4.2.2.11.2 are fulfilled.

The parameter NEUTRA_carrier for a UE configured with idle mode DC measurements (while T331 is running), is the combined number of configured E-UTRA carriers in the neighbour frequency list and E-UTRA carriers configured for idle mode DC measurements, excluding the configured E-UTRA carriers indicated to meet high speed requirements in the neighbour frequency list.

Note:	combined total number means that if a carrier is an E-UTRA carrier indicated by the serving cell for mobility and additionally a carrier configured for idle mode CA/DC measurements, it only counts as one carrier.

The UE shall filter RSRP and RSRQ measurements of each measured E-UTRA cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

An inter-RAT E-UTRA cell is considered to be detectable provided the following conditions are fulfilled:

-	the same conditions as for inter-frequency RSRP measurements specified in TS 36.133 [15, Annex B.1.2] are fulfilled for a corresponding Band, and

-	the same conditions as for inter-frequency RSRQ measurements specified in TS 36.133 [15, Annex B.1.2] are fulfilled for a corresponding Band.

-	SCH conditions specified in TS 36.133 [15, Annex B.1.2] are fulfilled for a corresponding Band

The UE shall be able to evaluate whether a newly detectable inter-RAT E-UTRAN cell meets the reselection criteria defined in TS38.304 [1] within NEUTRA_carrier_HST * Tdetect,EUTRAN_HST + NEUTRA_carrier  * Tdetect,EUTRAN when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities.

Cells which have been detected shall be measured at least every NEUTRA_carrier_HST * Tmeasure,EUTRAN_HST + NEUTRA_carrier  * Tmeasure,EUTRAN when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,EUTRAN. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell.

If the UE detects on an inter-RAT E-UTRAN carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall not consider an inter-RAT E-UTRA cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE is not configured with eDRX_IDLE cycle shall be capable of evaluating that an already identified inter-RAT E-UTRA cell has met reselection criterion defined in TS 38.304 [1] within NEUTRA_carrier_HST * Tevaluate,EUTRAN_HST + NEUTRA_carrier  * T evaluate,EUTRAN when Treselection = 0 as speficied in table 4.2.2.5-1 and 4.2.2.5-2 provided that the reselection criteria is met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities.

For a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE is configured with eDRX_IDLE cycle shall be capable of evaluating that an already identified inter-RAT E-UTRA cell has met reselection criterion defined in TS 38.304 [1] within NEUTRA_carrier  * T evaluate,EUTRAN when Treselection = 0 as speficied in table 4.2.2.5-3 provided that the reselection criteria is met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities.

If Treselection timer has a non zero value and the inter-RAT E-UTRA cell is satisfied with the reselection criteria which are defined in TS 38.304 [1], the UE shall evaluate this E-UTRA cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

For UE not configured with eDRX_IDLE cycle, Tdetect,EUTRAN, Tmeasure,EUTRAN and Tevaluate, E-UTRAN are specified in Table 4.2.2.5-1 and . 4.2.2.5-2

For UE configured with eDRX_IDLE cycle, Tdetect,EUTRAN, Tmeasure,EUTRAN and Tevaluate, E-UTRAN are specified in Table 4.2.2.5-3, where the requirements apply provided that the serving cell is configured with eDRX_IDLE and is the same in all PTWs during any of Tdetect,EUTRAN, Tmeasure,EUTRAN and Tevaluate, E-UTRAN when multiple PTWs are used.

Table 4.2.2.5-1: Tdetect,EUTRAN, Tmeasure,EUTRAN, and Tevaluate,EUTRAN

| DRX cycle length [s] | Tdetect,EUTRAN [s] (number of DRX cycles) | Tmeasure,EUTRAN [s] (number of DRX cycles) | Tevaluate,EUTRAN [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 (36) | 1.28 (4) | 5.12 (16) |
| 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
| 1.28 | 32(25) | 1.28 (1) | 6.4 (5) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |

Table 4.2.2.5-2: Tdetect,EUTRAN_HST, Tmeasure,EUTRAN_HST, and Tevaluate,EUTRAN_HST for UE configured with highSpeedMeasFlag-r16

| DRX cycle length [s] | Tdetect,EURAN_HST [s] (number of DRX cycles) | Tmeasure,EUTRAN_HST [s] (number of DRX cycles) | Tevaluate,EUTRAN_HST [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 4.16 (13) | 0.64 (2) | 0.96 (3) |
| 0.64 | 7.68 (12) | 1.28 (2) | 1.92 (3) |
| 1.28 | 8.96 (7) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [interRAT-MeasurementEnhancement-r16]. |  |  |  |

The requirements in Table 4.2.2.5-2 apply only when the UE supports measurementEnhancement-r16 or interRAT-MeasurementEnhancement-r16. For UE not supporting either measurementEnhancement-r16 or interRAT-MeasurementEnhancement-r16, the UE is not required to meet the requirements specified in Table 4.2.2.5-2.

Table 4.2.2.5-3: Tdetect,EUTRAN, Tmeasure,EUTRAN, and Tevaluate,EUTRAN for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,EUTRAN [s] (number of DRX or eDRX cycles Note 3) | Tmeasure,EUTRAN [s] (number of DRX or eDRX cycles Note 3) | Tevaluate,E-UTRAN [s] (number of DRX or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 2.56 | N/A | N/A | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| 5.12 | N/A | N/A | 117.76 (23) | 5.12 (1) | 10.24 (2) |
| 10.24 | N/A | N/A | 235.52 (23) | 10.24 (1) | 20.48 (2) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥1.28 (1) | (23) | 0.32 (1) | 0.64 (2) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 5.12s, number of DRX cycles otherwise. NOTE 4: The lower bound of PTW length is derived based on Tevaluate,E-UTRAN*DRX_cycle1.28*1.28. NOTE 5: When eDRX=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX cycles. |  |  |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

End of Change 1