3GPP TSG-RAN3 Meeting #123	R3-241173

Athens, Greece, 26 Feb - 1 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0326 | rev | 3 | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IAB-node authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei, Qualcomm Incorporated, ZTE, Nokia, Nokia Shanghai Bell, Xiaomi, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2024-2-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-16 IAB, the MME/AMF authorizes the IAB-node during network integration. TS 38.401 presently does not describe the RAN’s behavior in case the IAB-node is not authorized. TS 38.401 presently does not describe the RAN’s behavior when receiving an authorization change from the MME/AMF. RAN3 agreed that during IAB-MT migration for an IAB-node operating in SA, the authorization status is also included in the XnAP HANDOVER REQUEST message and in the NGAP PATH SWITCH ACKNOWLEDGE message to the target IAB-donor-CU. However, TS 38.401 presently does not describe the behavior of the receiving IAB-donor. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A section on IAB-node authorization is added to TS 38.401. This section describes the RAN's behavior when receiving the IAB-node authorization indicator for the IAB-node. The general behavior of the IAB-donor-CU receiving the IAB-node’s authorization status is the following: - In case the authorization status is “not authorized”, the IAB-donor-CU does not configure BAP address, BAP configuration or IP address, or, in case these parameters are already configured, it removes the configuration of these parameters. - In case the authorization status is “authorized”, the IAB-donor-CU follows phase 2 and phase 3 of the respective network integration procedure. In case the IAB-node is dual-connected to an eNB and an IAB-donor operating in ENDC. In this case, the authorization status is passed via the SeNB Addition/Modification Request message. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has impact on the functional point of view, will only impact the RAN handling for the authorization status for an Rel-17 IAB-node from CN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CN sends authorization status for an Rel-17 IAB-node the RAN, the RAN does not know how to use the received authorization status. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.Y (new), 8.9.Y.1 (new), 8.9.Y.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.423 CR 1769 TS 36.413 CR 1925 TS 38.413 CR 1059 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: | - Revision version 1 updates the text based on R3-240167 (version 0). - Revision version 2 updates the text based on R3-241003 (version 1). - Revision version 3 corrects style and a editorial change based on R3-241094 (version 2). |
| --- | --- |

-------------------------------------------Start of changes-------------------------------------------

### 8.9.Y IAB-node authorization

#### 8.9.Y.1	IAB-node in NSA

During the IAB-node integration procedure, the eNB receives the authorization status of the IAB-node from the EPC. The eNB forwards the authorization status to the IAB-donor-CU in the SGNB ADDITION REQUEST message. If the authorization status is “not authorized”, the IAB-donor-CU neither establishes the backhaul resources nor allocates any BAP address, TNL address or default BAP configuration for this IAB-node.

When the authorization status for the IAB-node changes, the EPC sends an updated authorization status to the IAB-MT’s eNB. The eNB forwards the authorization status to the IAB-donor-CU in the SGNB ADDITION REQUEST message or SGNB MODIFICATION REQUEST message.

In case the authorization status is “not authorized”, the IAB-donor-CU performs the following actions in this order: it attempts to hand over the UEs served by the IAB-node to other cell(s), releases the F1 interface towards the IAB-DU, and releases all backhaul resources (including the BAP address, TNL address and default BAP configuration) for this IAB-node. In case the authorization status is changed back from “not authorized” to “authorized”, the phase 2 and phase 3 of the IAB-node integration procedure as defined in clause 8.12 are carried out.

#### 8.9.Y.2	IAB-node with single IAB-donor in SA

During the IAB-node network integration, the IAB-donor-CU receives the authorization status of the IAB-node from the 5GC. If the authorization status is “not authorized”, the IAB-donor-CU neither establishes the backhaul resources nor allocates any BAP address, TNL address or default BAP configuration for this IAB-node. When the authorization status for the IAB-node changes, the 5GC sends an updated authorization status to the IAB-donor-CU. Upon reception of the authorization status, the IAB-donor-CU performs the SA equivalent of the steps described for NSA in 8.9.Y.1.

-------------------------------------------End of changes-------------------------------------------