3GPP TSG-RAN2 Meeting #125	R2-2401622

# Athens, Greece, 26th February – 1st March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4548 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Misc RRC corrections for SL relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, OPPO, Apple, ZTE, Philips International B.V |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_Relay-Core |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For the procedure of action related to transmission of SidelinkUEInformationNR message in clause 5.8.3.3, the filed name of “sl-InterestedFreqList” for NR sidelink discovery messages transmission is misaligned with “sl-TxInterestedFreqListDisc” in ASN.1 part, the filed name of “sl-InterestedFreqListL2U2N” for NR sidelink L2 U2N relay communication transmission is misaligned with “sl-TxInterestedFreqListL2U2N” in ASN.1 part, and the filed name of “sl-InterestedFreqList” for NR sidelink L3 U2N relay communication transmission is misaligned with “sl-TxInterestedFreqList” in ASN.1 part. - In R17 SL relay, L2 U2N remote UE could perform measurements for serving L2 U2N relay UE or candidate L2 U2N relay UEs and report measurement the results to gNB for path switching. However, the measurement for L2 U2N relay UE is missing in 4.2.1. - Different from the transmission of legacy SRBs using CCCH logical channel or DCCH logical channel, the SRBs of L2 U2N remote UE are transmitted via PC5 Relay RLC channel and Uu Relay RLC channel. Clarification is needed in 4.2.2. - In the description of LogicalChannelIdentity, it is said that this IE is used to identify the “PC5 Relay RLC Channel(SL-RLC-ChannelConfig)”. However, there is no logical channel identity configured in SL-RLC-ChannelConfig by NW via Uu interface. Instead, LCID is configured in PC5 link to identify a PC5 Relay RLC Channel in PC5 link(SL-RLC-ChannelConfigPC5). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 4.2.1, add the measurement for L2 U2N Relay UE. - In 4.2.2, clarify that the SRBs of L2 U2N remote UE are not using Uu CCCH or DCCH logical channel. - In 5.8.3.3, change “sl-InterestedFreqList” for NR sidelink discovery messages transmission to “sl-TxInterestedFreqListDisc” , change“sl-InterestedFreqListL2U2N” for NR sidelink L2 U2N relay communication transmission to“sl-TxInterestedFreqListL2U2N”, change“sl-InterestedFreqList” for NR sidelink L3 U2N relay communication to “sl-TxInterestedFreqList”. - Change “SL-RLC-ChannelConfig” to “SL-RLC-ChannelConfigPC5” in the description of LogicalChannelIdentity. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality: NR sidelink discovery, NR sidelink relay communication Inter-operability: If the network implements this CR but not the UE, there is no interoperability issue. If the UE implements this CR but not the network, there is no interoperabiilty issue. If one UE implement this change, but another UE is not implemented this change, there is no inter-operability issue . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong field name or unclear descriptions exist in specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1, 4.2.2, 5.8.3.3, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

4.2.1	UE states and state transitions including inter RAT

A UE is either in RRC_CONNECTED state or in RRC_INACTIVE state when an RRC connection has been established. If this is not the case, i.e. no RRC connection is established, the UE is in RRC_IDLE state. The RRC states can further be characterised as follows:

-	RRC_IDLE:

-	A UE specific DRX may be configured by upper layers;

-	At lower layers, the UE may be configured with a DRX for PTM transmission of MBS broadcast;

-	UE controlled mobility based on network configuration;

-	The UE:

-	Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5);

-	Monitors a Paging channel for CN paging using 5G-S-TMSI, except if the UE is acting as a L2 U2N Remote UE;

-	If configured by upper layers for MBS multicast reception, monitors a Paging channel for CN paging using TMGI;

-	Performs neighbouring cell measurements and cell (re-)selection;

-	Acquires system information and can send SI request (if configured);

-	Performs logging of available measurements together with location and time for logged measurement configured UEs;

-	Performs idle/inactive measurements for idle/inactive measurement configured UEs;

-	If configured by upper layers for MBS broadcast reception, acquires MCCH change notification and MBS broadcast control information and data.

-	RRC_INACTIVE:

-	A UE specific DRX may be configured by upper layers or by RRC layer;

-	At lower layers, the UE may be configured with a DRX for PTM transmission of MBS broadcast and/or a DRX for PTM transmission of MBS multicast;

-	UE controlled mobility based on network configuration;

-	The UE stores the UE Inactive AS context;

-	A RAN-based notification area is configured by RRC layer;

-	Transfer of unicast data and/or signalling to/from UE over radio bearers configured for SDT.

-	The UE:

-	Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5);

-	While T319a is running, monitors control channels associated with the shared data channel to determine if data is scheduled for it;

-	While T319a is not running, monitors a Paging channel for CN paging using 5G-S-TMSI and RAN paging using fullI-RNTI, except if the UE is acting as a L2 U2N Remote UE;

-	If configured by upper layers for MBS multicast reception, while T319a is not running, monitors a Paging channel for paging using TMGI;

-	Performs neighbouring cell measurements and cell (re-)selection;

-	Performs RAN-based notification area updates periodically and when moving outside the configured RAN-based notification area;

-	Acquires system information and, while SDT procedure is not ongoing, can send SI request (if configured);

-	While SDT procedure is not ongoing, performs logging of available measurements together with location and time for logged measurement configured UEs;

-	While SDT procedure is not ongoing, performs idle/inactive measurements for idle/inactive measurement configured UEs;

-	If configured by upper layers for MBS broadcast reception, acquires MCCH change notification and MBS broadcast control information and data;

-	If configured for MBS multicast reception in RRC_INACTIVE, acquires multicast MCCH change notification and MBS multicast control information and data;

-	Transmits SRS for Positioning.

-	RRC_CONNECTED:

-	The UE stores the AS context;

-	Transfer of unicast data to/from UE;

-	Transfer of MBS multicast data to UE;

-	At lower layers, the UE may be configured with a UE specific DRX;

-	At lower layers, the UE may be configured with a DRX for PTM transmission of MBS broadcast and/or a DRX for MBS multicast;

-	At lower layers, the UE may be configured with a cell specific cell DTX/DRX;

-	For UEs supporting CA, use of one or more SCells, aggregated with the SpCell, for increased bandwidth;

-	For UEs supporting DC, use of one SCG, aggregated with the MCG, for increased bandwidth;

-	Network controlled mobility within NR, to/from E-UTRA, and to UTRA-FDD;

-	Network controlled mobility (path switch) between a serving cell and a L2 U2N Relay UE, or vice versa.

-	The UE:

-	Monitors Short Messages transmitted with P-RNTI over DCI (see clause 6.5), if configured;

-	Monitors control channels associated with the shared data channel to determine if data is scheduled for it;

-	Provides channel quality and feedback information;

-	Performs neighbouring cell and/or L2 U2N relay measurements and measurement reporting;

-	Acquires system information;

-	Performs immediate MDT measurement together with available location reporting;

-	If configured by upper layers for MBS broadcast reception, acquires MCCH change notification and MBS broadcast control information and data.

Figure 4.2.1-1 illustrates an overview of UE RRC state machine and state transitions in NR. A UE has only one RRC state in NR at one time.

Figure 4.2.1-1:	UE state machine and state transitions in NR

Figure 4.2.1-2 illustrates an overview of UE state machine and state transitions in NR as well as the mobility procedures supported between NR/5GC, E-UTRA/EPC and E-UTRA/5GC.

Figure 4.2.1-2:	UE state machine and state transitions between NR/5GC, E-UTRA/EPC and E-UTRA/5GC

Figure 4.2.1-3 illustrates the mobility procedure supported between NR/5GC and UTRA-FDD.

Figure 4.2.1-3:	Mobility procedure supported between NR/5GC and UTRA-FDD

4.2.2	Signalling radio bearers

"Signalling Radio Bearers" (SRBs) are defined as Radio Bearers (RBs) that are used only for the transmission of RRC and NAS messages. More specifically, the following SRBs are defined:

-	SRB0 is for RRC messages using the CCCH logical channel (except SRB0 of L2 U2N Remote UE);

-	SRB1 is for RRC messages (which may include a piggybacked NAS message) as well as for NAS messages prior to the establishment of SRB2, all using DCCH logical channel (except SRB0 of L2 U2N Remote UE);

-	SRB2 is for NAS messages and for RRC messages which include logged measurement information, all using DCCH logical channel (except SRB2 of L2 U2N Remote UE). SRB2 has a lower priority than SRB1 and may be configured by the network after AS security activation;

-	SRB3 is for specific RRC messages when UE is in (NG)EN-DC or NR-DC, all using DCCH logical channel;

-	SRB4 is for RRC messages which include application layer measurement report information, all using DCCH logical channel. SRB4 has a lower priority than SRB1 and can only be configured by the network after AS security activation.

-	SRB5 is for RRC messages which include application layer measurement report information, all using DCCH logical channel. SRB5 has a lower priority than SRB1 and SRB3 and can only be configured by the SN serving the SCG when the UE is in NR-DC, after AS security activation.

In downlink, piggybacking of NAS messages is used only for one dependant (i.e. with joint success/failure) procedure: bearer establishment/modification/release. In uplink piggybacking of NAS message is used only for transferring the initial NAS message during connection setup and connection resume.

NOTE 1:	The NAS messages transferred via SRB2 are also contained in RRC messages, which however do not include any RRC protocol control information.

Once AS security is activated, all RRC messages on SRB1, SRB2, SRB3, SRB4 and SRB5, including those containing NAS messages, are integrity protected and ciphered by PDCP. NAS independently applies integrity protection and ciphering to the NAS messages, see TS 24.501 [23].

Split SRB is supported for all the MR-DC options as well as MP in both SRB1 and SRB2 (split SRB is not supported for SRB0, SRB3, SRB4 and SRB5).

For operation with shared spectrum channel access in FR1, SRB0, SRB1 and SRB3 are assigned with the highest priority Channel Access Priority Class (CAPC), (i.e. CAPC = 1) while CAPC for SRB2 is configurable.

Next change

5.8.3.3	Actions related to transmission of SidelinkUEInformationNR message

The UE shall set the contents of the SidelinkUEInformationNR message as follows:

1>	if the UE initiates the procedure to indicate it is (no more) interested to receive NR sidelink communication/positioning; or

1>	if the UE initiates the procedure to request (configuration/ release) of NR sidelink communication/positioning transmission resources or to report to the network that a sidelink radio link failure, sidelink RRC reconfiguration failure or sidelink carrier failure has been declared; or

1>	if the UE initiates the procedure to report to the network the sidelink DRX configuration for NR sidelink unicast reception; or

1>	if the UE initiates the procedure to report to the network the sidelink DRX assistance information or the sidelink DRX configuration reject information for NR sidelink unicast transmission; or

1>	if the UE initiates the procedure to report to the network the Destination Layer-2 ID and QoS profile(s) associated with its interested service(s) that sidelink DRX is applied for NR sidelink groupcast or broadcast reception; or

1>	if the UE initiates the procedure to report to the network the Destination Layer-2 ID and the sidelink DRX on/off indication for the corresponding destination for NR sidelink groupcast transmission; or

1>	if the UE initiates the procedure to indicate it is (no more) interested to receive NR sidelink discovery messages; or

1>	if the UE initiates the procedure to request (configuration/ release) of NR sidelink discovery messages transmission resources; or

1>	if the UE initiates the procedure to request (configuration/ release) of NR sidelink U2N or U2U relay communication transmission resources or report other parameters related to U2N relay operation (i.e. UE includes all concerned information, irrespective of what triggered the procedure):

2>	if SIB12 including sl-ConfigCommonNR is provided by the PCell:

3>	if configured by upper layers to receive NR sidelink communication:

4>	include sl-RxInterestedFreqList and set it to the frequency for NR sidelink communication reception;

3>	if configured by upper layers to transmit non-relay NR sidelink communication and/or to transmit NR sidelink relay communication; or

3>	if configured by upper layers to transmit NR sidelink L3 U2U relay communication [and SIB12 includes FFS gNB capability indication]:

4>	include sl-TxResourceReqList and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink communication resource:

5>	set sl-DestinationIdentity to the destination identity configured by upper layer for NR sidelink communication transmission;

5>	set sl-CastType to the cast type of the associated destination identity configured by the upper layer for the NR sidelink communication transmission;

5>	set sl-RLC-ModeIndication to include the RLC mode(s) and optionally QoS profile(s) of the sidelink QoS flow(s) of the associated RLC mode(s), if the associated bi-directional sidelink DRB has been established due to the configuration by RRCReconfigurationSidelink;

5>	set sl-QoS-InfoList to include QoS profile(s) of the sidelink QoS flow(s) of the associated destination configured by the upper layer for the NR sidelink communication transmission;

5>	set sl-TxInterestedFreqList to indicate the frequency of the associated destination for NR sidelink communication transmission;

5>	set sl-TypeTxSyncList to the current synchronization reference type used on the associated sl-TxInterestedFreqList for NR sidelink communication transmission;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from the associated peer UE;

5>	if sl-FreqInfoListSizeExt is included in SIB12-IEs:

6>	set sl-QoS-InfoList to include the frequency(ies), and Tx Profile mapped to the sidelink QoS flow(s) of the associated destination configured by the upper layer for the NR sidelink communication transmission;

4>	if a sidelink radio link failure or a sidelink RRC reconfiguration failure has been declared, according to clauses 5.8.9.3 and 5.8.9.1.8, respectively;

5>	include sl-FailureList and set its fields as follows for each destination for which it reports the NR sidelink communication failure:

6>	set sl-DestinationIdentity to the destination identity configured by upper layer for NR sidelink communication transmission;

6>	if the sidelink RLF is detected as specified in clause 5.8.9.3:

7>	set sl-Failure as rlf for the associated destination for the NR sidelink communication transmission;

6>	else if RRCReconfigurationFailureSidelink is received:

7>	set sl-Failure as configFailure for the associated destination for the NR sidelink communication transmission;

4>	if a sidelink carrier failure has been indicated by MAC layer;

5>	include sl-CarrierFailureList and set its fields as follows for each destination for which it reports the sidelink carrier failure:

6>	set sl-DestinationIdentity to the destination identity for which the concerned sidelink carrier failure is indicated;

6>	set sl-CarrierFailure to include the concerned carrier for which the sidelink carrier failure is indicated;

3>	if SIB12 includes sl-NonRelayDiscovery and if configured by upper layers to receive NR sidelink non-relay discovery messages, or if SIB12 includes sl-L2U2N-Relay and if configured by upper layers to receive NR sidelink L2 U2N relay discovery messages, or if SIB12 includes sl-L3U2N-RelayDiscovery and if configured by upper layers to receive NR sidelink L3 U2N relay discovery messages; or

3>	if SIB12 includes [FFS gNB capability indication] and if configured by upper layers to receive NR sidelink U2U relay discovery messages:

4>	include sl-RxInterestedFreqListDisc and set it to the frequency for NR sidelink discovery messages reception;

3>	if SIB12 includes sl-L2U2N-Relay and the UE is capable of L2 U2N remote UE:

4>	include sl-SourceIdentityRemoteUE and set it to the source identity configured by upper layer for NR sidelink L2 U2N relay communication transmission;

3>	if SIB12 includes sl-NonRelayDiscovery and if configured by upper layers to transmit NR sidelink non-relay discovery messages, or if SIB12 includes sl-L2U2N-Relay and if configured by upper layers to transmit NR sidelink L2 U2N relay discovery messages, or if SIB12 includes sl-L3U2N-RelayDiscovery and if configured by upper layers to transmit NR sidelink L3 U2N relay discovery messages; or

3>	if SIB12 includes [FFS gNB capability indication] and if configured by upper layers to transmit NR sidelink U2U relay discovery messages:

4>	include sl-TxResourceReqListDisc and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink discovery messages resource:

5>	set sl-DestinationIdentityDisc to the destination identity configured by upper layer for NR sidelink discovery messages transmission;

5>	if the UE is acting as L2 U2N Relay UE:

6>	set sl-SourceIdentityRelayUE to the source identity configured by upper layer for NR sidelink L2 U2N relay discovery messages transmission;

5>	set sl-CastTypeDisc to the cast type of the associated destination identity for the NR sidelink discovery messages transmission;

5>	set sl-TxInterestedFreqListDisc to indicate the frequency of the associated destination for NR sidelink discovery messages transmission;

5>	set sl-TypeTxSyncListDisc to the current synchronization reference type used on the associated sl-TxInterestedFreqListDisc for NR sidelink discovery messages transmission;

5>	set sl-DiscoveryType to the current discovery type of the associated destination identity configured by the upper layer for NR sidelink discovery messages transmission;

3>	if SIB12 includes sl-L2U2N-Relay and if configured by upper layers to transmit NR sidelink L2 U2N relay communication and the UE is acting as L2 U2N Relay UE:

4>	include sl-TxResourceReqL2U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink L2 U2N relay communication resource:

5>	set sl-DestinationIdentityL2U2N to the destination identity configured by upper layer for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TxInterestedFreqListL2U2N to indicate the frequency of the associated destination for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TypeTxSyncListL2U2N to the current synchronization reference type used on the associated sl-TxInterestedFreqListL2U2N for NR sidelink L2 U2N relay communication transmission;

5>	set sl-LocalID-Request to request local ID for L2 U2N Remote UE transiting to RRC_CONNECTED or in RRC_CONNECTED state;

5>	set sl-PagingIdentityRemoteUE to the paging UE ID received from peer L2 U2N Remote UE, if it is not released as in 5.8.9.8.3;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE;

4>	include ue-Type and set it to relayUE;

3>	if SIB12 includes sl-L2U2N-Relay and if configured by upper layers to transmit NR sidelink L2 U2N relay communication and the UE has a selected L2 U2N Relay UE:

4>	include sl-TxResourceReqL2U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows to request network to assign NR sidelink L2 U2N relay communication resource:

5>	set sl-TxInterestedFreqListL2U2N to indicate the frequency of the associated destination for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TypeTxSyncListL2U2N to the current synchronization reference type used on the associated sl-TxInterestedFreqListL2U2N for NR sidelink L2 U2N relay communication transmission;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE;

4>	include ue-Type and set it to remoteUE;

3>	if SIB12 includes sl-L3U2N-RelayDiscovery and if configured by upper layers to transmit NR sidelink L3 U2N relay communication:

4>	include sl-TxResourceReqL3U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink L3 U2N relay communication resource:

5>	set sl-DestinationIdentity to the destination identity configured by upper layer for NR sidelink L3 U2N relay communication transmission;

5>	set sl-CastType to the cast type of the associated destination identity configured by the upper layer for the NR sidelink L3 U2N relay communication transmission;

5>	set sl-RLC-ModeIndication to include the RLC mode(s) and optionally QoS profile(s) of the sidelink QoS flow(s) of the associated RLC mode(s), if the associated bi-directional sidelink DRB has been established due to the configuration by RRCReconfigurationSidelink;

5>	set sl-QoS-InfoList to include QoS profile(s) of the sidelink QoS flow(s) of the associated destination configured by the upper layer for the NR sidelink L3 U2N relay communication transmission;

5>	set sl-TxInterestedFreqList to indicate the frequency of the associated destination for NR sidelink L3 U2N relay communication transmission;

5>	set sl-TypeTxSyncList to the current synchronization reference type used on the associated sl-TxInterestedFreqList for NR sidelink L3 U2N relay communication transmission;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE;

4>	include ue-Type and set it to relayUE if the UE is acting as NR sidelink L3 U2N Relay UE or to remoteUE otherwise;

3>	if SIB12 includes [FFS gNB capability indication] and if configured by upper layers to transmit NR sidelink L2 U2U relay communication and the UE is acting as L2 U2U Relay UE:

4>	include sl-TxResourceReqL2-U2U and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink L2 U2U relay communication resource:

5>	set sl-DestinationIdentityL2-U2U to the destination identity configured by upper layer for NR sidelink L2 U2U relay communication transmission to peer L2 U2U Remote UE;

5>	set sl-TxInterestedFreqListL2-U2U to indicate the frequency of the associated destination for NR sidelink L2 U2U relay communication transmission;

5>	set sl-TypeTxSyncListL2-U2U to the current synchronization reference type used on the associated sl-InterestedFreqListL2-U2U for NR sidelink L2 U2U relay communication transmission;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer L2 U2U Remote UE;

5>	include sl-U2U-InfoList and set its fields (if needed) for each entry as follows, to report the related information of the connected L2 Remote UEs:

6>	include the source L2 U2U Remote UE's source L2 destination in sl-TargetUE-Identity;

6>	include sl-PerSLRB-QoS-InfoList, with each entry including the per-SLRB second-hop QoS profile and the corresponding sl-RemoteUE-SLRB-Identity which is set to the same value as the SLRB-PC5-ConfigIndex received in RRCReconfigurationSidelink message from the L2 U2U Remote UE for the same end-to-end SLRB;

3>	if SIB12 includes [FFS gNB capability indication] and if configured by upper layers to transmit NR sidelink L2 U2U relay communication and the UE has a selected L2 U2U Relay UE:

4>	include sl-TxResourceReqL2-U2U and set its fields (if needed) as follows to request network to assign NR sidelink L2 U2U relay communication resource:

5>	set sl-DestinationIdentityL2-U2U to the destination identity configured by upper layer for NR sidelink L2 U2U relay communication transmission to L2 U2U Relay UE;

5>	set sl-TxInterestedFreqListL2-U2U to indicate the frequency of the associated destination for NR sidelink L2 U2U relay communication transmission;

5>	set sl-TypeTxSyncListL2-U2U to the current synchronization reference type used on the associated sl-InterestedFreqListL2-U2U for NR sidelink L2 U2U relay communication transmission;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message received from L2 U2U Relay UE and the peer L2 U2U Remote UE, if any;

5>	include sl-U2U-InfoList and set its fields (if needed) for each entry as follows to report the related end-to-end and the first hop information for the end-to-end PC5 connection with each peer L2 U2U Remote UE:

6>	set sl-TargetUE-Identity to the destination identity configured by upper layer for NR sidelink L2 U2U relay communication transmission to peer L2 U2U Remote UE;

6>	set sl-E2E-QoS-InfoList to include end-to-end QoS profile(s) of the sidelink QoS flow(s) of the associated destination configured by the upper layer for the NR sidelink L2 U2U relay communication transmission to peer L2 U2U Remote UE;

6>	set sl-PerSLRB-QoS-InfoList to include the first-hop split PDB of the sidelink QoS flow(s) received from the sl-SplitQoS-InfoListPC5 in UEInformationResponseSidelink message for the associated destination in accordance with the received sl-TargetUE-Identity;

3>	if sl-DRX-ConfigCommonGC-BC is included in SIB12-IEs:

4>	if configured by upper layers to perform NR sidelink reception:

5>	include sl-RxDRX-ReportList and set its fields (if needed) as follows for each destination for which it reports to network:

6>	set sl-DRX-ConfigFromTx to include the accepted sidelink DRX configuration of the associated destination for NR sidelink unicast communication, if received from the associated peer UE;

5>	include sl-RxInterestedGC-BC-DestList and set its fields (if needed) as follows for each Destination Layer-2 ID for which it reports to network:

6>	set sl-RxInterestedQoS-InfoList to include the QoS profile of its interested service(s) that sidelink DRX is applied for the associated destination for NR sidelink groupcast or broadcast reception;

NOTE 1:	It is up to UE implementation to set the QoS profile in sl-RxInterestedQoS-InfoList for reception of NR sidelink discovery message or ProSe Direct Link Establishment Request message as described in TS 24.554 [72], or for reception of Direct Link Establishment Request message as described in TS 24.587 [57].

6>	set sl-DestinationIdentity to the associated destination identity configured by upper layer for NR sidelink groupcast or broadcast reception;

4>	if configured by upper layers to perform NR sidelink transmission and configured with sl-ScheduledConfig:

5>	include sl-TxResourceReqList and/or sl-TxResourceReqListCommRelay and/or sl-FailureList and set its fields (if needed) as follows for each destination for which it reports to network:

6>	set sl-DRX-InfoFromRxList to include the sidelink DRX assistance information of the associated destination, if any, received from the associated peer UE;

6>	if the RRCReconfigurationCompleteSidelink message includes the sl-DRX-ConfigReject:

7>	set sl-Failure as drxReject-v1710 for the associated destination for the NR sidelink communication transmission;

6>	set sl-DRX-Indication to include the sidelink DRX on/off indication for the associated destination for NR sidelink groupcast transmission;

2>	if SIB23 including sl-PosConfigCommonNR is provided by the PCell;

3>	if configured to transmit SL-PRS:

4>	include sl-PosTxResourceReqList and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink positioning resource:

5>	set sl-DestinationIdentity to the destination identity configured by upper layer for NR sidelink positioning transmission;

5>	set sl-CastType to the cast type of the associated destination identity configured by the upper layer for the NR sidelink positioning transmission;

5>	set sl-RLC-ModeIndication to include the RLC mode(s)

5>	set sl-InterestedFreqList to indicate the frequency of the associated destination for SL-PRS transmission;

5>	set sl-TypeTxSyncList to the current synchronization reference type used on the associated sl-InterestedFreqList for NR sidelink positioning transmission;

3>	if configured to receive sidelink control information for SL-PRS measurements;

4>	include sl-PosRxInterestedFreqList and set it to the frequency for NR sidelink positioning reception.

Editor's Note: FFS on sl-PosTxResourceReqList

1>	if the UE initiates the procedure while connected to an E-UTRA PCell:

2>	submit the SidelinkUEInformationNR to lower layers via SRB1, embedded in E-UTRA RRC message ULInformationTransferIRAT as specified in TS 36.331 [10], clause 5.6.28;

1>	else:

2>	submit the SidelinkUEInformationNR message to lower layers for transmission.

NOTE 2:	When multiple lists are reported in SidelinkUEInformationNR, a UE can report up to maxNrofSL-Dest-r16 SL destinations in sl-TxResourceReqList, sl-TxResourceReqListDisc and sl-TxResourceReqListCommRelay in total.

Next change

6.3.2	Radio resource control information elements

<text omitted>

–	LogicalChannelIdentity

The IE LogicalChannelIdentity is used to identify one logical channel (LogicalChannelConfig) and the corresponding RLC bearer (RLC-BearerConfig) or BH RLC channel (BH-RLC-ChannelConfig) or Uu Relay RLC channel (Uu-RelayRLC-ChannelConfig) or PC5 Relay RLC channel (SL-RLC-ChannelConfigPC5).

LogicalChannelIdentity information element

-- ASN1START

-- TAG-LOGICALCHANNELIDENTITY-START

LogicalChannelIdentity ::=          INTEGER (1..maxLC-ID)

-- TAG-LOGICALCHANNELIDENTITY-STOP

-- ASN1STOP

End of Changes