3GPP TSG RAN #107			RP-250039

Incheon, Korea, March 12-14, 2025

Title:	Moderator's summary for RAN4 led REL-20 topics

Source:	RAN4 Chair (Huawei)

Introduction

This document summarizes the discussion of all the companies’ proposals for RAN4 Rel-20 5G-Adavanced. All the topics are categorized into 7 groups by basically following the Rel-19 approach,

- UE RF centric evolution

- BS RF centric evolution

- UE OTA enhancement

- RRM enhancement

- Demodulation performance evolution

- NTN enhancements

- Cross RF and baseband topics

Offline discussions will attempt to collect companies’ views on how to build Rel-20 RAN4 package for 5G-Advanced. All the related contributions are provided in Annex.

UE RF centric evolution

Summary of contributions

| NO. | Proposed objectives for Rel-20 and supporting companies | High level objectives |
| --- | --- | --- |
| 1 | FR1 HPUE enhancement Nokia (0085), OPPO (0165), LGE (0240), Huawei (0284), Apple (0321), Qualcomm (0458), Ericsson (0540), T-Mobile (0638) | Proposed objectives: - High power PC1.5/PC1 UE for TN single carrier in NR FR1 - PC1 with 4Tx UL for FWA. - PC1 with 2Tx and UL-MIMO - PC1.5 FDD with 2Tx - PC1.5 with 1Tx - PC1.5 TDD and FDD with 3Tx - High power [PC1]/PC1.5/PC2 UE for TN single carrier in EN-DC and NR CA - PC2/PC1.5 EN-DC with PC2 LTE band with 2Tx or 3Tx - PC1.5 with 4Tx or 8Tx for UL intra-band and inter-band CA - PC1.5 with DL CA - 3UL CC in two bands for CA/EN-DC with PC2 / PC1.5 (2Tx) with 2CC in NR - PC1.5(2Tx) FR1 PDE for intra-band CA T-mobile proposal: - Continue with PC2 and PC1.5 band combinations - PC1.5 evolution - with 2Tx (TDD+TDD, TDD+FDD, FDD+FDD) - with 3Tx including uplink inter-band + intra-band CA (like CA_n25A-n41C) - with 4Tx (TDD+TDD, TDD+FDD, FDD+FDD) - FDD single band PC1.5 - PC1 with downlink Carrier Aggregation - PC1 with inter-band and intra-band UL CA (4Tx and 8Tx enhancements) - 4x26 dBm - 8x23 dBm - 2x29 dBm (New request for Rel-20) - Combination of 23 and 26 (and 29) dB |
| 2 | Power enhancement with relaxed UE Tx EVM and advanced BS receiver Samsung (0104), vivo (0209), Xiaomi (0353), China Telecom (0387), Huawei (0284) | Proposed objectives: - Reduce UE MPR with relaxed Tx EVM requirements and advanced BS receiver for 64QAM and 256QAM - Reduce UE MPR with relaxed Tx EVM requirements for QPSK and 16QAM |
| 3 | MSD optimization vivo (0209), Huawei (0284), Qualcomm (0458), ZTE (0566), T-Mobile (0638) | Proposed objectives: - Simplify MSD requirements for PC3 - Simplify MSD requirements for HPUE (Rel-19 leftover if any) - (T-Mobile) dynamic real time feedback for MSD issues, rather than a static UE capability |
| 4 | UL Tx switching enhancement OPPO (0165), Apple (0321), ZTE (0566), T-mobile (0638) | Proposed objectives: - Introduce non-zero switching period for 1Tx UE for SUL+NUL band combination - Uplink Tx switching for intra-band non-contiguous UL CA [with 3Tx] - 3Tx 3CC or 2CC Tx Switching T+F today 2Tx |
| 5 | Tx requirements enhancement for U6G Huawei (0284), China Telecom (0387), CATT (0518) | Proposed objectives: - Specify ACLR of 26dB and corresponding MPR requirements for U6GHz - Specify RF requirements for 200MHz CBW for U6GHz - (CATT) Support higher-order bandwidth class for intra-band uplink contiguous CA, such as introducing bandwidth class D. |
| 6 | Band/band combination requirements simplification OPPO (0165), Huawei (0284), Nokia (0085) | Proposed objectives: - Simplify the band/band combination specifications - Replace the current MS Word-based table listing for band combination specifications by using database. |
| 7 | Power classes framework Qualcomm (0458) | Proposed objectives: - High power class framework - Power class handling for CA (allowing maximum output power per band for different CA combos) - 4Tx with inter-band UL CA (2Tx UL-MIMO each band) |
| 8 | Optimize MPR/A-MPR Ericsson (0540) | Proposed objectives: - Overhaul UE TX model in order to selectively improve MPR/A-MPR in 5G-A and be prepared for 6G - Considering PC3, PC2, PC1 and multiple TX/MIMO |
| 9 | SRS enhancement Ericsson (0540) | Proposed objectives: - Further improvement to SRS performance for reciprocal DL MIMO including improved testing of SRS and IL reproting |
| 10 | SL-U enhancement LGE (0240) | Proposed objectives: - PC3 in single carrier and PC5 in intra-band contiguous CA for SL-U |
| 11 | MPR reduction by leveraging asymmetric ACLR Huawei (0284) | Proposed objectives: - Specify the UE RF requirements of MPR reduction by defining different regions corresponding to asymmetric ACLR |
| 12 | Basket WIs China Telecom (0387) | Proposed objectives: - HPUE basket WIs - Downlink interruption basket WI |
| 13 | Coherent UL MIMO ZTE (0566) | Proposed objectives: - Study how to make the UL-MIMO to be commercially available e.g., introducing certain mechanism by leveraging the current DPD feedback link to do the calibration between RF chains. |
| 14 | 256QAM evolution T-Mobile (0638) | Proposed objectives: - 8 layer DL 256QAM including performance requirements - 4-layer UL 256QAM |

Moderator’s observations

According to moderator’s observations, the following evolution/enhancement directions attract the relatively more common interests (≥4 sources):

- FR1 HPUE enhancement (PC2/PC1.5/PC1) for single carrier and EN-DC/NR-CA

- Power enhancement with relaxed UE Tx EVM and advanced BS receiver

- MSD optimization

- UL Tx switching enhancement

The other evolution/enhancement directions are also captured in this summary but seem get less supports and may need more discussions. Moderators would like to remind companies that the available TU for Rel-20 5G-Advanced would be relatively limited compared with the proposals, and would like to encourage companies to converge on a limited number of mainstream proposals, which would be more critical and will be commercialized.

Offline discussions and conclusions

From moderator’s point of view the following list of UE RF centric evolution and/or enhancement can be used as the starting point for further discussions to converge on Rel-20 5G-Advanced RAN4 package. It does not mean that the technique directions under this list must be part of a certain WI (e.g., UE RF enhancement in Rel-20), in other words, how to organize the final RAN4 project package will be further discussed separately.

The following list of high level candidate objectives can be used as the starting point for 5G-A UE RF centric evolution and/or enhancement in Rel-20:

- FR1 HPUE enhancement (PC2/PC1.5/PC1) for single carrier and EN-DC/NR-CA

- Power enhancement with relaxed UE Tx EVM and advanced BS receiver

- MSD optimization

- UL Tx switching enhancement

- Tx requirements enhancement for U6G

- Band/band combination requirements simplification

- Power classes framework

- Optimize MPR/A-MPR

- SRS enhancement

- SL-U enhancement

- MPR reduction by leveraging asymmetric ACLR

- Requirement enhancement to support coherent UL MIMO

- 256QAM evolution

Discussions

Comments:

Summary/conclusions

BS RF centric evolution

Summary of contributions

| NO. | Proposed objectives for Rel-20 and supporting companies | High level objectives |
| --- | --- | --- |
| 1 | Enhancement of co-location and co-existence requirements Nokia (0085), Ericsson (0540), ZTE (0566), Huawei (0284) | Proposed objectives: - The OTA co-location requirement was designed for BS up to 3 GHz and is not suitable for higher frequencies - Rel-20 should aim to define an improved co-location concept that will enable real and relevant requirements to be defined and tested OTA for relevant frequency ranges and deployment scenarios above 3 GHz - Essential to support robust performance in multi-operator/vendor shared tower deployments - RAN task to simplify co-existence and co-location requirements - (Huawei) Investigate OBUE requirements structure simplification solutions in (all network-nodes specifications; conducted and OTA), e.g. compacting tables, etc. |
| 2 | Additional Hybrid requirement sets for BS type 1-H/large antenna array Huawei (0284), Ericsson (540) | Proposed objectives: - Introduce additional Hybrid requirement sets with more OTA options for some TX and RX requirements for the BS type 1-H - Ensure specification support for hybrid beamforming Define unambiguous BS types for bands / large array sizes |
| 3 | OTA requirements enhancement Nokia (0085), Ericsson (0540) | Proposed objectives: - In Rel-20 RAN4 should re-evaluate RX blocking levels with respect to technical background assumptions (e.g., UE power, BS antenna) - Ensure that other OTA requirements are properly defined such that OTA testing becomes feasible |
| 4 | NB-IoT to AAS BS specifications Huawei (0284) | Proposed objectives: - Define the transmitter and receiver requirements and conformance testing for introduction of NB-IoT in-band, NB-IoT guard band, and standalone NB-IoT operation to AAS BS specifications. |
| 5 | LP-WUS in TN MSR spec ZTE (0566) | Proposed objectives: - Specify the LP-WUS related requirement in MSR specification which is similar as NB-IoT operation in NR in-band in Rel-16. |

Moderator’s observations

According to moderator’s observations, the following evolution/enhancement directions attract the relatively more common interests (≥2 sources):

- Enhancement of co-location and co-existence requirements

- Additional Hybrid requirement sets for BS type 1-H/large antenna array

- OTA requirements enhancement

The other evolution/enhancement directions are also captured in this summary but seem get less supports and may need more discussions. Moderators would like to remind companies that the available TU for Rel-20 5G-Advanced would be relatively limited compared with the proposals, and would like to encourage companies to converge on a limited number of mainstream proposals, which would be more critical and will be commercialized.

Offline discussions and conclusions

From moderator’s point of view the following list of BS RF centric evolution and/or enhancement can be used as the starting point for further discussions to converge on Rel-20 5G-Advanced RAN4 package. It does not mean that the technique directions under this list must be part of a certain WI (e.g., BS RF enhancement in Rel-20), in other words, how to organize the final RAN4 project package will be further discussed separately.

The following list of high level candidate objectives can be used as the starting point for 5G-A BS RF centric evolution and/or enhancement in Rel-20:

- Enhancement of co-location and co-existence requirements

- Additional Hybrid requirement sets for BS type 1-H/large antenna array

- OTA requirements enhancement

- NB-IoT to AAS BS specifications

- LP-WUS in TN MSR spec

Discussions

Comments:

Summary/conclusions

UE OTA enhancement

Summary of contributions

| NO. | Proposed objectives for Rel-20 and supporting companies | Details of objectives |
| --- | --- | --- |
| 1 | TRP/TRS and MIMO OTA test enhancement vivo (0209), Huawei (0284), Apple (0321), Xiaomi (0353), CAICT (0423), Thales (0457) | Proposed objectives: - Enhance test method - More UE types/advanced features, including - CA, 3T/6R, XR UE - NTN for Ku band, RedCap NTN, mobile VSAT - FR1 Dynamic MIMO OTA testing - Phantom-based dynamic MIMO OTA test method, Multi-TRP - Performance requirements - Specify TRP and TRS requirements for new bands: - Specify MIMO OTA requirements for new bands |

Offline discussions and conclusions

Moderator’s observations

The proposed evolution or enhancement technique directions for TRP/TRS and MIMO OTA are basically aligned. But moderators would like to remind companies that the available TU for Rel-20 5G-Advanced would be relatively limited compared with the proposals, and would like to encourage companies to converge on a limited number of mainstream proposals, which would be more critical and will be commercialized.

Offline discussions and conclusions

From moderator’s point of view the following list of UE OTA evolution and enhancement can be used as the starting point for further discussions to converge on Rel-20 5G-Advanced RAN4 package. It does not mean that the technique directions under this list must be part of a certain WI (e.g., UE OTA in Rel-20), in other words, how to organize the final RAN4 project package will be further discussed separately.

The following list of high level candidate objectives can be used as the starting point for 5G-A UE OTA evolution and/or enhancement in Rel-20:

- Enhance test method

- More UE types/advanced features, including

  - CA, 3T/6R, XR UE

- NTN for Ku band, RedCap NTN, mobile VSAT

- FR1 Dynamic MIMO OTA testing

  - Phantom-based dynamic MIMO OTA test method, Multi-TRP

- Performance requirements

- Specify TRP and TRS requirements for new bands:

- Specify MIMO OTA requirements for new bands

RRM enhancements

Summary of contributions

| NO. | Proposed objectives for Rel-20 and supporting companies | High level objectives |
| --- | --- | --- |
| 1 | RLM/BFD relaxation for RedCap UE OPPO (0165), vivo (0209), Huawei (0284), Xiaomi (0353), China Telecom (0387), Sony (0456), Mediatek (0483), CATT (0518), Ericsson (0540) | Proposed objectives: RLM/BFD relaxation requirements for RedCap UE |
| 2 | Fast SCell activation Nokia (0085), Samsung (0104), Huawei (0284), Apple (0321), China Telecom (0387) | Proposed objectives: - Further reduce the delay of measurement and procedure for SCell activation for intra-band CA - Reduce SCell activation delay based on Temporary RS and L3 measurement triggered by SCell activation command, and define specific requirements for unknown SCell |
| 3 | Uplink only SCell (NES SSB-less SCell enhancement) Samsung (0104), Intel (0465), CATT (0518), ZTE (0566) | Proposed objectives: - Support SCell with uplink only (SSB-less SCell without DL signals) - SSB-less SCell for intra-band NC CA |
| 4 | FR2 multi-Tx and multi-Rx Samsung (0104), LGE (0240) | Proposed objectives: - Multi-Tx: Enable UL STxMP (Simultaneous transmission across multiple panels) - Multi-Rx: Enhance simultaneous DL reception to support Rel-17/18 unified TCI state framework and inter-cell multi-TRP scenarios |
| 5 | Enhanced RRM requirements for NES OPPO (0165), Ericsson (0540) | Proposed objectives: - (OPPO) Specify the RRM requirements to support the combined OD-SBB and always on-SSB based L3 measurement (Ericsson) To support OD-SSB based multiple Scell activation delay requirements if not done in Rel-19 |
| 6 | Enhanced RedCap with LTM Ericsson (0540) | Proposed objectives: - Specify support for L1/L2 based mobility for RedCap - Based on the LTM requirements introduced in Rel-18 for non-RedCap UEs, define LTM requirements for RedCap UEs |
| 7 | R18 leftover on LTM RRM vivo (0209) | Proposed objectives: - Specify the RRM requirements to support the combined OD-SBB and always on-SSB based L3 measurement |
| 8 | Interruption at NR SRS antenna port switching CMCC (0262) | Proposed objectives: - Specify interruption requirements at NR SRS antenna port switching to cover the case that SRS resource(s) is not within the last 6 symbols of a slot |
| 9 | Parallel measurement CMCC (0262) | Proposed objectives: - Specify RRM requirements to support parallel measurements upon MG collision |
| 10 | Inter-frequency measurement enhancement for LTM CMCC (0262) | Proposed objectives: - Specify RRM requirements to support L1 inter-frequency measurement with NCSG and L1 inter-frequency measurement with NeedforGap |
| 11 | On-demand deactivated SCell measurement Huawei (0284) | Proposed objectives: - Introduce on-demand fast deactivated SCell measurement and specify corresponding SCell activation requirements |
| 12 | A-TRS based RRM enhancement Apple (0321) | Proposed objectives: - utilizing A-TRS for the enhancement to the corresponding UE activities - HO, - TCI switching - SCell activation - PSCell addition/change |
| 13 | Dynamic RTD/TTD status update Apple (0321) | Proposed objectives: - Dynamic RTD/TTD status update |
| 14 | AL/ML beam prediction testing Qualcomm (0458) | Proposed objectives: - AI/ML beam prediction testing to provide foundation to 6G |
| 15 | Handover interruption minimization Mediatek (0483) | Proposed objectives: - Early DL synchronization before handover - Introduce an enhanced UE behavior to start autonomous early pre-synchronization to neighboring cells - Study and identify the applicable conditions - Under the applicable conditions, define the reduced handover delay without additional time for synchronization - Postponed interruption (in case time for synchronization during handover is still needed) - Introduce an enhanced HO delay requirement by leveraging the waiting time of SSB to reduce interruption. - If needed, LS can be sent to RAN2 for necessary signaling change, based on RAN4 consensus RRM Handover interruption minimization [1/3] RP-250483 RRC Dec Proc. SS |
| 16 | Interruption-free DCI operation Mediatek (0483) | Proposed objectives: - Introduce a new UE type capable of interruption-free DCI reception with reduced RF re-tuning time - Identify which existing interruption requirements can be enhanced with the enhanced UE behavior - If needed, introduce new requirements or modify existing requirements for UEs capable of interruption-free DCI reception |
| 17 | Extend OD-SSB to PSCell in NR-DC ZTE (0566) | Proposed objectives: - Extend the application of OD-SSB to PSCell in NR-DC - To speed up the deactivated PSCell identification/measurement - To reduce the latency of PSCell addition, PSCell change, SCG activation |

Moderator’s observations

According to moderator’s observations, the following evolution/enhancement directions attract the relatively more common interests (≥4 sources):

- RLM/BFD relaxation for RedCap UE

- Fast SCell activation

- Uplink only SCell (NES SSB-less SCell enhancement)

The other evolution/enhancement directions are also captured in this summary but seem get less supports and may need more discussions. Moderators would like to remind companies that the available TU for Rel-20 5G-Advanced would be relatively limited compared with the proposals, and would like to encourage companies to converge on a limited number of mainstream proposals, which would be more critical and will be commercialized.

Offline discussions and conclusions

From moderator’s point of view the following list of RRM evolution and/or enhancement can be used as the starting point for further discussions to converge on Rel-20 5G-Advanced RAN4 package. It does not mean that the technique directions under this list must be part of a certain WI (e.g., RRM enhancement in Rel-20), in other words, how to organize the final RAN4 project package will be further discussed separately.

The following list of high level candidate objectives can be used as the starting point for 5G-A RRM centric evolution and/or enhancement in Rel-20:

- RLM/BFD relaxation for RedCap UE

- Fast SCell activation

- Uplink only SCell (NES SSB-less SCell enhancement)

- FR2 multi-Tx and multi-Rx

- Enhanced RRM requirements for NES

- Enhanced RedCap with LTM

- R18 leftover on LTM RRM

- Interruption at NR SRS antenna port switching

- Parallel measurement

- Inter-frequency measurement enhancement for LTM

- On-demand deactivated SCell measurement

- A-TRS based RRM enhancement

- Dynamic RTD/TTD status update

- AL/ML beam prediction testing

- Handover interruption minimization

- Interruption-free DCI operation

- Extend OD-SSB to PSCell in NR-DC

Discussions

Comments:

Summary/conclusions

Demodulation performance evolution

Summary of contributions

| NO. | Proposed objectives for Rel-20 and supporting companies | High level objectives |
| --- | --- | --- |
| 1 | SCM based requirements vs SCM further study Nokia (0085), Samsung (0104), CMCC (0262), Xiaomi (0353), China Telecom (0387), Qualcomm (0458), Intel (0465), Mediatek (0483), Ericsson (0540) | Proposed objectives: - (Nokia, CMCC) Introduce the following SCM based requirements for SU-MIMO and MU-MIMO - (Samsung) Further study on SCM impact on demodulation performance by considering testability and test repeatability for MU-MIMO, FR2, AI, ISAC and etc - (Qualcomm/Mediatek) If channel model can be aligned well in Rel-19, introduce the simple demodulation test focusing on SU-MIMO [with neighbour cell interference] - (Ericsson) Study spatial channel model for AI/ML |
| 2 | R-ML receiver for 8Rx SU-MIMO scenario Huawei (0284), Xiaomi (0353), China Telecom (0387), Mediatek (0483) | Proposed objectives: - Introduce the UE performance requirements for 8Rx SU-MIMO scenario with R-ML receiver |
| 3 | CSI Reporting Requirements Enhancement Apple (0321), China Telecom (0387) | Proposed objectives: - Study the framework and methodology for CRI reporting requirements - Use CRI reporting in LTE as a starting point - Define CRI reporting requirements for at least 32 TX ports for Type I codeboo |
| 4 | BS MMSE-IRC Xiaomi (0353), China Telecom (0387), ZTE (0566) | Proposed objectives: - MMSE-IRC for BS (async TDD) - Investigate the advanced receiver to address the co-channel CLI in the HetNet |
| 5 | 6-layer performance requirements for handheld UE Qualcomm (0458) | Proposed objectives: - 6-layer performance requirements for handheld devices |
| 6 | UE IC receiver ZTE (0566) | Proposed objectives: - Investigate the IC receiver for UE demod requirement |

Moderator’s observations

According to moderator’s observations, the following evolution/enhancement directions attract the relatively more common interests (≥4 sources):

- SCM based requirements vs SCM further study

- R-ML receiver for 8Rx SU-MIMO scenario

The other evolution/enhancement directions are also captured in this summary but seem get less supports and may need more discussions. Moderators would like to remind companies that the available TU for Rel-20 5G-Advanced would be relatively limited compared with the proposals, and would like to encourage companies to converge on a limited number of mainstream proposals, which would be more critical and will be commercialized.

Offline discussions and conclusions

From moderator’s point of view the following list of demodulation evolution and/or enhancement can be used as the starting point for further discussions to converge on Rel-20 5G-Advanced RAN4 package. It does not mean that the technique directions under this list must be part of a certain WI (e.g., demodulation in Rel-20), in other words, how to organize the final RAN4 project package will be further discussed separately.

The following list of high level candidate objectives can be used as the starting point for 5G-A demodulation evolution and/or enhancement in Rel-20:

- SCM based requirements vs SCM further study

- R-ML receiver for 8Rx SU-MIMO scenario

- CSI Reporting Requirements Enhancement

- BS MMSE-IRC

- 6-layer performance requirements for handheld UE

- UE IC receiver

Discussions

Comments:

Summary/conclusions

NTN enhancements

Summary of contributions

| NO. | Proposed objectives for Rel-20 and supporting companies | High level objectives |
| --- | --- | --- |
| 1 | NTN HPUE Samsung (0104), vivo (0209), CMCC (0262), Xiaomi (0353), China Telecom (0387), China Unicom (0423), ZTE (0566) | Proposed objectives: - PC1.5/PC1 for handheld NR-NTN and IoT-NTN devices - Higher power than PC1 for handheld NR-NTN and IoT-NTN devices - PC1/PC2 for RedCap NTN UE |
| 2 | NTN CA Samsung (0104), vivo, Xiaomi (0353), Thales (0457), Mediatek (0483) | Proposed objectives: - Intra-band and inter-band UL/DL CA for NR-NTN with necessary RF and RRM requirements being specified |
| 3 | Ku band NTN LGE (0240), Eutelsat (0291), CHTTL et al (0410), Mediatek (0483), CATT (0518), Sharp (0648) | Proposed objectives: - Mobile and other types VSAT RF and RRM requirements for single carrier and intra-band CA in Ku band - Support intra-SAN CA in Ku band - (Mediatek/CATT) Specify requirements for HD-FDD devices for NR-NTN |
| 4 | NTN mobile VSAT Samsung (0104), Xiaomi (0353),Thales (0457), BMW Group (0542) | Proposed objectives: - NGSO mobile VSAT in Ku and Ka bands - Ultra-small mobile VSAT: definition of type 6 and/or additional type in above 10GHz band(s) |
| 5 | Testing method for VSAT Thales (0457), vivo (0209), Xiaomi (0353) | Proposed objectives: - Investigate the applicability of current 3GPP test methods for the following example VSAT device types |
| 6 | NTN power domain enhancements Apple (0321), Xiaomi (0353) | Proposed objectives: - NTN power domain enhancements - MPR reduction with emission relaxation under NTN deployment |
| 7 | MSR for in-band IoT-NTN in NR Thales (0457), ZTE(0518) | Proposed objectives: - Create TS 37.181 (NTN equivalent of TS 37.141 NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing) and TS 37.108 (NTN equivalent of TS 37.104 NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception) |
| 8 | Irregular CBW 7MHz for NR-NTN CATT (0518) | Proposed objectives: - Include 7MHz channel bandwidth for NR NTN |
| 9 | Mobility between TN and NR-NTN CATT (0518), ZTE (0566) | Proposed objectives: - Mobility enhancement between TN and NR NTN in the RRC_CONNECTED state. - Define the RRM requirement for NTN-TN handover |
| 10 | Repeater for NTN ZTE (0566) | Proposed objectives: - Mobility enhancement between TN and NR NTN in the RRC_CONNECTED state. |

Moderator’s observations

According to moderator’s observations, the following evolution/enhancement directions attract the relatively more common interests (≥4 sources):

- NTN HPUE

- Ku band NTN

- NTN CA

- NTN mobile VSAT

The other evolution/enhancement directions are also captured in this summary but seem get less supports and may need more discussions. Moderators would like to remind companies that the available TU for Rel-20 5G-Advanced would be relatively limited compared with the proposals, and would like to encourage companies to converge on a limited number of mainstream proposals, which would be more critical and will be commercialized.

Offline discussions and conclusions

From moderator’s point of view the following list of NTN evolution and/or enhancement can be used as the starting point for further discussions to converge on Rel-20 5G-Advanced RAN4 package. It does not mean that the technique directions under this list must be part of a certain WI (e.g., NTN in Rel-20), in other words, how to organize the final RAN4 project package will be further discussed separately.

The following list of high level candidate objectives can be used as the starting point for 5G-A NTN evolution and/or enhancement in Rel-20:

- NTN HPUE

- Ku band NTN

- NTN CA

- NTN mobile VSAT

- Testing method for VSAT

- NTN power domain enhancements

- MSR for in-band IoT-NTN in NR

- Irregular CBW 7MHz for NR-NTN

- Mobility between TN and NR-NTN

- Repeater for NTN

Discussions

Comments:

Summary/conclusions

Cross RF and baseband topics

Summary of contributions

| NO. | Proposed objectives for Rel-20 and supporting companies | Motivation and High level objectives |
| --- | --- | --- |
| 1 | NR FR1 DL fragmented carrier Samsung(0104)*, vivo (0209), LGE (0240), Xiaomi (0353), Intel (0465), Mediatek (0483), ZTE (0566) * General principle: UE RF topics shall be selected to “address real and critical commercial deployment needs* ”. Identified demands from operators: (a) NR FR1 DL fragmented carriers and (b) low-low band CA via switching enhancement. | Motivation: - Follow-up WI of Rel-19 SI. Proposed objectives: - To introduce the mechanism and UE requirements to enable DL fragmented carrier with the proper assumptions - Specify necessary RF and RRM requirements to support fragmented DL carriers in FR1 following the outcome of Rel-19 SI - (Intel) limited to Rel-19 SI objectives/conclusions |
| 2 | Extension of low-low band CA via switching Samsung*(0104), vivo (0209), LGE (0240), Apple (0321), Xiaomi (0353), Qulalcomm (0458), ZTE (0566), Telus et al (0569) * General principle: UE RF topics shall be selected to “address real and critical commercial deployment needs* ”. Identified demands from operators: (a) NR FR1 DL fragmented carriers and (b) low-low band CA via switching enhancement. | Motivation: - Extend Rel-19 low-low band CA via switching to other cases Proposed objectives: - Extend Rel-19 Low-low band switching to other cases, for which the following candidates are proposed - To other FDD+SDL combo, if any, with dual DL being specified - To low band CA FDD+FDD - To switching between FDD UL/DL and FDD DL+SDL - Dynamic/rule based switching to improve performance |
| 3 | Irregular channel bandwidth (flexible channel bandwidth) Samsung (0104), OPPO (0165), China Telecom (0387), China Unicom (0423), Intel (0465), T-Mobile (0638) | Proposed objectives: - Support more irregular channel bandwidth - Support 6MHz for n29, n5, n12, n85, n28 with core and band specific requirements - Study and specify the support of flexible CBWs in TN and NTN with necessary guard bands and max transmission bandwidth being defined - Add support of 11MHz for Band n5 by Next-larger CBW Solution |
| 4 | 3MHz channel bandwidth for eRedcap devices Nokia (0085), Anterix (0093), Sony (0456), Intel (0465), Futurewei (0530), Ericsson (0540) | Motivation: Proposed objectives: - Introduce support of 3MHz channel bandwidth (15 RPB) for eRedcap devices in FR1 with 15KHz SCS [for both TN and NTN] - Example bands: n106, n85, n26, n28 - Introduce 3 MHz channel bandwidth support for n106 for 1Rx and 2Rx* IRedCap devices. - Utilize existing IRedcap capabilities defined in Rel-17, Rel-18 and Rel-19 unless otherwise specified. - Define the necessary RF and RRM Core and Performance requirements for UE and BS. - (Sony) Study the limiting factors in UE RF requirements in TS38.101-1 and specify the corresponding meachanism(s) to enable single SKU designs for HD-FDD eRedCap with baseband bandwidth reduction BW3/PR3 Note: SKU – stock-keeping unit |
| 5 | Intra-band non-collocated EN-DC/NR-CA deployment KDDI, et al (0277) | Proposed objectives: - Type 3 UE requirements assuming MRTD<CP and power imbalance < [xx] dB for handheld Without intended impacts on the network side |
| 6 | AI/ML assisted receiver Samsung (0104) | Proposed objectives: - AI/ML assisted non-linearity compensation (AI-NC) at BS - AI/ML channel estimation (AI-CE) at BS |
| 7 | ATG evolution LGE (0240) | Proposed objectives: - RF/RRM requirements of ATG for - FR1 intra/inter-band UL CA - FR2 in single carrier |
| 8 | UAV enhancement CMCC (0262) | Proposed objectives: - UAV enhancement - Use main lobe to enhance the UAV downlink coverage - Enhance the maximum output power of UAV - Beamforming for FR1 - Other RF/RRM requirements if needed |

Moderator’s observations

According to moderator’s observations, the following evolution/enhancement directions attract the relatively more common interests (≥4 sources):

- NR FR1 DL fragmented carrier

- Extension of low-low band CA via switching

- Irregular channel bandwidth (flexible channel bandwidth)

- 3MHz channel bandwidth for eRedcap devices

- Intra-band non-collocated EN-DC/NR-CA deployment

The other evolution/enhancement directions are also captured in this summary but seem get less supports and may need more discussions. Moderators would like to remind companies that the available TU for Rel-20 5G-Advanced would be relatively limited compared with the proposals, and would like to encourage companies to converge on a limited number of mainstream proposals, which would be more critical and will be commercialized.

Offline discussions and conclusions

From moderator’s point of view the following list of cross RF and baseband evolution and/or enhancement can be used as the starting point for further discussions to converge on Rel-20 5G-Advanced RAN4 package. It does not mean that the technique directions under this list must be part of certain WIs (e.g., fragmented DL in Rel-20), in other words, how to organize the final RAN4 project package will be further discussed separately.

The following list of high level candidate objectives can be used as the starting point for cross RF and baseband evolution and/or enhancement in Rel-20:

- NR FR1 DL fragmented carrier

- Extension of low-low band CA via switching

- Irregular channel bandwidth (flexible channel bandwidth)

- 3MHz channel bandwidth for eRedcap devices

- Intra-band non-collocated EN-DC/NR-CA deployment

- AI/ML assisted receiver

- ATG evolution

- UAV enhancement

Discussions

Comments:

Summary/conclusions

Conclusions

Annex reference contributions

The Tdocs submitted for RAN4 Rel-20 5G-Advanced are listed in the table below.

| TDoc | Title | Source | Type | Agenda ittem |
| --- | --- | --- | --- | --- |
| RP-250039 | Moderator's summary for RAN4 led REL-20 topics | RAN4 Chair (Huawei) | report | 15.3 |
| RP-250085 | RAN4-led topics for 5G-Advanced in Rel-20 | Nokia | discussion | 15.3 |
| RP-250093 | Proposal for <5 Mhz (e)RedCap Support in Rel-20 | Anterix | discussion | 15.3 |
| RP-250104 | Overview on Rel-20 RAN4-led 5G-Advanced Scope | Samsung | discussion | 15.3 |
| RP-250165 | View on RAN4 R20 | OPPO | other | 15.3 |
| RP-250209 | High level overview of 5G-A Rel-20 proposals - RAN4 | vivo | discussion | 15.3 |
| RP-250240 | Views on RAN4-led topics in Rel-20 | LG Electronics Deutschland | discussion | 15.3 |
| RP-250262 | Views on Rel-20 5G-A RAN4 led topics | CMCC | discussion | 15.3 |
| RP-250277 | Motivation on supporting Intra-band non-collocated EN-DC/NR-CA deployment in Rel-20 (Type 3 UE for handheld) | KDDI, NTT Docomo, SoftBank, KT, LG Uplus | discussion | 15.3 |
| RP-250284 | Views on Rel-20 RAN4-led topics | Huawei, HiSilicon | discussion | 15.3 |
| RP-250291 | Carrier Aggregation requirements for Ku band in 5G-Advanced | Eutelsat Group | discussion | 15.3 |
| RP-250321 | Apple’s view on Rel-20 RAN4 package | Apple | discussion | 15.3 |
| RP-250353 | Initial view for Rel-20 5G-A RAN4 scope | Xiaomi Technology | discussion | 15.3 |
| RP-250387 | Views on topics for RAN4 Rel-20 | China Telecom | discussion | 15.3 |
| RP-250410 | Views on the NTN Ku band related enhancement in Rel.20 | CHTTL | discussion | 15.3 |
| RP-250423 | Views on RAN4 Rel-20 OTA Topics | CAICT | discussion | 15.3 |
| RP-250430 | Views on Rel-20 5G-A RAN4 Topics | China Unicom | discussion | 15.3 |
| RP-250456 | (e)Redcap enhancement in Rel-20 | Sony | discussion | 15.3 |
| RP-250457 | Candidate RAN4 led NTN topics for Rel-20 5G Advanced | THALES | discussion | 15.3 |
| RP-250458 | Qualcomm Views on RAN4 Topics for Rel-20 | Qualcomm Incorporated | discussion | 15.3 |
| RP-250465 | Views on Rel-20 5G-Adv RAN4-led topics | Intel Corporation | discussion | 15.3 |
| RP-250483 | MediaTek views on Rel-20 RAN4 5G-A scope | MediaTek Inc. | discussion | 15.3 |
| RP-250518 | General views on Rel-20 RAN4 works | CATT, CSCN | discussion | 15.3 |
| RP-250530 | RedCap for Rel-20 | Futurewei | discussion | 15.3 |
| RP-250540 | 5G-Advanced RAN4 led topics in Rel-20 | Ericsson | discussion | 15.3 |
| RP-250542 | NTN in 5G-Advanced Rel-20 / Automotive Terminal / VSAT support | BMW AG, Fraunhofer IIS | discussion | 15.3 |
| RP-250566 | Views on Rel-20 RAN4-led topics | ZTE Corporation, Sanechips | discussion | 15.3 |
| RP-250569 | Motivation for low-band carrier aggregation via switching enhancement in Rel-20 | TELUS, Bell Mobility, Murata, AT&T, BT plc, Apple | discussion | 15.3 |
| RP-250638 | Priorities for RAN4-led 5G-A topics for Rel-20 | T-Mobile USA Inc. | discussion | 15.3 |
| RP-250648 | Motivation to specify RF requirements for ultra-small Mobile VSAT phased array antenna in Ku and Ka Band | SHARP Corporation | discussion | 15.3 |