3GPP TSG RAN #107 Meeting 	RP-250044

Incheon, Republic of Korea, March 12-14th, 2025

Title:	Candidate RAN1/2/3 NR-NTN topics for Rel-20 5GA

Source: 	Thales

Type:	discussion

Document for:	discussion

Agenda Item:	15.2.8.4 - NR-NTN

# Introduction

This document proposes to define new capabilities or performance optimizations for NR-NTN radio interface as part of Rel-20 5G Advanced work plan.

# Candidate RAN1/2/3 led NR-NTN related topics for Rel-20 5G Advanced

## Context

The following NTN related topics (capabilities or performance optimizations) have been identified for NR-NTN by different companies.

This includes the support of

- GNSS resilient operation (GNSS signal may be temporarily unavailable)

- Wide beams on DL and narrow beam on UL

- Resilient notification

- Multi orbit satellite support architecture

- LTE TN to NR-NTN and vice e versa mobility (i.e. hand-over) in connected mode

- NR NTN to NR TN and vice e versa mobility (i.e. hand-over) in connected mode

- DL throughput and capacity enhancement

- Inter RAT (NR-NTN/IoT-NTN) cell (re)selection

- Store and forward for NR-NTN

- SSB periodicity extension up to 320 ms

## Proposed NR-NTN enhancements for Rel-20 5G advanced

Thales recommend that the RAN1/2/3 led NTN topics in the table below to be considered in priority as part of the Rel-20 5G Advanced work plan:

### GNSS resilient operation for uplink time and frequency synchronization in NTN based access

#### Justification

5G NR-NTN and IoT-NTN specifications in Releases 17, 18 and 19 (presently in progress) assume that the UE uses its GNSS coordinates i.e., UE position - see TS 38.300 Section 16.14.2.2 and TS 36.300 Section 23.21.2.2 (together with the ephemeris of the serving satellite and additional information about the feeder link delay) to pre-compensate the time and frequency shift.

In most locations and at most times, GNSS provides a highly accurate position and time reference. However, evidence has been shown that GNSS jamming (leading to denial of service) and spoofing (leading to incorrect location reporting and potentially denial of service) may occur. Also, a terminal, or group of terminals attempting to connect over access channels or once connected to the network, could also cause denial or degradation of service for all other users in the cell if using an altered GNSS position.

Additionally, GNSS handling may cause delays to the UE operation due to positioning acquisition time (e.g., for a GNSS "cold start") and additional energy consumption.

Therefore, NTN UEs of all types including handheld terminals and mobile VSAT (*) devices do require an alternative means of achieving and maintaining successful uplink time and frequency synchronization in NTN based access.

(*) It is assumed that for a stationary device (e.g., fixed VSAT, static IoT device), the UE position can be pre-configured.

Other benefits provided by GNSS resilient NTN operation would include improved energy efficiency.

#### Objectives

Hypothesis

- UE (up to 250 km/h) with GNSS receiver (but GNSS service may be unavailable)

- NGSO, GSO

- In a given radio cell, it is assumed that either all UE can receive GNSS or not

- Multiple satellite in visibility of the UE

Objective

- Define for NR-NTN additional network assistance information broadcasted across multiple satellites (e.g. PRS) for the UE to determine its position with sufficient accuracy (RAN4 timing requirements in TS 38.133 and frequency requirements in TS 38.101-5) allowing the UE to perform UL timing/frequency synchronization in Idle (including initial access) and Connected modes

- Define, if needed, enhancement(s) to closed loops for timing / frequency corrections.

#### Working groups

- RAN1 [0.5 TU per meeting]

- RAN2 [0.25 TU per meeting]

- RAN4 RRM [0.5 TU per meeting]

- RAN4 RF [0.25 TU per meeting]

### Wide beam on DL and narrow beam on UL

#### Justification

NGSO systems face challenges due to significant slant range differences between beams generated at satellite nadir and those generated towards the edge of the satellite foot print, hence at low elevations. This imbalance results in uneven cell signal power across the beams and elongated radio cells at edge compared to nadir, which may cause in coverage gaps.

To address this issue, NTN satellite systems aim to deliver consistent DL Power Flux Density (PFD) and UL receive gain contours at ground level across the foot print. Such a setup increases power demands for edge-of-coverage beams. To manage this, re-balancing beam footprints by adjusting ABS (Adjacent Beam Spacing of adjacent beams from the central beam) can help maintain uniform coverage. This involves increasing nadir beam footprints while reducing edge beam sizes, thereby necessitating a mix of wide and narrow beams, which conserves DL power near nadir beams and facilitates power allocation for coverage edges, However, this does not improve the UL receive gain, which requires narrow beams to maintain gain and avoid unacceptable UL link budget losses, especially critical for signals like PRACH.

Therefore, it is proposed to consider potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels including PRACH may be transmitted using a narrower beam footprint. These enhancements were previously discussed during the Rel-19 WID NTN phase 3 but could not be pursued due to a lack of Time Units.

#### Objectives

Specify potential enhancements to support a dual-beam operation where a wide beam is used for transmitting SSB and downlink (DL) common channels, with narrower beams overlaid by the wide beam being utilized for data transfer on dedicated DL/UL channels, including the PRACH.

- Introduce technique (s) to direct the UE from wider DL beam to UL narrow beam at initial access, to allow early benefits of narrow beam starting from PRACH transmission;

#### Working groups

- RAN1 [0.25 TU per meeting]

- RAN2 [0.25 TU per meeting]

- RAN4 RRM [0.25 TU per meeting]

# Conclusion and Way forward

Proposal 1: RAN to start defining the necessary enabling functions for the RAN1/2/3 led NR-NTN topics as part of the Rel-20 5G Advanced work plan:

- GNSS resilient operation for uplink time and frequency synchronization in NR-NTN based access (i.e. support of multiple satellites positioning with PRS configuration indication in SIB)

- Wide beam on DL and narrow beam on UL

- Support of IMS voice call over NR-NTN via GEO

END