3GPP TSG RAN meeting #107									           		RP-250045

Incheon, Korea, March 12th-14th , 2025

## Status Report to TSG

Agenda item:			10.4.5

| WI / SI Name | Introduction of IoT-NTN TDD mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | IoT_NTN_TDD |  |  |  |  |
| Unique ID | 1050123 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-243293 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: N/A | Core part: Overall: 30% RAN1: 50% RAN2: 15% RAN4: 20% | Performance Part: Overall: 0% RAN4: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Andjela Ilic-Savoia |
|  | Company | Iridium Satellite LLC |
|  | Email | andjela.savoia@iridium.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1 Agreements during RAN1#120

Agreement

The guard period between the end of the downlink subframes and the beginning of the uplink subframes is fixed in specifications to be [48…50] ms at the ULSRP

- NOTE: This corresponds to the largest separation between DL and UL Iridium slot pairs. Adjustments to align with different UL/DL pair can be achieved by adjusting the common TA.

Agreement

The downlink subframes within D=8 are fixed to:

- Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames)

Agreement

NPSS/NSSS/NPBCH transmissions are dropped in non-D NB-IoT subframes

- NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

- FFS: handling for SIB1-NB

Agreement

The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” from the specifications

- NPUSCH is mapped as per current specifications.

- NOTE: “non-U NB-IoT subframes” are subframes not contained within the set of U=8 usable consecutive uplink subframes in the TDD structure.

Agreement

The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” from the specifications

- NPDCCH / NPDSCH (not containing SI) are mapped as per current specifications.

- FFS: additional NB-IoT TDD-specific rules for handling non “NB-IoT DL subframes”

- FFS: Introduction of new periodicities for NPDCCH monitoring (e.g. by the introduction of scaling factor)

- FFS: whether all the configuration values for NPDCCH are valid for NB-IoT NTN TDD.

- FFS: handling of DL transmission gap

- NOTE: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure.

Conclusion

For determining the offset between the 90ms TDD structure and the 10240ms H-SFN: the UE may derive the 90ms TDD structure based on observations of downlink signals.
No specification impact.

Agreement

For NPRACH, further consider the two options until RAN1#120bis:

- Option 1: New NPRACH periodicities (multiple of 90ms) are introduced for NB-IoT NTN TDD.

- Option 2: NPRACH transmissions are postponed in non-U NB-IoT subframes until the next U NB-IoT subframe(s)

  - FFS: The unit of postponement (e.g. PRU)

Agreement

NPRACH format 2 is not supported in NB-IoT NTN TDD.

Working assumption

Uplink transmission gaps do not apply in NB-IoT NTN TDD.

Conclusion

RAN1 does not further discuss the issue of Kmac extension unless triggered by RAN2 LS.

Conclusion

It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any.

Agreement

For segmented pre-compensation, RAN1 to further discuss at least the following options:

- Option 1: Dropped/postponed NPUSCH / NPRACH are counted for segment determination

- Option 2: A new pre-compensation segment is started at the beginning of each uplink burst

- Option 3: UL segment should only be defined in consecutive U subframes

- Option 4: Segmented pre-compensation does not apply for NB-IoT NTN TDD.

- FFS: How to handle the UL segment gap

## 2.2	RAN2

#### 2.2.1 Agreements during RAN2#129

Agreements:

1.	RAN2 will continue studying paging aspects based on RAN1 progress

2.	RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed.

3.	RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network.

4.	RAN2 thinks that a change of H-SFN duration (Option 1-1) and/or H-SFN total number (option 2-2) will impact RAN2 and SA2 specification regarding the support of idle mode eDRX in IoT-NTN TDD network and the impact should be evaluated.

5.	RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system.

6.	RAN2 can continue the discussion also on RAR window

7.	Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN)

2.2.2	Remaining open issues

- Overall description

Proposal 1: A description of IoT-NTN TDD mode shall be added in 36.300 [§5] as follows:

Downlink and uplink transmissions are organized into radio frames with 10 ms duration. Three radio frame structures are supported:

- Type 1, applicable to FDD and IoT-NTN TDD;

- Type 2, applicable to TDD;

…

For IoT-NTN TDD mode, Frame Structure Type-1 is used where uplink and downlink transmissions are separated in the time domain and constitute of set of D non-overlapping usable contiguous DL subframes and set of U usable contiguous UL subframes separated by fixed guard period (GP) as per Figure 1. This pattern is repeated every N radio frames. The values of N, D, U, [TBD for GP and start subframe for D] are fixed per IoT-NTN TDD band specified in [TBD]

RAN2 invites the WI rapporteur to submit a TP for Stage2 to RAN1 so that they can endorse it and send it back to RAN2 for final approval

- SI scheduling, SI transmissions postponement/ truncation, potential modifications to the SI scheduling mechanism

- RAN2 waits for further RAN1 progress before further discussing this

- UL SPS

Proposal 6: UL SPS transmission is dropped when the UL SPS overlaps with non-U NB-IoT subframes.

- RAN2 Can come back to this proposal in the next meeting

## 2.3	RAN3

## 2.4	RAN4

2.4.1 	Agreements during RAN4#114 UE/SAN RF

2.4.1.1 General requirements:

Issue 1-1-1: Work Plan presented under R4-2500136.

Agreement: Updated Work Plan to be agreed under R4-2502291 (see Annex for the updated work plan).

Issue 1-2-1: Band number

Agreement: Adopt [249] E-UTRA operating band number for the new TDD 1616-1626.5 MHz NTN NB-IoT frequency band introduction.

Issue 1-2-2: E-UTRA operating band

Agreement: Confirm the proposed band plan as follows:

E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | - | 2010 MHz | 2170 MHz | - | 2200 MHz | FDD |
| 255 | 1626.5 MHz | - | 1660.5 MHz | 1525 MHz | - | 1559 MHz | FDD |
| 254 | 1610 MHz | - | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| 2532 | 1668 MHz | - | 1675 MHz | 1518 MHz | - | 1525 MHz | FDD |
| 252 | 2000 MHz | - | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |
| [249] | 1616 MHz | - | 1626.5 MHz | 1616 MHz | - | 1626.5 MHz | TDD |
| NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations. NOTE 3: Band 249 is defined only for NB-IoT operation. |  |  |  |  |  |  |  |

Issue 1-2-3: Gap between EARFCN FDD NTN and EARFCN TDD NTN

Agreement: Consider a gap between previous introduced EARFCN FDD NTN (starting from 2^18-1=262143 for NUL and 2^18-2^15-1=229375 for NDL) and new (first) introduced EARFCN TDD NTN.

Issue 1-2-4: EARFCN for TDD NTN

Agreement: For the NTN E-UTRA TDD EARFCN number, consider to start the NUL/NDL upper range from 2^18-2^16-1=196607 (below FDD NTN NOffs-DL).

Issue 1-2-5: Channel raster for TDD NTN

Agreement: Use a channel raster of 100 kHz and a Step size of <1>.

Issue 1-3-1: TS 36.108 Table 5.4A.2-1: E-UTRA channel numbers

Agreement: Update table as part of E-UTRA channel numbers in Section 5.4A.2, TS 36.108:

Table 5.4A.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 256 | 100 | 2170 | 229076 | 229076 -<1>- 229375 | 1980 | 261844 | 261844 -<1>- 262143 |
| 255 | 100 | 1525 | 228736 | 228736 -<1>- 229075 | 1626.5 | 261504 | 261504 -<1>- 261843 |
| 254 | 100 | 2483.5 | 228571 | 228571 -<1>- 228735 | 1610 | 261339 | 261339 -<1>- 261503 |
| 253 | 100 | 1518 | 228501 | 228501 -<1>- 228570 | 1668 | 261269 | 261269 -<1>- 261338 |
| [249] | 100 | 1616 | 196607 | 196607-<1>-196711 | 1616 | 196607 | 196607-<1>-196711 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Issue 1-3-2: TS 36.108 5.7.3 Clause

Agreement: As an initial working assumption, with respect to the definition of IoT-NTN TDD L-band, add the following information in 5.7.3 for the carrier frequency and EARFCN for DL and UL respectively:

- “The carrier frequency of NB-IoT in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 – 262143 and the Offset of NB-IoT Channel Number to EARFCN in the range {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,-0.5,0,1,2,3,4,5,6,7,8,9} for FDD and TDD band 249.”

- “The carrier frequency of NB-IoT in the uplink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 –262143, and the Offset of NB-IoT Channel Number to EARFCN in the range {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,0,1,2,3,4,5,6,7,8,9} for FDD and TDD band 249.”

Moderator Note: Companies to further check.

Issue 1-4-1: TS 36.102 Table 5.4A.2-1: E-UTRA channel numbers

Agreement: Update TS 36.102 with:

Table 5.4A.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 256 | 100 | 2170 | 229076 | 229076 -<1>- 229375 | 1980 | 261844 | 261844 -<1>- 262143 |
| 255 | 100 | 1525 | 228736 | 228736 -<1>- 229075 | 1626.5 | 261504 | 261504 -<1>- 261843 |
| 254 | 100 | 2483.5 | 228571 | 228571 -<1>- 228735 | 1610 | 261339 | 261339 -<1>- 261503 |
| 253 | 100 | 1518 | 228501 | 228501 -<1>- 228570 | 1668 | 261269 | 261269 -<1>- 261338 |
| [249] | 100 | 1616 | 196607 | 196607-<1>-196711 | 1616 | 196607 | 196607-<1>-196711 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Issue 1-4-2: TS 36.102 5.4B Clause

Agreement: Update TS 36.102 with:

== Start Change == ## 5.4B Channel arrangement for category NB1 and NB2 ### 5.4B.1 Channel spacing Nominal channel spacing for UE category NB1 and NB2 in stand-alone mode is 200 kHz. ### 5.4B.2 Channel raster, carrier frequency and EARFCN The channel raster of UE category NB1/NB2 shall be as defined in clause 5.4A.2, and the channel raster per-frequency band shall be as defined in table 5.4A.2-1. The carrier frequency of UE category NB1/NB2 in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) as defined in Table 5.4A.2-1, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, -0.5, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD and TDD band 249. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL is the downlink carrier frequency of category NB1/NB2, FDL_low and NOffs-DL are given in table 5.4A.2-1, NDL is the downlink EARFCN, MDL is the Offset of category NB1/NB2 Channel Number to downlink EARFCN. FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL) The carrier frequency of UE category NB1/NB2 in the uplink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) as defined in Table 5.4A.2-1, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD and TDD band 249. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the uplink is given by the following equation, where FUL is the uplink carrier frequency of category NB1/NB2, FUL_low and NOffs-UL are given in table 5.4A.2-1, NUL is the uplink EARFCN, MUL is the Offset of category NB1/NB2 Channel Number to uplink EARFCN. FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL) NOTE 1: Guard-band operation and in-band operation for NB-IoT are not supported in this version of the specification. NOTE 2: For the carrier including NPSS/NSSS for stand-alone operation, MDL = 0. == End Change ==

2.4.1.2 UE requirements:

Issue 2-1-1: Discussion on UE REFSENS for IoT TDD in TS 36.102

Agreement: Reuse REFSENS -108.2dBm for the new TDD NTN NB-IoT operating band.

Issue 2-1-2: Clause 7.3B Update - Reference sensitivity power level for UE category NB1 and NB2

Agreement: RAN4 to adopt the following modification in the TS 36.102:

## 7.3B	Reference sensitivity power level for UE category NB1 and NB2

The reference sensitivity power level REFSENS is the minimum mean power applied to the single antenna port for UE category NB1 and category NB2, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise.

The category NB1 and NB2 for FDD and TDD UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in TS 36.101 [7] Annex A.3.2 with received signal level as specified in Table 7.3B-1. Requirement in Table 7.3B-1 applies for any uplink configuration.

Table 7.3B-1: Reference sensitivity for UE FDD and TDD category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B | - 108.2 |

Issue 2-1-3: Discussion on UE AMPR for NB-IoT TDD NTN in TS 36.102

Agreement: Further discuss AMPR for 249. Proponent to clarify at next meeting which band has to be protected through this AMPR mechanism.

Issue 2-1-4: Discussion on frequency error requirement for NB-IoT TDD NTN in TS 36.102

Agreement: Further discuss the potential update of frequency error requirement for TDD to allow UE to update pre-compensation between every UL slot/segment.

Moderator note1: further clarify if UL slot is the LTE Tx slot or is for the Tx transmission time U=8. In both cases, the issue seems to be implementation-related.

Moderator note2: [Clause 6.4B.1of TS36.102] “When a repetition period is configured on the uplink for which repetition period (R) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period, except in the transmission gaps as defined in clause 10.1.3.6 of TS 36.211[3]. When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.”

Moderator note3: According to the 2nd note, it seems that TDD UE behaviour will be similar to FDD UE behaviour over different segment transmissions. Also, the current FRC in UL for FDD seem to indicate much higher transmissions times than 8ms (proposed for TDD frame).

Issue 2-1-5: Discussion on EVM clause for NB-IoT TDD NTN in TS 36.102

Agreement: Add for EVM clause “For TDD the averaging in the time domain can be calculated from subframes of different frames and should have a minimum of 10 subframes averaging length. TDD special fields (DwPTS and GP) are not included in the averaging.”

2.4.1.3 SAN requirements:

Issue 3-1-1: Requirement set applicability TS 36.108

Agreement: RAN4 to update the following Requirement set applicability from TS 36.108:

Table 4.6-1: Requirement set applicability

| Requirement | Requirement set |  |
| --- | --- | --- |
|  | SAN type 1-H | SAN type 1-O |
| SAN output power | 6.2 |  |
| … | … |  |
| Transmit ON/OFF power | NA 6.4 |  |
| … | … |  |
| Transmitter intermodulation | NA | NA |
| … | … |  |
| Performance requirements | 8 |  |
| Radiated transmit power | 9.2 | 9.2 |
| … |  | … |
| OTA transmit ON/OFF power |  | NA 9.5 |
| … |  | … |
| OTA ACLR | NA | 9.7.3 |
| … |  | … |

Issue 3-1-2: Transmit ON/OFF power in TS 36.108

Agreement: Update the following Clause 6.4 in TS 36.108 with “The requirements in clause 6.4 are only applied for NB-IoT TDD SAN.” Also, update relevant clause “transmit ON/OFF power” for NB-IoT TDD SAN in TS 36.181 with TS 36.108 agreement for Transmit ON/OFF power.

## 6.4	Transmit ON/OFF power

The requirement is not applicable in this version of the specification.

The requirements in clause 6.4 are only applied for NB-IoT TDD SAN.

### 6.4.1	Transmitter OFF power

Transmitter OFF power is defined as the mean power measured over 70 µs filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the SAN (BWConfig) centred on the assigned channel frequency during the transmitter OFF period.

#### 6.4.1.1	Minimum Requirement

The transmitter OFF power spectral density shall be less than -85dBm/MHz.

For SAN capable of multi-band operation, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

### 6.4.2	Transmitter transient period

The transmitter transient period is the time period during which the transmitter is changing from the OFF period to the ON period or vice versa. The transmitter transient period is illustrated in Figure 6.4.2-1.

Figure 6.4.2-1 Illustration of the relations of transmitter ON period, transmitter OFF period and transmitter transient period.

#### 6.4.2.1	Minimum requirements

The transmitter transient period shall be shorter than the values listed in Table 6.4.2.1-1.

Table 6.4.2.1-1 Minimum requirements for the transmitter transient period

| Transition | Transient period length [us] |
| --- | --- |
| OFF to ON | [17] |
| ON to OFF | [17] |

Issue 3-1-3: OTA Transmit ON/OFF power in TS 36.108

Agreement: Update the following Clause 9.5 in TS 36.108 with “The requirements in clause 9.5 are only applied for NB-IoT TDD SAN”. Also, update relevant clause “transmit ON/OFF power” for NB-IoT TDD SAN in TS 36.181 with TS 36.108 agreement for OTA Transmit ON/OFF power.

## 9.5	OTA transmit ON/OFF power

The requirement is not applicable in this version of the specification.

### The requirements in clause 9.5 are only applied for NB-IoT TDD SAN.

### 9.5.2	OTA transmitter OFF power

#### 9.5.2.1	General

OTA transmitter OFF power is defined as the mean power measured over 70 us filtered with a square filter of bandwidth equal to the transmission bandwidth configuration of the SAN (BWConfig) centred on the assigned channel frequency during the transmitter OFF period.

For multi-band RIBs and single band RIBs supporting transmission in multiple bands, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

#### 9.5.2.2	Minimum requirement for SAN type 1-O

The total power from all co-location reference antenna conducted output(s) shall be less than -106 dBm/MHz.

### 9.5.3	OTA transient period

#### 9.5.3.1	General

The OTA transmitter transient period is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

This requirement shall be applied at each RIB supporting transmission in the operating band.

#### 9.5.3.2	Minimum requirement for SAN type 1-O

For SAN type 1-O, the OTA transmitter transient period shall be shorter than the values listed in the minimum requirement table 9.5.3.2-1.

Table 9.5.3.2-1: Minimum requirement for the OTA transmitter transient period for SAN type 1-O

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | [17] |
| ON to OFF | [17] |

Issue 3-2-1: Remove redundant A.3-1 FRC

Agreement: RAN4 to clean up the spec TS 36.108 as part of maintenance CR (not specifically related to this WID) and remove redundant data e.g. A.3-1 FRC (redundant with A.14-1) not cited at all in all the text.

Table A.3-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 | A14-2 |
| --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Diversity | No | No |
| Modulation | π/2 BPSK | π/2 BPSK |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) Note 1 | 4 | 16 |
| Number of NPUSCH repetition | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 |
| Payload size (bits) | 32 | 32 |
| Allocated resource unit | 2 | 2 |
| Code rate (target) | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 |
| Total symbols per resource unit | 96 | 96 |
| Tx time (ms) | 16 | 64 |
| Note: Channel estimation lengths are included in the table for information only. |  |  |

Issue 3-2-2: Remove redundant A.4-1 FRC

Agreement: RAN4 to clean up the spec TS 36.108 as part of maintenance CR (not specifically related to this WID) and remove redundant data e.g. A.4-1 FRC (redundant with A.15-1) not cited at all in all the text.

Table A.4-1 FRC parameters for NB-IoT dynamic range

| Reference channel | A15-1 | A15-2 |
| --- | --- | --- |
| Sub carrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Modulation | π/4 QPSK | π/4 QPSK |
| Diversity | No | No |
| Frequency offset | 0 | 0 |
| IMCS / ITBS | 7 / 7 | 7 / 7 |
| Payload size (bits) | 104 | 104 |
| Allocated resource units | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 |
| Coding rate (target) | 2/3 | 2/3 |
| Coding Rate | 0.67 | 0.67 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks – C | 1 | 1 |
| Total symbols per resource unit | 96 | 96 |
| Total number of bits per resource unit | 192 | 192 |
| Tx time (ms) | 8 | 32 |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| Note: Channel estimation lengths are included in the table for information only. |  |  |

Issue 3-3-1: NTN NB-IoT SAN FRC for reference sensitivity and in-channel selectivity

Agreement: RAN4 to further discuss FRC for reference sensitivity and in-channel selectivity in SAN TS 36.108 with respect to band 249 for better adaptation to 8ms TDD Tx time (corresponding to D/U=8).

- Option 1: Further discuss if to reuse A14-1 reference channel and segment the signal into 2 different 8 ms Tx/Rx times (e.g. with 90 ms periodicity between 2 transmissions, if 1 transmission can be postponed) for TDD NTN NB-IoT SAN requirements. Further discuss if A14-2 could be descoped for TDD NTN NB-IoT SAN requirements.

- Option 2: Further discuss if to define new FRC(s) for NTN NB-IoT to cope with 8 ms Tx/Rx time, other options are not precluded.

Table A.14-1 FRC parameters for reference sensitivity and in-channel selectivity

| Reference channel | A14-1 | A14-2 |
| --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Diversity | No | No |
| Modulation | π/2 BPSK | π/2 BPSK |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| Number of NPUSCH repetition | 1 | 1 |
| IMCS / TBS | 0 / 0 | 0 / 0 |
| Payload size (bits) | 32 | 32 |
| Allocated resource unit | 2 | 2 |
| Code rate (target) | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 |
| Total symbols per resource unit | 96 | 96 |
| Tx time (ms) | 16 | 64 |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |

Issue 3-3-2: NTN NB-IoT SAN FRC dynamic range

Agreement: RAN4 to further discuss FRC for dynamic range in SAN TS 36.108 with respect to band 249 for better adaptation to 8ms TDD Tx time (corresponding to D/U=8).

- Further discuss if reference channel A15-1 can be reused for NTN TDD IoT NTN.

- Further discuss if reference channel A15-2 can be descoped for NTN TDD IoT NTN.

- Other options are not precluded.

Table A.15-1 FRC parameters for NB-IoT dynamic range

| Reference channel | A15-1 | A15-2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Modulation | π/4 QPSK | π/4 QPSK |
| Diversity | No | No |
| Frequency offset | 0 | 0 |
| IMCS / ITBS | 7 / 7 | 7 / 7 |
| Payload size (bits) | 104 | 104 |
| Allocated resource units | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 |
| Coding rate (target) | 2/3 | 2/3 |
| Coding Rate | 0.67 | 0.67 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks – C | 1 | 1 |
| Total symbols per resource unit | 96 | 96 |
| Total number of bits per resource unit | 192 | 192 |
| Tx time (ms) | 8 | 32 |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) (NOTE) | 4 | 16 |
| NOTE: Channel estimation lengths are included in the table for information only. |  |  |

Issue 3-4-1: General requirements

Agreement: RAN4 to further discuss FRCs, related SNR targets for specific configurations and equations for driving requirements (e.g. REFSENS, in-channel selectivity, dynamic range).

  - NOTE: TR 36.802 can be used as starting point for the equations, FRC (when applicable) and related SNR values.

Issue 3-4-2: Number of tones

Agreement: RAN4 to discuss the number of tones to be used for deriving the requirements. Companies may consider increasing the number of tones in order to decrease transmission time less than 8 ms (for compatibility with D/U=8 for the

Issue 3-4-3: Repetition number

Agreement: RAN4 to discuss the repetition number to be used for TDD NB-IoT SAN Rx requirements compatible with D/U=8 (8 ms transmission/reception time) and transmission periodicity N=9 (90 ms) for the new TDD frame structure.

Issue 3-4-5: New REFSENS SAN requirements for NTN TDD in TS 36.108

Agreement: For SAN TS 36.108, adapt FRC configuration with new TDD frame structure (maximum 8 ms transmission time), for example:

  - Option 1: Adapt the existent SAN FRC.

    - For conducted specifications:

For SAN supporting NTN NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.1 with parameters specified in Table 7.2.2-3 and 7.2.2-4 for SAN type 1-H in all FDD operating bands and specified in Table 7.2.2-5 and 7.2.2-6 for SAN type 1-H in all TDD operating bands.

Table 7.2.2-3: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 |

Table 7.2.2-4: Reference sensitivity levels of SAN supporting NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 |

Table 7.2.2-5: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 | TBD |
| 200 | 3.75 | TBD in Annex A.14 | TBD |

Table 7.2.2-6: Reference sensitivity levels of SAN supporting NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 | TBD |
| 200 | 3.75 | TBD in Annex A.14 | TBD |

    - For OTA specifications:

For SAN supporting standalone NB-IoT operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameter specified in Table 10.3.2-3 and Table 10.3.2-4 for FDD bands and Table 10.3.2-5 and Table 10.3.2-6 when the OTA test signal is at the corresponding EISREFSENS level and arrives from any direction within the OTA REFSENS RoAoA.

Table 10.3.2-3: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -124.9 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -130.9 - ΔOTAREFSENS |

Table10.3.2-4: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in FDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | FRC A14-1 in Annex A.14 | -128.0 - ΔOTAREFSENS |
| 200 | 3.75 | FRC A14-2 in Annex A.14 | -134.0 - ΔOTAREFSENS |

Table 10.3.2-5: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (GEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 | TBD- ΔOTAREFSENS |
| 200 | 3.75 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |

Table10.3.2-6: Reference sensitivity levels of SAN supporting standalone NB-IoT operation in TDD (LEO class payload)

| SAN channel bandwidth (kHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 200 | 15 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |
| 200 | 3.75 | TBD in Annex A.14 | TBD - ΔOTAREFSENS |

  - Option 2: Reuse UE FRC (from TS 36.101) for SAN FRC.

Table A.3.2-2e Fixed Reference Channel for Receiver Requirements (TDD) – for CAT-NB1 and CAT-NB2

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | MHz | 0.2 |
| Number of subcarriers |  | 12 |
| Uplink-Downlink Configuration (Note 7) |  | 1 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of HARQ Processes | Processes | 1 |
| Maximum number of HARQ transmissions |  | 1 |
| Transport block size | Bits | 88 |
| Number of Sub-Frames per transport block |  | 1 |
| Transport block CRC | Bits | 24 |
| Binary Channel Bits Per Sub-Frame | Bits | 320 |
| LTE CRS port |  | N/A |
| Number of NRS ports |  | 1 |
| Number of NPDSCH repetitions Number of NPUSCH repetitions |  | 1 |
| UE DL Category UE UL Category |  | NB1 or NB2 |
| Note 1: Category NB1 or NB2 in stand-alone mode has been considered here. Note 2: Reference signal, Synchronization signals and NPBCH allocated as per TS 36.211. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) Note 4: Parameters related to NPDSCH scheduling are defined in Table A.3.2-1e to Table A.3.2-1g. Note 5: NPDCCH and information bit payload are not transmitted in the subframes used for transmission of SI messages. Note 6: SI messages transmission should be prioritized over NPDCCH transmission in case of collision. NPDCCH transmission is postponed until the next NB-IoT downlink subframe in case NPDCCH transmission occurs in a non NB-IoT downlink subframe, where an NB-IoT downlink subframe is a subframe that does not contain NPSS/NSSS/NPBCH/SIB1-NB transmission. Note 7: As per Table 4.2-2 in TS 36.211 [4] Note 8: Number of repetition NRep as defined in table 16.4.1.3-2 in TS 36.213 [6]. |  |  |

Issue 3-4-6: Add EVM phrase for NTN TDD in TS 36.108

Agreement: Add EVM TDD phrase in TS 36.108 “For TDD the averaging in the time domain can be calculated from subframes of different frames and should have a minimum of 10 subframes averaging length. TDD special fields (DwPTS and GP) are not included in the averaging.”

2.4.2 	Agreements during RAN4#114 RRM

## Issue 0: General aspects

Agreement:

For TDD NB-IoT in NTN, the following aspects will be further considered in future RAN4 meetings:

- The target SNR

- Applicable coverage (whether the EC needs to be considered)

- Applicable Rmax

- Applicable DRX cycle

  - Note: The DRX cycle might not be non-uniformly periodic due to the H-SFN not matching with the new TDD pattern. This issue will be addressed by other working groups. In the meantime, RAN4 assumes the DRX cycle is uniformly periodic.

## Issue 1: Cell reselection (4.6A.2)

Agreement:

For the cell reselection for TDD NB-IoT in NTN, discuss the following details in the next meeting:

- Tdetect, Tevaluation and Tmeasure

## Issue 3: RRC re-establishment (6.5A)

Agreement:

In the core requirement, For the delay requirement (TUL_grant + TUE_re-establish_delay_NB-IoT) of RRC re-establishment for TDD NB-IoT in NTN,

- No update to TUL_grant

- For the following parameters in TUE_re-establish_delay_NB-IoT

  - No update:

    - TPRACH_NB-IoT

    - TSI_NB1-NC [and TSI_NB1-EC]

  - Consider update, and the details are FFS:

    - Tsearch_NB1-NC,i [and Tsearch_NB1-EC,i] for unknown cell

## Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A)

Agreement:

For the latency requirement (TRRC_procedure_delay + Tperiod_DL_bitmap + TUL_grant + TDL-UL switch + TNPUSCH) of RRC Connection redirection to Non-Anchor Carrier for TDD NB-IoT in NTN,

- No update to TRRC_procedure_delay, TUL_grant, and TNPUSCH

- Update the following parameters

  - Tperiod_DL_bitmap: to be updated based on RAN1 design

  - TDL-UL switch: to be updated based on RAN1 design

## Issue 5: RLM (7.23A)

Agreement:

Further discuss the following aspects for RLM for TDD NB-IoT in NTN:

- FFS update the hypothetical NPDCCH format (including Rmax) specified in Table 7.23A.2-1 upon RAN1 design in future meetings.

- Evaluation period when DRX is not in use:

  - Option 1: Reuse the existing requirement defined in 7.23A.2.1, unless RAN1 changes cause impact to this requirement

  - Option 2: The evaluation period defined in 7.23A.2.1 is scaled

    - Option 2a: scaled by N=9

    - Option 2b: scaled by FFS

    - Option 2c: no scaling, and define requirement based on 8ms DL subframes

- Evaluation period when DRX is in use:

  - FFS whether the evaluation period defined in 7.23A.2.1 needs to be scaled.

## Issue 6: Channel quality report (8.14A.4)

Agreement:

Channel report requirement for TDD NB-IoT in NTN is defined as follows:

- FFS Reuse the existing measurement period

- In future meetings, may need to revisit the following aspects based on RAN1 design:

  - Hypothetical NPDCCH format specified in Table 8.14A.4-1

  - Report mapping tables defined in [9.1.22.15]

## Issue 7: PUR (4.6A.3)

Agreement:

The existing PUR requirement defined for NB-NTN applies to the TDD NB-IoT NTN, if PUR requirements is in the WI scope.

## Issue 8: Uplink transmit timing (7.20A)

Agreement:

The existing UE transmit timing requirement defined for NB-NTN applies to the TDD NB-IoT NTN.

- If any updates are identified upon RAN1 agreements related to 'uplink transmission gap' and 'segmented uplink transmission,' the corresponding updates will be made accordingly.

2.4.3 		Remaining open issues

## Issue 2: Cell measurement in connected mode (8.14A)

Further discuss the following:

For the cell measurement period in connected mode for TDD NB-IoT in NTN,

- For intra-frequency measurement, the delay requirements shall be scaled by N=9

- For inter-frequency measurement, the minimum length of measurement occasions is defined as the max {existing value, periodicity of corresponding RS}.

- Revise the existing cell measurements condition “at least one 1 DL subframe per radio frame has to be available at the UE during the measurement period.”

- Serving cell measurement requirements apply provided the UE is provided with the configuration about the “active DL” periods.

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1		SA2

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

## 4.1	RAN1

RAN1#120 meeting, Athens, Greece, February 17-21th, 2025:

- R1-2500523 	Work plan for WID: introduction of IoT-NTN TDD mode	Iridium

- R1-2500038	Discussion on IoT-NTN TDD mode	THALES

- R1-2500086	Discussion on IoT-NTN TDD mode	Huawei, HiSilicon

- R1-2500186	Discussion on IoT-NTN TDD mode	Spreadtrum, UNISOC

- R1-2500211	Physical layer design on IoT-NTN TDD mode	CATT

- R1-2500305	Discussion on the new NB-IoT NTN TDD mode	CMCC

- R1-2500370	Discussion on IoT-NTN TDD mode	vivo

- R1-2500386	Discussion on the IoT-NTN TDD mode	ZTE Corporation, Sanechips

- R1-2500447	Discussion on IoT-NTN TDD mode	OPPO

- R1-2500509	Discussion on IoT-NTN TDD mode	Nokia, Nokia Shanghai Bell

- R1-2500524	loT-NTN TDD mode scheduling, timing and channel decoding performance	Iridium

- R1-2500720	Discussion on IoT-NTN TDD mode	Xiaomi

- R1-2500807	Discussion on IoT-NTN TDD mode	Apple

- R1-2500870	Discussion on IoT-NTN TDD mode	Samsung

- R1-2500889	Discussion on IoT-NTN TDD mode	Lenovo

- R1-2500982	Discussion on the IoT-NTN TDD mode	TCL

- R1-2501044	Discussion on IoT-NTN TDD mode	LG Electronics

- R1-2501075	IoT NTN TDD frame structure and synchronization signal mapping	Sharp

- R1-2501177	IOT-NTN TDD mode	Qualcomm Inc

- R1-2501219	Discussion on IoT-NTN TDD mode	NTT DOCOMO, INC.

- R1-2501295	On R19 TDD IoT-NTN	Nordic Semiconductor ASA

- R1-2501366	Feature lead summary #1 on IoT-NTN TDD mode	Moderator (Qualcomm Inc)

- R1-2501367	Feature lead summary #2 on IoT-NTN TDD mode	Moderator (Qualcomm Inc)

- R1-2501614	Feature lead summary #3 on IoT-NTN TDD mode	Moderator (Qualcomm Inc)

## 4.2	RAN2

RAN2#129 meeting, Athens, Greece, February 17-21th, 2025:

- R2-2500175 	Work plan for WID: introduction of IoT-NTN TDD mode	Iridium

- R2-2500936	Discussion of adaptions to support IoT-NTN TDD	THALES

- R2-2500085	Discussion on RAN2 Imapcts of IoT-NTN TDD Mode	vivo

- R2-2500312	Discussion on IoT NTN TDD mode	OPPO

- R2-2500527	Discussion on support of IoT NTN TDD	CATT

- R2-2500536	Discussion on new NB-IoT NTN TDD mode	Qualcomm Incorporated

- R2-2500549	Discussion on support of TDD mode for IoT-NTN	Nokia, Nokia Shanghai Bell

- R2-2500621	Discussion on TDD support in IoT NTN	Lenovo

- R2-2500923	Signaling aspects of IoT-NTN TDD mode	TOYOTA Info Technology Center

- R2-2501036	Support of IoT-NTN TDD mode		CMCC

- R2-2501277	Initial aspects on IoT NTN TDD		Samsung

- R2-2501521	Draft LS on H-SFN aspects for IoT NTN TDD mode Samsung	LS out To:RAN1

- R2-2501333	Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE)

## 4.3	RAN3

## 4.4	RAN4

RAN4#114 meeting, Athens, Greece, February 17-21th, 2025:

- R4-2500579 	Topic summary for [114][318] IoT_NTN_TDD_UE_SAN_RF, Moderator (Thales)

- R4-2500546 	Topic summary for [114][232] IoT_NTN_TDD, Moderator (Qualcomm)

- R4-2502026	General Discussion on the NTN NB-IoT TDD	THALES, Iridium Satellite LLC

- R4-2502027	Discussion on SAN RF requirements for NTN NB-IoT TDD	THALES, Iridium Satellite

- R4-2502050	Discussion on UE RF requirements for NTN NB-IoT TDD	THALES, Iridium Satellite

- R4-2502052	Discussion on RRM requirements for NTN NB-IoT TDD	THALES, Iridium Satellite

- R4-2500611	Discussion on system parameters and UE RF requirements for the new NB-IoT TDD operating NTN band	ZTE, Sanechips

- R4-2500738	Discussion on band and system parameters for IoT-NTN TDD mode	vivo

- R4-2501178	Discussion on IoT NTN TDD mode	ZTECorporation, Sanechips

- R4-2502224	(IoT_NTN_TDD-Core) RRM requirements for IoT NTN in TDD mode	Qualcomm

- R4-2500136	Work plan for WID: introduction of IoT-NTN TDD mode, Iridium

- R4-2502291	Revised Work plan for WID: introduction of IoT-NTN TDD mode, Iridium

- R4-2502292	Way Forward for [114][318] IoT_NTN_TDD_UE_SAN_RF, Moderator (Thales)

- R4-2502607	Way Forward on RRM requirements IoT_NTN_TDD, Moderator (Qualcomm)