3GPP TSG-RAN Meeting #107	RP-250057

Incheon, Korea, 12–14 March, 2025

Agenda item:	15.2.8.1

Source:	Samsung

Title:	View on Rel-20 Mobility enhancements

Document for:	Discussion and Decision

# Introduction

3GPP RAN had a workshop to discuss the candidate items for Rel-20 last December, and RAN Chair summarized the discussion in RP-243292 [1].

For mobility enhancements, we observe strong support on the items as also summarized below [1], and it can be part of the Rel-20.

In this contribution, we discuss which objectives can be discussed in Rel-20 for the item.

# Discussion and justification

Quite many companies showed their interest to continue the mobility enhancements from the discussion in RAN#106 (e.g., RP-242468, RP-242525, RP-242584, RP-242792, RP-242822, RP-242848, RP-242862, RP-242923, RP-243162 and RP-243212).

From the contributions, we observe that companies are generally fine to complete the conditional LTM (L1/L2 Triggered Mobility) by expanding it to inter-CU case, if it cannot be done in Rel-19.

On top of the objective, we think that the following two items can also be discussed under this agenda as we expressed in RP-242525:

-	CSI acquisition and CSI report for SCell (RAN2)

-	Legacy HO enhancements, e.g., early UL sync, inter-gNB RACH-less HO support (RAN2/RAN3)

The LTM feature is increasingly attracting attention from operators. The feature is useful especially in the high frequency scenarios where multiple beams should be utilized. That is, it can reduce the actual handover latency after handover by performing the (CSI-based) beam measurement in advance. From that perspective, this feature can be extended to SCell addition case, as SCell is usually deployed in higher frequency than PCell. That is, if network configures CSI-RS for candidate SCell measurement (as it did for the handover purpose in LTM), UE can provide its measurement result in advance before adding the candidate SCell which results the reduction of the SCell addition delay. Hence, CSI acquisition and CSI report for SCell can be discussed under this work item as part of the evolvement of LTM feature.

In addition to the evolution of LTM feature, 3GPP may also need to consider the evolution of the legacy Layer 3 (L3)-based handover, since numerous operators utilize lower frequency for the coverage. In this case, they may not have enough motivation to adopt the LTM feature, but prefer maintaining the legacy L3-based handover. Even in such case, it is worth noting that RAN2 introduced several enhancements for LTM applicable to the legacy handover, so such enhancements can be considered as part of the evolution of the legacy L3-based handover. One example is 'early UL synchronization'. In this feature, UE transmits a random access preamble to the candidate target cell "before" performing handover, so that UE can skip the random access procedure at the point time in handover.

In addition, 'RACH-less handover' can be another candidate to reduce the handover latency in the legacy L3-based handover. The feature was already introduced for a normal UE in Rel-18 NR, but is limited to intra-gNB case only in the stage-2 text. As the feature is practical and easy to implement, 'RACH-less handover' can be extended to the inter-gNB case as well.

# Detailed objectives

The following can be the objectives for the WI:

-	Support of inter-CU conditional LTM (subject to Rel-19 progress) (RAN2/RAN3)

-	CSI acquisition and CSI report for SCell (RAN2)

-	Legacy HO enhancements, e.g., early UL sync, inter-gNB RACH-less HO support (RAN2/RAN3)

To accommodate the objectives above, 2 TU and 1 TU would be needed in RAN2 and RAN3, respectively.

# Reference

RP-243292, RAN Chair’s Summary of 5G-Advanced in Rel-20, RAN Chair, December 2024