3GPP TSG RAN Meeting #107						RP-250080

Incheon, Korea, March 12-14, 2025

Agenda item:	15.2.2

Source:	Ericsson

Title:	MIMO Phase 6

Document for:	Discussion

# Background

In the Rel.20 workshop at RAN plenary #106, a large amount of MIMO proposals for MIMO Phase 6 was proposed. In this contribution, to keep the scope of limited, we discuss our view on the urgency, commercial relevance, implementation complexity and potential benefits of these categories of proposals:

- Our own proposals

- Proposals that seem to have support from several companies

# Discussions on candidates for MIMO Phase 6

We have categories the proposals into Uplink PUSCH, SRS, CSI and FR2 respectively:

## Uplink PUSCH enhancements

### DFT-s-OFDM rank 2 for PUSCH

This proposal is to adopt the LTE rank 2 codebook for UEs with 2 and 4 Tx. Alternatively one could consider the use of the NR non-coherent codebook. It is motivated by the following observations:

- Even at cell edge, the channel supports rank 2, due to the use of dual polarization channel (see RP-242599). Hence, there is an untapped capacity potential, even for coverage limited UEs, if they can transmit rank 2 MIMO.

- Our simulation results (RP-242599 and copied below) verify this potential as it shows 10-16% -units gains over Rel.18 fast waveform switching between CP-OFDM and DFT-S-OFDM.

- The impact to gNB implementation is marginal to support rank 2 DFT-s-OFDM

- To minimize the scope, we suggest re-using the LTE rank 2 codebook for DFT-s-OFDM

- This feature applies to non-coherent UEs as well, hence addressing a wide market of the majority of UE implementations

- There is a commercial need as this will extend the uplink rank 2 coverage and thus better end user experience near cell edge

Support Rank 2 DFT-s-OFDM transmission for PUSCH for 2 and 4 TX UEs by reusing LTE codebook or NR non-coherent codebook.

To quantify the relative gains of fast DFT-S/CP-OFDM switching and multi-layer DFT-S-OFDM, we considered 3 scenarios where the two first ones are possible in today’s specifications:

- Rel-15 codebook-based UL MIMO using CP-OFDM,

- Rel-18 fast switching between CP-OFDM and DFT-S-OFDM for rank 1, but CP-OFDM for rank 2

- Proposed Rel-20 DFT-S-OFDM for both rank 1 and rank 2.

Table 1: Mean and cell edge throughput gains for fast switching and multi-layer DFT-S-OFDM: 2Tx, xpol.

|  | Mean Throughput Gain |  |  | Cell Edge Throughput Gain |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 20% | 50% | 70% | 20% | 50% | 70% |
| Rel.15 CP-OFDM | 0% | 0% | 0% | 0% | 0% | 0% |
| Rel.18 Fast-Switching | 4% | 2% | 1% | 17% | 27% | 34% |
| Rel.20 DFT-S-OFDM | 11% | 14% | 17% | 31% | 33% | 34% |

Table 2: Mean and cell edge gains over legacy for fast switching and multi-layer DFT-S-OFDM: 4Tx, directional.

|  | Mean Throughput Gain |  |  | Cell Edge Throughput Gain |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 20% | 50% | 70% | 20% | 50% | 70% |
| Rel.15 CP-OFDM | 0% | 0% | 0% | 0% | 0% | 0% |
| Rel.18 Fast-Switching | 5% | 8% | 9% | 68% | 63% | 59% |
| Rel.20 DFT-S-OFDM | 12% | 16% | 19% | 70% | 78% | 81% |

### Open loop precoder cycling for PUSCH

This feature has quite good support but the evidence among the proponents that this would provide a benefit is weak. Our simulation results in Section 2.3.2 of R1-2500539 doesn’t show any performance benefits, the feature only provides performance degradation at 3 km/h and at 60 km/h, at best, a comparable performance to Rel.15 wideband closed loop precoding. Almost none of the proponents have not yet provided any own simulation evidence. In addition, it has several other drawbacks which are listed below.

- Does not provide any gains in simulations (R1-2500539 and partly copied below)

- Requires significant gNB implementation work since a new per-PRG channel estimator in gNB needs to be implemented

- The use of a large PRG limits scheduling, since allocations must be large enough to cycle precoders and FDM’d UEs should align with PRG boundaries, while a small PRG can degrade channel estimation.

- It’s a scheme that doesn’t work with DFT-s-OFDM waveform

- It is unclear what is included in the proposal. How many Tx? Is it a cycling of precoders from a codebook or are the used precoders UE implementation? What mechanism shall be used at gNB to perform link adaptation, i.e., is a new SRS type needed?  What is the PRG bandwidth? How does introduction of a PRG in uplink impact the interference covariance estimate at the gNB receiver?

For example, the choice of a precoders to cycle was discussed already in LTE, for the PDCCH design for 4 Tx base stations and the use of FSTD technique was chosen due to its attractive features. It uses the precoders [1 0] and [0 1], per subcarrier, hence a simple Tx antenna hopping scheme, to obtain diversity. The choice of the precoder [1 0] and [0 1] has these benefits:

- Wideband DMRS channel estimation can still be used, thereby avoiding the channel estimation losses found in per-PRG precoder cycling and thus avoiding large impact to gNB implementation complexity.

- It can be used for DFT-S-OFDM without harming single carrier properties

- It is compatible with non-coherent UL MIMO transmission, which is the most commercially relevant UL MIMO transmission scheme at present.

- Full power transmission is achieved automatically, since all transmit antennas are used.

Support open loop transmit diversity if and only if gNB wideband channel estimation can be retained, DFT-S-OFDM can be supported, and coherence among transmit chains is not required.

Figure 1 and Figure 2 present the link-level UL throughput comparison between non-coherent Rel.15 CL wideband precoding, OL precoding cycling, and FSTD. Detailed simulation assumptions are given in Table 3 in the Appendix. The analysis considers scenarios where the UE moves at speeds of 3 km/h and 120 km/h, transmitting with a maximum rank of 1. The UE is equipped with 2 full-power non-coherent transmit chains, while the gNB has 4 receive chains. For each figure, the evaluation is done for SRS periodicity of 10ms and 80ms. Accordingly,

- When the UE moves at a pedestrian speed of 3 km/h, OL precoding cycling and FSTD experience a performance loss over a wide SNR range compared to CL wideband precoding (i.e. Rel.15 specification).

  - Specifically, at SNRs of -5, 0, 10 dB, losses are observed for OL precoding cycling and FSTD as -19, -16,-10% and -20, -14,-9%, respectively, for SRS periodicity of 10ms, and -16, -13,-7% and -16, -11,-7%, respectively, for SRS periodicity of 80ms, respectively. This performance gap arises because, in relatively static channels, non-coherent CL precoding can select the optimal transmit chain, whereas OL precoding randomly selects transmit chains per subband, and FSTD blindly transmits on the first transmit chain on odd subcarriers and the second transmit chain on even subcarriers.

- In contrast, at a vehicular UE speed of 120 km/h, OL precoding cycling and FSTD perform almost comparably to CL wideband precoding, regardless of the SRS periodicity. The rapid channel variations at this speed provides a diversity gain for OL precoding cycling and FSTD, enhancing their performance. Specifically, at SNRs of -5,  0, 10 dB, gains are observed for OL precoding cycling and FSTD as 16, 4, 4% and 11, 6, 2%, respectively, for SRS periodicity of 10ms, and 16, 8, 6% and 12, 9, 4%, respectively, for SRS periodicity of 80ms.

Figure 1	  UL throughout versus SNR, comparing non-coherent CL wideband precoding with OL precoding cycling and FSTD for 10ms (left figure) and 80ms (right figure) SRS periodicity with 2 transmit chains and Rel-16 Mode 0 power-scaling. Here, the UEs transmit with max rank 1 and is moving with pedestrian speed, i.e., 3 km/h.

Figure 2 UL throughout versus SNR, comparing non-coherent CL wideband precoding with OL precoding cycling and FSTD for 10ms (left figure) and 80ms (right figure) SRS periodicity with 2 transmit chains and Rel-16 Mode 0 power-scaling. Here, the UEs transmit with max rank 1 and is moving with vehicular speed, i.e., 120 km/h.

FSTD does have the drawback that it has less gain than other schemes at higher code rates. However, our expectation is that the principal benefit of open loop TxD in the scope of NR Rel-20 is that the UE can transmit at full power by combining the power on the antennas in a specified way in scenarios where closed loop MIMO fails, since we do not see performance gains as shown above. Therefore, if it is decided that it should be specified, any design for open loop TxD should focus on minimizing impact to UE and network complexity while maintaining reasonable performance, good PAPR for DFT-S-OFDM, and full power transmission.

Compared to non-coherent closed loop wideband precoding (Rel.15), and when the maximum rank is fixed to 1, precoder cycling and FSTD with two active Tx chains has a loss at pedestrian speeds over a wide range of SNRs and the comparable performance at 120 kmph.

The SRS periodicity does not seem to affect the relative gains of (frequency selective) precoder cycling, FSTD and (wideband) closed loop precoding.

Given their lack of gain over wideband closed loop precoding under conditions favorable to open loop TxD, a primary benefit of open loop TxD schemes seems to be full power transmission.

If open loop transmit diversity is to be specified for NR Rel-20, cycling over transmit chains within a PRB is supported, i.e. FSTD.

### Closed loop (TPMI based) frequency selective precoding for PUSCH

We will not support this enhancement, mainly because two major drawbacks:

- A very large impact to DCI payload, as TPMIs for every subband need to be indicated to the UE. The payload grows with the PUSCH scheduling bandwidth.

- There is a tremendous impact to gNB receiver and channel estimator since PRG is introduced in UL, the benefits of wideband channel estimation is lost, and it is uncertain whether and how (which requirements are needed on UE power and coherency, the codebook etc) to recover from this loss

We have carried out new simulations (See Section 2.3.2 in R1-2500539 and partly copied below) and uplink frequency selective precoding with MMSE-based channel estimation underperforms compared to wideband precoding with (wideband RS based) time-filtering-based channel estimation. This implies a need for exploration and improvement of channel estimation algorithms tailored for frequency selective precoding.

A proper study on DCI redesign vs performance trade-off is needed, and this is a big study suitable as a 6G SI topic. Significant studies are needed to reach an acceptable and workable scheme, on UE RF limitations such as maximum Tx chain power, coherence, or PAPR.

It seems this feature requires coherent codebook UEs, thus addressing a currently very small market segment. A proper study that also considers uplink interference suppressions and its relation to PRG bandwidth is needed.

To summarize, we believe this is a very large topic that spans multiple aspects, DCI, channel estimation, precoder codebook, PAPR etc. We don’t think there is enough TU for 5G-A MIMO Phase 6, instead suggests that it can be thoroughly explored in a 6G SI.

Do not support closed loop (TPMI indication based) frequency selective precoding for PUSCH in 5G-A.

Here follows some more theoretical background and evaluation results for closed loop subband precoders for PUSCH:

The state-of-the-art MMSE CHEST algorithm can be optimized for subband processing, where channel estimation is performed per subband. However, this approach reduces processing gain due to the limited number of subcarriers within each subband, restricting its ability to effectively suppress noise. While increasing the subband size can enhance performance, it comes at the expense of higher computational complexity, proportional to the number of subcarriers.

Moreover, MMSE CHEST assumes white noise, characterized by uncorrelated and uniform power across frequencies. Consequently, it struggles to handle colored noise, which exhibits non-uniform power spectral density and frequency correlations. This limitation becomes particularly significant in system-level scenarios, where interference from neighboring cells or users often appears as colored noise.

In the following, MMSE CHEST is compared to a time-filtering (TF) based CHEST, which is optimized for wideband precoding. By leveraging time-domain processing, TF-based CHEST effectively eliminates both noise and interference. However, TF-based filtering requires further enhancements to support FS precoding effectively, warranting additional studies.

Figure 3	 Mean and cell-edge (5-th percentile) UL throughout versus served traffic, comparing performance of wideband and FS precoding with MMSE CHEST to the performance of wideband precoding with TF-based CHEST. The channel bandwidth is 256 PRBs with subband size of 4 PRBs for FS precoding.

From Figure 3, it is evident that the performance of wideband and FS precoding with MMSE CHEST degrades compared to wideband precoding with TF-based CHEST as the traffic load increases. Specifically, FS precoding with MMSE CHEST experiences a performance loss of 10% in mean throughput and 34% in cell-edge throughput at 50% traffic load, relative to wideband precoding with TF-based CHEST.

This superior performance of TF-based CHEST is attributed to its ability to effectively suppress noise and interference, particularly at the cell edge. Consequently, the significant advantage of wideband precoding with TF-based CHEST over FS precoding with MMSE CHEST underscores the importance of continued study into CHEST algorithms optimized for FS precoding.

### Codebook extension for PUSCH

This enhancement aims to take advantage of unused DCI codepoints for TMPI indication to extend the QPSK precoder alphabet for 2Tx UEs to 8-PSK. We have not evaluated to verify the claimed benefits, especially under the rather large allowed phase error between antenna branches from RAN4 requirements. We are open to support this very limited scope feature if some benefits can be shown, although we must question the urgent commercial need for this feature.

Proponents of 2Tx codebook extension to explain more of the benefits and urgent need

## SRS enhancements

### Cross slot SRS transmissions

We support cross-slot span of an SRS resource, there is an urgent and commercial demand for this as some operators have problems utilizing the S slots for SRS as there is only two SRS symbols available. With this enhancement (which basically consists of removing a restriction in spec that an SRS resource can only map to one slot), the first symbol or few symbols of the subsequent slot can be used for SRS, which allows the UE to maintain phase coherence is it is a continuous transmission.

We support cross slot transmission of an SRS resource as it addresses an urgent commercial need and it has a very small 3GPP TU footprint.

### Enhancements to partial SRS

To us, the proposal seems reasonable, but it is uncertain it will ever by implemented by anyone. The urgency is unclear.

We are open to discuss enhancements to partial SRS more as a Rel.20 candidate but whether it ever will be used in 5G-A is uncertain.

## CSI enhancements

### Interference plus noise reporting for reciprocity (TDD)

The motivation for this enhancement is to make MU-MIMO operation using channel reciprocity more efficient and simplify operation, remove the need for estimation and replace this with a measurement. It is a request from our product side.

Today, the channel for co-scheduled UEs is obtained using reciprocity from the SRS, but the interference plus noise (IpN) level at the UE is initially unknown to the gNB. To compute the MU-MIMO precoders for each co-scheduled UE, the IpN, or even better, the spatial interference covariance matrix, is needed in the precoder calculation and to estimate the rank, MCS selection.

This forces the gNB to configure another full CSI report, such as a Type I CSI report, to obtain a CQI, which then can be processed to estimate the IpN as experienced for each UE.

The proposal is to configure the UE with a NZP CSI-RS, with 1 or 2 ports as an CMR, and an associated IMR, and then the UE is configured to report (as a stand alone report) a per subband IpN which is an L1-RSRP normalized with the  L1-SINR measured by the UE (reported per subband). This is then reflecting the interference plus noise ratio experienced by the UE. We believe this report has to have a high dynamic range (e.g. 8 bit) and a suitable defined range. With this report, the UE has everything it need for link adaptation and precoding, for both SU, MU-MIMO scheduling and for both single- TRP and multi-TRP.

We think reporting of spatial covariance is also interesting but to limit the scope and TU footprint (and toe maximize the likelihood this is implemented in 5G-A), we think a scalar value to be reported is suitable for Rel.20 MIMO phase 6.

Support introducing a standalone CSI report (similar to Rel.18 TDCP) as a scalar IpN report of a high dynamic range measure of the interference plus noise level measured by the UE. The measurement is relative to a DL RS.

### Early CSI-RS / SRS configuration and reporting

It has been an issue within carrier aggregation that activation of SCells takes time and due to the bursty small packet nature of typical eMBB traffic, the activation and use of SCells must be very fast in order to make use of the SCell. Typically, the scheduler uses the PCell to start transmitting the packets and many times it even completes the delivery before the SCell is ready to be used.

Now, the activation time has been reduced in Release 17 and we believe that the next step of this enhancement is to include fast CSI acquisition so that the activated SCell can also utilize MIMO transmissions directly. This would make carrier aggregation very useful also for smaller and bursty traffic, which is very common in reality.

To Ericsson, the early CSI reporting measured on an early CSI-RS for SCells is important but also similar enhancements for IDLE mode UEs are also important. To support higher MCS and MIMO, it can also be discussed whether this should include an early TRS trigger, to allow the UE to perform fine sync.

Another enhancement for the same purpose is the early SRS configuration and triggering, to be used for the case where there is a paired UL carrier to the DL carrier (which is not the typical case for all DL carriers in TDD since a UE supports more DL carriers than UL carriers).

Support Early CSI-RS, SRS and possibly also early aperiodic TRS resource triggering and early CSI reporting to enable MIMO as fast as possible after SCell activation and after IDLE CONNECTED and INACTIVE CONNECTED state change.

Figure 4 shows the reduction in number of slots needed to receive an FTP packet of size of 25 kilobytes with FTP model 1 for the following schemes:

- Early CSI Feedback:  case where single port CSI feedback with early CQI reporting which is taken into account by gNB for scheduling PDSCH

- Initial MCS = 0:  case where there is no early CSI and the gNB uses the most conservative MCS initially for scheduling PDSCH

- Initial MCS = 26:  case where there is no early CSI and the gNB uses the most aggressive MCS initially for scheduling PDSCH

Among the 2 last schemes, ‘Initial MCS=0’ is the most practical as the gNB will likely schedule PDSCH with an initial MCS that is most conservative in the absence of early CSI.  As can be seen from Figure 4, early CSI feedback can significantly reduce the number of slots needed to receive a packet when compared to other schemes where early CSI is not available.

Figure 4. Reduction in slots needed to receive a packet with early CSI

Figure 5 shows performance comparison between early CSI with different number of ports.  Here, PDSCH scheduling is according to RI/PMI/CQI reported as part of the respective early CSI reports.  As can be seen from the figure, notable performance improvement can be achieved when increasing the number of ports in the early CSI.

Figure 5. Performance comparison of early CSI with different number of ports

### CSI-RS overhead reduction

In Rel.19, up to 128 CSI-RS ports is supported and it’s getting near the pain point where CSI-RS overhead is becoming a real issue. There is a proposal to use frequency domain CSI-RS overhead reduction techniques which are straight forward, simple by extending the density values to lower than ½ that is the minimum supported today. This seems like a small specification change and should easily be adopted.

There is also a proposal to consider spatial domain overhead reduction, for example transmitting a subset of configured CSI-RS ports and let the UE use advanced receiver processing to recover of absent ports. This may require larger studies and specification impact and also the need for new receiver algorithms in the UE. We are not fully negative to this but considering the needed workload and the need for study phase, it is in our view better suited for 6G SI.

Support CSI-RS overhead reduction in frequency domain. Do not support  CSI-RS overhead reduction in spatial domain.

## FR2 enhancements

### UEI/ED beam switching based on Rel-19 UEI/ED beam reporting

We are ok to consider leftovers from Rel.19. A clarification is needed (add NW -controlled?) whether this is UE controlled beam management or whether this is UEI beam reporting only. If the former we are against, otherwise it is ok.

We are open to support completion of leftovers from Rel.19 work on the UEI/ED beam switching based on Rel-19 UEI/ED beam reporting topic and limit this to UE initiated beam reporting only.

### Asymmetric DL sTRP/UL mTRP enhancements in FR2

In our understanding, there are two issues addressed in this topic:

- Issue1:  PDCCH order PRACH to UL TRP is supported in R18 (i.e. apply PL-offset to PDCCH order PRACH) for TA acquisition, but transmit beam is up to UE implementation. It is our understanding that this is to specify “tx beam of PDCCH order PRACH is the indicated TCI state”. Then, gNB can configure SRS resource as QCL source of the indicated TCI state by gNB implementation

- Issue2: In case of SRS beam sweeping (i.e. SRS BM without TCI state configuration), gNB cannot determine which Rx beam to be used, because gNB has no idea which tx beam is used for each SRS tx occasion. (this is general issue of SRS beam sweeping). The solution is to introduce repetition to SRS, similar as for CSI-RS. Then, gNB can determine appropriate rx beam when gNB measures multiple SRS.

We are open to support the solutions of the two asymmetric DL sTRP/UL mTRP enhancements in FR2 if the scope is small and well defined.

### MAC CE based QCL updates for source TRS

The QCL source for the PDCCH/PDSCH reception is the TRS, and since a TRS always is associated with an SSB via QCL association (‘QCL-Type-C’ and ‘QCL-TypeD’ where applicable), it is preferred to configure a TRP at FR2 with a one-to-one mapping between an SSB and a TRS (i.e., for each transmitted SSB, a TRS is transmitted in the same beam as the SSB).

However, for TRPs at mmWave frequencies with many beams and carriers, it is not possible to configure the UE with one TRS per SSB per carrier due to that UEs support a limited number of TRSs across all configured carriers (indicated by maxConfiguredResourceSetsAllCC in 38.306).

This means that when a UE moves to a new SSB beam which is not associated with an TRS (for one or more of the DL carriers), the TRP needs to update the QCL of a TRSs such that it is associated with the new SSB. The QCL update of the TRS requires RRC signaling, which introduces additional latency and overhead signaling in the system.

To mitigate this issue, it would be beneficial to introduce a more dynamic QCL source indication for TRSs, hence we propose:

Support introduction of MAC CE based switching for QCL source of TRS.

In practice, this would imply that we allow that a semi-persistent TRS is used as QCL source for PDCCH/PDSCH DMRS. The UE would use this semi-persistent TRS also for radio link monitoring and beam failure detection. As the QCL source for semi-persistent CSI-RS can be updated using MAC CE, RRC reconfigurations are avoided.

# Proposals

Based on the above we propose for Rel.20 MIMO

Proposal 1	Support Rank 2 DFT-s-OFDM transmission for PUSCH for 2 and 4 TX UEs by reusing LTE codebook or NR non-coherent codebook.

Proposal 2	Support open loop transmit diversity if and only if gNB wideband channel estimation can be retained, DFT-S-OFDM can be supported, and coherence among transmit chains is not required.

Proposal 3	If open loop transmit diversity is to be specified for NR Rel-20, cycling over transmit chains within a PRB is supported, i.e. FSTD.

Proposal 4	Do not support closed loop (TPMI indication based) frequency selective precoding for PUSCH in 5G-A.

Proposal 5	Proponents of 2Tx codebook extension to explain more of the benefits and urgent need

Proposal 6	We support cross slot transmission of an SRS resource as it addresses an urgent commercial need and it has a very small 3GPP TU footprint.

Proposal 7	We are open to discuss enhancements to partial SRS more as a Rel.20 candidate but whether it ever will be used in 5G-A is uncertain.

Proposal 8	Support introducing a standalone CSI report (similar to Rel.18 TDCP) as a scalar IpN report of a high dynamic range measure of the interference plus noise level measured by the UE. The measurement is relative to a DL RS.

Proposal 9	Support Early CSI-RS, SRS and possibly also early aperiodic TRS resource triggering and early CSI reporting to enable MIMO as fast as possible after SCell activation and after IDLE CONNECTED and INACTIVE CONNECTED state change.

Proposal 10	Support CSI-RS overhead reduction in frequency domain. Do not support  CSI-RS overhead reduction in spatial domain.

Proposal 11	We are open to support completion of leftovers from Rel.19 work on the UEI/ED beam switching based on Rel-19 UEI/ED beam reporting topic and limit this to UE initiated beam reporting only.

Proposal 12	We are open to support the solutions of the two asymmetric DL sTRP/UL mTRP enhancements in FR2 if the scope is small and well defined.

Proposal 13	Support introduction of MAC CE based switching for QCL source of TRS.

# Appendix

Table 3: Parameters for UL link level simulations for open-loop precoding cycling and FSTD

| Link level simulation parameters |  |
| --- | --- |
| Metric | UL throughput |
| TDD pattern | DDDDU |
| Carrier frequency | 3.5 GHz |
| Subcarrier spacing | 30 kHz |
| Bandwidth | 52 PRBs (~ 20 MHz) |
| Channel model | CDL-C with 300 ns delay spread |
| Power mode | Rel-16 Mode 0 |
| SRS periodicity | 10ms or 80ms |
| Modulation | Up to 256 QAM |
| Channel estimation | Practical SRS and DMRS CHEST |
| Link adaptation | Practical |
| Receiver | MMSE-IRC |
| BS antenna configuration | (M,N,P,Mg,Ng,Mp,Np) = (8,2,2,1,1,1,2) with (dH, dV) = (4.0, 0.8)λ, tilt: 10° |
| BS antenna pattern | Directional (8 dBi, 65° BW) |
| BS noise figure | 5 dB |
| UE antenna configuration | 1 dual pol with isotropic antenna element |
| UE coherence | Non-coherent |
| Max rank | 1 |
| UE speed | 3 km/h or 120km/h |
| Subband granularity | 4 PRBs per subband (for precoding cycling) |