3GPP TSG RAN meeting #107	RP-250106

Incheon, South Korea, March 12-14, 2025

## Status Report to TSG

Agenda item:			9.3.2.3

| WI / SI Name | XR (eXtended Reality) for NR Phase 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_XR_Ph3 |  |  |  |  |
| Unique ID | 1020098 |  |  |  |  |
| TSG TDoc of latest approved WI/SI description (if any) | RP-243318 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 2025/09 | Performance part: 2026/03 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 67% | Performance Part: 0% | Testing part: N/A |  |

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Benoist Sébire |
|  | Company | Nokia |
|  | Email | benoist.sebire@nokia.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

### 2.1.1	Discussions and Agreements

#### 2.1.1.1	RAN1#120

During RAN1#120, the following was agreed:

Confirm the Working assumption in RAN1#118 for supporting Alt. 1-1:

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

Agreement from RAN1#119 is updated (red part)

For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3.

- Explicit indication can be configured for each DCI format individually by higher layer.

- The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field)

Above applies only for intra-band CA cases (same SCS for cells in the set of co-scheduled cells).

- Alt1: If skipping field is configured to DCI format 0_3/1_3, only cells that are intra-band (and have same SCS) can be co-scheduled. In other words, if co-scheduled cells could be inter-band, skipping indication field cannot be configured.

Alt2: When measurement gap/restriction is indicated to be skipped by explicit indication by DCI format 1_1/1_2 that does not schedule PDSCH and

- if CIF field is present, the scheduled cell for the skipping indication is determined by CIF field.

- if CIF field is not present, the scheduled cell for the skipping indication is the scheduling cell.

Following conclusion was made in RAN1#120:

Conclusion

It is RAN1 understanding that if measurement gap/restriction is indicated to be skipped by explicit indication by DCI, the TX/RX is enabled on all the serving cells in the applicable range of the skipped measurement gap/restriction.

- Note: Applicable range of measurement gap/restriction is based on RAN4 specifications

### 2.1.2	Main Open issues

The one RAN1-related objective (related to RRM measurements) remains open.

## 2.2	RAN2

### 2.2.1	Discussions and Agreements

#### 2.2.1.1	RAN2#129

Agreements for RRM measurement gap skipping:

1.	From MAC perspective, the UE behaves as if there is no activated measurement gap during a skipped/cancelled measurement gap occasion.

2.	“Cancelled” or “skipped” terminology will be aligned with RAN1 specifications when implementing changes in MAC.RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements.

3.	RAN2 will not work on semi-static MG skipping solutions unless requested by RAN4/RAN1.

Agreements on LCP enhancements:

1.	Only one additional priority is configured to an LCH for LCP enhancement.

2.	We keep an existing agreement (remaining time threshold is configured per LCH)

3.	There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority.

4.	Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact

5.	FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination

6.	No additional PBR is needed for priority adjusted data

7.	FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

Agreements on DSR enhancements:

1.	1.	One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE.

2.	FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds.

3.	We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size.

4.	Different LCGs may be configured with different number of reporting thresholds.

5.	If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold.

6.	Triggering threshold is not used as a reporting threshold (but one of reporting thresholds can be configured to the same value as triggering threshold).

7.	Do not support a configuration of an LCG without any triggering threshold but with DSR reporting threshold(s).

Agreements on RLC timely retransmissions:

1.	Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC

2.	Only a single autonomous retransmission will be triggered per RLC SDU.

3.	There is no dynamic activation/deactivation of the autonomous retransmission mechanism.

4.	We have separate thresholds for autonomous reTx and for polling

Agreements on avoiding unnecessary retransmissions:

5.	When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment.

6.	A new RLC timer at the TX is not introduced to determine outdated RLC SDUs.

7.	The new RLC timer at the RX is per RLC entity

8.	The duration of the new RLC timer is not lower than that of t-reassembly

9.	Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

Agreements on XR rate control

1.	RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2.

We may revisit UAI option based on SA2/RAN3 reply

2.	Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc.

3.	We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed

4.	Working assumption:

a.	Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE.

b.	The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy).

Companies to check with their SA4 colleagues whether there are any issues with this.

In addition, the first version of the Stage 3 CRs were provided and will be reviewed in POST email discussions:

-	38.321 in R2-2500090;

-	38.300 in R2-2500489;

-	38.331 in R2-2501246;

-	38.323 in R2-2501197;

-	38.322 in R2-2501205.

### 2.2.1	Main Open Issues

XR Rate Control.

## 2.3	RAN3

#### 2.3.1	Discussions and Agreements

#### 2.3.1.1	RAN3#127

RAN3 discussed the RAN2 LS on UL bit rate control. RAN3 agreed both options have an impact on the F1 interface. RAN3 will further discuss whether gNB-DU can support per QoS flow data bit rate control when TU is available. Reply LS to RAN2 agreed in R3-250805.

RAN3 discussed the SA2 LS on available data rate report. RAN3 agrees that the available data rate reported to CN refers to the data rate that gNB is currently able to provide for a QoS Flow with GFBR, and it is up to gNB's implementation to derive the value of available data rate. RAN3 would like to clarify that the available data rate is not able to directly reflect the gNB resources properly, as there are some other factors (e.g., the channel status) besides the resource conditions that may impact the available data rate. Regarding whether the available data rate reporting can be used together with the Alternative QoS Profiles mechanism, RAN3’s understanding is that this should be up to CN configuration. RAN3 assumes that the SMF will not request Alternative QoS Profiles together with available data rate reporting for one QoS flow. Reply LS to SA2 agreed in R3-250806.

#### 2.3.2	Main Open issues

The Stage-2/3 TP to support the SA2 decision on MMSID, PDU Set Information Marking Support, etc., were not discussed yet.

## 2.4	RAN4

#### 2.4.1	Discussions and Agreements

#### 2.4.1.2	RAN4#114

Way forward on RRM requirements for XR_Ph3 agreed in (R4-2502700):

For deployment scenarios:

-	Deprioritize FR2-2 for enabling transmission/reception in gaps/restrictions that are caused by RRM measurements

For L3 measurements without gaps with scheduling restrictions and L1 measurements with scheduling restrictions:

-	During the skipped measurement gap occasion, the UE is not required to perform measurements (i.e. L3 measurements and L1 measurements), and the UE shall not cause scheduling restrictions.

-	Outside a gap occasion, skipping of L3 Measurements without gaps with scheduling restrictions is not supported.

-	Outside a gap occasion, skipping of L1 Measurements with scheduling restrictions is not supported.

Regarding the impact of XR skipping on RRM requirements, the following principle for measurement delay calculation is agreed:

-	Follow NR-U-like approach for designing the formulas for measurement delay extension.

Regarding UE assistance information, the following is agreed:

-	Agree to define UAI based on the following assumptions, and further discuss the content of UAI

-	RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective)

-	Not define UE behaviour and the corresponding requirements based on UAI

-	Supporting of UAI is optional for UE with optional UE capability.

Work split is agreed in R4-2502700 and the first CRs are expected in RAN4#114bis.

#### 2.4.2	Main Open issues

The one RAN4-related objective (related to RRM measurements) remains open. See agreed WF in R4-2502700.