3GPP TSG RAN meeting #107									   RP-250113

Incheon, Korea, Mar 12th – 14th, 2025

Status Report to TSG

Agenda item:	9.3.3.1

| WI / SI Name | Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_ENDC_SON_MDT_Ph4-Core |  |  |  |  |
| Unique ID | 1021092 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-234038 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2025 | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 60% | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG3 |
| --- | --- | --- |
| Rapporteur | Name | Xufei Dong |
|  | Company | China Unicom |
|  | Email | dongxf37@chinaunicom.cn |
|  | Name | Miaoqi Zhang |
|  | Company | CMCC |
|  | Email | zhangmiaoqi@chinamobile.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2#129 (Feb 2025)

- Running CRs

[3] (R2-2500918) is to be used as baseline in a post meeting email discussion.

- MRO enhancements for Rel-18 mobility features

  - MRO for LTM

-	Send an LS to RAN3 indicating scenario of beam failure recovery shortly after successful LTM (“near failure”), also say that we will not specify a UE based solution for this, but that RAN3 can consider a NW based solution if they want. RAN3 can see if/how the solution (if specified) could also be used for the case of LTM failure. The LS [57] on SON for LTM is approved.

  - MRO for CHO with candidate SCG(s)

-	Enhance RLF report for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any.

-	Enhance RLF report for CHO with candidate SCGs to include the associations between CHO and CPAC.

-	Enhance RLF report for CHO with candidate SCGs to include at least the following information:

-	Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered;

-	Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered;

-	The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

-	Enhance SCGFailureInformation for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any.

-	We should avoid specifying the procedure in a way that the UE sends redundant information.

  - MRO for S-CPAC

None.

- SON MDT for Slicing

None.

- SON MDT for NTN

-	Existing IEs for CHO are used to report the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”.

- Leftovers from Rel-18

  - SDT

-	Include cell re-selection as a failure cause for failed RA-SDT in RA-Report

-	Introduce a new field in RA-Report to indicate the elapsed time since the execution of RA-SDT. Value in seconds. The maximum value is 172800 seconds.

  - Other

-	We will introduce a UE based solution for SCG act/deact MHI, as an optional UE feature.

-	UE reports the time the UE has spent in PSCell with SCG activated, TBD details, e.g. percentage of time or absolute time, etc.

2.2.2	Remaining Open issues

-	Details on time information regarding condition fulfilment for CHO with candidate SCGs are FFS.

-	If RA-based LTM failure happens, whether the UE logs and reports additional info in the RLF report is TBD.

-	UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS.

-	Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

2.3	RAN3

2.3.1	Agreements

RAN3#127(Feb 2025)

- MRO enhancements

  - MRO for LTM

- CU can adjust the candidate Cell list for LTM based on UHI.

- DU should be able to resolve LTM ping-pong based on information over F1.

- Network based solution is used for LTM candidate cell list.

  - MRO for CHO with candidate SCG(s)

- In case of too late CHO execution, reusing Handover Report message from the last serving MN to the relevant candidate MN.

- New message is defined for the serving MN to forward the SCGFailureInformation to the respective MN which should perform the optimization.

  - MRO for S-CPAC

- The forwarding mechanism in BLCR for TS37.340 need to be refined.

- SON/MDT for Intra-NTN Mobility

- Existing timers can be reused to identify when RLF happens with 100ms accuracy.

- No need to identify whether RLF happens before or after T1 for time-based CHO.

- Revert the WA: There is no need to report any other info in the RLF report for the location based CHO.

- For earth moving case, the accuracy and complexity issue of the network-based solution is acknowledged.

- SON/MDT for Network Slicing

- The UE is unable to find suitable cell in the frequency corresponding to the prioritized NSAG, UE falls back to legacy Cell reselection procedure.

- R18 Leftovers

- A TP that adds SCG Failure Information Report and SCG Failure Transfer in X2AP was agreed in [64]

- LS Out

- LS to RAN2 on SON for Network Slicing in[70]

2.3.2	Remaining Open issues

- MRO will cover the scenario that RACH-less LTM fails including outdate TA and UE performs RACH based LTM?

- Network based or UE based solution for TA acquisition type?

- FFS on whether Handover Report message is needed for case 7a.

- FFS on the additional information included in handover report.

- When to send the message? Immediately or not?

- Check whether respective MN is only a candidate?

- In addition to SCGFailureInformation, information needed in the new message?

- -CPC failure type

- Will TA acquisition type always be available in the CU or is there a need for the DU to store and retrieve (partial) UE Context?

- FFS whether more accuracy is needed.

- FFS on option 1 or option 2.

- -Option 1: to introduce the absolute UTC time of when RLF/HOF happens in the RLF report.

- -Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell (when RLF occurred).

- MRO mechanism for intra-NTN mobility shall be enhanced to enable handling of local and temporary propagation changes (e.g. due to clouds, rain etc.)?

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2#129

- R2-2500025	Reply LS on MHI enhancement solution for SCG deactivation/activation (R3-247906; contact: Huawei)	RAN3	LS in	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	To:RAN2

- R2-2500026	LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT (R3-247908; contact: Samsung)	RAN3	LS in	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	To:RAN2

- R2-2500918	Running CR for SONMDT features	Ericsson	draftCR	Rel-19	38.331	18.4.0	B	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500234	MRO Enhancements for LTM	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500363	MRO for LTM	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500419	Discussion on random access report for LTM	ASUSTeK	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500558	Outstanding issues of MRO	Apple	discussion

- R2-2500622	Discussion on MRO enhancements for LTM	Lenovo	discussion	Rel-19

- R2-2500706	Discussion on MRO enhancement for LTM	China Unicom	discussion	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500831	MRO for LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500859	MRO enhancements for LTM	Samsung	discussion

- R2-2500916	Further considerations on MRO for LTM	Ericsson	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2501044	MRO enhancements for LTM	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2501104	Discussion on MRO for LTM	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2501212	MRO for LTM	LG Electronics Inc.	discussion	Rel-19

- R2-2501255	SON and MDT for LTM	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500235	MRO Enhancements for CHO with Candidate SCGs	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500364	MRO for CHO with candidate SCG	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500623	Discussion on MRO enhancements for CHO with candidate SCGs	Lenovo	discussion	Rel-19

- R2-2500674	MRO for CHO with candidate SCG(s)	NEC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500707	Discussion on MRO enhancement for CHO with candidate SCGs	China Unicom	discussion	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500832	MRO for CHO with candidate SCGs	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500855	MRO enhancements for CHO with Candidate SCG(s)	Samsung	discussion

- R2-2501084	Discussion on new triggering condition of SHR for CHO with candidate SCGs	vivo	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2501105	Discussion on MRO for CHO with candidate SCGs	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2501200	SON enhancement for CHO with candidate SCG	SHARP Corporation	discussion

- R2-2501213	MRO for CHO and Associated CPAC	LG Electronics Inc.	discussion	Rel-19

- R2-2501256	SON and MDT for CHO with candidate SCGs	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

- R2-2501285	SON support for CHO with Candidate SCG	Ericsson	discussion

- R2-2500236	Consideration on SONMDT enhancements for intra-NTN mobility	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500571	Discussion on SON of intra-NTN mobility	China Telecom	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500624	Discussion on MRO for intra-NTN mobility	Lenovo	discussion	Rel-19

- R2-2500708	Discussion on SONMDT for NTN mobility	China Unicom	discussion	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500833	MRO and MDT for NTN	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500861	SON/MDT for NTN	Samsung	discussion

- R2-2500917	Discussion on the reply LS to RAN3 on SON-MDT enhancements for NTN	Ericsson	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500978	Area Scope for the logged MDT for NTN	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2501046	SON/MDT enhancements for NTN	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2501106	Discussion on SONMDT for NTN	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2501214	NTN Logging for Unchanged PCI Mobility	LG Electronics Inc.	discussion	Rel-19

- R2-2501257	SON and MDT for NTN	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500365	MHI/UHI Enhancement for SCG Deactivation/Activation	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500834	Rel-18 leftovers and LS from RAN3	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2501199	RA report optimization for SDT	SHARP Corporation	discussion

- R2-2501289	On Rel.18 leftovers	Ericsson	discussion

- R2-2500625	Discussion on RACH optimization for SDT	Lenovo	discussion	Rel-19

- R2-2501085	RA report optimization for SDT	vivo	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2501107	Discussion on leftovers from Rel-18	Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2501258	SON and MDT for SDT	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500237	MHI Enhancement for SCG Activation and Deactivation	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2500867	MHI Enhancement for SCG Deactivation/Activation	Samsung	discussion

- R2-2501045	MHI and UHI Enhancement for SCG Deactivation Activation	CMCC, CATT, Ericsson, Huawei, Lenovo, ZTE	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

- R2-2501047	[Draft] Reply LS on MHI enhancement solution for SCG deactivation/activation	CMCC	LS out	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	To:RAN3

- R2-2501108	Draft Reply LS on MHI enhancement solution for SCG deactivation/activation	Huawei, HiSilicon	LS out	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	To:RAN3

- R2-2501354	[Post129][601][SONMDT] Running NR RRC CR (Ericsson)

- R2-2501355	[Post129][602][SONMDT] Running LTE RRC CR (Huawei)

- R2-2501378	LS on SON for LTM	RAN2

RAN3#127

- R3-250033	(BL CR to 36.300 for SON) Addition of SON enhancements	Lenovo

- R3-250034	(BL CR to 36.423) Addition of SON enhancements	CATT, Samsung, Lenovo, Huawei, Ericsson, ZTE, CMCC, Nokia

- R3-250035	(BL CR to 37.340 for SON) on MRO for S-CPAC	CMCC

- R3-250036	(BL CR to 38.413 for MDT)  Addition of MDT enhancements	Nokia

- R3-250037	(BL CR to 38.423 for MDT) Addition of MDT enhancements	CATT

- R3-250038	(BL CR to 38.423 for SON) Addition of SON enhancements	ZTE Corporation, Samsung, Nokia, Lenovo, Jio

- R3-250039	(BL CR to 38.473 for SON) Addition of SON enhancements	Huawei, Nokia, Jio

- R3-250040	(BL CR to 38.300 for SON) Addition of SON enhancements	China Unicom, Nokia, Huawei, Lenovo, CATT, Ericsson, Qualcomm Inc., Samsung, LG Electronics Inc, ZTE Corporation

- R3-250041	(BL CR to 38.401 for SON) Addition of SON enhancements	ZTE Corporation, Nokia, Huawei, Lenovo, CATT, Ericsson, Samsung

- R3-250076	[TP for SON BL CR for TS 37.340] MRO Enhancements for optimisation of the S-CPAC selection	Nokia

- R3-250077	[TP to BL CR to 38.423, SON] Solution to avoid restarting MRO in NTN deployments	Nokia

- R3-250078	[TP to BL CR to 38.300, MDT] MDT solution for slice support and slice-related mobility enhancements	Nokia

- R3-250084	(TP for SON BL CR for TS 38.473, TP for SON BL CR for TS 37.340, TP for SON BL CR for TS 38.300) MRO Enhancements for LTM and CHO with Candidate SCG(s)	Nokia

- R3-250098	MRO enhancements for R18 mobility mechanisms	Qualcomm Incorporated

- R3-250099	SON MDT for NTN	Qualcomm Incorporated

- R3-250100	SON MDT for network slicing	Qualcomm Incorporated

- R3-250167	(TP to BLCR of 38.473) Discussion on MRO for LTM	NEC

- R3-250180	Discussion on SON/MDT enhancements for NTN	NEC

- R3-250239	(TP for SON BL CR for TS 38.473) Discussion on LTM MRO	Google

- R3-250246	(TP for BLCR for SON for 38.300, 38.401,38.473, 38.413, 37.340, 38.423) MRO enhancement for R18 mobility mechanisms	Huawei

- R3-250247	SON and MDT for Network Slicing	Huawei

- R3-250248	(TP for BLCR for SON for 38.413, 36.423) Rel-18 SON and MDT leftovers	Huawei

- R3-250269	(TP for SON BLCR for 38.473) Discussion on MRO for LTM	Lenovo

- R3-250270	(TP for SON BLCR for 37.340) Discussion on MRO for subsequent CPAC and CHO with candidate SCG(s)	Lenovo

- R3-250271	Discussion on MRO for intra-NTN mobility	Lenovo

- R3-250272	Discussion on MRO for R18 leftovers	Lenovo

- R3-250300	(TP for MDT BLCRs for TS38.413, TS38.423, TS37.320): SON for Intra-NTN mobility	Huawei

- R3-250325	(TP for 37.340, 38.473, 38.300 and 38.401) MRO for R18 mobility mechanisms	CATT

- R3-250326	(TP for 38.300 and 38.413) Intra-NTN mobility for SONMDT	CATT

- R3-250327	Network slicing for SONMDT	CATT

- R3-250328	(TP for 37.340, 38.423, 36.423 and 38.413) SON enhancement for R18 leftover	CATT

- R3-250407	(TP for BL CR to 38.473 for SON) MRO enhancements for LTM	Ericsson

- R3-250408	SON-MDT enhancements for NTN	Ericsson

- R3-250409	On MDT enhancements for Network Slicing	Ericsson, Jio Platforms (JPL), Interdigital, FiberCop

- R3-250410	(TP for BL CR to 38.413 for MDT) Addition of MDT enhancements [TP on deferred MDT]	Ericsson, Jio Platforms (JPL), Interdigital, FiberCop

- R3-250411	On SON enhancements for Network Slicing	Ericsson

- R3-250412	(TP for BL CR to 38.413 for SON) UHI for SCG activation and deactivation	Ericsson

- R3-250499	(TP to BL CR for 38.300) Further discussion on SONMDT enhancement for NTN	ZTE Corporation

- R3-250500	Further discussion on Rel-18 leftovers	ZTE Corporation

- R3-250530	Discussion on Enhancements in SON/MDT Data Collection and Mobility Robustness Optimization (MRO)	Jio Platforms (JPL)

- R3-250606	(TP to BL CR TS38.401) MRO for LTM	Samsung

- R3-250610	(TP for SON BLCR TS38.300, TS37.340 and TS38.423) MRO for CHO with candidate SCGs and S-CPAC	Samsung

- R3-250611	Failure scenarios on MRO for CHO with candidate SCGs	Samsung, Cybercore, Lenovo

- R3-250612	(TP for SON BLCR for 38.423, 36.423 and TS37.340) Rel-18 SON and MDT leftovers	Samsung

- R3-250613	Discussion on MDT for Network Slicing	Samsung, Lenovo, Cybercore

- R3-250626	(TP for MDT BLCRs for TS38.413, TS38.423) SON enhancements for NTN	Harman GmbH

- R3-250666	Discussion on SONMDT enhancement for NTN	CMCC, Huawei

- R3-250683	Discussion on SON/MDT for network slicing	CMCC

- R3-250724	(TP for 38.401 38.473 38.300)Discussion on MRO	ZTE Corporation

- R3-250725	Further consideration on SON/MDT for Slicing	ZTE Corporation

- R3-250733	Workplan for Rel-19 SON_MDT Enhancement	China Unicom, CMCC

- R3-250734	Discussion on MRO Enhancements	China Unicom

- R3-250735	Discussion on SONMDT enhancements for NTN mobility	China Unicom

- R3-250736	draft CR for TS 38.413 MDT enhancement for NTN	China Unicom

- R3-250737	Discussion on SONMDT enhancements for network slicing	China Unicom

- R3-250738	(TP for MDT BLCRs for TS38.413, TS38.423) SON enhancements for NTN	Samsung

- R3-250755	Reply LS on SON for Network Slicing and enhanced area scope	SA5(Ericsson)

- R3-250823	SoD of MRO for LTM	Ericsson

- R3-250824	SoD of MRO for others	Samsung

- R3-250825	SoD of SONMDT NTN related	Huawei

- R3-250826	SoD of SONMDT slicing related	ZTE

- R3-250827	SoD of SONMDT leftovers	CATT

- R3-250850	(TP for SON BLCR for 38.423) MRO for MR-DC SCG failure	Samsung

- R3-250851	(TP for SON BLCR for 36.423) MRO for MR-DC SCG failure	Samsung

- R3-250853	(TP for SON BL CR for TS38.423) MRO for CHO with Candidate SCG(s)	Samsung, Nokia, Ericsson, Lenovo, CATT, ZTE

- R3-250854	(TP for SON BL CR for TS 37.340) MRO enhancement for CHO with Candidate SCG(s)	Nokia, Samsung, CATT, ZTE

- R3-250855	(TP for SON BL CR for TS38.300) MRO for CHO with Candidate SCG(s)	ZTE Corporation, Samsung, Nokia, Lenovo

- R3-250856	(TP for SON BL CR for TS38.300) MRO for S-CPAC	Samsung

- R3-250859	CB:#SONMDT1_LTM	Ericsson

- R3-250869	LS on SON for Network Slicing	RAN3(ZTE)

- R3-250900	(TP for SON BLCR TS37.340) MRO for S-CPAC	Samsung, Lenovo, CATT

- R3-250914	LS on SON for Network Slicing	RAN3(ZTE)

- R3-250926	(BL CR to 36.423 for SON) Addition of SON enhancements	CATT, Samsung, Lenovo, Huawei, Ericsson, ZTE, CMCC, Nokia

- R3-250927	(BL CR to 37.340 for SON) on MRO for S-CPAC	CMCC

- R3-250928	(BL CR to 38.300 for SON) Addition of SON enhancements	China Unicom, Nokia, Huawei, Lenovo, CATT, Ericsson, Qualcomm Inc., Samsung, LG Electronics Inc, ZTE Corporation

- R3-250929	(BL CR to 38.423 for SON) Addition of SON enhancements	ZTE Corporation, Samsung, Nokia, Lenovo, Jio

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template