3GPP TSG RAN Meeting #107						RP-250122

Incheon, Korea, 12 - 14 March 2025

Agenda item:		15.2.5	IoT NTN (RAN2)

Title:		NTN for IoT Phase 4

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

Following the Rel-20 5G-Advanced workshop at RAN#106, the endorsed RAN Chair’s Summary [1] identifies IMS Voice over GEO in IoT NTN for FDD as a candidate work item for Rel-20 5G-Advanced (see Annex). The last RAN1 meeting concluded that RAN1 did not reach a consensus on whether OCC for NPRACH is beneficial or not [2]. In this contribution, we further discuss the potential required works for IMS Voice over GEO and PRACH enhancement with OCC based on [3].

# 2. Discussion

2.1	Motivation

GEO satellites offer global coverage with minimal constellation complexity, enabling accelerated commercialization of 5G IoT NTN. NB-IoT devices, with their inherent ultra-coverage capabilities, are particularly suited for GEO scenarios with constrained link budgets. The low-cost NB-IoT devices and relatively easy GEO network construction make the NB-IoT NTN commercial attractive.

IMS Voice is both an attractive and important service for any telecommunication network. Supporting IMS Voice over GEO in IoT NTN would enable critical commercial use cases, particularly for emergency communications without available TN.

3GPP works on IoT NTN FDD since Rel-17. However, due to both the NB-IoT standard restriction and limited link budget under GEO, IoT NTN IMS voice is not supported in Rel-17, 18, and 19 yet.

Given Rel-20’s limited timeline, prioritizing emergency IMS Voice over GEO (i.e., voice communication between IoT NTN terminals and Public Safety Answering Point) is critical for early commercialization.

Proposal 1: Support working on IMS Voice over GEO in IoT NTN for FDD in Rel-20 5G-Advanced at least for emergency calls.

NPRACH enhancement with OCC could enhance the initial access capacity of the network and reduce the initial access delay of a terminal by enhancing the initial access success rate. Companies’ simulations show the gain during the Rel-19 study phase. Under one GEO cell, there will be a huge amount of devices that require NTN services. Due to the limited IoT-NTN spectrum, enhancing the initial access capacity is necessary and beneficial for IMS Voice in Rel-20.

Proposal 2: Support working on initial access capacity enhancement in IoT NTN in Rel-20 5G-Advanced at least for GEO.

2.2	Works on WGs and Cooperations with SA

To support IMS Voice over GEO, there are several challenges from the perspective of the RAN.

VoIP Packetisation Challenges

While the AMR codec is supported in IoT standards, its 20 ms packetization intervals (multiples of 20 ms) introduce frequent packet arrivals, increasing L1/L2 control overhead.

Observation 1: The frequent arrival/transmission of VoIP packets means a large control overhead for lower layers (L1/L2) in the radio protocol stack.

Latency Considerations

- Regenerative payload GEO: RTT ≥ 200 ms (depending on elevation, marked as green in the above figure)

- Transparent payload GEO: RTT ≥ 400 ms (depending on elevation, marked as orange in the above figure)

Observation 2: Efficient VoIP packet scheduling is essential for ensuring the IMS voice user experience.

Type-B Half-duplex FDD Constraints

For NB-IoT NTN, only type-B half-duplex FDD operation is supported[3]. In this type of operation, guard periods (GP), each referred to as a half-duplex guard subframe, are created by the UE by

- not receiving a downlink subframe immediately preceding an uplink subframe from the same UE, and

- not receiving a downlink subframe immediately following an uplink subframe from the same UE.

In emergency communication, using voice may not be the only effective method for ensuring accurate exchanges of information. Engaging in both cross-RAN and SA efforts could help minimize potential collisions.

Observation 3: Preventing UL and DL voice data collision on the terminal side with SPS scheduling enhancements.

Observation 4: Preventing UL and DL voice data collision on the terminal requires the SA's assistance on the IMS voice's service layer.

Initial Access Bottleneck

In a large GEO cell with limited bandwidth, the initial access can become a bottleneck for network capacity. To enhance uplink capacity, an enhancement to the NPRACH with OCC is being studied in Rel-19. During the Rel-19 study phase, issues related to power imbalances, timing offsets, and frequency offsets were identified. By focusing on the GEO scenario in Rel-20, we can address these issues and achieve the orthogonal properties of the OCC in the code domain. This approach will allow Release 20 to leverage most of the OCC scheme study results from Rel-19.

Observation 5: By focusing on the GEO scenario in R-20, we can reuse many of the initial access capacity enhancement study results from R-19, which would expedite the progress of the work in R-20.

Based on the discussions above, we consider the following objects in Rel-20.

Proposal 3: Support of efficient scheduling (e.g. Semi-Persistent Scheduling, SPS) for NB-IoT for UL and DL VoIP packets (RAN1/RAN2).

Proposal 4: Support jointly resolving the UL and DL VoIP packet collisions between RAN and SA (RAN1/RAN2/SA2/SA4).

Proposal 5: Support NPRACH enhancement with OCC to increase the initial access capacity for IoT NTN GEO (RAN1/RAN2).

# 3. Conclusion

In this contribution, we further discussed the potential required works for IoT NTN in RAN WGs and cooperation with SA and have the following observations and proposals.

Working Directions for Rel-20 5G-Advanced:

Proposal 1: Support working on IMS Voice over GEO in IoT NTN for FDD in Rel-20 5G-Advanced at least for emergency calls.

Proposal 2: Support working on initial access capacity enhancement in IoT NTN in Rel-20 5G-Advanced at least for GEO.

Objects for Rel-20 5G-Advanced WI:

Observation 1: The frequent arrival/transmission of VoIP packets means a large control overhead for lower layers (L1/L2) in the radio protocol stack.

Observation 2: Efficient VoIP packet scheduling is essential for ensuring the IMS voice user experience.

Observation 3: Preventing UL and DL voice data collision on the terminal side with SPS scheduling enhancements.

Observation 4: Preventing UL and DL voice data collision on the terminal requires the SA's assistance on the IMS voice's service layer.

Observation 5: By focusing on the GEO scenario in R-20, we can reuse many of the initial access capacity enhancement study results from R-19, which would expedite the progress of the work in R-20.

Proposal 3: Support of efficient scheduling (e.g. Semi-Persistent Scheduling, SPS) for NB-IoT for UL and DL VoIP packets (RAN1/RAN2).

Proposal 4: Support jointly resolving the UL and DL VoIP packet collisions between RAN and SA (RAN1/RAN2/SA2/SA4).

Proposal 5: Support NPRACH enhancement with OCC to increase the initial access capacity for IoT NTN GEO (RAN1/RAN2).

# References

[1] RP-243292, RAN Chair’s Summary of 5G-Advanced in Rel-20, RAN Chair

[2] RAN1 Chair’s Notes, RAN1#120

[3] RP-242504, Views on Rel-20 5G-Advanced, NEC

[4] TS 36.211

# Annex for reference [1]