3GPP TSG-RAN Meeting #107	RP-250127

Incheon, Korea, 12 - 14 March 2025

Agenda Item:	15.2.8.4

Source:	NEC

Title:	Views on Rel-20 NR NTN Phase 4

Document for:	Discussion and Decision

1	Introduction

IOT NTN and NR NTN were firstly specified in Rel-17 and further enhancements has been introduced in Rel-18 and Rel-19. Due to strong market and companies interest, it is our view that 3GPP should continue the NR NTN enhancement in Rel-20, and this topic has been listed as an additional RAN2-led topics based on the guidance of RAN Chair’s Summary of 5G-Advanced in Rel-20 [1].

In this paper, we discuss and give our views on Rel-20 NR NTN for 5G-A.

2	Discussion

2.1	Rel-20 potential scope

Considering the good progress made from Rel-17 to Rel-19, for Rel-20 5G-A scope, the work on NR NTN could include any leftover topics firstly; especially mobility enhancement aspects should be prioritized to overcome more frequent cell changes in NTN than TN for both transparent and regenerative architectures. Secondly,  NTN/NTN and NTN/TN integration and/or co-ordination could be considered in this release before we head to ubiquitous, unified 6G.  Below, some areas are listed for consideration:

- Enable regenerative mode with CU-DU split architecture, with minimum specification changes

- Handover enhancement for satellite switch with regenerative mode

- Generalization of CB(Contention-Based)-data transmission

- Multi-orbit/layer coordination for paging and service redirection

- Downlink throughput/capacity enhancement

2.2	Regenerative mode with CU-DU split architecture

In Rel-17,  transparent mode was firstly supported for NR NTN.  Later, in Rel-19, regenerative mode with whole gNB on board is supported in addition. For IOT NTN, it is going even further to have whole or part of CN on board to support store and forward services. One missed regenerative mode is with DU on board.

Comparing to other regenerative modes, one benefit of CU-DU split architecture is to make the payload on satellite smaller, and change inter-gNB switch into inter-DU switch upon satellite switch. Another benefit of CU-DU split architecture is to allow more deployment options.  CU could be on the ground as a basic option, alternatively, CU could be on another orbit, e.g. GSO, to allow more stable F1 interface. The figure below is just a possible example.

Figure 1  Deployment example of regenerative mode with CU-DU split

Thanks to the introduction of gNB on-board regenerative mode, we could envision no or limited work in RAN1 and RAN2. The main consideration would be dynamic F1 management. Dynamic NG interface management required for whole gNB on-board will pave the way for it. Alternatively, F1 management over feeder link could be left to implementation.

Observation 1: It would be possible to enable DU on-board regenerative mode with minimum specification change.

2.3	Satellite switch with regenerative mode

LEO and MEO satellites constantly move with high speed, handover/cell reselection happen much more frequently in NTN than TN network. Frequent handovers is one of the main challenges for both UE and network in terms of signalling overhead, processing overhead and service interruption.  Handover signalling reduction is the key aspect for the success of NR NTN deployments. In general, it should not be excluded to consider further mobility enhancement, providing feasibility and gain are justified comparing to existing mechanisms.

In the scenario of satellite switch, i.e. a new satellite will replace an old satellite to provide cell coverage/service to the same area,  RAN2 has worked out a solution of uplink re-synch without PCI change which effectively avoid handover procedure for all connected UEs in the cell at the switch time. This feature can only be used in transparent mode, where the gNB on ground is unchanged upon satellite switches.

For later introduced regenerative mode, in the same satellite switch scenario, the need of handing over all UEs in the coverage area remains the same,  but a solution for handover signalling reduction is not in place yet. Due to the fact that gNB is on board, PCI unchanged solution cannot work directly, so a new solution should be studied.

Observation 2: There is no solution available yet to improve handover performance for satellite switch with regenerative mode.

2.4	Generalization of CB-data transmission

CB-Msg3 transmission is being discussed in the context of EDT in IOT NTN; this is to avoid to pre-configure dedicated uplink resource for a small amount of data transmission. This feature would increase radio resource efficiency and also reduce data transmission delay.

RACH-less CHO is a very important handover mechanism in NTN network. However, RACH-less CHO is more feasible for time-triggered CHO, where the handover time is known by network and dedicated uplink grant could be reserved/scheduled at the predicted handover window. For location and or RRM-triggered CHO, it is difficult to schedule the very first uplink resource in the target cell. CB-data transmission would fit well in this case.

Observation 3: It would also be beneficial to generalize CB-data transmission to NR NTN for RACH-less CHO and other cases, e.g. SDT in RRC inactive mode.

2.5	Multi-orbit/layer Coordination

Mobility, including handover, cell reselection, and redirection between NTN/NTN and TN/NTN, has been generally specified to integrate different network types for service continuity, especially in case UE moves out or into TN coverage. However, it is notable that the coverage of different network types including GSO/MEO/LEO/TN would be likely overlapped and there are huge cell size difference between them. These characteristics should be exploited for UEs under multi-orbit or NTN/TN cells’ coverage, for network efficiency.

As we know, generally it is more efficient for big cell to provide paging and other type of broadcasting services/signal;  in contrast, small cell could provide higher throughput and faster connectivity. One example multiple-orbit/layer coordination is to encourage UEs to camp on GSO cell for paging/broadcast, but reselect or redirect to LEO/TN cell for data transmission as shown below:

Figure 2: Example of Multi-orbit/layer Coordination

Observation 4: It would be useful to consider multi-orbit/layer coordination for paging and service redirection.

2.6	Downlink Throughput/Capacity Enhancement

There is consistently high demand for increased throughput at terminals and greater capacity within the network. Regarding our progress on the NR NTN standards, 3GPP has successfully enhanced uplink coverage in Rel-18, followed by enhancements in uplink throughput/capacity and downlink coverage in Rel-19. It is now the appropriate time to focus on enhancing downlink throughput and capacity in Rel-20.

Based on the throughput and capacity enhancement methods discussed in 3GPP, including MIMO technology, downlink throughput/capacity enhancement for NR NTN is a low-hanging fruit.

In line with the Rel-20 timeline, we can work on improving downlink throughput/capacity for both FR1-NTN and FR2-NTN bands.

Observation 5: There is still potential to improve downlink throughput/capacity for both FR1-NTN and FR2-NTN with limited efforts.

3	Conclusion

In this contribution, we give our views on Rel-20 NR NTN for 5G-A. We propose that:

Proposal : Support NR NTN enhancements Ph4 for Rel-20 5G-A as a RAN2-led work, including:

  - Enable DU on-board regenerative mode with minimum specification changes (RAN2,RAN3)

  - Handover enhancement for satellite switch with regenerative mode  (RAN2,RAN3)

  - Generalization of CB-data transmission (RAN2, RAN1)

  - Multi-orbit/layer coordination for paging and service redirection (RAN2, RAN3)

  - Downlink Throughput/Capacity Enhancement (RAN1, RAN2)

References

[1]	RP-243292, RAN Chair’s Summary of 5G-Advanced in Rel-20, TSG RAN#106.