3GPP TSG-RAN Meeting #107	RP-250129

Incheon, Korea, 12 - 14 March 2025

Agenda Item:	15.2.9.2

Source:	NEC

Title:	Views on Rel-20 SON/MDT enhancement

Document for:	Discussion and Decision

1	Introduction

Self-Organizing Network (SON) and Minimization of Drive Tests (MDT), which encompasses solutions for network self-configuration and self-optimisation, was introduced in LTE to support deployment of the system and performance optimization. SON for NR was first introduced in Rel-16 and further enhanced from rel-17 to 19. And today, SON/MDT functionality is an important item for operators to optimize network operation efficiently.

In addition, in the RAN#106 meeting, the SON/MDT Enhancement was listed as a RAN3-led candidate topic for Rel-20 [1] as follows:

- SON/MDT Enhancements: - There is reasonably strong interest to continue Self-Organizing Networks (SON)/Minimization of Drive Tests (MDT) in Rel-20 - Particularly for some of the new features (e.g., inter-CU LTM) in Rel-19 - Rel-19 leftovers? - Important to focus on a very limited set of topics targeting real and urgent commercial needs in Rel-20

In Rel-20, where the 6G discussion is held in parallel, specific commercial benefits should be taken into account, as well as stability and technical maturity, with the introduction of SON/MDT functionality as the object that will most improve network efficiency, for example, Inter-CU LTM, Conditional LTM, SON/MDT enhancements for UAV, LP-WUS, etc.

In this contribution, we described our views on Rel-20 SON/MDT enhancement.

2	Discussion

2.1	MRO enhancement for R19 mobility mechanisms

In Rel-19, some mobility enhancements are introduced in NR mobility enhancements Phase 4 [2]. For SON/ MDT, following enhancements of these mobility features and extending SON functions such as MRO is essential for optimization in a network with Rel-19 mobility. Therefore, these fundamental mobility enhancements were specified in Rel-19 mobility, and it should be covered in SON feature.

- inter-CU Layer1/ 2 Triggered Mobility (LTM)

- Conditional LTM

Proposal 1: MRO enhancement for R19 mobility mechanisms, including Inter-CU LTM, Conditional LTM

2.2	SON enhancements for UAV

Uncrewed Aerial Vehicles (UAV) [3] was introduced in Rel-18 for NR.  It has highly commercial demand today. The radio conditions for Aerial UE may differ from those for General UE because the radio conditions for Aerial UE are less affected by obstacles on the ground when it is in the sky and more likely to be in line-of-sight communication. Therefore, it is important for the SON function to identify whether it is an Aerial UE or a Ground UE and operate accordingly. In addition, considering specific MRO functions for Aerial UEs, which have different characteristics from Ground UEs, could potentially accelerate service delivery.

Proposal 2: Support of SON enhancements for UAV

2.3	SON/MDT enhancements for LP-WUS/WUR

Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [4] was introduced in Rel-19 and also not assuming further enhancement. It is a highly demanded feature that provides significant power savings, especially in IoT devices. On the other hand, LP-WUS/WUR in IDLE/INACTIVE mode can be potentially less efficient than normal signal and receiver characteristics.Thus considering a SON/MDT specifically for LP-WUS/WUR can help to set the appropriate conditions for LP-WUS monitoring. Even in CONNECTED mode, the LP-WUS provides functionality for proactive power reduction in a different manner than conventional DRX, and the addition of SON/MDT functionality will help to configure the relevant parameters.

Proposal 3: Support of SON/MDT enhancements for LP-WUS/WUR

2.4	Support of the leftovers in Rel-19 SON/MDT

If some rel-19 SON/MDT items will not be completed, RAN WG can continue to discuss in next release as usual.

Proposal 4: Support of the leftovers in Rel-19 SON/MDT, if needed

3	Conclusion

In this contribution, we give our views on Rel-20 SON/MDT enhancement. We propose that:

Proposal: Support SON/MDT enhancement for Rel-20 5G-A as a RAN3-led work, including:

  - MRO enhancement for R19 mobility mechanisms, including Inter-CU LTM, Conditional LTM [RAN3, RAN2]:

    - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

    - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

  - Support of SON enhancements for the following features [RAN3, RAN2]:

    - UAV

  - Support of SON/MDT enhancements for the following features [RAN3, RAN2]:

    - LP-WUS/WUR

  - Support of the leftovers in Rel-19 SON/MDT, if needed

References

[1]	RP-243292, RAN Chair’s Summary of 5G-Advanced in Rel-20, TSG RAN#106.

[2]	RP-242356, NR mobility enhancements Phase 4, TSG RAN #105.

[3]	RP-233386, NR Support for UAV (Uncrewed Aerial Vehicles), TSG RAN #102.

[4]	RP-240801, Low-power wake-up signal and receiver for NR (LP-WUS/WUR), TSG RAN#103.