3GPP TSG-RAN Meeting #107	RP-250152

Incheon, South Korea, 12th – 14th March, 2025

Source:	Nokia

Title:	On Less than 5MHz demodulation requirements for CA

Agenda item:	9.3.4.4

Document for:	Discussion

Introduction

The Rel-19 WI  “NR_FR1_lessthan_5MHz_BW_Ph2-Perf” has the objective of introducing CA requirements for NR less than 5 MHz. While the performance part of the WID includes “Specify necessary performance requirements (RAN4)”, the WID does not include TS38.101-4 as an impacted TS, and because of this, demod performance requirements are not currently considered for the work in RAN4.

In this contribution we ask RAN to verify interest, whether RAN4 shall introduce UE demodulation performance requirements for CA in Rel-19 “NR_FR1_lessthan_5MHz_BW_Ph2-Perf”.

Discussion

The Rel-19 WI “NR_FR1_lessthan_5MHz_BW_Ph2-Perf” aims to introduce requirements for CA [1]:

# 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI - Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band

In the relevant UE demod specification (TS 38.101-4) there exist CA requirements that cover different CBWs down to and including 5 MHz, supported by performance requirements tables.

Concerning the performance part of “NR_FR1_lessthan_5MHz_BW_Ph2-Perf” in Rel-19, the WID does not include TS38.101-4 as one of the impacted specifications, which has been interpreted in RAN4 to mean that no demod performance requirements are to be considered.	
We ask RAN to verify interest, whether RAN4 shall discuss to introduce UE demodulation performance requirements for CA in Rel-19 “NR_FR1_lessthan_5MHz_BW_Ph2-Perf”. WG work would entail updating CA performance requirements table to also include 3 MHz, and checking if other amendments are needed. To enable such work and discussion in WG4, RAN would need to include TS 38.101-4 in the list of affected TSs.

There are two RAN4 meetings left until “NR_FR1_lessthan_5MHz_BW_Ph2-Perf” is planned to be finalized, and our assessment is that the changes will likely be small and would be possible to handle in the remaining 2 meetings assuming existing requirements for 5MHz can be reused.

In Rel-19, CA is introduced, hence it should be decided whether UE demodulation requirements for CA are to be included.

RAN to verify interest, whether RAN4 shall introduce UE demodulation performance requirements for CA in Rel-19 “NR_FR1_lessthan_5MHz_BW_Ph2-Perf, and if so, add 38.101-4 to the list of impacted TSs.

Conclusion

In this paper we have provided our view on observations related to UE performance requirements for CA not having been considered in “NR_FR1_lessthan_5MHz_BW_Ph2-Perf”. We have made the following observations and proposal:

- In Rel-19 CA is introduced, hence it should be decided whether UE demodulation requirements for CA are to be included.

- RAN to verify interest, whether RAN4 shall introduce UE demodulation performance requirements for CA in Rel-19 “NR_FR1_lessthan_5MHz_BW_Ph2-Perf, and if so, add 38.101-4 to the list of impacted TSs.

- References

- RP-243309 - Revised WID: NR channel BW less than 5MHz for FR1 Phase 2