3GPP TSG RAN Meeting #107	RP-250187

Incheon, South Korea, 12 - 14 March, 2025

Status Report to TSG

Agenda item:			9.3.2.4

| WI / SI Name | NR sidelink multi-hop relay |  |  |  |
| --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No | Testing part: No |
| Acronym | NR_SL_relay_multihop |  |  |  |
| Unique ID | 1040114 |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242349 |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: 09/2025 | Performance part: mm/yyyy | Testing part: mm/yyyy |
| Overall Completion level | Study Item: xx % | Core part: 60% | Performance Part: x% | Testing part: xx% |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | Youngdae Lee |
|  | Company | LG Electronics Inc. |
|  | Email | youngdae.lee@lge.com |
| Rapporteur | Name | Martino Freda |
|  | Company | InterDigital Communications |
|  | Email | martino.freda@interdigital.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2#129 (February 2025)

60 contributions ([1] ~ [60]) were submitted in this meeting. The following agreements were made:

General

- From RAN2 perspective, the extension of the multihop relay WI to two additional hops is feasible with the current time allocation.

Relay discovery and (re)selection for multi-hop U2N relay

- The last relay UE over one cell cannot be operated as an intermediate relay UE towards a different cell.  FFS if there can be exceptions for control plane approach 2 when the intermediate relay UEs are in idle/inactive.

- From RAN2 perspective, when an intermediate/last relay UE serves two different downstream UEs, there is no assumption on whether it presents the same or different L2IDs for connection establishment.

- Reuse the same Uu RSRP thresholds for discovery transmission as in the legacy (i.e., single hop L2 U2N relay) for L2 Remote UE and L2 U2N last Relay UE in multi-hop U2N relay.

- Reuse the same discovery resource pool(s) and configurations as in the legacy (i.e., single hop L2 U2N relay) in multi-hop U2N relay.

- For model A discovery, there are no additional AS criteria on discovery message forwarding (the existence of the PC5 link is sufficient).

- For model B discovery, if the PC5 link on which the discovery message is received has already been established, no additional AS criteria on discovery message forwarding are applied.  FFS if the PC5 link is not already established.

- For multi-hop U2N relay selection at the remote UE, it reuses the trigger condition of R17 single-hop U2N relay, i.e.: 1) Direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold; 2) Indicated by upper layer of the U2N Remote UE.

- For multi-hop U2N Relay reselection trigger at the remote UE, reuse the following R17 single-hop U2N Relay reselection trigger condition:

  - PC5 signal strength of current first relay UE is below a (pre)configured signal strength threshold.

  - When U2N Remote UE receives a PC5-S link release message from current first relay UE.

  - When U2N Remote UE detects PC5 RLF with the current first relay UE.

  - Indicated by upper layer.

- L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating (FFS on the detailed PC5-RRC signaling design):

  - cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network.

  - release of RRC connection between an intermediate relay UE and the network.

  - PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s).

  - FFS: link quality degradation of the upstream links resulting in a notification.

  - FFS if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE).

  - FFS on the signalling contents of the PC5-RRC indication from the first relay UE.

  - FFS applicability to L3.

- The intermediate relay UE can adopt the same AS triggering condition of relay (re)selection of the R17 remote UE. FFS upper layer conditions and whether the spec impact will be modelled as a separate relay UE behaviour or through functioning as a remote UE.

- Upon intermediate relay UE performing relay (re)selection, it can notify its child node.  If the cause of (re)selection already causes a notification, there is no double-trigger of the notification.

- FFS if (re)selection as such is captured as a cause of notification (intention to maintain no double-trigger)

- FFS if the notification is required in all cases

- FFS the content in the notification

- FFS the child node behaviour after receiving this notification

- FFS other condition/timing that the intermediate relay UE notifies its child node

Control plane procedures for multi-hop U2N relay

- RAN2 assumes that discovery and PC5 connection establishment can be performed in each intermediate UE prior to processing the received RRCSetupRequest by the remote UE.  The related FFS can be removed from the stage 2 description.

- For the baseline solution, the last relay sending SUI on behalf of other relay UEs is not supported.  The related FFS can be removed from the stage 2 description.

- For the baseline solution, Rel17 SUI message and format is re-used.

- No further clarification in stage 2 description is needed to clarify that a relay UE can establish its RLC channel for relaying of SRB1 from its immediate child node during its own connection establishment and details will be clarified in stage 3 (i.e., follow legacy U2N procedure from Rel-17 to set up the RLC channel for SRB1).  The related FFS can be removed from the stage 2 description.

- For system information acquisition in multi-hop, the remote UE:

  - When RRC_CONNECTED, uses end-to-end RRC signaling to obtain its system information directly from its connected cell.

  - When RRC_IDLE/RRC_INACTIVE, can request SI using PC5-RRC signaling (e.g., RemoteUEInformationSidelink message)

  - When in RRC_IDLE/RRC_INACTIVE, receives the required SI from PC5-RRC signaling (e.g., UuMessageTransferSidelink)

- For system information acquisition by the remote UE in multi-hop, the same triggers as Rel17 are supported for sending the PC5-RRC message (e.g., RemoteUEInformationSidelink) namely:

  - when there is a change in the required SI while in RRC_IDLE/RRC_INACTIVE, or when entering RRC_IDLE/RRC_INACTIVE

  - when it entering RRC_CONNECTED, a PC5-RRC message (e.g., RemoteUEInformationSidelink) is sent to cancel a previously sent required SI

- The last relay UE in multihop can forward SI (e.g., in a UuMessageTransferSidelink) to an intermediate Relay upon:

  - acquisition of the SIB(s) requested (in a hop by hop manner) by a connected child node (intermediate node and/or remote UE, but the last relay UE is not required to determine which node originated the request)

  - reception of updates of any SIBs requested by a remote UE or another a child relay UE (in a hop-by-hop manner), including SIB1

  - deciding to perform unsolicited SIB1 forwarding

- An intermediate relay UE that is RRC_CONNECTED may use end-to-end RRC signaling to obtain its system information directly from its connected cell (i.e., behaving as a remote UE).

- The intermediate UE, when it receives SI from a parent relay, forwards the SI message only to the child UEs which requested the SI (i.e., the intermediate UE keeps track of the required SI for each child node).  FFS on the need for including a “remote UE ID” in the request.

- The PC5-RRC message containing the required SI that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least the requested SIB list.  FFS if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI.

- Re-use RemoteUEInformationSidelink as the PC5-RRC message transmitted by the remote UE or by the intermediate relay UE to the parent node (intermediate relay or last relay) to request the required SI.

- The PC5-RRC message transmitted by the last relay UE or by the intermediate relay UE that provides the SI to a child UE contains at least containers with SIB1 and other system information requested by the child UE.

- Re-use UuMessageTransferSidelink as the PC5-RRC message transmitted by the Last relay or by the intermediate relay UE that provides SI to the child UE.

- The remote UE in multi-hop:

  - When RRC_IDLE/RRC_INACTIVE, can request to receive paging by sending its paging information using PC5-RRC signaling (e.g., RemoteUEInformationSidelink message)

  - When in RRC_IDLE/RRC_INACTIVE, can receive paging record from PC5-RRC signaling (e.g., UuMessageTransferSidelink)?

- For a remote UE in multi-hop, the same triggers as Rel17 are supported for sending the PC5-RRC message (e.g., RemoteUEInformationSidelink) namely:

  - when there is a change in the paging information while in IDLE/INACTIVE, or when entering RRC_IDLE/RRC_INACTIVE

  - when it entering RRC_CONNECTED, a PC5-RRC message (e.g., RemoteUEInformationSidelink) is sent to release the paging information

- The last relay UE in multihop can forward paging to an intermediate Relay upon receiving paging message related to a multihop remote UE (including the case of an intermediate relay UE functioning as a remote UE).

- When the intermediate UE receives a paging message from a parent relay on PC5, it forwards the paging message only to the remote UE being paged or the intermediate relay UE serving a remote UE being paged.

- The PC5-RRC message containing the paging information that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least paging UE ID and paging cycle of the remote UE and any intermediate relay UEs serving the remote UE.

- Re-use RemoteUEInformationSidelink as the PC5-RRC message transmitted by the remote UE or by the intermediate relay UE to the parent node (intermediate relay or last relay) to provide the paging information.

- The PC5-RRC message transmitted by the last relay UE or by the intermediate relay UE contains at least one or multiple paging record(s) associated with intermediate relay UE(s) and/or remote UE(s).

- Re-use UuMessageTransferSidelink as the PC5-RRC message transmitted by the Last relay or by the intermediate relay UE that provides paging record to the child UE(s).

- When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain the SI required by it or requested by the remote UE by requesting SI from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink).  FFS if it can also receive it directly from SIB broadcast by the last relay UE’s serving cell on Uu (when the intermediate relay UE is in coverage of the same cell that serves the last relay UE).

- The intermediate relay UE sends SI request in PC5-RRC (e.g., in RemoteUEInformationSidelink) to the parent relay (intermediate relay or last relay), following legacy remote UE behaviour:

  - when there is a change in the SI required by the intermediate UE for its own use (as a remote UE)

  - when the intermediate UE enters RRC_IDLE/RRC_INACTIVE

  - when the intermediate UE enters RRC_CONNECTED (to cancel a previously sent SI request; if a child node requested the SI, the intermediate UE acquires it by dedicated signalling)

- The intermediate relay UE sends SI request in PC5-RRC (e.g., in RemoteUEInformationSidelink) to the parent relay (intermediate relay or last relay):

  - upon reception of new/changed required SI received from a remote UE/child relay UE (where the concerned SI was not previously requested)

  - FFS when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay.

- The intermediate relay UE can send SI (e.g., in UuMessageTransferSidelink) to a child node:

a)	Upon reception of SI received from intermediate relay or last relay containing SI requested by a child node (intermediate relay or remote UE)

b)	Upon acquisition (from the network) of SI requested by a child node (intermediate relay or remote UE)

c)	Upon receiving updated SIBs from the network which have been requested by a child node (intermediate relay or remote UE)

d)	Upon reception of SIB1 received from a parent relay (i.e., this case may correspond to SIB1 update detected by the last relay, or unsolicited SIB1 forwarding by the last relay)

e)	Upon receiving updated SIB1 from the network (as in Rel17)

f)	Upon unsolicited SIB1 forwarding to a connected child node (intermediate relay UE or remote UE)

FFS if b), c), and e) are limited to RRC_CONNECTED relay UE or apply in all states.  FFS whether to consolidate some of the conditions in stage 3 specification text.

- If multiple remote UEs are connected to a single intermediate relay UE, the multiple remote UEs must connect through the same upstream path (same parent relay UE(s)).

- An intermediate relay UE in RRC_CONNECTED releases its paging-related information in the parent UE at least for itself.  FFS whether it also releases it for the child UEs and how the child UEs receive the paging when the intermediate relay UE is in RRC_CONNECTED.

- When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain paging from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink).  FFS if it can also receive it directly on Uu (when the intermediate relay UE is in coverage).

- What triggers the intermediate relay UE to request paging monitoring by the parent relay (intermediate relay or last relay) in PC5-RRC (e.g., in RemoteUEInformationSidelink)?

  - FFS: when there is a change in the paging information of the intermediate UE or child UE

  - when the intermediate UE enters RRC_IDLE/RRC_INACTIVE

  - when the intermediate UE enters RRC_CONNECTED (to cancel paging monitoring request)

  - upon reception of paging monitoring request from a remote UE/child relay UE

  - FFS: upon change in the ability of the intermediate UE to monitor paging on Uu (e.g., moving in/out of coverage) to initiate/cancel paging monitoring by the parent relay).

- The intermediate relay UE sends paging message (e.g., in UuMessageTransferSidelink) to a child node

  - Upon reception of paging message received from a parent node (intermediate relay or last relay) that is intended for a child node (intermediate relay or remote UE)

  - FFS upon acquisition (from the network) of paging message that is for a child node (intermediate relay or remote UE)

- In multi-hop L2 U2N relay, besides the agreed remote UE ID and BEARER ID, in addition, at least include D/C field in the SRAP PDU header.

- In multi-hop L2 U2N relay, for SRAP header, at least support the legacy single-hop U2N SRAP header, i.e., the agreed UE ID field is used for the remote UE local ID (including the case of an intermediate relay UE acting as a remote UE). FFS on whether to have one additional format to include remote UE L2 ID and what use case it is applied for.

- Each Layer-2 Intermediate Relay UE has a single PC5 SRAP entity.

Service continuity for multi-hop U2N relay

- For multi-hop i2d, Remote UE evaluates both relay link (for the link between remote UE and serving first relay UE) and Uu link, where the sidelink relay measurement report shall include at least serving first relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

- For multi-hop i2d, first relay UE evaluates adjacent upstream relay link and Uu link, where the sidelink relay measurement report shall include at least measured relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.  FFS if intermediate relay UE reports.

- For multi-hop i2i, the remote UE reports serving first relay UE and candidate single-hop relay UE(s), including at least a source L2 ID, serving cell ID, and a sidelink measurement quantity information.

- For multi-hop i2i, first evaluates adjacent upstream relay link and candidate relay link, where the sidelink relay measurement report shall include at least serving intermediate/ last relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.  FFS if intermediate relay UE reports.

2.2.2	Remaining Open issues

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay.

  - Details of relay discovery and (re)selection

    - Details of triggering conditions and notifications

  - Impact on SRAP and QoS handling for multi-hop

    - Details of SRAP format

  - Control plane procedures

    - Details of system information delivery

    - Details of paging delivery

    - Details of RRC connection setup procedures

- Specify intra-gNB service continuity for multi-hop U2N relay

  - FFS if intermediate relay UE reports for intra-gNB multi-hop indirect to direct path switching and intra-gNB multi-hop indirect to single-hop indirect path switching

  - Intra-gNB direct to multi-hop indirect path switching

  - Intra-gNB single-hop indirect to multi-hop indirect path switching

2.3	RAN3

2.3.1	Agreements

RAN3#127 (February 2025)

- RAN3 can work on the new signaling flow for multi-hop relay for the purpose of Initial Access procedure based on the existing procedures in TS 38.401.

- RAN3 agrees to support the Multi-hop relay in CU-DU split architecture.

- The existing F1AP UE Context Setup and UE Context Modification procedure can be used as a baseline to establish the PC5/Uu Relay RLC channel(s) for relaying of Remote UE’s SRB/DRB.

- For the initial UL RRC transfer message, the gNB-DU informs the gNB-CU of the multi-hop relay related information including at least gNB-DU UE F1AP ID of the Last Relay UE and Remote UE Local ID.

2.3.2	Remaining Open issues

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay.

  - Current signaling flows are based on the agreements that the intermediate Relay UE is in RRC_CONNECTED state. Whether the intermediate Relay UE in RRC_IDLE/RRC_INACTIVE state is supported or not is pending to RAN2.

  - FFS whether each Relay UE needs to establish new PC5/Uu RLC channel for a Remote UE.

  - FFS how/what information is used to indicate the counterpart information of a configured PC5 Relay RLC channel to the gNB-DU.

  - FFS how to configure relay UEs for relaying of remote UE’s SRB/data over F1, e.g. via separate procedures of each relay UE or via an single procedure for all relay UEs’ configuration.

  - FFS whether the gNB-DU UE F1AP IDs of the U2N First Relay UE and the U2N Intermediate Relay UE are included in the INITIAL UL RRC MESSAGE TRANSFER message.

  - FFS whether and how the gNB-DU becomes aware that the U2N Remote UE is connected to the gNB-DU via the U2N First Relay UE, the U2N Intermediate Relay UE and U2N Last Relay UE.

- Specify intra-gNB service continuity for multi-hop U2N relay

  - Intra-gNB multi-hop indirect to direct path switching using existing framework

  - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework

  - Intra-gNB direct to multi-hop indirect path switching

  - Intra-gNB single-hop indirect to multi-hop indirect path switching

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2#129

- R2-2500064	LS on Authorization information for Layer-2 multi-hop U2N relaying to NG-RAN (S2-2501296; contact: LGE)	SA2

- R2-2500068	Reply LS on relay discovery announcement (S2-2501334; contact: LGE)	SA2

- R2-2500122	Discussion on multi-hop U2N relay discovery and relay selection	NEC

- R2-2500187	Discussion on Multi-hop Discovery and (Re)selection	CATT

- R2-2500188	Discussion on the Control Plane Procedures	CATT

- R2-2500189	Intra-gNB Service Continuity for Multi-hop U2N Relay	CATT

- R2-2500192	Discovery and relay (re)selection for multi-hop U2N relay	OPPO

- R2-2500193	Service continuity of multi-hop U2N relay	OPPO

- R2-2500194	Control plane procedures of multi-hop U2N relay	OPPO

- R2-2500300	Control Plane aspects for Multi-hop Relay	NEC

- R2-2500307	Considerations on relay discovery and (re)selection	Samsung

- R2-2500308	Consideration on CP issues for multi-hop SL relay	Samsung

- R2-2500420	One remaining issue on multi-hop U2N Relay Discovery message forwarding	ASUSTeK

- R2-2500421	RRC states of candidate Relay UEs for path switching	ASUSTeK

- R2-2500432	Relay discovery and selection for Multi-hop UE-to-NW Relay	Apple

- R2-2500433	Discussion on Control Plane for Multi-hop UE-to-NW Relay	Apple

- R2-2500434	Discussion on SRAP for Multi-hop Layer-2 U2N Relay	Apple

- R2-2500496	Report of [Post128][401][Relay] Control Plane Baseline Solution	InterDigital

- R2-2500497	Discovery and Relay (Re)Selection for Multi-hop U2N Relays	InterDigital

- R2-2500498	Control Plane Handling for Multi-hop U2N Relays	InterDigital

- R2-2500508	Discussion on the discovery and relay (re)selection for multi-hop U2N relay	LG Electronics Inc.

- R2-2500509	Discussion on the control plane procedure for multi-hop U2N relay	LG Electronics Inc.

- R2-2500510	Discussion on service continuity for multi-hop U2N relay	LG Electronics Inc.

- R2-2500561	Discussion on control plane aspects for NR sidelink multi-hop relay	China Telecom

- R2-2500562	Discussion on service continuity for multi-hop relay	China Telecom

- R2-2500570	Consideration on multi-hop relay discovery and reselection	China Telecom

- R2-2500632	Relay (re)selection in Multi-hop relay	Lenovo

- R2-2500633	Control plane in Multi-hop relay	Lenovo

- R2-2500634	Service continuity for Multi-hop system	Lenovo

- R2-2500700	Relay discovery and (re)selection for multi-hop Relay	Huawei, HiSilicon

- R2-2500701	Control plane procedures for multi-hop relay	Huawei, HiSilicon

- R2-2500702	Discussion on service continuity for Multi-hop Relay	Huawei, HiSilicon

- R2-2500723	[Draft] LS on legacy UE participation in multi-hop UE communication	Nokia

- R2-2500724	Relay discovery and (re)selection	Nokia

- R2-2500725	SRAP impacts on MH relay	Nokia

- R2-2500733	Considerations on Service Continuity of Multi-hop Relay	NEC

- R2-2500753	Multi-hop relay selection/re-selection	Sony

- R2-2500864	Discussion on control plane procedures	Ericsson, Apple, AT&T, InterDigital Inc, FirstNet, Qualcomm Incorporated

- R2-2500865	Discussion on relay discovery and relay (re)selection	Ericsson

- R2-2500866	Introduction of multi-hop U2N relay in TS 38.323	Ericsson

- R2-2500898	Discussion on control plane procedures for multi-hop SL Relay	ZTE Corporation, Sanechips

- R2-2500905	Discussion on multi-hop Relay discovery and (re)selection	ZTE Corporation, Sanechips

- R2-2500906	Discussion on service continuity for multi-hop SL relay	ZTE Corporation, Sanechips

- R2-2500913	SRAP design for R19 multi-hop SL relaying	Samsung R&D Institute UK

- R2-2500927	Intra-gNB Service Continuity for Multi-Hop Relays	Ericsson

- R2-2500933	Discovery and (re)selection under multihop relay	Kyocera

- R2-2500934	Control Plane under multihop L2 U2N relaying	Kyocera

- R2-2500935	Service Continuity for U2N multihop relay	Kyocera

- R2-2500953	Discussion on control plane procedure for SL multi-hop relay	KT Corp.

- R2-2501118	Discussion on topology and intermediate relay UE (re)selection	vivo

- R2-2501119	Discussion on CP and SRAP impact for baseline procedure	vivo

- R2-2501120	Discussion on Service continuity for multi-hop relay	vivo

- R2-2501174	Relay discovery and (re)selection	TCL

- R2-2501183	discussion on Relay discovery and (re)selection for multi-hop relay	Sharp

- R2-2501184	discussion on C-plane procedure for multi-hop relay	Sharp

- R2-2501185	discussion on service continuity for multi-hop relay	Sharp

- R2-2501259	Discovery and Relay (re)selection for multi-hop U2N relay	Qualcomm Incorporated

- R2-2501260	Open issue for control plane approach 1	Qualcomm Incorporated

- R2-2501421	[AT129][401][Relay] SRAP for Rel-19 multihop relay (OPPO)	OPPO

- R2-2501433	[AT129][410][Relay]Intermediate Relay UE (re)selection (vivo)	vivo

RAN3#127

- R3-250070	LS on Authorization information for Layer-2 multi-hop U2N relaying to NG-RAN	SA2 (LGE)

- R3-250101	Support for Multi-hop L2 U2N relays	Qualcomm

- R3-250164	Discussion for U2N multi-hop relay	NEC

- R3-250188	Discussion on support of multi-hop SL relay		ZTE Corporation

- R3-250189	(TP to TS 38.401, 38.470) on support of multi-hop SL relay	ZTE Corporation

- R3-250206	Work plan for NR sidelink multi-hop relay	LG Electronics

- R3-250207	(TP for TS 38.401) Support of Multi-hop relay	LG Electronics

- R3-250208	Discussion on service continuity in Multi-hop relay		LG Electronics

- R3-250225	Discussion on the support of multi-hop Layer-2 UE-to-Network relay	Nokia, Nokia Shanghai Bell

- R3-250226	(TP for TS 38.413) Support of multi-hop Layer-2/3 UE-to-Network relay	Nokia, Nokia Shanghai Bell

- R3-250255	Authorization for multi-hop relay		Huawei

- R3-250256	(TP for BLCR38.473) Authorization for multi-hop relay	Huawei, LG Electronics, Nokia, Nokia Shanghai Bell, NEC, CATT, Ericsson, Samsung, ZTE

- R3-250257	Support of multi-hop relay	Huawei

- R3-250331	Discussion on multi-hop Layer-2 UE-to-Network relay	CATT

- R3-250353	UE authorization for multi-hop relay	China Telecom

- R3-250354	Discussion on control plane procedures for multi-hop relay	China Telecom

- R3-250526	(TP for TS 38.423) – Authorization info for SL Multi-hop Relay	Ericsson, LG Electronics, Nokia, Nokia Shanghai Bell, Huawei, NEC, CATT, InterDigital, Samsung, ZTE

- R3-250527	Authorization of Intermediate Relay UEs in RRC_INACTIVE	Ericsson

- R3-250528	QoS handling for Multi-hop Relay	Ericsson

- R3-250638	Discussion on multi-hop for SL relay	Samsung

- R3-250878	(TP for TS 38.401) Support of Multi-hop relay	LG Electronics, ZTE, Nokia, Nokia Shanghai Bell, Ericsson, FirstNet

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template