3GPP TSG RAN Meeting #107	RP-250190

Incheon, South Korea, 12 - 14 March, 2025

Source:	LG Electronics, InterDigital, FirstNet

Title:	Discussion on check point for Rel-19 NR sidelink multi-hop relay

Agenda Item:	9.3.2.4

Document for: 	Discussion and Decision

- Background

In the current WID RP-242349 on NR sidelink multi-hop relay, there is a checkpoint for supporting two additional hops relay in Rel-19 on top of the Rel-17 U2N relay [1].

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. …

In this contribution, it is requested to remove the checkpoint and update WI objective to support two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB) in Rel-19 on top of Rel-17 U2N relay.

- Discussion

In the first meeting of this WI (RAN2#127, August 2024), following agreements were made to support 2 additional hops relays in Rel-19 from the start of the WI in order to efficiently utilize allocated TU for this WI.

Agreements:

RAN2 intend to minimize the impact of hop count on the multi-hop relay mechanisms.

RAN2 intend to design mechanisms from the start of the WI that are flexible enough to be adapted to at least two additional hops in Rel-19.

Since RAN2#127, RAN2 is considering support for 2 additional hops relays on top of Rel-17 U2N relay in all the developed mechanisms under this WI [2-6]. Thus, we can say that the solutions that RAN2 previously agreed satisfy the criterion for easy extensibility to two additional hop relays and forward compatibility for future extensions for additional relays.

In addition, the following agreement was recently made in RAN2#129 for the extension of the work to two additional hops:

Agreement:

From RAN2 perspective, the extension of the multihop relay WI to two additional hops is feasible with the current time allocation.

As agreed in RAN2, it is expected that the extension does not require additional TU request and change of the WI competition date. That is, TU does not need to be updated with the proposed update of WID.

Furthermore, WI progress is also on track to be completed on time with support of 2 additional hops relays considering the reported WI competition level indicated in the recent status report [3].

Therefore, we request to update the WID removing the checkpoint and specifying support for two additional hops relays on top of Rel-17 U2N relay.

- Conclusion and Proposal

RAN2 confirmed that it is feasible to extend the work to 2 additional hops relays on top of Rel-17 U2N relay in all the developed mechanisms under the current TU allocation and the WI progress is also on track for completion on time with the support of 2 additional hops relays.

Therefore, it is proposed to remove the checkpoint as proposed in the WID revision [7] and specify support for two additional hops relays on top of Rel-17 U2N relay.

Proposal: Approve the proposed WID revision [7].

- References

- RP-242349: Revised WID on NR sidelink multi-hop relay, 3GPP TSG RAN Meeting #105, Melbourne, Australia, 09 – 12 September, 2024.

- RP-241750: Status Report to TSG on NR sidelink multi-hop relay, 3GPP TSG RAN Meeting #105, Melbourne, Australia, 09 – 12 September, 2024.

- RP-250187: Status Report to TSG on NR sidelink multi-hop relay, 3GPP TSG RAN Meeting #107, Incheon, South Korea, 12 - 14 March, 2025.

- R2-2410305: Report of [POST127][401][Relay] MH relay discovery and (re)selection, 3GPP TSG-RAN WG2 Meeting #128, Orlando, Florida, 18th – 22nd November 2024.

- R2-2410006: Report of [Post127][402][Relay] Multi-hop relay control plane, 3GPP TSG-RAN WG2 Meeting #128, Orlando, Florida, 18th – 22nd November 2024.

- R2-2500496: Report of [POST128][401][Relay] Control plane baseline solution, 3GPP TSG-RAN WG2 Meeting #129, Athens, Greece, Feb 17th– 21st, 2025.

- RP-250188: Revised WID on NR sidelink multi-hop relay, 3GPP TSG RAN Meeting #107, Incheon, South Korea, 12 - 14 March, 2025.