3GPP TSG RAN Meeting #107									RP-250196

Incheon, Korea, March 12-14, 2025

Status Report to TSG

Agenda item:		9.3.4.8

| WI / SI Name | WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | TRP_TRS_MIMO_OTA_Ph3 |  |  |  |  |
| Unique ID | 1030085 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-243277 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: Sep/2025 | Performance part: Mar/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 65% | Performance Part: 35% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Wang, Ruixin |
|  | Company | vivo |
|  | Email | ruixin.wang@vivo.com |
| Rapporteur | Name | Zhu, Siting |
|  | Company | CAICT |
|  | Email | zhusiting@caict.ac.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.4	RAN4

2.4.1	Agreements

RAN4 #113

General

- Topic summary for WI in [15][16].

- WF for MIMO OTA and TRP TRS was approved in [41] and [48], respectively.

- Framework for Rel-19 MIMO OTA performance requirements was agreed in [40].

- Ad-hoc meeting minutes for MIMO OTA and TRP TRS was captured in [46] and [45], respectively.

- TR 38.762 v0.2.0 was approved in [28].

Agreements for TRP TRS topic:

Sub-topic 1-1 General for WI

Issue 1-1-1: Rel-19 CR for general part of TRP TRS test method

Agreements

  - Approve the CR R4-2500748

Sub-topic 2-1 XR test methods and configurations

Issue 2-1-1: Discussions on impact of Reply LS from CTIA

Agreements:

  - XR phantom development in CTIA would potentially delay the discussion in RAN4. The timeline and WI scope should be further discussed in next RAN plenary. The XR requirement work may be removed out of scope.

Issue 2-1-2: Minimum measurement distance for 50cm QZ

Agreements:

  - The missing minimum measurement distance for AC system with 50cm QZ should be added into spec.

Issue 2-1-3: General XR TRP TRS test procedure

Agreements:

  - The general XR test method in R4-2502300 is agreed.

  - The TRP/TRS MU for low-battery capacity devices is FFS

Issue 2-1-5: Alternative Low-UL test procedure for XR

Agreements:

  - Update the endorsed SPOT method at a single pol: The peak value is defined as the maximum EIS measured in a single polarization direction for a given channel.

  - The corresponding updated CR in R4-2502300 is agreed.

  - The MU of this test method can be further discussed.

Issue 2-1-6: Alternative split measurement grids method for XR

Agreements:

  - CR for this test method to TR 38.870 will be provided next meeting.  The applicability of this method based on the MU analysis should be further discussed.

Sub-topic 2-2 Performance metric for XR devices

Issue 2-2-1: General views on Performance requirements for XR devices

Agreements:

  - Whether XR performance requirements will be removed out of Rel-19 scope will depend on next RAN plenary meeting discussion.

Sub-topic 3-1 UE type, usage scenarios and performance metric for NTN (NR-NTN and IoT-NTN)

Issue 3-1-1: UE manufacturer declaration of supported usage scenario for different NTN UE

Agreements:

  - OEM declaration of scenario applicability can be used to declare the supported usage scenario.

  - Details on declaration and spec language can be further discussed.

Issue 3-1-2: performance metric for NR-NTN and IoT-NTN only support scenario 2

Agreements:

  - RAN4 further discuss the performance metric for UE declared only support scenario 2.

  - Details on performance metric data processing should also be discussed.

Issue 3-1-5: Whether performance metric should be same for GEO and NGEO for handheld UEs (both NR/IoT NTN)

Agreements

  - Same performance metric for GSO and NGSO handheld UEs.

Sub-topic 3-2 NTN OTA test methodologies

Issue 3-2-1: NTN OTA test system, LP or CP for measurement antenna

Agreements:

  - From a test system perspective, the baseline test method for FR1 NTN is LP-based OTA test system.

  - For the full sphere performance metric TRP/TRS for NTN scenario 1, it is same as that defined in TR 38.870.

Issue 3-2-3: Whether MU is impacted if NTN OTA test system go with LP architecture

Agreements:

  - No additional MU contributor is identified for NTN OTA test system.

Sub-topic 4-1 Lab alignment activity

Issue 4-1-1: working procedure update for 2Tx lab alignment activity

Agreements:

- The LAD measurement time at each volunteer lab can be recommended as [5] working days as in previous releases.

- Update working procedure for Rel-19 Performance Campaign next meeting to incorporate agreement to confirm browsing mode only for 2Tx requirements in Rel-19.

Issue 4-1-2: latest status of Lab Alignment Devices (LADs) for 2Tx lab alignment activity

- Proposals

  - Proposal 1: update the test case based on latest status, one for band n41 and the other one for n78. (moderator)

- Current status of Rel-19 2Tx Lab Alignment Devices (LADs).

| Rel-19 2Tx Lab Alignment Devices (LADs) for non-coherent UL MIMO |  |  |  |  |
| --- | --- | --- | --- | --- |
| DUT | UL-MIMO Configuration | Provider/Owner | Contact | issue |
| LAD5 (new device for Rel-19 2Tx) | TPMI index 2 | vivo | Ruixin Wang ruixin. wang@vivo.com | Only support n41. |
| LAD6 (new device for Rel-19 2Tx) | TPMI index 2 | Xiaomi |  | For n78 testing. Checking configuration |

Agreements:

  - The above LAD information is updated for Rel-19 2Tx lab alignment. Also update the test case based on latest status, one for band n41 and the other one for n78.

Issue 4-1-3: 2Tx related MU impacts

Agreements:

  - The MU term under B.2.7.1.1 in TS 38.870 for offset of DUT phase centre from axis(es) of rotation should remain zero for 2Tx cases. The final value will be decided by RAN5.

Issue 4-1-4: For RC low band harmonization

Way forward:

  - WI rapporteur will offline discuss with RC test labs on how to organize the work in Rel-19. And make decision next meeting. Further input from companies on how to improve RC vs AC harmonization is encouraged.

Sub-topic 4-2 1Tx FR1 TRP TRS requirements

Issue 4-2-1: How to specify PDA hand related TRP/TRS requirements (size 2)

Way Forward:

  - RAN4 further discuss how to specify PDA hand related TRP/TRS requirements (size 2).

Issue 4-2-3: band number for 1Tx requirements in Rel-19

Agreements:

  - RAN4 further discuss how to select 4 bands from n3, n5, n7, n8, and n20, encourage companies to provide feedback by RAN4 #114bis on available number of devices each band and provide devices to test labs for data collecting.

Issue 4-2-4: Working procedure for 1Tx requirements related performance measurement campaign

Way Forward:

- Limit to legacy grid for Rel-19 measurement campaign.

Issue 4-2-5: How to specify 1Tx requirements in Rel-19

Way Forward:

- The working procedure in Rel-18 on how to specify the requirement can be a baseline.

Agreements for dynamic MIMO OTA topic:

Sub-topic 1-1 Dynamic channel model emulation and validation

Issue 1-1-1: Dynamic channel model validation targets in different SDOs

Agreement:

Pending the alignment of path loss models with different SDOs, the channel models and their applicability might need to be further clarified and/or renamed by RAN4#116.

Issue 1-1-2: Dynamic channel model PDP target values

Agreement:

- Alignment among different CE vendors is encouraged. It is also recommended to freeze the channel model values no later than RAN4#115 (May 2025), to make sure the core part WI can be completed on time at RAN#109 (Sep. 2025).

- Include original PDP value without any processing related to the delay resolution into TR next meeting.

- Use Chebyshev window with 120dB sidelobe factor as a starting point to produce PDP target values. The window applied should be indicated in the TR.

Issue 1-1-3: TP to 38.762 on Dynamic channel model validation

Agreement:

- Approve the TP (R4-2502176) with the understanding that additional changes and modifications can be made.

Issue 1-1-4: TP to 38.762 on terms and abbreviations

Agreement:

- TP R4-2501746 is approved.

Sub-topic 1-2 Dynamic MIMO OTA test and UE performance metric

Issue 1-2: Dynamic MIMO OTA test and UE performance metric

Agreement:

- All measured throughput values for different device orientations and rotations shall be combined to derive a single CDF curve for the dynamic MIMO OTA test.  FFS whether the performance metric should be based on three (10%/50%/90%) percentiles at the CDF.

- Use the following as a starting point:

  - include both UMa and UMi channels via post measurement data processing. FFS how to do the post data processing.

Sub-topic 2-1 General issues on Rel-19 static MIMO OTA requirements

Agreement:

- Down select the following four bands as a starting point: n3, n7, n77, (n8 or n20) and choose either n8 or n20 to represent low band.

- Encourage companies to provide feedback on available number of devices each band and provide devices to test labs for data collecting.

Sub-topic 2-2 Framework for Rel-19 MIMO OTA requirements

Issue 2-2: Framework for Rel-19 MIMO OTA requirements

Agreement:

- The revision of Framework (R4-2502295) is approved.

2.4.2	Remaining Open issues

- Core part

    - Further discuss XR OTA test method and UE performance metric

    - Further discuss NTN OTA test method and UE performance metric

    - Further discuss FR1 dynamic MIMO OTA test method and UE performance metric

    - TR for FR1 dynamic MIMO OTA

- Performance part

    - How to treat XR OTA requirements scope due to phantom issue in CTIA reply LS [49]

    - 2Tx lab alignment activity for TRP TRS

      - Framework and requirements for 2Tx TRP TRS

    - Measurement campaign for 1Tx TRP TRS and static MIMO OTA

    - Final Requirements and recommended TT for 1Tx TRP TRS and static MIMO OTA

      - Includes Requirements for size 2 smartphone

2.5	RAN5

2.5.1	Agreements

RAN5 #106

- No discussion this meeting

2.4.2	Remaining Open issues

- Core part

  - Detailed RAN5 Workplan for Rel-19 OTA MU assessment

  - XR OTA test methods related MU assessment

  - FR1 NTN OTA (NR-NTN and IoT-NTN) test methods related MU assessment

  - FR1 dynamic MIMO OTA related MU assessment

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A

4.	References

RAN4 #114

- R4-2500041	on OTA NTN tests	Huawei, HiSilicon

- R4-2500042	on OTA XR tests	Huawei, HiSilicon

- R4-2500043	on MIMO tests under dynamic channels	Huawei, HiSilicon

- R4-2500044	On TRP and TRS tests	Huawei, HiSilicon

- R4-2500107	CR for split measurement grids method for TRP/TRS measurements	Nokia, Keysight Technologies UK Ltd

- R4-2500295	CR to TR 38.870 for an alternate TRP test procedure for XR devices	Apple

- R4-2500296	CR to TS38.161 for an alternate TRS test procedure for XR devices	Apple

- R4-2500323	On NTN TRP TRS test methodology	Apple

- R4-2500400	Discussion on FR1 NTN devices OTA	Qualcomm Incorporated

- R4-2500480	Views on Rel19 TRP TRS Performance Part	Apple

- R4-2500481	On FR1 MIMO OTA Dynamic Channel Model	Apple

- R4-2500505	On OTA performance metric and polarization for handheld NTN UE	Samsung

- R4-2500506	On performance metric for FR1 dynamic MIMO	Samsung

- R4-2500507	Some initial test for RC harmonization at FR1 low band	Samsung

- R4-2500588	Topic summary for [114][327] TRP_TRS_Ph3	Moderator (vivo)

- R4-2500589	Topic summary for [114][328] MIMO_OTA_Ph3	Moderator (CAICT)

- R4-2500627	Discussion on OTA test for FR1 NTN devices	Xiaomi

- R4-2500748	CR to TR 38.870 on general part	vivo

- R4-2500749	CR to 38.870 on XR OTA test method	vivo

- R4-2500750	draft CR to 38.870 on NTN OTA test method	vivo

- R4-2500751	Discussions on NTN OTA test method	vivo

- R4-2500752	Discussions on Rel-19 TRP TRS requirements work	vivo

- R4-2501375	Discussion on OTA Test for XR Devices	Meta

- R4-2501613	Further discussion on XR test method	Ericsson-LG Co., LTD

- R4-2501746	TP to 38.762 on terms and abbreviations	CAICT, SAICT

- R4-2501747	On dynamic MIMO OTA channel models in 3GPP and CTIA	CAICT, CMCC

- R4-2501748	Framework and time plan for Rel-19 MIMO OTA performance requirements development (Feb 2025)	CAICT

- R4-2501749	TR 38.762 V0.2.0 on MIMO OTA dynamic test methodology for FR1 UEs	CAICT

- R4-2501801	CR for split measurement grids method for TRP/TRS measurements	Nokia, Keysight Technologies UK Ltd

- R4-2501805	CR for split measurement grids method for TRP/TRS measurements	Nokia, Keysight Technologies UK Ltd

- R4-2501806	CR for split measurement grids method for TRP/TRS measurements	Nokia, Keysight Technologies UK Ltd

- R4-2501828	On dynamic MIMO OTA channel models PDP target values	CMCC

- R4-2501973	Discusion on XR OTA	CAICT

- R4-2502175	On Channel Models in CTIA and 3GPP for Dynamic FR1 MIMO OTA	Keysight Technologies UK Ltd, Spirent Communications, CAICT, Huawei, AT&T, Verizon Wireless

- R4-2502176	TP for TR 38.762: DynMIMO CM Validation Details and Corrections	Keysight Technologies UK Ltd, Spirent Communications

- R4-2502177	On Circular Polarization for FR1 NTN OTA Conformance Testing	Keysight Technologies UK Ltd, Samsung, Nokia, Verizon Wireless, AT&T, Mediatek, ZTE, Ericsson, ETS-Lindgren, Rohde&Schwarz, OPPO, Xiaomi, CAICT

- R4-2502208	CR to TR 38.870 for an alternate TRP test procedure for XR devices	Apple

- R4-2502209	CR to TS38.161 for an alternate TRS test procedure for XR devices	Apple

- R4-2502238	Proposals for Rel-19 TRP TRS requirements topic	Telecom Italia S.p.A., Vodafone, Deutsche Telekom, Orange

- R4-2502295	Framework and time plan for Rel-19 MIMO OTA performance requirements development (Feb 2025)	CAICT

- R4-2502296	Way Forward for [114][328] MIMO_OTA_Ph3	CAICT

- R4-2502299	CR to TR 38.870 for an alternate TRP test procedure for XR devices	Apple

- R4-2502300	CR to 38.870 on XR OTA test method	vivo

- R4-2502301	draft CR to 38.870 on NTN OTA test method	vivo

- R4-2502302	Ad-hoc meeting minutes for [114][327] TRP_TRS_Ph3	vivo

- R4-2502303	Ad-hoc meeting minutes for [114][328] MIMO_OTA_Ph3	CAICT

- R4-2502367	CR for split measurement grids method for TRP/TRS measurements	Nokia, Keysight Technologies UK Ltd

- R4-2502370	Way Forward for [114][327] TRP_TRS_Ph3	vivo

- R4-2500017	Reply LS on Head Phantom for XR devices OTA testing	CTIA OTA NFP

RAN5 #106

N/A

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template