3GPP TSG RAN Meeting #107										RP-250246

Incheon, Korea (Republic Of), March 12-14, 2025

## Status Report to TSG

Agenda item:			9.3.4.11

| WI / SI Name | Low band carrier aggregation via switching |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_LBCA_Sw |  |  |  |  |
| Unique ID | 1060082 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-243317 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 09/2025 | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 10% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Ronald Borsato |
|  | Company | AT&T |
|  | Email | ronald.borsato@att.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN4

#### 2.3.1	Agreements

RAN4 #114 – RF (Core)

General aspects

-	The work plan was presented for information in [1]

-	The topic summary was prepared in [35]

-	The skeleton TR 38.768 was approved in [30]

UE RF aspects

-	A WF in [37] was approved with the following agreements:

Issue 1: Scope clarification - Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI - It is not precluded that such two low bands are further combined with the additional band - The combination of those two low bands with other band can be further discussed in the future release Issue 2: Switching period values Summary of company proposals (for information): 5us: 3 (Spreadtrum, Huawei, Ericsson) 13us, assuming no LO retuning: 2 (Ericsson, Huawei) 35us: 6 (Huawei, Qualcomm, ZTE, CATT, Spreadtrum, Xiaomi) 70us: 1 (Samsung) 100us: 2 (Apple, Murata) 140us: 9 (Apple, Qualcomm, ZTE, CATT, MediaTek, Spreadtrum, Samsung, Xiaomi, Murata) 140 + FFS us: 1 (Skyworks) 200us: 1 (Google) 210us: 4 (Qualcomm, ZTE, Samsung, Xiaomi). - Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations - 35us, 140us - RAN4 to further discuss 13us - LS will capture the currently confirmed values - It is not precluded to send another LS with more values in future meetings Issue 3: Switching period capability - Discuss whether the switching delay capability can be per band combination Issue 4: Time mask for switching - Discuss the time mask for switching based on the following options: - Option 1: Switching period can be either within carrier 1 (FDD) or within carrier 2 (SDL) - Option 2: Switching period is always within the switch-from carrier - Option 3: Switching period is contained within the switching gap configured by the network - Other options are not precluded Issue 5: Switching period location - Discuss the switching period location based on the following options: - Option 1: Switching period location is entirely up to RAN1 design - Option 2: Switching period location is defined relative to the slot boundary - Other options are not precluded Issue 6: UE reference architecture - Companies are encouraged to contribute proposals on the UE reference architecture as a basis for developing RF requirements for the feature Issue 7: Test methodology - Companies are encouraged to contribute proposals related to the testability of the time mask requirement

Liaison to other working groups

-	A liaison to RAN1 (with RAN2 in CC) was approved with the following content [36]:

# 1 Overall description RAN4 would like to update RAN1 on the progress of the discussions related to the switching time for low NR band carrier aggregation via switching. To accommodate different UE implementations RAN4 has reached the following agreement: - Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations - 35us, 140us. Additional value(s) are under discussion. If any additional value(s) are confirmed, RAN4 will provide them in a future LS. RAN4 respectfully requests RAN1 to take this information into account in their work. # 2 Actions To RAN WG1 ACTION: RAN4 respectfully asks RAN1 to take the above information into account.

RAN4 #114 – RRM (Core)

General aspects

-	The topic summary was prepared in [31]

-	An RRM adhoc was held, with the minutes captured in [33]

RRM aspects

-	A WF in [32] was approved with the following agreements:

Issue 1-1-1: RRM work scope - Interruption requirement - Alt 1: Needs interruption requirement for carrier switching between FDD CC and SDL CC (Apple, Xiaomi, CATT, HW, Samsung) - Capabilities for UEs with and without interruption (Samsung) - Alt2: RRM to reuse the RF defined switch delay and time mask. RAN4 RRM not to define additional interruption or scheduling restriction in RRM - Session Chair (RRM session): check the progress in RF session, and discuss if any aspects are not covered by the RF discussion. - Switch delay requirements Agreement: - The switching delay between operating the FDD and SDL carrier is to be discussed in the RF session. RAN4 RRM not to define additional switch delay in RRM. - Synchronization requirements for FDD and synchronization accuracy at BS antenna connectors or radiated interface boundaries Agreement: - No need to define new cell phase synchronization requirement in RRM spec for R19 LB CA via switching. - SDL SCC related RRM requirement - activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL activated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL activated SCell L3 measurement requirement if SSB based SCell operation is used. - de-activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL deactivated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL deactivated SCell L3 measurement requirement if SSB based SCell operation is used. - other SDL SCell requirements Agreement: - FFS: Whether and how - RAN4 to define a new SDL SCell activation/deactivation delay and related interruption for R19 LB CA - the existing SCell activation/deactivation delay and related interruption requirement can be used as baseline for R19 LB CA - FFS: Whether and how - RAN4 to define a new BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for R19 LB CA - the existing BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement can be used as baseline for R19 LB CA Agreement: - Not to change definition of intra-frequency/inter-frequency measurement with/without gap measurement - FFS on the use case for MG application. - Potential impacts to the PCell related requirement - FFS Issue 1-1-2: Assumptions for R19 LB CA via switching - (1) FFS: For a UE supporting low band CA via switching, SDL SCell operation is SSB based? or SSB-less based? or both? Agreement: - For a UE supporting low band CA via switching, RAN4 to discuss requirement for SDL SCell operation for both SSB-based SCell and SSB-less SCell based, and decide whether to define requirements for both cases or start with SSB-based in the next meeting. - (2) FFS: MRTD between the two low bands is CP? Or wait for TAE conclusion from RF session? Agreement: As generic assumption for discussing the RRM requirement, the received timing difference between the two bands at UE is within a value in option 1 or option 2, unless other TAE requirement will be defined in RF session. - Option 1: 4.67us - Option 2: 3 us

Liaison to other working groups

-	A liaison to RAN1 (with RAN2 in CC) was approved with the following content [34]:

# 1 Overall description RAN4 discussed the impact of LB CA via switching on RAN4 RRM requirements and would like to share the following information to RAN1. - In TS38.133, UL transmit timing requirements are applicable provided that at least one SSB is available at the UE during the last 160 ms. - The required availability of SSB on PCell and/or SCell for RRM requirements is still under discussion in RAN4. RAN1 does not need to wait for RAN4 discussion. # 2 Actions To RAN1: ACTION: RAN4 respectfully asks RAN1 to take the above into account in their future work.

#### 2.4.2	Remaining Open issues

Core part:

-	Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2}

-	Specify only a semi-static switching pattern based on RRC configuration [RAN1]

-	Specify the switching delay and time mask for carrier switching [RAN4]

-	Specify necessary RRM requirements [RAN4]

-	Define the corresponding UE capabilities [RAN4, RAN2, RAN1]

-	Specify requirements for the example band combination (CA_n5A-n29A) [RAN4]

Performance part:

-	Specify RRM test cases [RAN4]

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A.

## 4.	References

- R4-2500089	AT&T	Work plan for low NR band carrier aggregation via switching

- R4-2501119	MediaTek Inc.	Discussion on scope clarification for low-band CA switching

- R4-2500139	Huawei, HiSilicon	Discussion on LB-LB switching

- R4-2500313	Apple	Initial Views on UE RF requirements for LB CA via switching

- R4-2500405	Nokia	NR_LBCA_Sw Introduction

- R4-2500492	Qualcomm Incorporated	Initial aspects of RX Switching

- R4-2500644	Xiaomi	LB (FDD) LB (SDL) CA with switching

- R4-2500768	vivo	Discussion of UE RF impact for LB-LB switching

- R4-2500940	ZTE Corporation, Sanechips	Discussion on low-low band CA via switching

- R4-2501031	CATT	Discussion on time mask of low band carrier aggregation

- R4-2501121	MediaTek Inc.	Discussion on low-band CA switching

- R4-2501122	Spreadtrum, UNISOC	Discussion on UE RF requirements for LB-LB CA switching

- R4-2501169	Google	Discussion on low band carrier aggregation via switching

- R4-2501410	Samsung	Views on UE RF requirements for LB CA switching

- R4-2501572	Ericsson	Time masks for LB-LB CA switching

- R4-2501988	Skyworks Solutions, Inc.	NR Low-Band CA Switching

- R4-2502255	LG Electronics	UE RF requirements and UE capability of LB carrier aggregation

- R4-2500237	Apple	On RRM requirement for R19 LB CA

- R4-2500645	Xiaomi	LB (FDD) LB (SDL) CA with switching

- R4-2500828	vivo	Discussion on RRM requirements for low band carrier aggregation via switching

- R4-2500894	LG Electronics	Discussion on RRM requirements for Low Band CA operation

- R4-2500914	ZTE Corporation, Sanechips	Discussion on RRM aspects of R19 Low band carrier aggregation

- R4-2501048	CATT	Discussion on the impacts of low band CA via switching on RRM requirements

- R4-2501236	Huawei, HiSilicon	Discussion on low band carrier aggregation via switching

- R4-2501680	Samsung	General discussion on RRM requirements for LB CA switching

- R4-2502061	Ericsson	Discussion on low band carrier aggregation via switching

- R4-2502081	MediaTek inc.	Discussion on low-band CA switching

- R4-2502148	Nokia	On LBCA switching RRM requirements

- R4-2502170	Qualcomm Incorporated	RRM requirements for low band carrier aggregation via switching

- R4-2502875	Apple Benelux B.V.	Skeleton TR 38.768 on low NR band carrier aggregation via switching v0.0.2

- R4-2500527	Moderator (Apple)	Topic summary for [114][213] NR_LBCA_Sw

- R4-2502588	Moderator (Apple)	WF on RRM requirements for NR_LBCA_Sw

- R4-2502663	Moderator (Apple)	AdHoc minutes for [114][213] NR_LBCA_Sw

- R4-2502697	Apple	LS of RAN4 RRM agreements on LB CA via switching

- R4-2502853	Moderator (Apple)	Topic summary for [114][126] NR_LBCA_Sw

- R4-2502877	AT&T	LS on switching periods for low-low band switching

- R4-2503016	Apple	WF on low-low band switching

04.02.2025		minor adaptations for RAN #107

17.10.2024		minor adaptations for RAN #106 and 2.6 for RAN6 removed

12.08.2024		minor adaptations for RAN #105

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template