3GPP TSG RAN Meeting #107								             RP-250269

Incheon, Korea, March 12-14, 2025

Agenda item:	9.3.1.7

Source:	CMCC, Huawei, HiSilicon, Spreadtrum

Title:	Discussion on the scope of Rel-19 Ambient loT WID

Document for:	Discussion and decision

- Introduction

In RAN#106, a new WID for solutions for Ambient IoT in NR has been approved [1].

In RAN1#120 meeting, companies shared different interpretation on whether no use of FEC is allowed for D2R transmissions. It should be obvious that not using FEC is permitted, but in case clarifications are needed in RAN, in this contribution, we provide our views as to why.

- Discussion

In the WID, the RAN1 scope on D2R FEC is recapped as follows:

The following objectives are set, within the General Scope: - RAN1 scope: - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis).

The adverb “only” in the above sentence refers to convolutional code with generator polynomials as per TS 36.212, which means that convolutional code with generator polynomials other than that defined in LTE is not allowed. It has no relation to whether convolutional code should be always enabled to be used. That's a separate discussion.

Observation 1: The WID precludes convolutional code with generator polynomials other than that defined in LTE. No statement can be found in the WID to imply that FEC should always be applied for all D2R transmissions.

In the study item, the evaluation for D2R link performance assumes FEC as baseline due to the reason that the feasibility studies in study item are more related to the coverage evaluations. To improve the coverage and evaluate the edge of coverage, D2R transmission should adopt FEC. However, in practical applications, there are typical use cases such as inbound logistics, where a large number of tagged merchandises passes through the gate of an indoor factory within a few seconds and requires completing the inventory of all the merchandise for several rounds [2]. In such a case, the reader is known in advance, as a fact of the deployment choices, to be very close to devices which do not require large coverage. Instead, high data rates and high peak inventory rates are important to meet the service requirements. From this perspective, it is not reasonable to always use FEC for D2R transmission with a much lower code rate and data rate.

Notably, some competing radio technologies do not use FEC at all, meaning while they have poorer coverage, they have a higher uncoded ‘baseline’ data rate by 640 kbps.

Observation 2: System design should be designed with scalability in mind to support diverse use cases. Typical use cases such as inbound logistics require completing the inventory of large number merchandise for several rounds in a few seconds. High data rates are important to meet the requirement of such services. It is obviously advantageous to disable FEC as needed for D2R transmission with a higher data rate.

In addition, the design of Ambient IoT should make sure that its peak data rate for both R2D and D2R links are competitive enough. During the discussion in RAN1#120 meeting, it has been agreed that the maximum M value (i.e. chips per OFDM symbol) for R2D OOK-4 modulation should be no less than 16 to enable a high R2D data rate. Similarly, for D2R, we should also guarantee that the peak data rate is competitive, e.g., up to 640 kbps. Note that if the device sampling clock is limited to around 2 MHz and one D2R chip should contain at least 2-3 samples, in this regard, with FEC 1/3 code rate applied all the time, combined with line coding (e.g., at least 2 chips for one bit), it is impossible for D2R transmission to achieve 640 kbps.

Observation 3: if the device sampling clock is around 2Msps, it is impossible to achieve up to 640 kbps peak data rate in the D2R link, if FEC is always applied.

Therefore, we propose to clarify that no use of FEC for D2R transmission is permitted in RAN1 scope.

Proposal 1: RAN plenary confirms that the D2R convolutional code is not always applied.

- Conclusion

This contribution discusses the clarifications on the WID for D2R FEC, and provides the following observations and proposals:

Observation 1: The WID precludes convolutional code with generator polynomials other than that defined in LTE. No statement can be found in the WID to imply that FEC should always be applied for all D2R transmissions.

Observation 2: System design should be designed with scalability in mind to support diverse use cases. Typical use cases such as inbound logistics require completing the inventory of large number merchandise for several rounds in a few seconds. High data rates are important to meet the requirement of such services. It is obviously advantageous to disable FEC as needed for D2R transmission with a higher data rate.

Observation 3: if the device sampling clock is around 2Msps, it is impossible to achieve up to 640 kbps peak data rate in the D2R link, if FEC is always applied.

Proposal 1: RAN plenary confirms that the D2R convolutional code is not always applied.

References

- RP-242236, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA

- RP-250455, Rel-19 Ambient IoT WI specifications and other issues, Huawei, HiSilicon