3GPP TSG RAN meeting #107										RP-250310

Incheon, S. Korea, March 12-14, 2025

## Status Report to TSG

Agenda item:			9.3.1.3

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_AIML_air |  |  |  |  |
| Unique ID | 1020093 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-243244 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: Sept ‘25 | Performance part: March ‘26 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 65% | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Juan Montojo (RAN1); Xiaofeng Liu (RAN4); Marco Belleschi (RAN2/RAN3) |
|  | Company | Qualcomm; CAICT; Ericsson |
|  | Email | juanm@qti.qualcomm.com; liuxiaofeng1@caict.ac.cn; marco.belleschi@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

##### 2.1.1.1	RAN1#120

# Specification support for Beam Management

Agreement

For report content of inference results for UE-sided model, where the largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size.

Note: the model output is UE implementation and it doesn’t have to be RSRP subject to dBm value.

Agreement

For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

Note: how to capture it in the spec is a separate discussion.

Agreement

For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances,

- the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP.

  - The time instance information of the beam with the largest RSRP are additionally indicated in the report.

Agreement

For inference, for BM-Case 2 of UE-side model,

- The time gap between two consecutive future time instances is configured by RRC, and the number of future time instance(s) N is configured by RRC.

  - time gap is [10ms, 20ms, 40ms, 80ms, 160ms]

  - N = [1, 2, 4, 8]

- Reference time of the earliest time instance for the predicted results is based on the most recent occasion of the CSI-RS/SSB resource in Set B for measurement

  - Where the most recent occasion of the CSI-RS/SSB resource of set B is the latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource of the corresponding inference report.

Agreement

For UE-sided model, for configuring the resource for data collection purpose, support

- CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report.

  - One CSI-ResourceConfigId is configured for Set A.

  - One CSI-ResourceConfigId is configured for Set B.

  - Note: UE performs measurement on all resources

  - One or two associated IDs can be configured in CSI-ReportConfig

    - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured,

    - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B

- FFS: whether/how to support 'aperiodic' CSI RS

Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

Agreement

For UE-sided model, in CSI-ReportConfig for inference

- One or two associated IDs can be configured in CSI-ReportConfig

  - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured,

  - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B

- FFS: At least BM-Case 1, the applicability for 'aperiodic' CSI RS

# Specification support for positioning accuracy enhancement

Conclusion

For AI/ML based positioning Case 3a, regarding the time stamp in a measurement report from gNB to LMF,

- Existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective.

Agreement

For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting:

- k = {0...5}

Agreement

For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt' values” in RAN1#119 agreement is resolved by supporting:

- Nt' = {8, 16, 24}

Agreement

For AI/ML based positioning Case 1, from RAN1 perspective, when the label data of location is generated by LMF and transferred from LMF to UE, label and quality indicator of label can be provided by reusing existing IEs.

- From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label.

Agreement

For AI/ML based positioning,

- When channel measurement is to be reported/sent, time stamp of channel measurement refers to the time instance when the channel measurement is performed.

- When label is to be reported/sent, time stamp of label is the time instance for which the label is valid.

Agreement

Send Reply LS R1-2501522 to SA2 about Case 2b with the content of Section 2 of R1-2501521 by revising

“RAN1 would continue work and may provide additional input in the future to SA2 for Case 2b if any new agreement reached in RAN1.”

To

“RAN1 may provide additional input in the future to SA2 for Case 2b if any new agreement reached in RAN1.”

Agreement

Send Reply LS R1-2501524 to SA2 about Case 3b with the content of Section 3 of R1-2501521 by adding all the agreement made in this meeting for Case 3b.

Agreement

Final LS R1-2501523 is endorsed.

Final LS R1-2501525 is endorsed.

Agreement

For AI/ML based positioning Case 3b, with the existing definition of two channel measurement types (A) or type (B):

(A) path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), (B) Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b).

- Descriptive info of the channel measurement is included together with the channel measurement values, either explicitly or implicitly.

- The descriptive info of the channel measurement includes at least:

  - measurement type (A) or type (B) and

  - measurement parameters.

- Regarding measurement parameters,

  - For (A) path-based measurement, the measurement parameters include: number of additional paths, k.

  - For (B) Rel-19 enhanced measurement, the measurement parameters include at least: Nt', k.

    - FFS: Nt

- Note: Type (A) and Type (B) are used for discuss purpose only.

Conclusion

For Rel-19 AI/ML based positioning Case 3b,

- If the gNB cannot report measurements according to measurement type (A) or measurement type (B) requested by LMF, the gNB responds to LMF that: the gNB cannot provide measurement with the requested measurement type.

  - The gNB does not respond to LMF with a measurement report where the measurement type is different from requested.

- It is up to RAN3 to decide the signaling details.

  - Note: It is RAN1 understanding that this is the same behavior as in legacy when the NG-RAN node cannot support the requested measurement.

Agreement

RAN1 send an LS to RAN3 and RAN2 to inform them above agreements and conclusions.

Agreement

- For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed:

- The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted).

- FFS: other information is not precluded (e.g., if Part B is valid for a duration)

Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications.

RAN1 send an LS to RAN3 and RAN2 to inform the above.

Agreement

Draft LS R1-2501627 is endorsed in principle.

Agreement

Final R1-2501628 is endorsed.

# Specification support for CSI prediction

Agreement

For CSI prediction using UE-side model, at least for inference, Rel-18 CSI framework is reused.

- For CSI-RS resource type for CMR, periodic, semi-persistent, and aperiodic CSI-RS are supported.

- For inference report,

  - for N4>=1, support Rel-18 codebook ('typeII-Doppler-r18')

Agreement

For CSI prediction using UE-side model, if performance monitoring type 1 or 3 is supported, for calculation of monitoring metric, support

- Based on intermediate KPI

  - Down select between SGCS and NMSE by RAN1#120bis

  - FFS on the definition of SGCS/NMSE and how to calculate monitoring metric

Agreement

For CSI prediction using UE-side model, for CSI processing criteria and timeline, at least for inference further study on

- Whether the CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting

- Whether the Processing Unit should be shared or separately counted among AI/ML related features/functionalities.

- Whether new timeline is needed/updated for inference, and whether a different timeline is needed when functionality switches/activates.

- Whether legacy framework for active CSI-RS resource and port counting can be reused

Note: Strive to study CSI processing criteria considering both BM and CSI case, and take the existing solutions as starting point.

Agreement

For CSI prediction using UE-side model, for data collection for training,

- For resource configuration,

  - For CSI-RS resource type for CMR, periodic and semi-persistent are supported.

    - FFS on support of aperiodic CSI-RS resource

  - FFS on whether to separately configure CSI-RS resource(s) used for measurements for model input and CSI-RS resource(s) used for measurements for ground-truth CSI

  - FFS whether/how to enhance CSI-RS resource configuration to reduce CSI-RS signalling overhead.

- FFS on CSI report configuration

#### 2.1.2	Remaining Open issues

- Finalize specification support for beam management

- Finalize specification support for positioning accuracy enhancements

- Finalize specification support for CSI prediction

## 2.2	RAN2

#### 2.2.1	Agreements

##### 2.2.1.1	RAN2#129

RAN2#129 discussed LCM for UE-sided model for Beam Management use case and made the following agreements:

- Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

- The full inference configuration is sent in CSI-ReportConfig

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability

- FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

- Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.   FFS if we report explicit cause

- If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

- The provided periodic CSI configuration should be consistent with reported UE capabilities

- FFS option B

- Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

  - Semi-persistent reporting, activated by MAC CE/DCI

  - Aperiodic CSI reporting, activated by DCI

RAN2#129 discussed NW-side data collection for the beam management use case, and made the following agreements:

- Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.    FFS what to log

- Low power bit indication is supported

- Data availability indication is supported.  FFS when this would be triggered

- As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

- UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

- UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

- FFS how to handle idle/inactive and RLF cases and whether we have a unified

RAN2#129 discussed UE-side data collection for the UE-side use cases (including the beam management use case), focusing on data collection configuration, and made the following agreements:

- Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration:

  - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration.

  - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop).  FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network.

  - The network can provide or release the data collection configuration (at any point in time), with or without UE request.

  - The following methods for network control of the initiation and configuration for data collection:

    - The network can decide when to start/stop the data collection and send configuration.

    - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

RAN2#129 discussed LCM for UE-sided model for Positioning use case and made the following agreements:

- RAN2 has not and will not study Case 2a and Case 2b in Rel-19. An LS was sent to SA2 to notify this agreement (LS available in R2-2501507)

- Introduce AI/ML positioning Case 1 as a new positioning method

- Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extention can be disucssed while drafting the stage 3 CR.

- FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.  FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation.

- The content of error cause is discussed while drafting stage3 CRs.

- As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list.

- UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality.   FFS if any additional LMF control is needed.

RAN2 also discussed organizational aspects related to the handling of running CRs.

Further, the following post-meeting email discussions were agreed:

[POST129][024][AI PHY] Stage 2 running CR (Vivo)

Intended outcome:  update running CR to be submitted to next meeting

Deadline:  Mar.  21st 10:00 UTC

[POST129][025][AI PHY] RRC running CR (Ericsson)

Intended outcome:  update running CR to be submitted to next meeting

Deadline:  Mar.  21st 10:00 UTC

#### 2.2.2	Remaining Open issues

Related to the topic of LCM for UE-sided model for Beam Management use case, RAN2 can continue analyzing the details of LCM-related signalling procedures, focusing on the FFS agreed during RAN2#129. Further, RAN2 should start the discussion on how to capture in normative procedures the agreements reached so far on this topic, with focus on the running CRs for TS 38.300 and TS 38.331.

Related to the topic of NW-side data collection, RAN2 can continue the discussion on configuration and reporting mechanisms to enable NW-side data collection, focusing on the FFS agreed during RAN2#129. Further, RAN2 should start the discussion on how to capture in normative procedures the agreements reached so far on this topic, with focus on the running CRs for TS 38.300 and TS 38.331.

Related to the topic of LCM for Positioning use case, RAN2 can continue the discussion on the FFS agreed during RAN2#129, e.g. on how to switch between AIML and non-AIML positioning methods.

Related to the topic of UE-side data collection, RAN2 can continue the discussion on the mechanisms to configure the UE (with or without UE request) for the UE-side data collection. Further, RAN2 should start the discussion on how to capture in normative procedures the agreements reached so far on this topic, with focus on the running CRs for TS 38.300 and TS 38.331.

## 2.3	RAN3

#### 2.3.1	Agreements

##### 2.3.1.1	RAN3#127

RAN3#127 discussed the topic of Positioning Accuracy Enhancements, a work plan was presented in R3-250488.

A summary of discussion is available in R3-250873. Based on the discussion, RAN3 made the following agreements:

- Turn the “WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.” to an agreement.

- For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner.

- For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs.

- WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB

- To be continued: Stage 2 (TS 38.305) text proposals for case 3a data collection. Companies are encouraged to consolidate solutions, if possible

Further, RAN3 agreed to send an LS to SA2 (Cc: RAN1, RAN2), on LMF-based AI/ML Positioning for case 3b responding to SA2 questions on the editor’s notes in TS 23.273 pending RAN3 signalling. The LS is available in R3-250796.

#### 2.3.2	Remaining Open issues

Related to the topic of Positioning Accuracy Enhancements, RAN3 should continue the discussion on the following FFSs discussed during RAN3#127:

- FFS how to support data collection in case 3a

- Continue the discussion on the stage 2 (TS 38.305) and stage 3 text proposals for support of case 3a and case 3b

## 2.4	RAN4

#### 2.4.1	Agreements

##### 2.4.1.1	RAN4#114

CSI reporting requirement framework for CSI prediction

Issue 1-2: Reference model for CSI prediction

Agreement:

- Assume no reference AI model for CSI prediction as baseline

  - If it is difficult to align the results corresponding to test metric, RAN4 will discuss the reference model.

Issue 1-3: Training and test data consistency

Agreement:

- further discuss the following options

  - Option 1: Consistency between training data and test data should be ensured

    - RAN4 to study how to ensure consistency between training data(simulation) and test data (conformance test) and whether anything special should be done during the test

  - Option 2: Consistency will be implicit as test emulated channel will be very similar to the one used in RAN4 simulations

Issue 1-4: Generalization

Agreement:

- Postpone this discussion until the requirement and test definition is clear

Issue 1-5: Simulation parameters

Agreement:

Simulation assumptions for CSI prediction is agreed in R4-2503026

Testability and interoperability issues for beam management

Issue 2-1: Metrics/KPIs for beam prediction

Agreement:

If RSRP and beam ID are reported:

- RSRP accuracy will be one of the KPIs.

  - How option 1 is applied is FFS

- Beam ID prediction accuracy will be one of the KPIs

  - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options

If only beam ID is reported:

- Beam ID prediction accuracy will be used as the KPI

  - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options

Issue 2-4: QCL Source RS for TCI states

Agreement:

wait for RAN1 discussion to conclude without sending any LS

Issue 2-6:	Simulation results

Agreement:

Updated simulation assumptions for beam prediction is agreed in R4-2503021

Requirements for positioning

Issue 3-4: Report mapping for UL SRS-RSRP

Agreement:

- Reuse the report mapping for UL SRS-RSRP to Case 3a

#### 2.4.2	Remaining Open issues

- CSI reporting requirement framework for CSI prediction

  - Test baseline

  - Reference model for alignment

  - Training data and test consistency

  - Generalization

  - Parameters for simulations/tests

- Requirements for Beam managements

  - Beam prediction KPIs

  - Relative RSRP accuracy

  - Measurement period for inference

  - QCL source RS for TCI states

  - Requirements based on relative difference

  - Simulation results

  - Measurement error modelling

- Requirements for Positioning Simulation results

  - Requirements for case 1

  - LOS/NLOS indicator requirements

  - Timing information reporting from UE/gNB to LMF

  - Report mapping for UL SRS-RSRP for Case 3a/3b

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

## 4.1	RAN1

#### 4.1.1	RAN1#120

R1-2501143	Rapporteur view on higher layer signalling of Rel-19 AI-ML for NR air interface	Qualcomm Incorporated

# Specification support for beam management

R1-2501594	FL summary #5 for AI/ML in beam management	Samsung (Moderator)

R1-2501443	FL summary #4 for AI/ML in beam management	Samsung (Moderator)

R1-2501442	FL summary #3 for AI/ML in beam management	Samsung (Moderator)

R1-2501441	FL summary #2 for AI/ML in beam management	Samsung (Moderator)

R1-2501440	FL summary #1 for AI/ML in beam management	Samsung (Moderator)

R1-2500050	Discussion on specification support for AI/ML-based beam management	FUTUREWEI

R1-2500066	Discussion on AI/ML-based beam management	ZTE Corporation, Sanechips

R1-2500089	Discussion on AI/ML for beam management	Huawei, HiSilicon

R1-2500159	Discussion on AIML for beam management	Spreadtrum, UNISOC

R1-2500201	Discussion on AI/ML-based beam management	CATT

R1-2500254	Discussion on AI/ML for beam management	China Telecom

R1-2500274	Discussion on specification support for beam management	CMCC

R1-2500337	Specification support for beam management	vivo

R1-2500390	Specification Support for AI/ML for Beam Management	Kyocera

R1-2500391	AI/ML for beam management	Ericsson

R1-2500404	Specification support for beam management	Tejas Networks Limited

R1-2500465	On specification for AI/ML-based beam management	OPPO

R1-2500512	Discussion on AI/ML for beam management	Ofinno

R1-2500529	Discussion on AI/ML for beam management	InterDigital, Inc.

R1-2500545	AI/ML based Beam Management	Google

R1-2500555	Discussion on AIML beam management	TCL

R1-2500560	Discussion on specification support for beam management	Panasonic

R1-2500565	Discussions on AI/ML for beam management	LG Electronics

R1-2500590	Discussion on specification support for beam management	NEC

R1-2500635	AI/ML specification support for beam management	Lenovo

R1-2500642	Discussion on Specification Support for Beam Management	Sony

R1-2500669	Discussion on specification support for beam management	Ruijie Networks Co. Ltd

R1-2500686	Specification support for AI-enabled beam management	NVIDIA

R1-2500710	Discussion on AI/ML for beam management	Xiaomi

R1-2500766	Enhancements for AI/ML enabled beam management	Apple

R1-2500834	Discussion for supporting AI/ML based beam management	Samsung

R1-2500877	Specification support for beam management	KDDI Corporation

R1-2500900	Discussion on specification support for beam management	ETRI

R1-2500925	Discussion on specification support on AI/ML for beam management	Fujitsu

R1-2500962	Discussion on specification support for AI/ML beam management	Transsion Holdings

R1-2500970	AI/ML for Beam Management	Nokia

R1-2501013	Discussion on specification support for AIML-based beam management	MediaTek Inc.

R1-2501085	AI/ML for Beam Management	Meta

R1-2501103	Specification support for beam management	Intel Corporation

R1-2501104	Discussion on AI/ML for beam management	HONOR

R1-2501130	Discussions on specification support for beam management	Sharp

R1-2501144	Specification support for AI-ML-based beam management	Qualcomm Incorporated

R1-2501190	Discussion on AI/ML for beam management	NTT DOCOMO, INC.

R1-2501226	Discussion on AI/ML based beam management	KT Corp.

R1-2501235	Discussion on AIML based beam management	ASUSTeK

R1-2501262	Discussions on AI/ML for beam management	CAICT

R1-2501270	Specification support for beam management	Fraunhofer HHI, Fraunhofer IIS

R1-2501333	Specification support for AI/ML beam management	ITL

# Specification support for positioning accuracy enhancement

R1-2501414	Summary #5 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2501413	Summary #4 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2501412	Summary #3 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2501411	Summary #2 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2501410	Summary #1 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

R1-2501521	Summary#1 on reply LS to SA2 on LMF-based AI/ML Positioning for case 2b and 3b	Moderator (vivo)

R1-2500060	AI/ML for Positioning Accuracy Enhancement	Ericsson

Late submission

R1-2500067	Discussion on AI/ML-based positioning enhancement	ZTE Corporation, Pengcheng Laboratory

R1-2500090	Discussion on  AI/ML for positioning accuracy enhancement	Huawei, HiSilicon

R1-2500202	Discussion on AI/ML-based positioning	CATT, CICTCI

R1-2500275	Discussion on specification support for positioning accuracy enhancement	CMCC

R1-2500338	Specification support for positioning accuracy enhancement	vivo

R1-2500405	Specification support for positioning accuracy enhancement	Tejas Networks Limited

R1-2500466	On specification for AI/ML-based positioning accuracy enhancements	OPPO

R1-2500518	Discussion on specification support for AI-ML based positioning accuracy enhancement	Baicells

R1-2500546	AI/ML based Positioning	Google

R1-2500556	Discussion on AIML positioning	TCL

R1-2500606	Discussion on specification support for AIML based positioning accuracy enhancement	NEC

R1-2500636	Specification impacts for AI/ML positioning	Lenovo

R1-2500643	Specification support for AI/ML for positioning accuracy enhancement	Sony

R1-2500670	Discussion on specification support for positioning accuracy enhancement	Ruijie Networks Co. Ltd

R1-2500687	Specification support for AI-enabled positioning	NVIDIA

R1-2500711	Discussion on AI/ML-based positioning accuracy enhancement	Xiaomi

R1-2500746	Discussion on support for AIML positioning	InterDigital, Inc.

R1-2500767	Discussion on Specification Support for AI/ML-based positioning	Apple

R1-2500835	Discussion for supporting AI/ML based positioning accuracy enhancement	Samsung

R1-2500901	Discussion on specification support for positioning accuracy enhancement	ETRI

R1-2500926	Discussion on specification support for AIML-based positioning accuracy enhancement	Fujitsu

R1-2500971	AI/ML for Positioning Accuracy Enhancement	Nokia

R1-2500991	AI/ML positioning accuracy enhancement	Fraunhofer IIS, Fraunhofer HHI

R1-2501131	Discussion on specification support for AI/ML based positioning accuracy enhancements	Sharp

R1-2501145	Specification support for AI-ML-based positioning accuracy enhancement	Qualcomm Incorporated

R1-2501191	Discussion on AI/ML for positioning accuracy enhancement	NTT DOCOMO, INC.

R1-2501247	Discussion on Specification Support of AI/ML for Positioning Accuracy Enhancement	Indian Institute of Tech (M)

R1-2501259	Design for AI/ML based positioning	MediaTek Korea Inc.

R1-2501273	Discussion on specification support for AI/ML Positioning Accuracy enhancement	CEWiT

R1-2501283	Discussions on specification support for positioning accuracy enhancement for AI/ML	ITL

# Specification support for CSI prediction

R1-2501610	Summary #4 of CSI prediction	Moderator (LG Electronics)

R1-2501530	Summary #3 of CSI prediction	Moderator (LG Electronics)

R1-2501529	Summary #2 of CSI prediction 	Moderator (LG Electronics)

R1-2501528	Summary #1 of CSI prediction 	Moderator (LG Electronics)

R1-2500057	AI/ML for CSI prediction	Ericsson

R1-2500068	Discussion on specification support for AI CSI prediction	ZTE Corporation, Sanechips

R1-2500091	Discussion on AI/ML for CSI prediction	Huawei, HiSilicon

R1-2500160	Discussion on AIML for CSI prediction	Spreadtrum, UNISOC

R1-2500203	Discussion on AI/ML-based CSI prediction	CATT

R1-2500276	Discussion on AI/ML for CSI prediction	CMCC

R1-2500319	Discussion on CSI Prediction	TCL

R1-2500339	Specification support for CSI prediction	vivo

R1-2500406	Specification support for CSI Prediction	Tejas Networks Limited

R1-2500467	On specification for AI/ML-based CSI prediction	OPPO

R1-2500533	On AI/ML-based CSI prediction	InterDigital, Inc.

R1-2500547	AI/ML based CSI Prediction	Google

R1-2500557	Discussion on AIML CSI prediction	TCL

Withdrawn

R1-2500566	Discussions on CSI prediction	LG Electronics

R1-2500600	Discussion on specification support for CSI prediction	NEC

R1-2500637	Specification support for CSI prediction	Lenovo

R1-2500644	Specification Support for AI/ML CSI prediction	Sony

R1-2500688	Specification support for AI-enabled CSI prediction	NVIDIA

R1-2500712	Discussion on AI/ML model based CSI prediction	Xiaomi

R1-2500768	Discussion on AI/ML based CSI prediciton	Apple

R1-2500816	Discussion on AI/ML-based CSI prediction	Panasonic

R1-2500836	Views on AI/ML based CSI prediction	Samsung

R1-2500902	Discussion on specification support for CSI prediction	ETRI

R1-2500927	Discussion on specification support for CSI prediction	Fujitsu

R1-2500972	AI/ML for CSI Prediction	Nokia

R1-2501014	AI/ML - Specification support for CSI Prediction	MediaTek Inc.

R1-2501080	Discussion on AI/ML for CSI prediction	AT&T

R1-2501132	Discussion on specification support for AI/ML based CSI prediction	Sharp

R1-2501146	Specification support for CSI prediction	Qualcomm Incorporated

R1-2501192	Discussion on AI/ML for CSI prediction	NTT DOCOMO, INC.

## 4.2	RAN2

### 4.2.1	RAN2#129

80 contributions (for details see agenda items 8.1.1, 8.1.2, 8.1.3 in Tdoc list)

## 4.3	RAN3

### 4.3.1	RAN3#127

29 contributions (for details see agenda item 20 in Tdoc list)

## 4.3	RAN4

#### 4.3.1	RAN4#114

// General aspects

R4-2500045	Discussion on general aspects for AI/ML air

Type: discussion		For: Discussion
					Source: Korea Testing Laboratory

Decision:		Noted.

R4-2500224	Discussion on General Aspects on AI/ML for NR Air Interface

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2500511	Discussion on general aspects of AI/ML for NR air interface

Type: discussion		For: Discussion
					Source: CAICT

Decision:		Noted.

R4-2500616	Discussion on general aspect for AI

Type: discussion		For: Discussion
					Source: Xiaomi

Decision:		Noted.

R4-2500821	Discussion on general aspects for AIML

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2501034	Discussion on general aspects for AIML for NR air

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2501094	Discussion on general aspects

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2501172	Discussion on the AI/ML general aspects

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision:		Noted.

R4-2501526	Discussion on general aspects on AIML for NR air interface

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2501610	On LCM requirements for AI/ML

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

On LCM requirements for AI/ML

Decision:		Noted.

R4-2501956	General aspects of AI/ML for NR Air interface

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2502167	Post Deployment Testing of AI-ML

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

// CSI reporting requirement framework for CSI prediction

R4-2500228	Discussion on CSI Prediction: CSI reporting requirement framework for CSI prediction

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2500270	CSI reporting requirement framework for CSI prediction

Type: other		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2500338	Discussion on CSI reporting requirement framework for CSI prediction

Type: discussion		For: Discussion
					Source: MediaTek inc.

Decision:		Noted.

R4-2500512	Discussion on RRM core requirement for CSI prediction

Type: discussion		For: Discussion
					Source: CAICT

Decision:		Noted.

R4-2500822	Discussion on CSI reporting requirement framework for CSI prediction

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2501035	Discussion on CSI reporting requirement framework for CSI prediction

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2501093	Discussion on CSI prediction

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2501499	Discussion on CSI prediction

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

This contribution discusses the AI/ML-based CSI prediction.

Decision:		Noted.

R4-2501508	Discussion on testability and interoperability issues for CSI prediction

Type: other		For: Approval
					Source: ZTE Corporation, Sanechips

Decision:		Noted.

R4-2501527	Discussion on CSI reporting requirement framework for AIML CSI predictio

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2502032	View on CSI reporting requirements framework for CSI prediction

Type: discussion		For: Discussion
					Source: Samsung

Decision:		Noted.

R4-2502070	CSI reporting requirement framework for CSI prediction

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2502166	CSI Prediction: Core Requirements and Testability Issues

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

// RRM core requirement for beam management

R4-2500085	Views on outstanding Testability and Interoperability Issues for Beam Management

Type: discussion		For: Discussion
					Source: VIAVI Solutions

Decision:		Noted.

R4-2500225	Discussion on Beam Management: RRM Core Requirements

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2500274	RRM core requirement for beam management

Type: other		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2500513	Discussion on RRM core requirements for beam management

Type: discussion		For: Discussion
					Source: CAICT

Decision:		Noted.

R4-2500593	Discussions on RRM core requirement for beam management

Type: discussion		For: Discussion
					Source: NTT DOCOMO, INC.

Decision:		Noted.

R4-2500612	Discussion on RRM requirement for beam management

Type: discussion		For: Discussion
					Source: Jio

Decision:		Noted.

R4-2500618	Discussion on core part requirement for AI beam management

Type: discussion		For: Discussion
					Source: Xiaomi

Decision:		Noted.

R4-2500823	Discussion on RRM core requirements for beam management

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2500888	Discussion on RRM core requirements of AI/ML BM use case

Type: discussion		For: Discussion
					Source: MediaTek Inc.

Decision:		Noted.

R4-2501036	Discussion on RRM core requirements for BM

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2501092	Discussion on RRM requirements for beam management

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2501170	Discussion on the RRM requirements of AI/ML Beam management

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision:		Noted.

R4-2501284	Discussion on RRM core requirements for beam management

Type: discussion		For: Discussion
					Source: Ericsson

Abstract:

Discussion on RRM core requirements for beam management

Decision:		Noted.

R4-2501528	Discussion on RRM core requirement for AIML Beam management

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2501692	RRM core requirement for AI/ML based beam management

Type: discussion		For: Discussion
					Source: Tejas Network Limited

Decision:		Noted.

R4-2501726	Discussion on test setup for AI/ML based beam management

Type: discussion		For: Discussion
					Source: Rohde & Schwarz

Decision:		Noted.

R4-2501957	RRM core requirements for AI/ML Based Beam Management

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2501964	Further discussion on beam management core requirements

Type: other		For: Approval
					Source: Samsung

Decision:		Noted.

R4-2502165	RRM Core Requirements for AI/ML Beam Management

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

// RRM core requirement for Positioning accuracy enhancement

R4-2500229	Discussion on RRM Core Requirements for positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Apple

Decision:		Noted.

R4-2500272	RRM core requirement for Positioning accuracy enhancement

Type: other		For: Approval
					Source: OPPO

Decision:		Noted.

R4-2500514	Discussion on RRM core requirements for positioning

Type: discussion		For: Discussion
					Source: CAICT

Decision:		Noted.

R4-2500824	Discussion on RRM core requirements for positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: vivo

Decision:		Noted.

R4-2501037	Discussion on RRM core requirements for positioning

Type: discussion		For: Discussion
					Source: CATT

Decision:		Noted.

R4-2501095	Discussion on RRM requirements for positioning

Type: discussion		For: Discussion
					Source: CMCC

Decision:		Noted.

R4-2501171	Discussion on the RRM requirements of AI/ML positioning

Type: other		For: Approval
					Source: ZTECorporation,Sanechips

Decision:		Noted.

R4-2501529	Discussion on RRM core requirement for AIML Positioning

Type: discussion		For: Discussion
					Source: Huawei, HiSilicon

Decision:		Noted.

R4-2501758	On requirements for AI/ML based positioning

Type: other		For: Approval
					Source: Ericsson

Abstract:

This paper discusses issues related to the AI/ML based positioning.

Decision:		Noted.

R4-2502071	RRM core requirements for AI/ML positioning accuracy enhancement

Type: discussion		For: Discussion
					Source: Nokia

Decision:		Noted.

R4-2502168	RRM Core Requirements for Positioning Accuracy

Type: discussion		For: Discussion
					Source: Qualcomm Incorporated

Decision:		Noted.

// Moderator summary and conclusions

R4-2500683	Topic summary for [114][131] NR_AIML_air_part1

Type: other		For: Information
					Source: Moderator(Qualcomm)

Abstract:

Summary for AI 7.19, 7.19.2, 7.19.3, 7.19.4, 7.20.2

Decision:		Noted.

Newly allocated tdocs in the first round

R4-2503012	Ad hoc minutes on AI/ML topic thread [131]

Type: other		For: Approval
					Source: Qualcomm

Decision:		Noted.

R4-2502855	Summary of simulation results for AI beam prediction

Type: other		For: Information
					Source: Vivo

Decision:		Noted.

R4-2503020	Summary of simulation results for AI beam prediction

Type: other		For: Information
					Source: Vivo

Decision:		Withdrawn.

R4-2502856	WF on the requirements for AI/ML air interface

Type: other		For: Approval
					Source: Qualcomm

Decision:		Approved.

R4-2502969	Updated simulation assumptions for beam prediction

Type: other		For: Approval
					Source: Vivo

Decision:		Revised to R4-2503021 (from R4-2502969).

R4-2503021	Updated simulation assumptions for beam prediction

Type: other		For: Approval
					Source: vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT

Decision:		Approved.

R4-2502970	Way forward on simulation assumptions for CSI prediction

Type: other		For: Approval
					Source: OPPO, CAICT, vivo, Nokia, Ericsson, Qualcomm, Huawei, Hisilicon, Mediatek, CATT, Samsung, ZTE Corporation, Apple

Qualcomm: add option 4 PC7 for CSI overhead.

Decision:		Revised to R4-2503026 (from R4-2502970).

R4-2503026	Way forward on simulation assumptions for CSI prediction

Type: other		For: Approval
					Source: OPPO, CAICT, vivo, Nokia, Ericsson, Qualcomm, Huawei, Hisilicon, Mediatek, CATT, Samsung, ZTE Corporation, Apple

Decision:		Approved.

Minutes and agreements in the online session and ad hoc

Please refer to the hyperlink below for the detailed minutes of the first round discussions on Wednesday

https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_114/Inbox/Drafts/%5B114%5D%5B100%5D%20Main%20Session/3.Wednesday/1.%5B131%5D_R4-2500683_online.docx

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template