3GPP TSG RAN #107			RP-250315

Incheon, Korea – March 12-14, 2025

Agenda item:	7

Source: 	Qualcomm Incorporated

Title: 	On PWS reception in RRC_CONNECTED mode for NB-IoT NTN

Document for:	Discussion and Decision

# Background

In [1], RAN2 inquired whether an NB-IoT NTN UE can receive PWS notifications in RRC_CONNECTED mode:

RAN2 discussed whether to introduce support for reception of PWS (ETWS and CMAS) notifications in RRC_CONNECTED for NB-IoT but were not able to conclude.

PWS notifications are delivered via paging or direct indication to trigger acquiring ETWS system information and CMAS system information.

Currently NB-IoT UEs are not required to monitor for paging or receive system information triggered by the paging in RRC_CONNECTED, and RAN2 would like to understand from RAN1 perspective whether it is feasible to introduce this in Rel-19.

ACTION:	 Please respond with an indication of whether it is feasible from RAN1 perspective to introduce support in Rel-19 for NB-IoT UEs reception of PWS notifications (in paging, direct indication, and system information) while in RRC_CONNECTED.

RAN1 replied in [2] to RAN2 and RAN as follows (emphasis added):

From RAN1 perspective, it may be possible to find solutions with specification impact for supporting NB-IoT UEs in RRC connected mode monitor PWS notifications, while some companies expressed concerns due to increased implementation complexity, power consumption at UE side and timely delivery of PWS notifications in NTN context. However, the amount of RAN1 workload for specifying this may not fit in the Rel-19 TU allocation for NTN. RAN1 would like to ask RAN plenary to decide whether RAN1 should work on specifying support of NB-IoT UEs in RRC connected mode monitor PWS notifications in Rel-19.

In this contribution, we present our views on this issue.

# Discussion

As explained in the original RAN2 LS, currently the UE is not required to monitor the NPDCCH search space and/or SIBs during RRC_CONNECTED mode. The main reason to do this back in Rel-13 was to reduce the UE complexity in terms of parallel monitoring of search spaces and number of decodes / DCI sizes to handle.

Supporting the monitoring of PWS notifications in RRC_CONNECTED would be beneficial for the following reasons::

- Currently, in the absence of monitoring PWS notifications during RRC_CONNECTED, the eNB would need to release all UEs to RRC_IDLE for them to receive the PWS message.

- Releasing all connected UEs may result in a very large overhead for the eNB (due to the need to send one release message to each individual UE), which will increase the delay in acquiring the PWS notification.

- Furthermore, due to releasing the RRC connection, the ongoing session may be terminated, impacting UE experience for some use cases and potentially becoming critical if voice support is introduced in Rel-20.

We think it would be feasible and beneficial to specify monitoring of CSS to receive PWS during connected mode, as long as it does not increase the UE complexity. For example, if the USS and CSS are TDM'd (i.e., there is no simultaneous monitoring of both), the UE could monitor both without incurring additional UE complexity. This behavior could be introduced with minimal RAN1 specification impact. Alternatively, a solution similar to the one specified for eMTC could be reused, although this may increase the specification work (since NB-IoT does not support CSS in connected mode).

There were some comments in RAN1 regarding potential impact on UE complexity and UE power consumption. The solution highlighted above would have minimal impact on these aspects:

- UE complexity: By TDM’ing the search spaces, the UE complexity (in terms of blind decodes) is not increased with respect to current specifications. The eNB (or some constraint on RAN1 specifications) should ensure that the UE is not required to do simultaneous monitoring (i.e., in the same slots) of both search spaces.

- UE power consumption: The UE should only be required to monitor the CSS for PWS with the same frequency as in RRC_IDLE mode (e.g. every 5s, or whatever the RRC_IDLE DRX cycle is), which is expected to be much larger than the NPDCCH monitoring periodicity in RRC_CONNECTED. Therefore, the increased UE power consumption due to the PWS monitoring will be minimal.

Proposal 1: RAN to instruct RAN1 to specifies mechanisms for the UE to monitor PWS notification in RRC_CONNECTED mode

- The CSS for PWS notification and USS are TDM’d, with a minimum separation of 4ms between them (legacy NB-IoT constraint). With this constraint, UE complexity is not increased.

- The UE may only monitor the CSS for PWS notifications once every X seconds (to minimize power consumption)

- To simplify the scope, PWS notification can only be carried in DCI through direct indication.

# Summary

In this contribution we presented our views on the LS in [2]. We made the following proposal:

Proposal 1: RAN to instruct RAN1 to specifies mechanisms for the UE to monitor PWS notification in RRC_CONNECTED mode

- The CSS for PWS notification and USS are TDM’d, with a minimum separation of 4ms between them (legacy NB-IoT constraint). With this constraint, UE complexity is not increased.

- The UE may only monitor the CSS for PWS notifications once every X seconds (to minimize power consumption)

- To simplify the scope, PWS notification can only be carried in DCI through direct indication.

# References

[1] R2-2411194, “LS on PWS support in RRC_CONNECTED for NB-IoT NTN”

[2] R1-2501613, “Reply LS on PWS support in RRC_CONNECTED for NB-IoT NTN”