3GPP TSG RAN Meeting #107									RP-250354

Incheon, Korea, March 12-14, 2025

Agenda Item:	16

Source:	Xiaomi

Title:	Discussion on 6G SI and TPR

Document for:	Discussion and Decision

- Introduction

In RAN#106, for 6G RAN plenary study, it was endorsed that a single SID will cover two RP SIs (ITU focused & general), where part I (ITU focused) for approval in RAN#106 and part II (general) for approval in RAN#107 [1].

In addition, the following study plan guidance for study focusing on ITU-R (part 1) in RAN#107 meeting was endorsed [1]:

- Aim to finalize the candidate set of TPRs

- Initial discussion on the associated values of the identified TPRs

  - A corresponding summary of the values as an input for future progress

- Possible LS to ITU-R as necessary, depending on the progress of the study

In this contribution, our views on scope of general 6G study, candidate items and values of TPRs are provided.

- General 6G study

To address the future demands of new business opportunity and technology development, 6G system is expected to:

- Provide better user experience on throughput, latency, reliability, etc. for new services such as immersive communication

- Enable native support for new network capabilities such as sensing, AI, NTN, etc.

- Enable lean system design with combination of energy saving and energy charging/harvesting

- Support varieties of devices including light-capability devices

In [2], six usage scenarios for future 6G system have been recommended. However, 3GPP RAN needs to identify the typical usage scenarios and radio requirements from RAN perspective, and for 6G day 1 study. The recommendation in [2] can be considered as the baseline.

The general 6G study shall also develop the deployment scenarios for identified usage scenarios. Further, the evaluation of the radio requirements shall take the output of ITU-R focused 6G study (part I) into consideration as well, e.g., the evaluation methodology of identified TPR items shall also be developed.

Proposal 1: the general 6G study shall aim to:

- Identify the typical usage scenarios and radio requirements for RAN

- Consider the usage scenarios in Recommendation ITU-R M.216 as the baseline

- Develop the deployment scenarios for usage scenarios, considering the identified minimum TPR items in ITU-R focused study

- Candidate set of minimum TPRs

In RAN#106 meeting, an initial list of candidates IMT-2030 TPRs has been agreed and an LS has been sent to ITU-R [3], which includes:

1.	Peak data rate

2.	Peak spectral efficiency

3.	User experienced data rate

4.	5th percentile user spectral efficiency

5.	Average spectral efficiency

6.	Sustainability/Energy efficiency

7.	Area traffic capacity

8.	User plane latency

9.	Control plane latency

10.	Connection density

11.	Reliability

12.	Mobility

13.	Mobility interruption time

14.	Bandwidth

However, the definitions for some of items still need further discussion. In addition, candidate TPR items for IMT-2030 new capabilities such as AI and ISAC are not included in the initial list, which needs to be further discussed.

  - Definition of initial list of TPRs

The definition of IMT-2020 [4] on the same item of most TPRs in the initial candidate list can be reused, except that of sustainability/energy efficiency.

In IMT-2020, both network energy efficiency and device energy efficiency are defined, and energy efficiency is considered from two aspects including efficient data transmission in a loaded case, and low energy consumption when there is no data. However, the requirement is only defined as the capability to support high sleep ratio and long sleep duration, which is verified by inspection for both network and device.

Considering that sustainability/energy efficiency is important for 6G success. it would be preferred to have an TPR item definition which can be quantitively evaluated.

Proposal 2: the definition in IMT-2020 for TPRs in initial candidate list can be reused, except for sustainability/energy efficiency

- a TPR for sustainability/energy efficiency which can be quantitively evaluated is preferred

  - TPR items for new 6G capabilities

Two new usage scenarios have been recommended by IMT-2030 including Integrated Sensing and Communication (ISAC) and AI and communication (AI), which are very important for 6G success. Therefore, TPR items shall be considered for Sensing-related capabilities and AI-related capabilities.

Sensing related capabilities

For integrated Sensing and Communications (ISAC) scenario, quantitative TPR items for sensing-related capabilities needs to be defined.

The following TPR items can be considered:

- Missed detection probability and False alarm probability

Missed detection probability is defined as the probability of not detecting the presence of a target object when the target object is present, and False alarm probability is defined as the probability of falsely detecting the presence of a target object when the target object is not present.

- Sensing accuracy

The sensing accuracy can be defined as accuracy on position (m) and/or accuracy on velocity (m/s). The accuracy on position is defined as the difference of the sensed position of the target object to its true position value; and the accuracy on velocity is defined as the difference of the sensed velocity of the target object to its true velocity value. Sensing accuracy on position can be further derived into a horizontal accuracy and a vertical accuracy.

- Sensing resolution

Sensing resolution can be defined as the minimum difference in the measured magnitude of target objects to be allowed to detect objects in different magnitude, e.g., range resolution (m), velocity resolution (m/s).

Proposal 3: Quantitative TPR items for sensing related capabilities should be defined, including Missed detection probability and False alarm probability, Sensing accuracy, and Sensing resolution.

AI-related capabilities

AI-related capabilities for AI and Communication scenario can be categorized from 3 aspects:

- AI/ML for IMT-2030 e.g., Air-interface enhancement, RAN optimization, automation, and management enhancements, etc.

- AI/ML services provided by IMT 2030, e.g., Model inference service, Model training service,  AI/ML model provision, AI/ML data provision & provision, etc.

- IMT-2030 for AI/ML: Further study is needed to identify which aspects should be enhanced to support the functionality of “AI/ML for IMT-2030” and “AI/ML service provided by IMT 2030”

Qualitative evaluation (by inspection) for the 3 categories of capabilities related to AI and communication are preferred, due to the following reasons:

- AI and Communication comprise various usage scenarios and different requirements should be considered for different application scenarios there is no need and impractical to converge to one or several representative KPIs;

- Some factors impacting the performance depends on implementation or deployment strategy, which are beyond IMT-2030 design scope;

- Some factors impacting the performance can be included in the existing connection-related metrics.

Proposal 4: Qualitative TPR items for AI related capabilities should be defined, which is evaluated by inspection.

  - TPR candidate values

Peak data rate

Peak data rate is defined as the maximum achievable data rate under ideal conditions (in bit/s), which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e., excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

The value of peak data rate depends on both the peak spectral efficiency and the available bandwidth. The available bandwidth can be the aggregated bandwidth across multiple bands. Assuming that the available bandwidth in 6G can be 2 times of that in 5G (e.g., 200MHz per CC), and the peak spectral efficiency can be 1.5-2 times of that in 5G, the peak data rate in 6G can be 3-4 times of that in 5G.

Therefore, the minimum requirements for peak data rate for IMT-2030 can be:

- Downlink peak data rate is 60-80 Gbit/s

- Uplink peak data rate is 30-40 Gbit/s

Peak spectral efficiency

In IMT-2020, the peak spectral efficiency is the maximum data rate under ideal conditions normalized by channel bandwidth. The value of peak spectral is defined assuming 8 streams in DL and 4 streams in the uplink. For 6G, considering potential higher modulation orders in DL (e.g., 1024 QAM) and potential more streams to be supported in UL (e.g., 8 streams in UL), the peak spectral efficiency can be increased by at least 2 times.

The minimum requirements for peak spectral efficiency for IMT-2030 can be:

- Downlink peak spectral efficiency is 60 bit/s/Hz

- Uplink peak spectral efficiency is 30 bit/s/Hz

User experienced data rate

User experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput, which can be expressed as:

Ruser = W × SEuser

where W denotes the channel bandwidth and SEuser denotes the 5th percentile user spectral efficiency. Assuming that the channel bandwidth in 6G can be 2 times of that in 5G, and the 5th percentile user spectral efficiency can be 1.5-2 times of that in 5G, the user experienced data rate can be increased by 3-4 times.

The minimum requirements for user experienced data rate for IMT-2030 can be:

- Downlink user experienced data rate is 300-400Mbps

- Uplink peak spectral efficiency is 150-200Mbps

5th percentile user spectral efficiency

The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. Considering that more antenna elements can be deployed at BS side (e.g., 1024 antenna elements or even more), 1.5-2 times of performance improvements can be expected.

Average spectral efficiency

Average spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e., the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP. Considering that more antenna elements can be deployed at BS side (e.g., 1024 antenna elements or even more), 1.5-2 times of performance improvements can be expected.

Sustainability/Energy efficiency

As discussed in section 3.1, a quantitative TPR items for energy efficiency is preferred. The detailed value of the TPR can be discussed after the item definition is clarified.

Area traffic capacity

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2), which can be expressed as

Carea = ρ × W × SEavg

where W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). Assuming that the same TRxP density as 5G, 1.5-2 times of improvements on average spectral efficiency, and 2 times increase on channel bandwidth, totally 3-4 times improvements on area traffic capacity can be expected.

User plane latency

User plane latency is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state.

The requirement of user plane latency for IMT-2030 can be 1ms for Immersive communication, and 0.5ms for HRLLC communications.

Control plane latency

Control plane latency refers to the transition time from a most “battery efficient” state (e.g., Idle state) to the start of continuous data transfer (e.g., Active state).

The control plane latency for Immersive communication shall be reduced from IMT-2020, the minimum requirement of which can be 5-10ms for IMT-2030.

Connection density

Connection density is the total number of devices fulfilling a specific quality of service (QoS) per unit area (per km2).

The minimum requirement for connection density for IMT-2030 can be 106-107 devices/km2.

Reliability

Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time. For IMT-2020, the minimum requirement for the reliability is 1-10−5 success probability of transmitting a layer 2 PDU (protocol data unit) of 32 bytes within 1 ms.

For IMT-2030, similar requirement of success probability, i.e., 1-10-5, can be considered with larger PDU or smaller latency constraints.

Mobility

Mobility is the maximum mobile station speed at which a defined QoS can be achieved (in km/h).

For IMT-2030, mobile station with speed of up to 1000 km/h can be supported.

Mobility interruption time

Mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions.

For IMT-2030, the minimum requirement for mobility interruption time shall be 0 ms.

Bandwidth

Bandwidth is the maximum aggregated system bandwidth.

For IMT-2030, the minimum requirement for bandwidth for immersive communication devices can be 200 MHz.

Sensing-related capabilities

The TS 22.137 [5] can be referred for the value ranges of the interested TPR items. Note: Exact value for a TPR item is further dependent on the use case.

The minimum requirement for missed detection probability and false alarm probability can be:

-	Missed detection probability is 1-5%

-	False alarm probability is 1-5%

The minimum requirement for sensing accuracy can be:

-	Range accuracy: 0.5-1m for horizontal and 0.5-1m for vertical, with confidential level 95%

-	Velocity accuracy: 1-1.5m/s for horizontal and 1-1.5m/s for vertical, with confidential level 95%

The minimum requirement for sensing resolution can be:

-	Range resolution: 0.5-1m

-	Velocity resolution: 0.5-1m/s

In summary, the proposed TPR values are summarized as below:

Table 1: Summary of TPR values for IMT-2030

| TPR items | IMT-2020 | IMT-2030 |
| --- | --- | --- |
| Peak data rate (Gbit/s) | DL: 20; UL: 10 | 3-4 times of IMT-2020 DL: 60-80 ; UL: 30-40 |
| Peak spectral efficiency (bit/s/Hz) | DL: 30; UL: 15 | 2 times of IMT-2020 DL: 60; UL: 30 |
| User experienced data rate (Mbit/s) | Dense Urban: DL: 100; UL: 50 | 3-4 times of IMT-2020 |
| 5th percentile user spectral efficiency (bit/s/Hz) | InH: DL: 0.3; UL: 0.21 Dense Urban: DL: 0.225; UL: 0.15 Rural: DL: 0.12; UL: 0.045 | 1.5-2 times of IMT-2020 |
| Average spectral efficiency (bit/s/Hz/TRxP) | InH: DL: 9; UL: 6.75 Dense Urban: DL: 7.8; UL: 5.4 Rural: DL: 3.3; UL: 1.6 | 1.5-2 times of IMT-2020 |
| Sustainability/Energy efficiency | By inspection | TBD |
| Area traffic capacity (bit/s/m2) | InH: 10 M | 3-4 times of IMT-2020 |
| User plane latency (ms) | 4 for eMBB, 1 for URLLC | 1 for IC, 0.5 for HRLLC |
| Control plane latency (ms) | 20 | 5-10 |
| Connection density (devices per km2) | 106 | 106-107 |
| Reliability | 1-10-5 | 1-10-5 |
| Mobility (km/h) | 500 | 1000 |
| Mobility interruption time (ms) | 0 | 0 |
| Bandwidth (MHz) | 100 | 200 |
| Sensing - missed detection probability and false alarm probability | / | 1-5% |
| Sensing - sensing accuracy | / | Range accuracy: 0.5-1m for horizontal and 0.5-1m for vertical, with confidential level 95% Velocity accuracy: 1-1.5m/s for horizontal and 1-1.5m/s for vertical, with confidential level 95% |
| Sensing - sensing resolution | / | Range resolution: 0.5-1m Velocity resolution: 0.5-1m/s |

Proposal 5: Consider values in Table 1 as the candidate values for IMT-2030

- Conclusion

In this contribution, the scope of general 6G study, candidate items and values of TPRs are discussed, and our views are given:

Proposal 1: the general 6G study shall aim to:

- Identify the typical usage scenarios and radio requirements for RAN

- Consider the usage scenarios in Recommendation ITU-R M.216 as the baseline

- Develop the deployment scenarios for usage scenarios, considering the identified minimum TPR items in ITU-R focused study

Proposal 2: the definition in IMT2020 for TPRs in initial candidate list can be reused, except for sustainability/energy efficiency

- a TPR for sustainability/energy efficiency which can be quantitively evaluated is preferred

Proposal 3: Quantitative TPR items for sensing related capabilities should be defined, including Missed detection probability and False alarm probability, Sensing accuracy, and Sensing resolution.

Proposal 4: Qualitative TPR items for AI related capabilities should be defined, which is evaluated by inspection.

Proposal 5: Consider values in Table 1 as the candidate values for IMT-2030

- References

- RP-243276, RAN Chair’s Guidance for 6G Study: ITU Focused, RAN#106, Dec., 2024.

- Recommendation ITU-R M.2160-0, Framework and overall objectives of the future development of IMT for 2030 and beyond, Nov. 2023.

- PR-243325, Reply Liaison Statement on Minimum requirements related to technical performance for IMT-2030 radio interface(s), RAN#106, Dec. 2024

- REPORT ITU-R M.2410-0, Minimum requirements related to technical performance for IMT-2020 radio interface(s), 2017

- 3GPP TS 22.137, Service requirements for Integrated Sensing and Communication; Stage 1 (Release 19), V19.1.0, 2024