3GPP TSG RAN Meeting #107					RP-250461

Incheon, Korea, March 12-14, 2025

Agenda Item:	16

Source:	MITRE

Title:	Views on IMT-2030 Performance Requirements for RAN's 6G Study

Document for:	Discussion

# This contribution provides our views on aspects of the IMT-2030 Performance Requirements -candidate items and TPR values- in the context of the 3GPP RAN Study on 5G Scenarios and Requirements [1]

# 1. Introduction

RAN#106 decided to endorse a single Study Item Description (SID) for 6G encompassing 2 distinct parts:

- Part I: ITU-focused study

  - Approved in RAN#106 (Dec 2024)

  - Focuses on IMT-2030 requirements

- Part II: General 6G study

  - To be approved in RAN#107 (March 2025)

  - Covers broader 6G aspects

For the ITU-focused SI (Part I), the following guidance was endorsed for RAN#107 [2]:

- Aim at finalizing the candidate set of Technical Performance Requirements (TPRs) for IMT-2030

- Initiate discussions on potential values for the identified TPRs

- Prepare a summary of the proposed TPR values to guide future work

In addition, RAN#106 approved and LS to ITU-R WP5D [3] providing a preliminary view on the list of TPRs.

- It provided RAN's initial input on aspects related to technical performance requirements (TPRs) for IMT-2030.

- The LS covered legacy TPRs, which are performance requirements common to both IMT-2020 and IMT-2030 technologies

- The LS did not address new TPRs, such as those related to AI, Resiliency, Security and Interoperability

- It did not provide specific recommendations about TPR values.

# 2. TPR List and TPR Values

RAN#106 an initial list of candidate IMT-2030 Technical Performance Requirements (TPRs) was agreed upon and communicated to ITU-R [3].

- Peak data rate

- Peak spectral efficiency

- User experienced data rate

- 5th percentile user spectral efficiency

- Average spectral efficiency

- Sustainability/Energy efficiency

- Area traffic capacity

- User plane latency

- Control plane latency

- Connection density

- Reliability

- Mobility

- Mobility interruption time

- Bandwidth

Observation 1: RAN#106 did not address neither new TPRs like Sensing nor recommended TPR values.

Observation 2: The RAN#106 LS to ITU-R did not include candidate TPR items for new IMT-2030 capabilities such Resiliency, Security, and ISAC.

Observation 3: The qualitative or quantitative nature of some the TRPs and there is a need to discuss clarify this issue within 3GPP.

Based on these observations, our proposal is to consider legacy and new TRPs as part of the 6G study and also review quantitative and qualitative categorization and values.

# 3. Focus on New Services, Markets and Key Drivers

Furthermore, essential to 3GPP 6G work and specifically of the work related to TPRs needs is the goal to support new services and new markets. Security and Resiliency (see below Annex 5.2.16) is a foundational TPR for us that we recommend to study and include in 6G TRPs and aligns with the outcome of the 3GPP IMT-2030 Use Case Workshop in terms of identifying one of them, security, as a driver for all 3GPP MRPs and partners in the workshop.

4. References

[1]   RP-243327, New SID: Study on 6G Scenarios and requirements

[2]   RP-243276, RAN Chair’s Guidance for 6G Study: ITU Focused, RAN#106, Dec 2024.

[3]   RP-243325, Response to LS on Minimum requirements related to technical performance for IMT-2030 radio interface(s)

[4]   3GPP Stage-1 Workshop on IMT2030 Use Cases, https://www.3gpp.org/technologies/stage1-imt2030-uc-ws

Annex

Summary of Candidate Items for Minimum TPRs Listed in Recommendation ITU-R M.2160

### 5.2.1	Peak data rate

Peak data rate is the maximum achievable data rate under ideal conditions (in bit/s), which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

Peak data rate is defined for a single mobile station. In a single band, it is related to the peak spectral efficiency in that band. Let W denote the channel bandwidth and SEp denote the peak spectral efficiency in that band. Then the user peak data rate Rp is given by:

Rp = W × SEp	(1)

Peak spectral efficiency and available bandwidth may have different values in different frequency ranges. In case bandwidth is aggregated across multiple bands, the peak data rate will be summed over the bands. Therefore, if bandwidth is aggregated across Q bands then the total peak data rate is

R=i=1Q  Wi × SEpi 	(2)

where Wi and SEpi (i = 1,…Q) are the component bandwidths and spectral efficiencies respectively.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for peak data rate are as follows:

–	Downlink peak data rate is TBD Gbit/s.

–	Uplink peak data rate is TBD Gbit/s.

According to latest ITU discussion, the target value should be quantitative.

### 5.2.2	User experienced data rate

User experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time.

In case of one frequency band and one layer of transmission reception points (TRxP), the user experienced data rate could be derived from the 5th percentile user spectral efficiency through equation (3). Let W denote the channel bandwidth and SEuser denote the 5th percentile user spectral efficiency. Then the user experienced data rate, Ruser is given by:

Ruser = W × SEuser 	(3)

In case bandwidth is aggregated across multiple bands (one or more TRxP layers), the user experienced data rate will be summed over the bands.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The target values for the user experienced data rate are as follows in the TBD test environment(s):

–	Downlink user experienced data rate is TBD Mbit/s.

–	Uplink user experienced data rate is TBD Mbit/s.

According to latest ITU discussion, the target value should be quantitative.

### 5.2.3	Spectral efficiency

#### 5.2.3.1	Peak spectral efficiency

Peak spectral efficiency is the maximum data rate under ideal conditions normalised by channel bandwidth (in bit/s/Hz), where the maximum data rate is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for peak spectral efficiencies are as follows:

–	Downlink peak spectral efficiency is TBD bit/s/Hz.

–	Uplink peak spectral efficiency is TBD bit/s/Hz.

According to latest ITU discussion, the target value should be quantitative.

#### 5.2.3.2	Average spectral efficiency

Average spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

Let Ri (T) denote the number of correctly received bits by user i (downlink) or from user i (uplink) in a system comprising a user population of N users and M TRxPs. Furthermore, let W denote the channel bandwidth and T the time over which the data bits are received. The average spectral efficiency, SEavg is then defined according to equation (4).

(4)

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for average spectral efficiency are as follows:

–	Downlink average spectral efficiency is TBD bit/s/Hz.

–	Uplink average spectral efficiency is TBD bit/s/Hz.

According to latest ITU discussion, the target value should be quantitative.

#### 5.2.3.3	5th percentile user spectral efficiency

The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. The normalized user throughput is defined as the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time, divided by the channel bandwidth and is measured in bit/s/Hz.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

With Ri (Ti) denoting the number of correctly received bits of user i, Ti the active session time for user i and W the channel bandwidth, the (normalized) user throughput of user i, ri, is defined according to equation (5).

(5)

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for 5th percentile user spectral efficiency are as follows:

–	Downlink 5th percentile user spectral efficiency is TBD bit/s/Hz.

–	Uplink 5th percentile user spectral efficiency is TBD bit/s/Hz.

According to latest ITU discussion, the target value should be quantitative.

### 5.2.4	Area traffic capacity

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time.

This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g. TRxP (site) density) and bandwidth.

Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through equation (6).

Carea = ρ × W × SEavg	(6)

In case bandwidth is aggregated across multiple bands, the area traffic capacity will be summed over the bands.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The target value for Area traffic capacity in downlink is TBD Mbit/s/m2 in the TBD test environment.

According to latest ITU discussion, the target value should be quantitative.

### 5.2.5	Connection density

Connection density is the total number of devices fulfilling a specific quality of service (QoS) per unit area (per km2).

Connection density should be achieved for a limited bandwidth and number of TRxPs. The target QoS is to support delivery of a message of a certain size within a certain time and with a certain success probability.

This requirement is defined for the purpose of evaluation in the Massive Communication usage scenario.

The minimum requirement for connection density is TBD devices per km2.

According to latest ITU discussion, the target value should be quantitative.

### 5.2.6	Mobility

Mobility is the maximum mobile station speed at which a defined QoS can be achieved (in km/h).

The following classes of mobility are defined:

[TBD]

Table 1 defines the mobility classes that shall be supported in the respective test environments.

TABLE 1

Mobility classes

|  | Test environments for TBD |  |  |
| --- | --- | --- | --- |
| Mobility classes supported |  |  |  |

A mobility class is supported if the traffic channel link data rate on the uplink, normalized by bandwidth, is as shown in Table 2. This assumes the user is moving at the maximum speed in that mobility class in each of the test environments.

This requirement is defined for the purpose of evaluation in the Immersive communication and Massive Communication usage scenarios.

According to latest ITU discussion, the target value should be quantitative.

TABLE 2

Traffic channel link data rates normalized by bandwidth

| Test environment | Normalized traffic channel link data rate (bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |

#### 5.2.6.1	Mobility interruption time

Mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions.

The mobility interruption time includes the time required to execute any radio access network procedure, radio resource control signaling protocol, or other message exchanges between the mobile station and the radio access network, as applicable to the candidate RIT/SRIT.

This requirement is defined for the purpose of evaluation in the TBD usage scenarios.

The minimum requirement for mobility interruption time is TBD ms.

According to latest ITU discussion, the target value should be quantitative.

### 5.2.7	Latency

#### 5.2.7.1	User plane latency

User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state.

This requirement is defined for the purpose of evaluation in the Immersive communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirements for user plane latency are:

–	TBD ms for TBD

assuming unloaded conditions (i.e. a single user) for TBD IP packets), for both downlink and uplink.

According to latest ITU discussion, the target value should be quantitative.

#### 5.2.7.2	Control plane latency

Control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state).

This requirement is defined for the purpose of evaluation in the Immersive communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirement for control plane latency is TBD ms. Proponents are encouraged to consider lower control plane latency, e.g. TBD ms.

According to latest ITU discussion, the target value should be quantitative.

### 5.2.8	Reliability

Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability.

Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a TBD data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

This requirement is defined for the purpose of evaluation in the Hyper Reliable and Low Latency Communication usage scenario.

The minimum requirement for the reliability is TBD success probability of transmitting a layer 2 PDU (protocol data unit) of TBD bytes within TBD ms in channel quality of coverage edge for the Hyper Reliable and Low Latency Communication test environment, assuming TBD application data .

According to latest ITU discussion, the target value should be quantitative.

### 5.2.9	Joint requirement on data rate, latency, reliability and capacity

The joint performance requirement relates to the capability of transmitting a given amount of traffic within a predetermined latency with high success probability. In addition, the capacity is defined as the number of users or connections per cell that can be satisfied with the fulfilled joint requirement of data rate, latency and reliability.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The joint minimum requirement for various test environments is summarized in Table 3.

TABLE 3

Joint requirement on data rate, latency, reliability and capacity

| Test environment | Data rate |  | Latency |  | Reliability |  | Number of satisfied users |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink |

### 5.2.10	Coverage

Coverage refers to the ability to provide access to communication services for users in a desired service area. Coverage is defined as the cell edge distance of a single cell.

The RIT/SRIT shall have a function to provide a larger cell coverage in specific area than that supported by a ground IMT base station using the IMT-2020 technologies. This function may include, but not limited to, inherent functionalities within the RIT/SRIT and utilization of higher altitude applications (e.g. UAS and HIBS).

### 5.2.11	Positioning

Positioning is the ability to calculate the approximate position of connected devices. Positioning accuracy is defined as the difference between the calculated horizontal/vertical position and the actual horizontal/vertical position of a device.

According to latest ITU discussion, the target value should be quantitative.

### 5.2.12	Bandwidth

Bandwidth is the maximum aggregated system bandwidth. The bandwidth may be supported by single or multiple radio frequency (RF) carriers. The bandwidth capability of the RIT/SRIT is defined for the purpose of IMT-2030 evaluation.

The requirement for bandwidth is TBD.

According to latest ITU discussion, the target value should be quantitative.

### 5.2.13	Sensing-related capabilities

Sensing-related capabilities refer to the ability to provide functionalities in the radio interface to support sensing tasks, including passive object presence/absence detection, object classification, movement characterization, object localization, mapping, and composite sensing tasks such as tracking, SLAM, etc.

A sensing task is a processing function ISAC-capable system, which is performed to achieve a certain sensing goal with respect to one or more sensing target(s), based on radio frequency measurements. Performance of a sensing task is characterized by at least one minimum requirement.

According to latest ITU discussion, the target value should be quantitative.

#### 5.2.13.1	Detectability (Detection / False alarm probability)

Detection probability, PD, is the probability of correctly detecting the object status (e.g., presence, category, and state) within sensing range, and False alarm probability, PFA, is the associated probability that an object status is detected given that the object is not present within sensing range.

#### 5.2.13.2	Localization accuracy

Localization accuracy refers to the closeness of the measured sensing result of position of the target object to its true position value within sensing range and is the difference between the calculated location and the actual location of an object.

#### 5.2.13.3	Velocity accuracy

Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing target within sensing range.

#### 5.2.13.4	Sensing resolution

Sensing resolution is defined as the minimum difference of the measured value (e.g, range, location) of two target objects, in order to distinguish these two target objects.

### 5.2.14	AI-related capabilities

Applicable AI-related capabilities refer to the ability to provide certain functionalities throughout IMT-2030 to support AI enabled applications. These functionalities include distributed data processing, distributed learning, AI computing, AI model execution and AI model inference, etc.

### 5.2.15	Sustainability / Energy efficiency

Sustainability, or more specifically environmental sustainability, refers to the ability of both the network and devices to minimize greenhouse gas emissions and other environmental impacts throughout their life cycle.

Energy efficiency is a quantifiable metric of sustainability. It refers to the quantity of information bits transmitted or received, per unit of energy consumption (in bit/Joule). Energy efficiency is expected to be improved appropriately with the capacity increase in order to minimize overall power consumption.

Network energy efficiency is the capability of a RIT/SRIT to minimize the radio access network energy consumption in relation to the traffic capacity provided. Device energy efficiency is the capability of the RIT/SRIT to minimize the power consumed by the device modem in relation to the traffic characteristics.

Energy efficiency of the network and the device can relate to the support for the following two aspects:

- Efficient data transmission in a loaded case;

- Low energy consumption when there is no data, namely, in an un-loaded case.

Efficient data transmission in a loaded case is demonstrated by the average spectral efficiency.

This requirement is defined for the purpose of evaluation in the TBD usage scenario.

A quantitative requirement on Energy efficiency can be defined in the following

- A quantitative evaluation to demonstrate the level of sleep ratio and long sleep duration supported by the RIT (Analysis; With TPR limits on sleep ratio and sleep duration)

- A quantitative evaluation to demonstrate the power savings of sleep ratio and sleep duration for the RIT using power models (Analysis without user data traffic; With TPR limits on relative energy savings)

- A quantitative evaluation to demonstrate the power savings of sleep ratio and sleep duration for the RIT using power models (Simulation with user data traffic; With TPR limits on relative energy savings)

According to latest ITU discussion, the target value of energy efficiency should be quantitative.

### 5.2.16	Security and resilience

Security refers to preservation of confidentiality, integrity, and availability of information, such as user data and signaling, and protection of networks, devices and systems against cyberattacks such as hacking, distributed denial of service, man in the middle attacks, etc.

Resilience refers to capabilities of the networks and systems to continue operating correctly during and after a natural or man-made disturbance, such as the loss of primary source of power, etc.  The RIT/SRIT shall have functionalities to continue operations in emergency situations, either within itself or with the assistance of specific reliable applications with minimal ground network infrastructure (e.g. UAS, HIBS and shipboard base stations), etc.

### 5.2.17	Interoperability

Interoperability refers to the radio interface being based on member-inclusivity and transparency, so as to enable functionality(ies) between different entities of the system.