3GPP TSG RAN meeting #107								     		RP-250466

Incheon, Korea, March 12th - 14th , 2025

## Status Report to TSG

Agenda item:			9.4.1.6

| WI / SI Name | Introduction of Ku Bands for NR NTN |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_NTN_Ku_bands |  |  |  |  |
| Unique ID | 1040135 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-243314 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 09/2025 | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 50% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Manook Soghomonian |
|  | Company | Intelsat |
|  | Email | Manook.soghomonian@intelsat.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN4

Summary

Following RAN #106, a checkpoint was added to the WID for the RAN4 LS in RP-242499 to RAN1 and RAN2 regarding the feasibility of specifying two sets of overlapping NR bands using FR1-NTN 30 kHz numerology and FR2-NTN 120 kHz numerology. RAN1 confirmed in R1-2501609 and RAN2 confirmed in R2-2501530 that no reasons were identified that two sets of Ku bands with different numerologies could not be specified and so the checkpoint is met and RAN4 will continue to define two sets of bands. An updated WID is provided in RP-25YYYY.

Generally, there was good progress at RAN #114 in all areas. Important decisions were made on the initial two Ku bands and associated ARFCN and GSCN definitions and the approach for specifying support for mobile VSAT served by NGSO that enable work in drafting new specifications. Work starting at this meeting for the RRM part.

Based on the updated WID in RP-243314, RAN4 has started the work on defining two set of bands for the NR NTN Ku band, one for FR1 numerology and the other for FR2.

Based on this approach RAN4 has decided to extend both FR1-NTN and FR2-NTN frequency ranges to cover 10.7~14.5GHz frequency ranges. The detail band plan for priority 1 Ku band was also discussed and updated to minimize the number for the bands. The corresponding ARFCN and GSCN numbers were also agreed.

RAN4 has decided to progress the work to specify mobile VSAT served by NGSO based on the existing mobile VSAT type 4 & 5. Land ESIM, maritime ESIM and airborne ESIM will be considered for the requirements, however, only Land ESIM will be considered in the coexistence study as maritime and airborne ESIM default to Land ESIMbehaviour when on or near land.

RAN4 has agreed to define SAN type 1-O requirements for the FR1-NTN Ku bands, and define SAN type 2-O requirements for the FR2-NTN Ku bands, and the SAN type 1-H is not needed and will not be defined for the Ku band.

RAN4 has started the RRM part of the work, identifying potential impacts on the current RRM requirements as the starting point.

#### 2.1.1	Agreements

Core part:

RAN WG4 Meeting #114, Athens, Greece, 17th – 21st Feb, 2025:

The following agreements documented in the Way Forward for NR_NTN_Ku_Band_General in [49] were approved:

###### 2.1.1.1 WF for NTN Ku band General and work plan

Sub-topic 1-1: Approval of the new work plan

Agree Proposal 1 from Intelsat in R4-2500434: Approve the proposed updated Work Plan proposed in Section 2.5.

Sub-topic 1-2: Definition of extended FR1-NTN and FR2-NTN frequency ranges

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 14500 MHz |
| FR2-NTN (Note 2) | 10700 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2 band when references from other specifications. |  |

Sub-topic 1-3: Ku bands definition

Table 5.2.3-1: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n509 | 14000 MHz - 14500 MHz | 10700 MHz – 127504 MHz | FDD |
| n508 | 13750 MHz - 14000 MHz | 10700 MHz – 127504 MHz | FDD |
| NOTE 4: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT.. |  |  |  |

Table 5.2.3-2: Satellite operating bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n248 | 14000 MHz - 14500 MHz | 10700 MHz – 127501 MHz | FDD |
| n247 | 13750 MHz - 14000 MHz | 10700 MHz – 127501 MHz | FDD |
| NOTE 1: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT.. |  |  |  |

.

.

###### 2.1.1.2 	WF for NTN Ku band Coexistence study based on ITU regulations

Sub-topic 2-1: Baseline simulation assumptions

In order to coordinate the simulation assumptions, a Tdoc containing the aggregate agreements from all the WF will be maintained. For this meeting that will be in R4-2502315 which will be noted.

Sub-topic 2-3: BS TN antenna pattern

Agree Proposal 1 from Ericsson in R4-2501398: RAN4 to use TN BS antenna parameters of 7125 to 8400 MHz from TR 38.922 Table 5.4.1.2-1 for 11 GHz assumptions.

Sub-topic 2-6: Tx power control model

Agree Proposal 8 from Xiaomi in R4-2500637: It is proposed to reuse transmission power control model in the clause 6.2.6 of TR38.863.

Sub-topic 2-7: Rx power control model

Proposal 9 from Xiaomi in R4-2500637: It is proposed to reuse Received power model in the clause 6.2.7 of TR38.863.

Sub-topic 2-8: Performance metrics

Agree Proposal 10 from Xiaomi in R4-2500637: It is proposed to reuse Performance metric in the clause 6.2.8 of TR38.863.

Sub-topic 2-9: Throughput SNR mapping

Proposal 11 from Xiaomi in R4-2500637: It is proposed to reuse Throughput ~ SNR mapping model in the clause 6.2.9 of TR38.863.

Sub-topic 2-10: Conducted power per antenna element

Agree to specify conducted power per antenna port but the figure is dependent on the decisions on EIRP made in the NTN Ku-band UE RF discussions.

Sub-topic 2-12: Co-existence study scope

Agree Proposal 2 from Ericsson in R4-2501398 assuming that A-ESIM and M-ESIM shall be shut down when approaching land: RAN4 to confirm the scope of the co-existence evaluation based only on L-ESIM.

###### 2.1.1.3	WF for NTN Ku band System parameters

Sub-topic 3-6: FR1-NTN ARFCN definitions

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| n247 | 30 | 1316668 – <2> – 1333332 | 1113334 – <2> – 1250000 |

Sub-topic 3-7: FR2-NTN ARFCN definitions

Table 5.4.2.3-3: Applicable NR-ARFCN per operating band in FR2-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n509 | 120 | 1333336 – <8> – 1366664 | 1113336 – <8> – 1250000 |
| n508 | 120 | 1316672 – <8> – 1333328 | 1113336 – <8> – 1250000 |

    - Sub-topic 3-8: FR1-NTN GSCN definitions

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n248 | 30 kHz | Case C | 12850 – <1> – 14266 |
| n247 | 30 kHz | Case C | 12850 – <1> – 14266 |

    - Sub-topic 3-9: FR2-NTN GSCN definitions

Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n509 | 120 kHz | Case D | 12864 – <12> – 14256 |
| n508 | 120 kHz | Case D | 12864 – <12> – 14256 |

Sub-topic 3-10: Default Tx Rx frequency separation

Agreement: Variable TX RX spacing is assumed, how it is specified is FFS.

Ka band FR2 did not specify Tx Rx separation due to the large separation but may need to specify for Ku with close separation.

The following agreements documented in the Way Forward for NR_NTN_Ku_Band_UE_SAN_RF in [47] were approved:

###### 2.1.1.4	WF for NTN Ku band UE RF requirements

Sub-topic 1-1: General issues for Ku band UE RF requirements

Issue 1-1-1: VSAT types to be considered for NTN Ku band after RAN#106 update

VSAT type 1-3 will not be specified for Ku bands based on the revised WID

The mobile VSAT support of the Ku bands will be added based on the existing mobile VSAT type 4 & 5

The following changes can be the starting point

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52,3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. Note 3: LEO support is only defined for Ku bands (TBD) |  |  |

Issue 1-1-3: Mobile VSAT type to be considered

The land ESIM, maritime ESIM and airbone ESIM can be considered for the requirements of the Ku band.

However, only Land ESIM will be considered in the coexistence study.

Such justification will be captured in the TR.

Issue 1-1-5: General RF requirements impact on numerologies dependency

Only radiated requirements shall be updated with the new channel bandwidths to be supported.

It is confirmed the requirements in section 9.2 Transmitted power, 9.4 Transmitter signal quality, 10.8 Receiver antenna off-axis are numerology independent.

The requirements in section 9.5 Output RF spectrum emission, 10.3 OTA reference sensitivity level, 10.4 Maximum input level, 10.5 ACS & 10.6 blocking needs to be updated due to the new channel bandwidth at least.

Issue 1-1-4: General update to support NTN VSAT in FR1-NTN

The content in the draft CR R4-2501358 (Ericsson) to prepare TS 38.101-5 to support NTN VSAT in FR1-NTN can be considered.

Sub-topic 1-2: Tx UE requirements for the NTN Ku band

Issue 1-2-1: Maximum EIRP requirement

The maximum EIRP requirement for the Rel.19 Ku bands VSAT is set at 88.2 dBm

Additional limitations for VSATs in CEPT regions will be captured, including the following

54.5 dBW (or captured as 84.5 dBm) for land ESIM operating in CEPT regions

Issue 1-2-2: Maximum TRP requirements

Adopt the following maximum TRP requirements for the Rel.19 Ku bands VSAT

47dBm

Issue 1-2-3: Minimum EIRP requirements

Companies are encouraged to provide views on the minimum peak EIRP requirement for the NTN Ku band in the next meeting.

Issue 1-2-5: Off-axis EIRP requirements for the CEPT Ku band

For the Ku band concerning requirements from ECC regulation and ETSI standards, the off-axis EIRP density limits from EN 302 977 will be considered as the starting point

Companies are encouraged to further check the condition for φ > 70° and the application on the maximum number of the VSATs for this requirement

Issue 1-2-6: On-axis spurious requirements for the CEPT Ku band

For the CEPT Ku band on-axis spurious requirements

The On-axis spurious requirements for the “emissions disabled” and “carrier-off” will be set as EIRP limit [9] dBm with measurement bandwidth [100 kHz]

FFS for the “carrier-on” state

Regulations of other countries can be further checked whether further adjustment is needed for the on-axis spurious requirement

Issue 1-2-7: Off-axis spurious requirements for the CEPT Ku band

For the Off-axis spurious requirements for the CEPT Ku band

Specify the Off-axis spurious general requirement at 10 meters distance according to table 1.2.6-1 for the CEPT Ku band

Table 1-2-6.1

| Frequency range (MHz) | EIRP Limit (dBμV/m) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 30 – 230 | 30 | 120 |
| 230 – 1000 | 37 | 120 |

Off-axis spurious limits in “Carrier-on”, “Carrier-off” and the "Emissions-disabled" state in EN 302 977 as a starting point for mobile VSAT with GSO for Ku band

Off-axis spurious limits in “Carrier-on”,  “Carrier-off”  and the "Emissions-disabled" state specified in EN 303 980, EN 303 981 as a starting point for mobile VSAT with NGSO for Ku band

Regulations of other countries can be further checked whether further adjustment is needed for the off-axis spurious requirement

Issue 1-2-8: PFD limits for the CEPT Ku band

FFS to the next meeting

###### 2.1.1.5	WF for NTN Ku band SAN RF requirements

Issue 2-1-1: Discussion on the supported SAN type for NR NTN Ku band

Regarding the SAN requirement for the NR NTN Ku band:

Define SAN type 1-O requirements for the FR1-NTN Ku bands

Define SAN type 2-O requirements for the FR2-NTN Ku bands

The SAN type 1-H will not be defined for the FR1-NTN Ku bands

Issue 2-1-2: Alignment of SAN requirement between FR1-NTN and FR2-NTN

Aim to align the following SAN requirements between the FR1-NTN Ku bands and the FR2-NTN Ku bands at least

The tolerance for the radiated transmit power and SAN output power for FR1-NTN and FR2-NTN

The spurious frequency range for FR1-NTN and FR2-NTN

Companies are encouraged to further check other requirements between FR1-NTN and FR2-NTN, including the following aspects

Companies are encouraged to further check whether the RE power control dynamic range, OTA sensitivity and Rx dynamic range requirements shall be specified for NTN Ku-bands

FFS on other requirements

Issue 2-1-4: PFD limits

The PFD limits at the Earth’s surface specified by FCC (47 CFR 25.208) for the Ku-band will not be specified in the 3GPP specification.

Issue 2-1-5: Impact on SAN RF requirements for the NTN Ku band

When introducing radiated SAN requirement for the Ku band, there will be no specification impact on the following sections, (i.e., the existing requirements applies):

Section 9.5 (OTA transmit ON/OFF power), 9.6 (OTA transmitted signal quality), 9.7.2 (OTA occupied bandwidth), 9.7.4 (OTA out-of-band emissions), 9.7.5 (OTA transmitter spurious emissions), 10.2 (OTA sensitivity)

Issue 2-1-6: Work split on SAN RF requirements for the NTN Ku band

Consider the Ku band SAN requirement work split for the draft CRs to TS 38.108 based on table 2.1.6-1 in [47] as the starting point

The following agreements documented in the Way Forward for NR_NTN_Ku_Bands (RRM part) in [52] were approved:

###### 2.1.1.6	WF for RRM requirements for NR_NTN_Ku_bands

Issue 1-1: RRM scope for NR NTN on Ku bands

Agreement:

For SCS, consider SCS=30kHz in FR1-NTN numerology and 120kHz in FR2-NTN numerology, following the discussion in BDaT session.

Mobile VSAT UE shall be considered from RAN4 RRM requirement perspective. There are four cases shall be considered for mobile VSAT UE:

Case 1: mobile VSAT UE served by GSO in FR1-NTN numerology

Case 2: mobile VSAT UE served by NGSO in FR1-NTN numerology

Case 3: mobile VSAT UE served by GSO in FR2-NTN numerology

Case 4: mobile VSAT UE served by NGSO in FR2-NTN numerology

Issue 1-2: impact on RRM requirements from NR NTN on Ku bands

Take option 1 as baseline. Discuss which requirements shall be removed and whether other requirements need to be added.

Option 1: RAN4 RRM group to consider the sections identified in the Table 1 in [1] and determine which of these can be reused unmodified, which need to be revised for editorial modification and which need new technical work.  (Eutelsat Group)

Issue 1-3: UL transmit timing requirement

Agreement:

RAN4 to define timing accuracy requirements for case 1, 2:

For FR1-NTN numerology, existing requirements in Table 7.1C.2-1 of 38.133 (FR1-NTN) are re-used.

For the applicable SCS, follow the agreement in BDaT session.

RAN4 to define timing accuracy requirements for case 3:

For FR2-NTN numerology, existing requirements in Table 7.1C.2-3 of 38.133 (FR2-NTN mobile VSAT served by GSO) are re-used.

For the applicable SCS, follow the agreement in BDaT session.

The endorsed CR with 7.5 Ts under the same side conditions as Rel-18 are considered.

RAN4 to define timing accuracy requirements for case 4:

For FR2-NTN numerology, FFS existing requirements in Table 7.1C.2-3 of 38.133 (FR2-NTN mobile VSAT served by GSO) are re-used.

For the applicable SCS, follow the agreement in BDaT session.

The endorsed CR with 7.5 Ts is considered. FFS whether the same or different side condition from Rel-18 can be used.

Issue 1-4: terminology related issue

Proposals:

Option 1: An overarching task in RAN4 is to determine how terminology currently used in TS 38.133 can be aligned to, clearly and unambiguously, identify Ku bands defined for FR1-NTN and Ku bands defined for FR2-NTN. (Eutelsat Group)

Recommended WF:

Continue discussion.

##### Performance part:

Not started.

#### 2.1.2	Remaining Open issues

##### Core part:

Define additional bands for Ku (discussion on the Priority 2 Ku band #2a and #2b in RP-243314)

Completion of co-existence study

Define system parameters

Remaining system parameters for Priority 1 Ku bands including Tx Rx frequency separation, support on asymmetric channel bandwidth, channel bandwidth configuration

System parameters for Priority 2 Ku bands

Specify RF requirements

SAN RF requirements

UE RF Requirements

Specify RRM requirements

##### Performance part:

RRM performance requirements and test cases

UE demodulation and CSI reporting requirements

Satellite access node demodulation requirements

Satellite access node conformance tests

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

#### Not applicable.

## 4.	References

Contributions submitted to RAN4#112bis:

| Ref | Tdoc | Subject | Source | Status |
| --- | --- | --- | --- | --- |
| [1] | R4-2500135 | RRM updates required to support the introduction of Ku Bands for NR-NTN | Eutelsat Group | Noted |
| [2] | R4-2500210 | Ku Band: Narrow Band Services with 15 kHz Sub-Carrier Spacing | Intelsat | Noted |
| [3] | R4-2500211 | Ku Band: Band Numbering, ARFCN and GSCN Calculations | Intelsat | Noted |
| [4] | R4-2500223 | Minimization of number of Ku bands | Eutelsat Group | Noted |
| [5] | R4-2500387 | Discussion on RRM requirements for Ku bands for NR NTN | MediaTek inc. | Noted |
| [6] | R4-2500434 | Updated work plan for NTN Ku Band | Intelsat | Noted |
| [7] | R4-2500435 | Ku Band Co-existence Simulation Assumptions (Draft 1) | Intelsat | Noted |
| [8] | R4-2500529 | Topic summary for [114][215] NR_NTN_Ku_bands | Moderator (Apple) | Noted |
| [9] | R4-2500574 | Topic summary for [114][313] NR_NTN_Ku_Band_General | Moderator (Eutelsat) | Noted |
| [10] | R4-2500575 | Topic summary for [114][314] NR_NTN_Ku_Band_UE_SAN_RF | Moderator (CHTTL ) | Noted |
| [11] | R4-2500607 | Discussion on coexistence study based on ITU regulations for NTN Ku band | ZTE Corporation, Sanechips | Noted |
| [12] | R4-2500608 | Discussion on system parameters for NTN Ku band | ZTE Corporation, Sanechips | Noted |
| [13] | R4-2500609 | Discussion on UE RF requirements for NTN Ku band | ZTE Corporation, Sanechips | Noted |
| [14] | R4-2500610 | Discussion on SAN RF requirements for NTN Ku band | ZTE Corporation, Sanechips | Noted |
| [15] | R4-2500637 | Discussion the assumptions and parameters for NR NTN Ku Band coexistence study | Xiaomi | Noted |
| [16] | R4-2500642 | System parameters for NTN Ku band | Xiaomi | Noted |
| [17] | R4-2500643 | VSAT Type for Ku band | Xiaomi | Noted |
| [18] | R4-2500883 | Discussion on RRM requirements for Ku-band NTN | LG Electronics Inc. | Noted |
| [19] | R4-2500988 | Further discussion on system parameters for Ku-band for NR NTN | CATT | Noted |
| [20] | R4-2500989 | Further discussion on SAN RF core requirements for Ku-band for NR NTN | CATT | Noted |
| [21] | R4-2500990 | Further discussion on coexistence evaluations for Ku-band for NR NTN | CATT | Noted |
| [22] | R4-2501175 | Discussion on Ku bands for NR NTN | ZTECorporation,Sanechips | Noted |
| [23] | R4-2501271 | Initial discussion on RRM requirements for NTN in Ku band | Huawei, HiSilicon | Noted |
| [24] | R4-2501306 | Discussion on the system parameters for NR NTN Ku band | CHTTL | Noted |
| [25] | R4-2501328 | Discussion on the SAN RF requirement for the NTN Ku band | CHTTL | Noted |
| [26] | R4-2501338 | Discussion on the support VSAT types and EIRP, TRP, EIS requirement for the Rel.19 NTN Ku band | CHTTL | Noted |
| [27] | R4-2501354 | NTN Ku-band - WI updates impact | Ericsson | Noted |
| [28] | R4-2501355 | NTN Ku-band - System parameters | Ericsson | Noted |
| [29] | R4-2501356 | NTN Ku-band - UE RF requirements | Ericsson | Noted |
| [30] | R4-2501357 | NTN Ku-band - SAN requirements | Ericsson | Noted |
| [31] | R4-2501358 | Draft CR to 38101-5 - Needed updates to support NTN VSAT in FR1-NTN | Ericsson | Postponed |
| [32] | R4-2501398 | On Co-existence simulation assumptions for NR NTN Ku band | Ericsson | Noted |
| [33] | R4-2501399 | On Co-existence simulation results for NR NTN Ku band | Ericsson | Noted |
| [34] | R4-2501456 | Discussion on VSAT requirements for NR NTN Ku band | Huawei, HiSilicon | Noted |
| [35] | R4-2501621 | Mobile VSAT gradual timing adjustment | Ericsson | Noted |
| [36] | R4-2501691 | Discussion on UE RF requirements for NR NTN Ku band | SKY Perfect JSAT Corporation | Noted |
| [37] | R4-2501700 | Discussion on VSAT UE parameters for Ku-band co-existence study | SKY Perfect JSAT Corporation | Noted |
| [38] | R4-2501814 | On Ku-band coexistence studies | Samsung | Noted |
| [39] | R4-2501815 | On Ku-band UE RF requirements | Samsung | Noted |
| [40] | R4-2501822 | NR NTN system parameters for Ku band | Nokia | Noted |
| [41] | R4-2502173 | Discussion on Ku-band VSAT output power and different antenna sizes | LG Electronics Finland | Noted |
| [42] | R4-2502205 | UL Timing modifications to enable mobile VSAT served by NGSO | Eutelsat Group | Noted |
| [43] | R4-2502207 | CR to 38.133 to modify uplink timing requirements to enable mobile VSAT served by NGSO by NGSO | Eutelsat Group | Postponed |
| [44] | R4-2502211 | Discussion on Ku-band FR2-NTN configuration | THALES | Noted |
| [45] | R4-2502227 | (NR_NTN_Ku_bands-Core) RRM requirements for VSAT UE in Ku band | Qualcomm Incorporated | Noted |
| [46] | R4-2502232 | Addition of structure for support for mobile VSAT served by NGSO | Eutelsat Group | Noted |
| [47] | R4-2502288 | Way Forward for [114][314] NR_NTN_Ku_Band_UE_SAN_RF | CHTTL | Approved |
| [48] | R4-2502289 | Ad-hoc meeting minutes for [114][313] NR_NTN_Ku_Band_General | Eutelsat | Noted |
| [49] | R4-2502290 | Way Forward for [114][313] NR_NTN_Ku_Band_General | Eutelsat | Approved |
| [50] | R4-2502315 | Ku band simulation assumption agreements | Ericsson | Noted |
| [51] | R4-2502590 | WF on RRM requirements for NR_NTN_Ku_bands | Apple | Approved |

12.08.2024		minor adaptations for RAN #105

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v01	13.11.2009		First version of the template