3GPP TSG RAN Meeting #107										                RP-250518

Incheon, Korea, 12th – 14th March, 2025

Agenda Item:	15.3

Source:	CATT, CSCN

Title:	General views on Rel-20 RAN4 works

Document for:	Discussion and decision

# Introduction

With the imminent arrival of the 6G era, Release 20 will encompass efforts for both the evolution of 5G-A and the study of 6G within the RAN WGs. In particular, RAN4 has been under heavy load for a long time, highlighting the critical need for careful planning in the Release 20 efforts.

In this contribution, we present our initial general perspectives on the RAN4 activities for this upcoming release.

# Discussion

As usual, RAN4 is responsible for the following tasks in each release: (1) RAN4-related work required by RAN1/2/3-led items, (2) RAN4-led non-spectrum new items, (3) RAN4-led spectrum new items, and (4) maintenance of previous releases. Notably, maintenance work for previous releases, including Rel-15/16/17/18, continues to demand significant effort within RAN4, with more than 40% of the total contributions submitted in Q4 2024 dedicated to this [1]. With the potential additional Rel-19 maintenance, it is expected that a substantial amount of effort will still need to be reserved for maintenance tasks within the Rel-20 timeline.

Observation 1: In the Rel-20 timeline, a substantial amount of effort will still need to be reserved for maintenance tasks for releases from Rel-15 to Rel-19.

Additionally, RAN4 needs to work on Rel-20 in two parallel tracks: 5G-A evolution and the 6G study. As a result, the release is expected to be smaller than Rel-19, requiring careful planning for the 5G-A evolution. In our opinion, RAN4's focus will be on addressing commercial demands and interests, technical issues identified in real-world scenarios, and making necessary corrections to the current specifications for both NTN and TN in Rel-20.

Observation 2: From RAN4 perspective Rel-20 5G-A would likely be a smaller release than Rel-19 with a focus on addressing commercial demands and interests, resolving technical issues identified in real-world scenarios, and making necessary corrections to the current specifications for both NTN and TN.

Regarding potential NTN enhancements in this release, there is one clear commercial demand to support HD-FDD mobile VSAT, driven by the need for smaller and more cost-effective phased array antennas. In this configuration, UE device can use one antenna array to fulfill the requirement of transmission and reception with half duplex mode. It can significantly reduce the size and weight of NTN device and simplify RF design, which is very necessary in the scenarios of vehicle mounted terminals, ship-borne terminals and portable terminals.  Consequently, it is critical to make NTN VSAT device more popular, and in the end achieve commercial success.

Observation 3: NTN HD-FDD mobile VSAT can be considered to support in Rel-20.

In addition, only a small effort is required to introduce a 7MHz channel bandwidth into NR NTN by reusing similar work done in TN. Furthermore, mobility between TN and NR NTN in the RRC_CONNECTED state is currently lacking, as shown in Fig. 1. RAN4 may specify necessary requirements to enhance mobility between TN and NR NTN.

Observation 4: For other potential NTN enhancements in Rel-20 could include 7MHz channel bandwidth for NR NTN and mobility enhancement between TN and NR NTN in the RRC_CONNECTED state.

Fig. 1, TN and NR NTN mobility enhancement

For potential enhancements in TN, there is notable interest in improving spectrum utilization by introducing a wider channel bandwidth, such as 200MHz. The new wider channel bandwidth introduced in such a later stage of 5G plays a significant impact on both products and specifications, and much standardization effort might also be required for other potential channel bandwidths between 100MHz and 200MHz in a short while. Considering that it could be supported under current CA framework requiring a small overhead, the spectrum utilization improvement is limited. Therefore, the new wider channel bandwidth could be considered for the next generation, with the broader spectrum potentially supported through the CA framework in 5G. Additionally, RAN4 may look into promoting the support for a higher-order bandwidth class for intra-band uplink contiguous CA, such as introducing bandwidth class D, to meet the growing demands for uplink throughput.

- Observation 5:  Uplink bandwidth class promotion could be considered for spectrum utilization enhancement for a wider bandwidth to meeting the growing demands for uplink throughput.

Furthermore, regarding RRM related works, relaxed RLM/BFD for RedCap UEs could be considered via a small effort by reusing Rel-17 relaxed RLM/BFD work for non-RedCap UEs, and similarly less than 5MHz support could also be enabled by defining additional RRM requirements based on punctured SSB for RedCap UE, leveraging the relevant RRM requirements established for normal UE with less than 5MHz.

Observation 6: The enhancement of RedCap with relaxed RLM/BFD requirements and support for less than 5 MHz could be considered in Rel-20 for RRM-related tasks.

Finally, SSB-less SCell enhancement for intra-band intra-band non-contiguous CA (NCCA) is currently missing and could be included in Rel-20 as NES enhancement where the corresponding SCell activation/deactivation delay requirements could be specified.

Observation 7: As NES enhancement, SSB-less SCell enhancement for intra-band NCCA could be considered in Rel-20.

# Conclusions

In this contribution, we share our views on with the following observations for Rel-20 RAN4 works:

Observation 1: In the Rel-20 timeline, a substantial amount of effort will still need to be reserved for maintenance tasks for releases from Rel-15 to Rel-19.

Observation 2: From RAN4 perspective Rel-20 5G-A would likely be a smaller release than Rel-19 with a focus on addressing commercial demands and interests, resolving technical issues identified in real-world scenarios, and making necessary corrections to the current specifications for both NTN and TN.

Observation 3: NTN HD-FDD mobile VSAT can be considered to support in Rel-20.

Observation 4: For other potential NTN enhancements in Rel-20 could include 7MHz channel bandwidth for NR NTN and mobility enhancement between TN and NR NTN in the RRC_CONNECTED state.

Observation 5:  Uplink bandwidth class promotion could be considered for spectrum utilization enhancement for a wider bandwidth to meeting the growing demands for uplink throughput.

Observation 6: The enhancement of RedCap with relaxed RLM/BFD requirements and support for less than 5 MHz could be considered in Rel-20 for RRM-related tasks.

Observation 7: As NES enhancement, SSB-less SCell enhancement for intra-band NCCA could be considered in Rel-20.

# References

RP-242424, “Status Report RAN WG4”, RAN4 Chair (Huawei)