3GPP TSG RAN meeting #107	                                                RP-250555

Incheon, Korea, March 12-14, 2025

Title: 	Views on further network energy saving enhancement in 5G-A Rel-20

Source: 	ZTE Corporation, Sanechips

Agenda item:	15.2.7.2

Document for:	Discussion and decision

Introduction

In R19, we are specifying on demand SSB SCell operation for UE in connected mode configured with CA, on demand SIB1 for UE in idle/inactive mode as well as SSB adaption, PRACH adaption in time domain and paging adaption. While for R20, we still see room for further energy saving at network side and would like to share our thoughts in this paper.

Further enhancement on common channel/signal adaption

The common signals/channels transmission reduces the network sleep opportunity, especially in the case of empty/low load.

The energy consumption caused by periodic SSB transmission is a non-negligible part of that of the gNB. Reducing or even eliminating SSB transmission can bring considerable energy saving gains. In order to reduce the SSB transmission, SSB-less SCell for inter-band CA, on-demand SSB SCell for connected mode UE, and SSB adaptation are specified in Rel-18 and Rel-19. However, due to the limited application scenarios, these energy saving technologies cannot achieve the maximum energy saving gain. For example, SSB-less SCell is limited to FR1 and co-located scenarios, on-demand SSB for SCell only supports NCD-SSB and SSB off sync-raster, and the SSB adaptation also needs to minimize the impacts of initial access.

The applicable scenarios of SSB transmission reduction specified in Rel-18 and Rel-19 are limited.

To provide initial access for idle/inactive mode UE, the maximum SSB transmission cycle is 20ms, which lead to significant network energy consumption. In Rel-20, the common signals/channels reduction for idle/inactive mode UE, such as larger SSB transmission cycle (e.g. larger than 160ms), on-demand SSB transmission, should be further considered.

For a legacy UE, cells with SSB cycle of 160ms can be indicated in the pci-List parameter in smtc2-LP for intra/inter- frequency cell re-selection. That is, SSB cycle of 160ms is already supported for idle/inactive mode UEs in this case. Therefore, extension of SSB cycle can be further considered in other initial access scenarios for idle/inactive mode UEs.

Cell re-selection based on SSB periodicity of 160ms has already been supported for idle/inactive mode UEs.

In addition, the frequency of a UE to perform initial access is very low. The initial cell selection delay caused by a large SSB transmission cycle can be acceptable.

The additional delay of initial cell search caused by a long SSB transmission cycle can be acceptable considering initial access frequency is very low.

For the on-demand SSB transmission for idle/inactive mode UE, a wake up signal to considered to trigger the transmission, similar to the on-demand SIB1 introduced in Rel-19. In addition, the NES cell supporting on-demand SSB can function as a capacity cell and operate in heterogeneous manner with a coverage cell. The idle/inactive mode UEs can acquire the synchronization and WUS configuration of the NES cell through the coverage cell before the SSB is triggered.

The idle/inactive mode UEs can acquire the synchronization and WUS configuration of the NES cell through the coverage cell before the SSB is triggered.

What’s more, considering the impact on the legacy UE, the common signals/channels reduction can be further considered in the new spectrum without commercialized UEs, such as U6GHz, etc. In addition, leaner common signals/channels is also beneficial to better spectrum sharing between 5G-A and 6G.

The common common signals/channels reduction can be further enhanced considering the new spectrum without commercialized UEs, and leaner common signals/channels design to provide better spectrum sharing between 5G-A and 6G.

Therefore, solutions such as on-demand SSB transmission for idle/inactive mode UEs, longer period SSB transmission than 160ms should be supported. Further, SSB-less cell can also be extended to more scenarios.

Solutions such as on-demand SSB transmission for idle/inactive mode UEs, longer periodicity or even no SSB transmission for cell and are beneficial for the NES.

In order to obtain the initial access requests, SI requests, etc., from UEs, the gNB needs to wake up to detect each RO, which leads to energy consumption. PRACH adaptation in time domain is specified in Rel-19 to reduce network energy consumption. In addition, the enhancement of PRACH in spatial domain can also effectively reduce the number of ROs and reduce the energy consumption of the gNB, as shown in Figure 1.

Figure 1 RO-to-SSB mapping

PRACH enhancement in spatial domain is beneficial for NES.

Therefore, further enhancement on common signal/channel adaptation should be discussed and specified in Rel-20.

Further enhancement on common signal/channel adaptation, such as on-demand SSB for idle/inactive mode UEs, larger periodicity or even no SSB transmission, PRACH enhancement in spatial domain, can be specified in 5G-Advanced Rel-20.

SIB-less NES cell for UE in idle/inactive

On demand SIB1 for on-demand SIB1 for UEs in idle/inactive mode will be specified in R19 with the following objective focus on case#2, in which UE will acquire UL WUS configuration from cell A, send on demand request to NES Cell and acquire SIB1 from NES Cell [1].

Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

Also, there has been discussion with agreements reached in RAN2 on the triggering condition of the WUS transmission [2]:

RAN2#126 agreements

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

And the above agreement means that UE will trigger the on demand SIB1 request to NES cell based on the cell reselection priority and measurement results, which infers that UE will trigger such procedure when it moves near the cell center of a NES cell even when there is no upcoming traffic. As a result, it would be quite difficult for a NES cell to find some gap to omit the SIB1 transmission thus the energy saving gain would be quite limited.

With the above consideration in mind, we would like to propose further simplified procedure for UE to camp on cell A as much as possible, move to NES cell when traffic arrives and get back to cell A when traffic ends:

Figure 2: Rel-20 SIB-less NES cell

For Rel-20 SIB-less NES cell, as shown in Figure 2:

- Cell A provide access configuration of NES cell(s)

- NW/UE triggered (MT/MO) access to NES cell

- Release with redirection to Cell A when traffic ends in NES cell

Thus, there is no need for a NES cell to broadcast SIB since the access configuration will be provided via Cell A and UE will move back to cell A thus most of the services in idle and inactive state will be provided via Cell A instead of NES cell if we only want to provide basic services in SIB-less NES cell, e.g. data transmission, as the information contained in SIB1 is mainly for initial access, SIB2-5 are for cell reselection, SIB7/8 for PWS, SIB9 provides time information, SIB10-25 are for specific features like NPN, early measurements, sidelink, V2X, disaster roaming, slice, TRS, NTN. MBS, ATG. Then NES cell would only be used to provide basic service when there is real traffic to arrive, which would bring more power saving gain for such cells.

Support SIB-less NES cell for UE in idle/inactive in Rel-20:

- Cell A provide access configuration of NES cell(s)

- NW/UE triggered (MT/MO) access to NES cell

- Release with redirection to Cell A when traffic ends in NES cell

Dynamic UE specific signal/channel adaptation

In addition to the common signal/channel, periodic UE specific reference signals also reduce network sleep opportunity. Assuming that the transmission cycle of CSI-RS is 10ms (baseline), 20ms (case 1) and 40ms (case 2) respectively, the network energy consumption ratio and the network energy saving gain under different traffic loads are shown in Figure 3.

Figure 3 Network energy consumption and energy saving gain for different CSI-RS configurations

According to the simulation results in Figure 3, the power consumption caused by small CSI-RS transmission cycle accounts for a large part of network energy consumption. Reducing CSI-RS transmission has the opportunity to obtain over 30% energy saving gain in empty/low load scenarios. Hence, dynamic adaptation of CSI-RS/TRS can be considered for Rel-20 NES enhancement.

Dynamic UE specific signal/channel adaptation, e.g., CSI-RS/TRS adaptation, can be specified in Rel-20.

Conclusion

Based on the discussion about Rel-20 NES techniques above, we have the following observations and proposals.

Observation 1: The applicable scenarios of SSB transmission reduction specified in Rel-18 and Rel-19 are limited.

Observation 2: Cell re-selection based on SSB periodicity of 160ms has already been supported for idle/inactive mode UEs.

Observation 3: The additional delay of initial cell search caused by a large SSB transmission cycle can be acceptable considering initial access frequency is very low.

Observation 4: The idle/inactive mode UEs can acquire the synchronization and WUS configuration of the NES cell through the coverage cell before the SSB is triggered.

Observation 5: The common common signals/channels reduction can be further enhanced considering the new spectrum without commercialized UEs, and leaner common signals/channels design to provide better spectrum sharing between 5G-A and 6G.

Observation 6: Solutions such as on-demand SSB transmission for idle/inactive mode UEs, larger periodicity or even no SSB transmission for cell and are beneficial for the NES.

Observation 7: PRACH enhancement in spatial domain is beneficial for NES.

Proposal 1: Further enhancement on common signal/channel adaptation, such as on-demand SSB for idle/inactive mode UEs, larger periodicity or even no SSB transmission, PRACH enhancement in spatial domain, can be specified in 5G-Advanced Rel-20.

Proposal 2: Support SIB-less NES cell for UE in idle/inactive in Rel-20:

Proposal 3: Dynamic UE specific signal/channel adaptation, e.g., CSI-RS/TRS adaptation, can be specified in Rel-20.

References

[1] RP-242354 Revised WID: Enhancements of network energy savings for NR

[2] R2-2406202 RAN2#126 Meeting Report