3GPP TSG-RAN4 Meeting #114	R4-2501278

# Athens, Greece, 17 - 21 February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5351 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Perf) CR on the correction of band group notations in the SSB_RP and SSB Ês/Iot conditions tables R18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2025-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In chapter B.X, several tables show the conditions for measurement, such as SSB-RP, with the incorrect band group notations. Based on the clause in chapter 3.5.2, some band group notations are missing in these tables. Here are the all band groups notations in chapter 3.5.2. Table 3.5.2-1: NR frequency band groups for FR1 Group; NR FDD; NR TDD; NR SDL; NR CCA10. Band group notation; Operating bands; Band group notation; Operating bands; Band group notation; Operating bands; Band group notation; Operating bands. A; NR_FDD_FR1_A; n1, n18, n24, n70, n744, n91, n92, n93, n94, n100, n109; NR_TDD_FR1_A; n34, n389, n39, n40, n50, n51, n53, n54, n101; NR_SDL_FR1_A; n67, n75, n76; NR_CCA_FR1_A; -. B; NR_FDD_FR1_B; n65, n66, n743; NR_TDD_FR1_B; n387; NR_SDL_FR1_B; -; NR_CCA_FR1_B; -. C; NR_FDD_FR1_C; n30; NR_TDD_FR1_C; n48, n771, n78, n79; NR_SDL_FR1_C; -; NR_CCA_FR1_C; -. D; NR_FDD_FR1_D; n28; NR_TDD_FR1_D; n772; NR_SDL_FR1_D; -; NR_CCA_FR1_D; -. E; NR_FDD_FR1_E; n2, n5, n7; NR_TDD_FR1_E; n41, n90; NR_SDL_FR1_E; -; NR_CCA_FR1_E; -. F; NR_FDD_FR1_F; n266; NR_TDD_FR1_F; -; NR_SDL_FR1_F; -; NR_CCA_FR1_F; -. G; NR_FDD_FR1_G; n3, n8, n12, n13, n1412,, n20, n71, n85, n10511, n106; NR_TDD_FR1_G; n104; NR_SDL_FR1_G; n29; NR_CCA_FR1_G; -. H; NR_FDD_FR1_H; n25; NR_TDD_FR1_H; -; NR_SDL_FR1_H; -; NR_CCA_FR1_H; -. I; NR_FDD_FR1_I; -; NR_TDD_FR1_I; NR_SDL_FR1_I; -; NR_CCA_FR1_I; n46. J; NR_FDD_FR1_J; -; NR_TDD_FR1_J; n478,12; NR_SDL_FR1_J; -; NR_CCA_FR1_J; n96, n102. K; NR_FDD_FR1_K; -; NR_TDD_FR1_K; -; NR_SDL_FR1_K; -; NR_CCA_FR1_K; -. L; NR_FDD_FR1_L; -; NR_TDD_FR1_L; -; NR_SDL_FR1_L; -; NR_CCA_FR1_L; -. M; NR_FDD_FR1_M; -; NR_TDD_FR1_M; -; NR_SDL_FR1_M; -; NR_CCA_FR1_M; -. N; NR_FDD_FR1_N; n31, n72; NR_TDD_FR1_N; -; NR_SDL_FR1_N; -; NR_CCA_FR1_N; -. NOTE 1: Except 3.8 GHz to 4.2 GHz. NOTE 2: Only 3.8 GHz to 4.2 GHz. NOTE 3: Except 1475.9 MHz to 1510.9 MHz. NOTE 4: Only when the band is confined in 1475.9 MHz to 1510.9 MHz. NOTE 5: These bands are used only in NR carrier aggregation with other NR bands according to NR CA band combinations specified in TS 38.101-1 [18] and TS 38.101-3 [20]. NOTE 6: The minimum Io condition is reduced by 0.5 dB when the carrier frequency of the assigned NR channel bandwidth is within 865-894 MHz. NOTE 7: When this band is used for V2X SL service, the band is exclusively used for NR V2X in particular regions [18]. NOTE 8: This band is unlicensed band used for V2X service. There is no expected network deployment in this band. [18]. NOTE 9: When this band is only used for WAN service. NOTE 10: Operating bands where operation on carrier frequencies with CCA is supported. NOTE 11: The minimum Io condition is reduced by 0.5 dB when the downlink channel overlap the 612-617 MHz frequency range and the channel bandwidth is 5 MHz. NOTE 12: This band can be used for V2X SL operation.. |
| Summary of change: |  | The changes are in following part: - Add the missed band group notations in the tables. - Some typos have been edited. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | All the band group related test cases are not fully specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.1.2, B.1.4, B.2.2, B.2.3, B.2.4.1, B.2.4.2, B.2.5, B.2.8.1, B.2.8.2.1, B.2.8.2.2, B.2.8.3.1, B. 2.8.3.2, B.2.12, B.2.13, B.2.14, B.2.15, B.2.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

## B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection

This clause defines the following conditions for NR intra-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.1.2-1 for FR1 NR cells.

The conditions are defined in Table B.1.2-2 for FR2 NR cells.

Table B.1.2-1: Conditions for intra-frequency cell re-selection in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -4 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_F | -121.5 | -118.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
|  | NR_FDD_FR1_N | -117.5 | -114.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

### <End of Change 1>

### <Start of Change 2>

## B.1.4	Conditions for measurements on NR intra-frequency cells for cell re-selection for RedCap

This clause defines the following conditions for RedCap NR intra-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.1.4-1 and Table B.1.4-2 for 2 Rx and 1 Rx RedCap respectively for FR1 NR cells.

The conditions are defined in Table B.1.4-3 for FR2 NR cells.

Table B.1.4-1: Conditions for intra-frequency cell re-selection in FR1 for 2Rx RedCap

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_ FR1_A, NR_TDD_ FR1_A, NR_SDL_FR1_A | -124 | -121 | -4 |
|  | NR_FDD_ FR1_B, NR_TDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_FDD_ FR1_C NR_TDD_ FR1_C | -123 | -120 |  |
|  | NR_FDD_ FR1_D, NR_TDD_ FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_ FR1_E, NR_TDD_ FR1_E | -122 | -119 |  |
|  | NR_FDD_ FR1_F | -121.5 | -118.5 |  |
|  | NR_FDD_ FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -121 | -118 |  |
|  | NR_FDD_ FR1_H | -120.5 | -117.5 |  |
|  | NR_FDD_ FR1_N | -117.5 | -114.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.1.4-2: Conditions for intra-frequency cell re-selection in FR1 for 1Rx RedCap

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_ FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -4 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_FDD_ FR1_C NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_F | -121.5 | -118.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
|  | NR_FDD_ FR1_N | -117.5 | -114.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

### <End of Change 2>

### <Start of Change 3>

## B.2.2	Conditions for NR intra-frequency measurements

This clause defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.2-2 for FR2 NR cells.

Table B.2.2-1: Conditions for intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_TDD_FR1_C | -126 | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  |
|  | NR_FDD_FR1_F | -124.5 | -121.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -124 | -121 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 |  |
|  | NR_FDD_ FR1_N | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

### <End of Change 3>

### < Start of Change 4>

## B.2.3	Conditions for NR inter-frequency measurements

This clause defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.3-1 for FR1 NR cells.

The conditions are defined in Table B.2.3-2 for FR2 NR cells.

Table B.2.3-1: Conditions for inter-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_FDD_FR1_C , NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_F | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
|  | NR_FDD_ FR1_N | -118.5 | -115.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

### <End of Change 4>

### < Start of Change 5>

### B.2.4.1	Conditions for SSB based L1-RSRP reporting

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.1-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.1-2 for FR2 NR cells.

Table B.2.4.1-1: Conditions for SSB based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -3 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_F | -121.5 | -118.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
|  | NR_FDD_ FR1_N | -117.5 | -114.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

### <End of Change 5>

### < Start of Change 6>

### B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on CSI-RS: CSI-RS_RP and CSI-RS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.2-2 for FR2 NR cells.

Table B.2.4.2-1: Conditions for CSI-RS based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum CSI-RS_RP |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -118 | -3 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B | -123.5 | -120.5 | -117.5 |  |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C | -123 | -120 | -117 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 | -116.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 | -116 |  |
|  | NR_FDD_FR1_F | -121.5 | -118.5 | -115.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -121 | -118 | -115 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 | -114.5 |  |
|  | NR_FDD_ FR1_N | -117.5 | -114.5 | -111.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

### <End of Change 6>

### < Start of Change 7>

## B.2.5	Conditions for RRC connection release with redirection to NR

This clause defines the following conditions for RRC connection release with redirection to NR: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.5-1 for FR1 NR cells.

The conditions are defined in Table B.2.5-2 for FR2 NR cells.

Table B.2.5-1: Conditions for for RRC connection release with redirection to NR in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_F | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
|  | NR_FDD_ FR1_N | -118.5 | -115.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

### <End of Change 7>

### < Start of Change 8>

### B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured

This clause defines the following conditions for NR L1-SINR measurement reporting and corresponding procedures performed based on CSI-RSs: CSI-RS_RP and CSI-RS Ês/Iot, applicable for a corresponding operating band.

The conditions defined in Table B.2.8.1-1 for FR1 NR cells.

The conditions defined in Table B.2.8.1-2 for FR2 NR cells.

Table B.2.8.1-1: Conditions for L1-SINR measurements with CSI-RS based CMR only in FR1

| Parameter | NR operating band groups Note1 | Minimum CSI-RS_RP |  |  | CSI-RS CMR Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -118 | -3 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B | -123.5 | -120.5 | -117.5 |  |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C | -123 | -120 | -117 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 | -116.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 | -116 |  |
|  | NR_FDD_FR1_F | -121.5 | -118.5 | -115.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -121 | -118 | -115 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 | -114.5 |  |
|  | NR_FDD_ FR1_N | -117.5 | -114.5 | -111.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

### <End of Change 8>

### < Start of Change 9>

#### B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured

This clause defines the following conditions for NR L1-SINR measurement reporting and corresponding procedures performed based on SSBs and ZP-IMRs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions defined in Table B.2.8.2.1-1 for FR1 NR cells.

The conditions defined in Table B.2.8.2.1-2 for FR2 NR cells.

Table B.2.8.2.1-1: Conditions for L1-SINR measurements with SSB based CMR and ZP-IMR in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -3 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_F | -121.5 | -118.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
|  | NR_FDD_ FR1_N | -117.5 | -114.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

### <End of Change 9>

### < Start of Change 10>

#### B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured

This clause defines the following conditions for NR L1-SINR measurement reporting and corresponding procedures performed based on SSBs and NZP-IMRs: SSB_RP, SSB Ês/Iot and NZP-IMR Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.8.2.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.8.2.2-2 for FR2 NR cells.

Table B.2.8.2.2-1: Conditions for L1-SINR measurements with SSB based CMR and NZP-IMR in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot | NZP-IMR Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | 0 | 0 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B | -120.5 | -117.5 |  |  |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C | -120 | -117 |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 |  |  |
|  | NR_FDD_FR1_F | -118.5 | -115.5 |  |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -118 | -115 |  |  |
|  | NR_FDD_FR1_H | -117.5 | -114.5 |  |  |
|  | NR_FDD_ FR1_N | -114.5 | -111.5 |  |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

### <End of Change 10>

### < Start of Change 11>

#### B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured

This clause defines the following conditions for NR L1-SINR measurement reporting and corresponding procedures performed based on CSI-RSs and ZP-IMRs: CSI-RS_RP and CSI-RS Ês/Iot, applicable for a corresponding operating band.

The conditions defined in Table B.2.8.3.1-1 for FR1 NR cells.

The conditions defined in Table B.2.8.3.1-2 for FR2 NR cells.

Table B.2.8.3.1-1: Conditions for L1-SINR measurements with CSI-RS based CMR and ZP-IMR in FR1

| Parameter | NR operating band groups Note1 | Minimum CSI-RS_RP |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -118 | -3 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B | -123.5 | -120.5 | -117.5 |  |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C | -123 | -120 | -117 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 | -116.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 | -116 |  |
|  | NR_FDD_FR1_F | -121.5 | -118.5 | -115.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -121 | -118 | -115 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 | -114.5 |  |
|  | NR_FDD_FR1_N | -117.5 | -114.5 | -111.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

### <End of Change 11>

### < Start of Change 12>

#### B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured

This clause defines the following conditions for NR L1-SINR measurement reporting and corresponding procedures performed based on CSI-RSs and NZP-IMRs: CSI-RS_RP, CSI-RS Ês/Iot and NZP-IMR Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.8.3.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.8.3.2-2 for FR2 NR cells.

Table B.2.8.3.2-1: Conditions for L1-SINR measurements with CSI-RS based CMR and NZP-IMR in FR1

| Parameter | NR operating band groups Note1 | Minimum CSI-RS_RP |  |  | CSI-RS Ês/Iot | NZP-IMR Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  |  | dB | dB |
|  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | 0 | 0 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B | -120.5 | -117.5 | -114.5 |  |  |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C | -120 | -117 | -114 |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 |  |  |
|  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 |  |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -118 | -115 | -112 |  |  |
|  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 |  |  |
|  | NR_FDD_ FR1_N | -114.5 | -111.5 | -108.5 |  |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |  |

### <End of Change 12>

### < Start of Change 13>

## B.2.12	Conditions for NR CSI-RS based intra-frequency measurements

This clause defines the following conditions for NR CSI-RS based intra-frequency measurements and corresponding procedures performed based on CSI-RS: CSI_RP and CSI-RS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.12-1 for FR1 NR cells.

The conditions are defined in Table B.2.12-2 for FR2 NR cells.

Table B.2.12-1: Conditions for CSI-RS based intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum CSI_RP |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -6 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B | -126.5 | -123.5 | -120.5 |  |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C | -126 | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 |  |
|  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -124 | -121 | -118 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 |  |
|  | NR_FDD_ FR1_N | -120.5 | -117.5 | -114.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

### <End of Change 13>

### < Start of Change 14>

## B.2.13	Conditions for NR CSI-RS based inter-frequency measurements

This clause defines the following conditions for NR CSI-RS based inter-frequency measurements and corresponding procedures performed based on CSI-RS: CSI_RP and CSI-RS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.13-1 for FR1 NR cells.

The conditions are defined in Table B.2.13-2 for FR2 NR cells.

Table B.2.13-1: Conditions for CSI-RS based inter-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum CSI_RP |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -125 | -122 | -119 | -6 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B | -124.5 | -121.5 | -118.5 |  |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C | -124 | -121 | -118 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 | -117.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 | -117 |  |
|  | NR_FDD_FR1_F | -122.5 | -119.5 | -116.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -122 | -119 | -116 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 | -115.5 |  |
|  | NR_FDD_ FR1_N | -118.5 | -115.5 | -112.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

### <End of Change 14>

### < Start of Change 15>

## B.2.14	Conditions for NR PRS-based measurements

This clause defines the following conditions for NR PRS-based measurements and corresponding procedures performed based on PRS: PRP and PRS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.14-1 for FR1 NR cells.

The conditions are defined in Table B.2.14-2 for FR2 NR cells.

Table B.2.14-1: Conditions for NR PRS-based measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum PRP1,2 |  |  | PRS Ês/Iot |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSPRS |  |  | dB |  |
|  |  | SCSPRS = 15 kHz | SCSPRS = 30 kHz | SCSPRS = 60 kHz |  |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -6 Note2 -13 Note3 -3 Note4 -10 Note8 | -6 Note5 -3 Note6 0 Note7 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B | -126.5 | -123.5 | -120.5 |  |  |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C | -126 | -123 | -120 |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 |  |  |
|  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 |  |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -124 | -121 | -118 |  |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 |  |  |
|  | NR_FDD_FR1_N | -120.5 | -117.5 | -114.5 |  |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. NOTE 2: PRS Ês/Iot for RSTD measurement reference cell PRS resource, and RSCPD measurement reference cell PRS resource. NOTE 3: PRS Ês/Iot for RSTD measurement neighbor cell PRS resource, and RSCPD measurement neighbor cell PRS resource, PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement, and DL RSCP measurement. NOTE 4: PRS Ês/Iot for PRS-RSRP measurement, PRS-RSRPP measurement, UE Rx-Tx time difference measurement, DL RSCP measurement, and PRS-RSRP measurement when performed by 1Rx RedCap UE. NOTE 5: PRS Ês/Iot for RSTD measurement neighbor cell PRS resource, RSCPD measurement neighbor cell PRS resource, PRS-RSRP measurement, PRS-RSRPP measurement, UE Rx-Tx time difference measurement when performed with reduced number of samples, and DL RSCP measurement. NOTE 6: PRS Ês/Iot for RSTD measurement reference cell PRS resource when performed with reduced number of samples, and DL RSCPD measurement reference cell PRS resource. NOTE 7: PRS Ês/Iot for PRS-RSRP measurement, PRS-RSRPP measurement, UE Rx-Tx time difference measurement when performed with reduced number of samples, and DL RSCP measurement. NOTE 8: PRS Ês/Iot for RSTD measurement neighbor cell PRS resource, UE Rx-Tx measurements when performed by 1Rx RedCap UE, and PRS-RSRPP measurement when performed by 1Rx RedCap UE. |  |  |  |  |  |  |

### <End of Change 15>

### < Start of Change 16>

## B.2.15	Conditions for NR intra-frequency measurements for RedCap

This clause defines the following conditions for RedCap NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.15-1 and Table B.2.15-2 for 1 Rx and 2 Rx RedCap respectively for FR1 NR cells.

The conditions are defined in Table B.2.15-3 for FR2 NR cells.

Table B.2.15-1: Conditions for intra-frequency cell re-selection in FR1 for 1Rx RedCap

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_ FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_ FR1_B, NR_TDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_FDD_FR1_C, NR_TDD_ FR1_C | -126 | -123 |  |
|  | NR_FDD_ FR1_D, NR_TDD_ FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_ FR1_E, NR_TDD_ FR1_E | -125 | -122 |  |
|  | NR_FDD_ FR1_F | -124.5 | -121.5 |  |
|  | NR_FDD_ FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -124 | -121 |  |
|  | NR_FDD_ FR1_H | -123.5 | -120.5 |  |
|  | NR_FDD_FR1_N | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.15-2: Conditions for intra-frequency cell re-selection in FR1 for 2Rx RedCap

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_ FR1_A, NR_TDD_ FR1_A, NR_SDL_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_ FR1_B, NR_TDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_FDD_FR1_C, NR_TDD_ FR1_C | -126 | -123 |  |
|  | NR_FDD_ FR1_D, NR_TDD_ FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_ FR1_E, NR_TDD_ FR1_E | -125 | -122 |  |
|  | NR_FDD_ FR1_F | -124.5 | -121.5 |  |
|  | NR_FDD_ FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -124 | -121 |  |
|  | NR_FDD_ FR1_H | -123.5 | -120.5 |  |
|  | NR_FDD_FR1_N | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

### <End of Change 16>

### < Start of Change 17>

## B.2.16	Conditions for NR inter-frequency measurements for RedCap

This clause defines the following conditions for RedCap NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.16-1 and Table B.2.16-2 for 1 Rx and 2 Rx RedCap for FR1 NR cells.

The conditions are defined in Table B.2.16-3 for FR2 NR cells.

Table B.2.16-1: Conditions for inter-frequency measurements in FR1 for 1 Rx RedCap

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_F | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.16-2: Conditions for inter-frequency measurements in FR1 for 2 Rx RedCap

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B, NR_TDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_FDD_FR1_C, NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_F | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

### <End of Change 17>