3GPP TSG-RAN4 Meeting #114	R4-2501923

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5407 | rev | - | Current version: | 16.22.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Perf) Corrections in A.4.7.1.2 and A.6.7.1.2 on derived parameters (Rel 16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2025-02-20 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Derived parameters SS-RSRP and Io for NR serving cell in Test 2 in A.4.7.1.2 and A.6.7.1.2 do not meet with Noc level and SNR requirements. Considering Noc level ( for Cell 3 +8dB) and SNR requirement (13dB) for NR serving cell, derived parameter SS-RSRP should be (RSRP for Cell 3 +24dB) instead of (RSRP for Cell 3 +25dB) and Io parameter should be (Io for Channel 3 +19.45dB) instead of (Io for Channel 3 +19.75dB) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Derived parameters SS-RSRP and Io for NR serving cell in Test 2 in A.4.7.1.2 and A.6.7.1.2 to meet with Noc level and SNR requirements. SS-RSRP updated to (RSRP for Cell 3 +24dB) for NR serving cell and Io parameter updated to (Io for Channel 3 +19.45dB) for NR serving cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformance UE will be measuring different SS-RSRP and Io for NR serving cell than the values stated in test parameter table |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.7.1.2, A.6.7.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.533 CR 3811 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Cat A CR aligned with R4-2502634 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.4.7.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Clauses 10.1.4.1.1 and 10.1.4.1.2 for inter-frequency measurements with the testing configurations in Table A.4.7.1.2.1-1.

Table A.4.7.1.2.1-1: Applicable NR configurations for FR1 inter-frequency SS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations on each supported band |  |

##### A.4.7.1.2.2	Test parameters

In this set of test cases there are three cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2) and a FR1 neighbour cell (Cell 3) on a different frequency than the PSCell. The test parameters and applicability for Cell 1 are defined in A.3.7.2. The test parameters for the Cell 2 and Cell 3 are given in Table A.4.7.1.2.2-1 below. Both absolute and relative accuracy of RSRP inter-frequency measurements are tested by using the parameters in Table A.4.7.1.2.2-1. The inter-frequency measurements are supported by a measurement gap.

Table A.4.7.1.2.2-1: SS-RSRP inter-frequency test parameters

| Parameter |  | Config | Unit | Test 1 |  |  |  | Test 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 |  | Cell 3 |  | Cell 2 |  | Cell 3 |  |
| SSB ARFCN |  | 1~6 |  | freq1 |  | freq2 |  | freq1 |  | freq2 |  |
| BWchannel |  | 1,4 | MHz | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |  |
|  |  | 2,5 |  | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |  |
|  |  | 3,6 |  | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |  |
| Gap pattern ID |  |  |  | 0 |  |  |  | 0 |  |  |  |
| Duplex mode |  | 1,4 |  | FDD |  |  |  | FDD |  |  |  |
|  |  | 2,5 |  | TDD |  |  |  | TDD |  |  |  |
|  |  | 3,6 |  | TDD |  |  |  | TDD |  |  |  |
| TDD configuration |  | 1,4 |  | N/A |  |  |  | N/A |  |  |  |
|  |  | 2,5 |  | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |  |
|  |  | 3,6 |  | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |  |
| PDSCH Reference measurement channel |  | 1,4 |  | SR.1.1 FDD |  | - |  | SR.1.1 FDD |  | - |  |
|  |  | 2,5 |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |
|  |  | 3,6 |  | SR.2.1 FDD |  |  |  | SR.2.1 FDD |  |  |  |
| RMSI CORESET Reference Channel |  | 1,4 |  | CR.1.1 FDD |  | - |  | CR.1.1 FDD |  | - |  |
|  |  | 2,5 |  | CR.1.1 TDD |  | - |  | CR.1.1 TDD |  | - |  |
|  |  | 3,6 |  | CR.2.1 FDD |  | - |  | CR.2.1 FDD |  | - |  |
| Dedicated CORESET Reference Channel |  | 1,4 |  | CCR.1.1 FDD |  | - |  | CCR.1.1 FDD |  | - |  |
|  |  | 2,5 |  | CCR.1.1 TDD |  | - |  | CCR.1.1 TDD |  | - |  |
|  |  | 3,6 |  | CCR.2.1 TDD |  | - |  | CCR.2.1 TDD |  | - |  |
| SSB configuration |  | 1,4 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |  |
|  |  | 2,5 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |  |
|  |  | 3,6 |  | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |  |  |
| OCNG Patterns |  | 1~6 |  | OP.1 |  |  |  | OP.1 |  |  |  |
| TRS configuration |  | 1,4 |  | TRS.1.1 FDD |  |  | - | TRS.1.1 FDD |  |  | - |
|  |  | 2,5 |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |  |
|  |  | 3,6 |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |  |
| Initial BWP Configuration |  | 1~6 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP configuration |  | 1~6 |  | DLBWP.1.1 ULBWP.1.1 |  |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| Time offset with Cell 2 |  | 1,4 | ms | - | 3 |  |  | - | 3 |  |  |
|  |  | 2,3,5,6 | s | - | 3 |  |  | - | 3 |  |  |
| SMTC configuration |  | 1,4 |  | SMTC.2 |  |  |  | SMTC.2 |  |  |  |
|  |  | 2,3,5,6 |  | SMTC.1 |  |  |  | SMTC.1 |  |  |  |
| EPRE ratio of PSS to SSS |  | 1~6 | dB | 0 |  | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1~6 | dBm/15kHz | -94.65 |  |  |  | ( for Cell 3 +8dB) |  | -115 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -114.5 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -114 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -113.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -113 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -112 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -111.5 |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2,4,5 | dBm/SSB SCS | -94.65 |  |  |  | ( for Cell 3 +8dB) |  | -115 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -114.5 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -114 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -113.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -113 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -112 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -111.5 |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3,6 |  | -91.65 |  |  |  | ( for C 3 +8dB) |  | -112.00 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -111.50 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -111.00 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -110.50 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -110.00 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -109.00 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -108.50 |  |
|  |  | 1~6 | dB | 10 |  | 10 |  | 13 |  | -3 |  |
| SS-RSRPNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2,4,5 | dBm/SCS | -84.65 |  |  |  | (RSRP for Cell 3 +24dB) |  | -118.00 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -117.50 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -117.00 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -116.50 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -116.00 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -115.00 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -114.50 |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3,6 |  | -81.65 |  |  |  | (RSRP for Cell 3 +24dB) |  | -115.00 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -114.50 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -114.00 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -113.50 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -113.00 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -112.00 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -111.50 |  |
| IoNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, | 1,2,4,5 | dBm/ 9.36MHz | -56.28 |  |  |  | (Io for Channel 3 +19.45dB) |  | -85.28 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -84.78 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -84.28 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -83.78 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -83.28 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -82.28 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -81.78 |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, | 3,6 | dBm/ 38.16MHz | -50.19 |  |  |  | (Io for Channel 3 +19.45dB) |  | -79.19 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -78.69 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -78.19 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -77.69 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -77.19 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -76.19 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -75.69 |  |
|  |  | 1~6 | dB | 10 |  | 10 |  | 13 |  | -3 |  |
| Propagation condition |  | 1~6 | - | AWGN |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5 The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |  |  |

##### A.4.7.1.2.3	Test Requirements

The SS-RSRP measurement accuracy for Cell 2 and Cell 3 shall fulfil the Absolute requirement in clause 10.1.4.1.1 and Relative requirement in clause 10.1.4.1.2.

## <<< Skip unchanged sections >>>

#### A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.4.1.1 and 10.1.4.1.2 for inter-frequency measurements with the testing configurations for NR cells in Table A.6.7.1.2.1-1.

Table A.6.7.1.2.1-1: Applicable NR configurations for FR1 inter-frequency SS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.1.2.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on a different frequency than the PCell. The test parameters for the Cell 1 and Cell 2 are given in Table A.6.7.1.2.2-1 below. Both absolute and relative accuracy of RSRP inter-frequency measurements are tested by using the parameters in Table A.6.7.1.2.2-1. The inter-frequency measurements are supported by a measurement gap.

Table A.6.7.1.2.2-1: SS-RSRP inter-frequency test parameters

| Parameter |  | Config | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 |  | Cell 2 | Cell 1 |  | Cell 2 |
| SSB ARFCN |  | 1~3 |  | freq1 |  | freq2 | freq1 |  | freq2 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 2 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 3 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| Duplex mode |  | 1 |  | FDD |  |  | FDD |  |  |
|  |  | 2 |  | TDD |  |  | TDD |  |  |
|  |  | 3 |  | TDD |  |  | TDD |  |  |
| TDD configuration |  | 1 |  | N/A |  |  | N/A |  |  |
|  |  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD |  | - | SR.1.1 FDD |  | - |
|  |  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD |  | - | CR.1.1 FDD |  | - |
|  |  | 2 |  | CR.1.1 TDD |  | - | CR.1.1 TDD |  | - |
|  |  | 3 |  | CR.2.1 TDD |  | - | CR.2.1 TDD |  | - |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD |  | - | CCR.1.1 FDD |  | - |
|  |  | 2 |  | CCR.1.1 TDD |  | - | CCR.1.1 TDD |  | - |
|  |  | 3 |  | CCR.2.1 TDD |  | - | CCR.2.1 TDD |  | - |
| SSB configuration |  | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD |  | - | TRS.1.1 FDD |  |  |
|  |  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| Time offset with Cell 1 |  | 1 | ms | - | 3 |  | - | 3 |  |
|  |  | 2,3 | s | - | 3 |  | - | 3 |  |
| SMTC configuration |  | 1 |  | SMTC.2 |  |  | SMTC.2 |  |  |
|  |  | 2,3 |  | SMTC.1 |  |  | SMTC.1 |  |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 |  | 0 | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 |  |  | ( for Channel 2 +8dB) |  | -115 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -114.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/SSB SCS | -94.65 |  |  | ( for Channel 2 +8dB) |  | -115 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -114.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -91.65 |  |  | ( for Channel 2 +8dB) |  | -112.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -111.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -111.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -110.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -110.00 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -109.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -108.50 |
|  |  | 1~3 | dB | 10 |  | 10 | 13 |  | -3 |
| SS-RSRPNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2,4,5 | dBm/SCS | -84.65 |  |  | (RSRP for Cell 2 +24dB) |  | -118.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -117.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -117.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -116.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -116.00 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -115.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -114.50 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -81.65 |  |  | (RSRP for Cell 2 +24dB) |  | -115.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -114.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -114.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113.00 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -112.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.50 |
| IoNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/ 9.36MHz | -56.28 |  |  | (Io for Channel 2 +19.45dB) |  | -85.28 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -84.78 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -84.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -83.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -83.28 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -82.28 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -81.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 | dBm/ 38.16MHz | -50.19 |  |  | (Io for Channel 2 +19.45dB) |  | -79.19 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -78.69 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -78.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -77.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -77.19 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -76.19 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -75.69 |
|  |  | 1~3 | dB | 10 |  | 10 | 13 |  | -3 |
| Propagation condition |  | 1~3 | - | AWGN |  |  | AWGN |  |  |
| Antenna configuration |  | 1~3 |  | 1x2 |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |

##### A.6.7.1.2.3	Test Requirements

The SS-RSRP measurement accuracy for Cell 1 and Cell 2 shall fulfil the absolute requirement in clause 10.1.4.1.1 and relative requirement in clause 10.1.4.1.2.

## <<< END OF CHANGES >>>