3GPP TSG-RAN WG4 Meeting #114	R4-2502349

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.181 | CR | 0056 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TEI18) CR to 38.181 NB-IoT inband operation with NR NTN [NTNNBIoT_inbandNTNNR] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | EchoStar, Thales, Inmarsat, Viasat |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-02-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is strong NTN industry demand to enable NB-IoT operation in NTN NR in-band. SAN conformance testing is only supported for stand-alone NB-IoT NTN but not for in-band operation in NTN NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates RF test methods and conformance requirements for NB-IoT operation in NTN NR in-band. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SAN cannot be tested for NB-IoT operation in NTN NR in-band |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 3.1, 3.3, 4.6, 4.7.2, 4.7.3.1, 4.8.3, new clause 4.9.2.2.7, new clause 4.9.2.6, new clause 4.9.3, 5, 6.2.5, new clause 6.3.4, 6.5.2.5, 6.5.3.5, 6.6.2.4.1, 6.6.3, 6.6.3.4.2, 6.6.4.1, 6.6.4.4.2, 6.6.5.1, 6.6.5.4.2, 7.1, 7.2.4.2, 7.2.5, 7.3.4.2, 7.3.5, 7.4.1.4.2, 7.4.1.5, 7.5.4.2, 7.5.5, 7.8.4.2, 7.8.5, 9.2.5, 9.3.5.1, new clause 9.4.4, 9.6.2.5, 9.6.3.5.1, 9.7.2.4.1, 9.7.3.1, 9.7.3.4.2, 9.7.4.1, 9.7.4.4.2, 9.7.5.2.1, 9.7.5.2.4.2, 10.1, 10.2.4.2, 10.2.5.2, 10.3.4.2, 10.3.5.2, 10.4.4.2, 10.4.5.2, 10.5.1.4.2, 10.5.1.5.2, 10.6.5, 10.9.4.2, 10.9.5.1, Annex A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR has been prepared re-using CR 0093 for 38.141-1 as the basis (R4-2002743). Moreover, since conducted and radiated conformance tests for standalone NB-IoT NTN have already been specified, this CR captures updates to both conducted and radiated tests. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 1	Scope

The present document specifies the Radio Frequency (RF) test methods and conformance requirements for NR Satellite Access Node (SAN) type 1-H, SAN type 1-O and SAN type 2-O. The document also specifies the RF test methods and conformance requirements for NB-IoT operation in NTN NR in-band SAN type 1-H and SAN type 1-O. These have been derived from and are consistent with the conducted requirements for SAN type 1-H and radiated requirement for SAN type 1-H, SAN type 1-O and SAN type 2-O in NR SAN specification defined in TS 38.108 [2].

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.108: “Satellite Access Node radio transmission and reception”

[3]	ITU-R Recommendation M.1545, "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000"

[4]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain"

[5]	ITU-R Recommendation SM.328: "Spectra and bandwidth of emissions"

[6]	ITU-R Recommendation SM.1541-6: "Unwanted emissions in the out-of-band domain".

[7]	3GPP TS 38.212: "NR; Multiplexing and channel coding"

[8]	3GPP TS 38.211: "NR; Physical channels and modulation"

[9]	3GPP TS 38.214: "NR; Physical layer procedures for data"

[10]	ITU-T Recommendation O.150, "Equipment for the measurement of digital and analogue/digital parameters"

[11]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz"

[12]	3GPP TS 38.101-5: "NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements”

[13]	3GPP TR 37.941: "Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements"

[14]	IEC 60 068-2-1 (2007): "Environmental testing - Part 2: Tests. Tests A: Cold"

[15]	IEC 60 068-2-2: (2007): "Environmental testing - Part 2: Tests. Tests B: Dry heat"

[16]	IEC 60 068-2-6: (2007): "Environmental testing - Part 2: Tests - Test Fc: Vibration (sinusoidal)"

[17]	3GPP TR 38.811:  "Study on New Radio (NR) to support non-terrestrial networks (Release 15)"

[18]	3GPP TS 38.141-2: "NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing"

[19]	IEC 60721-3-3: "Classification of environmental conditions –Part 3-3: Classification of groups of environmental parameters and their severities – Stationary use at weatherprotected locations"

[20]	IEC 60721-3-4: "Classification of environmental conditions - Part 3-4: Classification of groups of environmental parameters and their severities - Stationary use at non-weatherprotected locations"

[21]	IEC 60721-1: "Classification of environmental conditions Part 1: Environmental parameters and their severities"

[22]	3GPP TS 36.108: "Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node radio transmission and reception"

[23]	3GPP TS 36.181: "Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node radio conformance testing"

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

<< Unchanged text skipped>>

non-terrestrial networks: Networks, or segments of networks, using an airborne or space-borne vehicle to embark a transmission equipment relay node or SAN.

NB-IoT operation in NTN NR in-band: NB-IoT is operating in-band when it is located within a NR transmission bandwidth configuration plus 15 kHz at each edge but not within the NR minimum guard band GBChannel.

NB-IoT operation in NTN NR guard band: NB-IoT is operating in guard band when it is located within a NR SAN channel bandwidth but is not NB-IoT operation in NR in-band.

operating band: frequency range in which NR operates (paired or unpaired), that is defined with a specific set of technical requirements.

NOTE:	The operating band(s) for a SAN is declared by the manufacturer according to the designations in tables 5.2-1 and 5.2-2.

<< Unchanged text skipped>>

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AA	Antenna Array

AAS	Active Antenna System

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AoA	Angle of Arrival

AWGN	Additive White Gaussian Noise

BW	Bandwidth

CA	Carrier Aggregation

CATR	Compact Antenna Test Range

CP-OFDM	Cyclic Prefix-OFDM

CW	Continuous Wave

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DM-RS	Demodulation Reference Signal

EIRP	Equivalent Isotropic Radiated Power

EIS	Equivalent Isotropic Sensitivity

EUT	Equipment Under Test

EVM	Error Vector Magnitude

FDD	Frequency Division Duplex

FR	Frequency Range

FRC	Fixed Reference Channel

GEO	Geostationary Earth Orbiting

GSCN	Global Synchronization Channel Number

ICS	In-Channel Selectivity

LEO	Low Earth Orbiting

MCS	Modulation and Coding Scheme

NB-IoT	Narrowband – Internet of Things

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

NTN	Non-Terrestrial Network

OOB	Out-of-band

OOBE	Out-of-band Emissions

OSDD	OTA Sensitivity Directions Declaration

OTA	Over-The-Air

PRB	Physical Resource Block

PT-RS	Phase Tracking Reference Signal

QAM	Quadrature Amplitude Modulation

RB	Resource Block

RDN	Radio Distribution Network

RE	Resource Element

REFSENS	Reference Sensitivity

RF	Radio Frequency

RIB	Radiated Interface Boundary

RMS	Root Mean Square (value)

RoAoA	Range of Angles of Arrival

RF	Radio Frequency

RX	Receiver

SAN 	Satellite Access Node

SCS	Sub-Carrier Spacing

SSB	Synchronization Signal Block

TAB	Transceiver Array Boundary

TRP	Total Radiated Power

TT	Test Tolerance

TX	Transmitter

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

## 4.6	Manufacturer declarations

The following SAN declarations listed in table 4.6-1, when applicable to the SAN under test, are required to be provided by the manufacturer for the conducted requirements testing of the SAN type 1-H, and radiated requirements testing of SAN type 1-H, SAN type 1-O and SAN type 2-O.

Table 4.6-1 Manufacturers declarations for SAN type 1-H conducted test requirements, and for SAN type 1-H, SAN type 1-O and SAN type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | SAN type 1-H (Note 2) | SAN type 1-O | SAN type 2-O |
| D.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the SAN enclosure. | x | x | x |
| D.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the SAN enclosure. | x | x | x |
| D.3 | Beam identifier | A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. The vendor may declare any number of beams with unique identifiers. The minimum set to declare for conformance, corresponds to the beams at the reference beam direction with the highest intended EIRP, and covering the properties listed below: 1) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ possible when narrowest intended BeWθ is used. 2) A beam with the narrowest intended BeWϕ and narrowest intended BeWθ possible when narrowest intended BeWϕ is used. 3) A beam with the widest intended BeWθ and widest intended BeWϕ possible when widest intended BeWθ is used. 4) A beam with the widest intended BeWϕ and widest intended BeWθ possible when widest intended BeWϕ is used. 5) A beam which provides the highest intended EIRP of all possible beams. When selecting the above five beam widths for declaration, all beams that the SAN is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. (Note 3) | x | x | x |
| D.4 | Operating bands and frequency ranges | List of NR operating band(s) supported by the SAN and if applicable, frequency range(s) within the operating band(s) that the SAN can operate in. Supported bands declared for every beam for SAN type 1-O (D.3), or every TAB connector for SAN type 1-H. (Note 4) | c | x | x |
| D.5 | SAN requirements set | Declaration of one of the NR satellite access node requirement's set as defined for SAN type 1-H, or SAN type 1-O. | c | x | x |
| D.6 | SAN class | Declared as GEO SAN, or LEO SAN. | c | x | x |
| D.7 | SAN channel band width and SCS support | SAN supported SCS and channel bandwidth per supported SCS. Declared for each beam for SAN type 1-O (D.3) or each TAB connector for SAN type 1-H, and each operating band (D.4). | c | x | x |
| D.8 | OTA peak directions set reference beam direction pair | The beam direction pair, describing the reference beam peak direction and the reference beam centre direction. Declared for every beam (D.3). | x | x | x |
| D.9 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam (D.3). | x | x | x |
| D.10 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). | x | x | x |
| D.11 | Rated beam EIRP (Prated,c,EIRP) | The rated EIRP level per carrier (Prated,c,EIRP) at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.8). Declared for every beam (D.3). (Note 11) | x | x | x |
| D.12 | Beamwidth | The beamwidth for the reference beam direction pair and the four maximum steering directions. Declared for every beam (D.3). | x | x | x |
| D.13 | Equivalent beams | List of beams which are declared to be equivalent. Equivalent beams imply that the beams are expected to have identical OTA peak directions sets and intended to have identical spatial properties at all steering directions within the OTA peak directions set when presented with identical signals. All declarations (D.4 – D.12) made for the beams are identical and the transmitter unit, RDN and antenna array responsible for generating the beam are of identical design. | x | x | x |
| D.14 | Parallel beams | List of beams which have been declared equivalent (D.13) and can be generated in parallel using independent RF power resources. Independent power resources mean that the beams are transmitted from mutually exclusive transmitter units. | x | x | x |
| D.15 | Number of carriers at maximum TRP | The number of carriers per operating band the SAN is capable of generating at maximum TRP declared for every beam (D.3). | n/a | x | x |
| D.16 | Maximum Satellite Access Node RF Bandwidth | Maximum Satellite Access Node RF Bandwidth in the operating band, declared for each supported operating band for each beam for SAN type 1-O, or for each TAB connector for SAN type 1-H (D.4). (Note 10) | c | x | x |
| D.17 | Total RF bandwidth (BWtot) | Total RF bandwidth BWtot of transmitter and receiver, declared per the band combinations (D.42). | c | x | x |
| D.18 | Contiguous spectrum operation support | Ability of SAN to support contiguous frequency distribution of carriers when operating multi-carrier in an operating band. Declared for each single-band RIB for SAN type 1-O or each single-band connector for SAN type 1-H, for each operating band. | c | x | x |
| D.19 | OSDD identifier | A unique identifier for the OSDD. | x | x | n/a |
| D.20 | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD (D.19). (Note 5) | x | x | n/a |
| D.21 | OTA sensitivity supported SAN channel bandwidth and SCS | The SAN supported SCS and channel bandwidth per supported SCS by each OSDD. | x | x | n/a |
| D.22 | Redirection of receiver target support | Ability to redirect the receiver target related to the OSDD. | x | x | n/a |
| D.23 | Minimum EIS for FR1 (EISminSENS) | The minimum EISminSENS requirement (i.e. maximum allowable EIS value) applicable to all sensitivity RoAoA per OSDD. Declared per NR supported channel BW for the OSDD (D.19). The lowest EIS value for all the declared OSDD's is called minSENS, while its related range of angles of arrival is called minSENS RoAoA. (Note 6) | x | x | n/a |
| D.24 | Receiver target reference direction Sensitivity Range of Angle of Arrival | The sensitivity RoAoA associated with the receiver target reference direction (D.26) for each OSDD. | x | x | n/a |
| D.25 | Receiver target redirection range | For each OSDD the associated union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD. | x | x | n/a |
| D.26 | Receiver target reference direction | For each OSDD an associated direction inside the receiver target redirection range (D.25). (Note 7) | x | x | n/a |
| D.27 | Conformance test directions sensitivity RoAoA | For each OSDD that includes a receiver target redirection range, four sensitivity RoAoA comprising the conformance test directions (D.28). | x | x | n/a |
| D.28 | Conformance test directions | For each OSDD four conformance test directions. If the OSDD includes a receiver target redirection range the following four directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. If an OSDD does not include a receiver target redirection range the following 4 directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. | x | x | n/a |
| D.29 | OTA coverage range | Declared as a single range of directions within which selected TX OTA requirements are intended to be met. (Note 8) | x | x | x |
| D.30 | OTA coverage range reference direction | The direction describing the reference direction of the OTA converge range (D.29). (Note 9) | x | x | x |
| D.31 | OTA coverage range maximum directions | The directions corresponding to the following points: 1) The direction determined by the maximum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. | x | x | x |
| D.32 | The rated carrier OTA SAN power, Prated,c,TRP | Prated,c,TRP is declared as TRP OTA power per carrier, declared per supported operating band. (Note 11) | n/a | x | x |
| D.33 | Rated transmitter TRP, Prated,t,TRP | Rated total radiated output power. Declared per supported operating band. (Note 11) | n/a | x | x |
| D.34 | Rated carrier output power (Prated,c,TABC) | Conducted rated carrier output power, per single band connector. Declared per supported operating band, per TAB connector for SAN type 1-H. (Note 11) | c | n/a | n/a |
| D.35 | Rated total output power (Prated,t,TABC) | Conducted total rated output power. Declared per supported operating band, per TAB connector for SAN type 1-H. (Note 11) | c | n/a | n/a |
| D.36 | Single band connector | List of single-band connector for the supported operating bands (D.4). | c | n/a | n/a |
| D.37 | Equivalent connectors | List of TAB connector of SAN type 1-H, which have been declared equivalent. Equivalent connectors imply that the TAB connector of SAN type 1-H, are expected to behave in the same way when presented with identical signals under the same operating conditions. All declarations made for the TAB connector of SAN type 1-H are identical and the transmitter unit and/or receiver unit driving TAB connector of SAN type 1-H are of identical design. | c | n/a | n/a |
| D.38 | Single-band RIB | List of single-band RIB for the supported operating bands (D.4). | n/a | x | x |
| D.39 | Single or multiple carrier | SAN capability to operate with a single carrier (only) or multiple carriers. Declared per supported operating band, per RIB for SAN type 1-O or per TAB connector for SAN type 1-H. | c | x | x |
| D.40 | Maximum number of supported carriers per operating band | Maximum number of supported carriers. Declared per supported operating band, per RIB for SAN type 1-O or per TAB connector for SAN type 1-H. (Note 10) | c | x | x |
| D.41 | Maximum supported power difference between carriers | Maximum supported power difference between carriers in each supported operating band. Declared per operating band (D.4), per RIB for SAN type 1-O or per TAB connector for SAN type 1-H. | c | x | x |
| D.42 | Operating band combination support | List of operating bands combinations supported by single-band RIB(s) of SAN type 1-O, or single-band connector(s) of SAN type 1-H. | c | x | x |
| D.43 | OTA REFSENS RoAoA | Range of angles of arrival associated with the OTA REFSENS. | n/a | x | x |
| D.44 | OTA REFSENS receiver target reference direction | Reference direction inside the OTA REFSENS RoAoA (D.43). | n/a | x | x |
| D.45 | OTA REFSENS conformance test directions | The following four OTA REFSENS conformance test directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. | n/a | x | x |
| D.46 | Relation between supported maximum RF bandwidth, number of carriers and Rated maximum TRP | If the rated transmitter TRP and total number of supported carriers are not simultaneously supported, the manufacturer shall declare the following additional parameters: - The reduced number of supported carriers at the rated transmitter TRP; - The reduced total output power at the maximum number of supported carriers. | n/a | x | x |
| D.47 | Relation between supported maximum RF bandwidth, number of carriers and Rated total output power | If the rated total output power and total number of supported carriers are not simultaneously supported, the manufacturer shall declare the following additional parameters: - The reduced number of supported carriers at the rated total output power; - The reduced total output power at the maximum number of supported carriers. | c | n/a | n/a |
| D.48 | TAB connectors used for performance requirement testing | To reduce test complexity, declaration of a representative (sub)set of TAB connectors to be used for performance requirement test purposes. At least one TAB connector mapped to each demodulation branch is declared. | c | n/a | n/a |
| D.49 | Prated,c,sys,GEO | The sum of Prated,c,TABC for all TAB connectors for a single carrier of the SAN GEO class. | c | n/a | n/a |
| D.50 | Prated,c,TABC,GEO | The rated carrier output power per TAB connector of the SAN GEO class. | c | n/a | n/a |
| D.51 | Prated,c,sys,LEO | The sum of Prated,c,TABC for all TAB connectors for a single carrier of the SAN LEO class. | c | n/a | n/a |
| D.52 | Prated,c,TABC,LEO | The rated carrier output power per TAB connector of the SAN LEO class. | c | n/a | n/a |
| D.53 | EIS REFSENS for FR2 (EISREFSENS_50M) | The EISREFSENS_50M level applicable in the OTA REFSENS RoAoA, (used as a basis for the derivation of the FR2 EISREFSENS for other channel bandwidths supported by SAN). (Note 12) | n/a | n/a | x |
| D.54 | SAN test conditions: Barometric pressure | Range of barometric pressure values for the Satellite Payload RF (SPRF) testing. | x | x |  |
| D.55 | SAN test conditions: Temperature | Range of temperature values for SPRF testing. | x | x |  |
| D.56 | SAN test conditions: Relative humidity | Range of relative humidity values for SPRF testing. | x | x |  |
| D.57 | SAN test conditions: Vibration | Range of vibration values for SPRF testing. | x | x |  |
| D.58 | SAN test conditions: Additional conditions | Additional (e.g. mission-specific) conditions for the SPRF testing. | x | x |  |
| D.59 | SAN test conditions: power supply | Manufacturer declaration related to the power supply. | x | x |  |
| D.60 | NB-IoT in-band operation | Manufacturer shall declare the support of NB-IoT operation in NTN NR in-band and the number of supported NB-IoT carriers in total and for each supported band, frequency range and channel bandwidth. | x | x |  |
| D.61 | NB-IoT sub-carrier spacing | If the SAN supports NB-IoT operation in NTN NR in-band, manufacturer shall declare if it supports 15 kHz sub-carrier spacing, 3.75 kHz sub-carrier spacing, or both for NPUSCH. | x | x |  |
| D.62 | NB-IoT power dynamic range | If manufacturer declares support for NB-IoT operation in NTN NR in-band, manufacturer shall declare the maximum power dynamic range it could support with a minimum of +6dB or +3dB as specified in clause 6.3.4 of TS 38.108 [2] (Note 13). | x | x |  |
| D.100 | PUSCH mapping type | Declaration of the supported PUSCH mapping type as specified in TS 38.211 [8], i.e., type A, type B or both. | c | x | n/a |
| D.101 | PUCCH format | Declaration of the supported PUCCH format(s) as specified in TS 38.211 [8], i.e., format 0, format 1, format 2, format 3, format 4. | c | x | x |
| D.102 | PRACH format and SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [8], i.e., format: 0, 2, B4, C2. Declaration of the supported SCS(s) per supported PRACH format with short sequence, as specified in TS 38.211 [8], i.e., 15 kHz, 30 kHz or both. | c | x | x |
| D.103 | Additional DM-RS for PUCCH format 3 | Declaration of the supported additional DM-RS for PUCCH format 3: without additional DM-RS, with additional DM-RS or both. | c | x | x |
| D.104 | Additional DM-RS for PUCCH format 4 | Declaration of the supported additional DM-RS for PUCCH format 4: without additional DM-RS, with additional DM-RS or both. | c | x | x |
| D.105 | PUCCH multi-slot | Declaration of multi-slot PUCCH support. | c | x | n/a |
| D.106 | MCS index table 3 | Declaration of support MCS index table 3 as specified in TS 38.214 [9]. | c | x |  |
| D.107 | PUSCH repetition type A | Declaration of support PUSCH repetition type A | c | x |  |
| D.108 | PUSCH DM-RS bundling | Declaration of supported SCS and FDD PUSCH DM-RS bundling, i.e., {15 kHz, 30 kHz} | c | x | n/a |
| NOTE 1: Manufacturer declarations applicable per SAN requirement set were marked as "x" or "c". Manufacturer declarations not applicable per SAN requirement set were marked as "n/a". NOTE 2: For SAN type 1-H, the only radiated declarations are related to EIRP and EIS requirements. For declarations marked as 'c', related conducted declarations apply, and for declarations marked as 'x', related radiated declarations apply. NOTE 3: Depending on the capability of the system some of these beams may be the same. For those same beams, testing is not repeated. NOTE 4: These operating bands are related to their respective single-band RIBs, or single-band TAB connectors. NOTE 5: As each identified OSDD has a declared minimum EIS value (D.23), multiple operating band can only be declared if they have the same minimum EIS declaration. NOTE 6: If the SAN type 1-H or SAN type 1-O is not capable of redirecting the receiver target related to the OSDD then there is only one RoAoA applicable to the OSDD. NOTE 7: For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA. NOTE 8: OTA coverage range is used for conformance testing of such TX OTA requirements as occupied bandwidth, frequency error or EVM. NOTE 9: The OTA coverage reference direction may be the same as the Reference beam direction pair (D.8) but does not have to be. NOTE 10: Parameters for contiguous spectrum operation in the operating band are assumed to be the same unless they are separately declared. When separately declared, they shall still use the same declaration identifier. NOTE 11: If a SAN is capable of 64QAM DL operation then up to two rated output power declarations may be made. One declaration is applicable when configured for 64QAM transmissions, and the other declaration is applicable when not configured for 64QAM transmissions. NOTE 12: Although EISREFSENS_50M level is based on a reference measurement channel with BWChannel = 50 MHz, it does not imply that SAN has to support 50 MHz channel bandwidth. NOTE 13: This manufacturer may declare two values, one with a minimum of +6dB and the other with a minimum of +3dB. |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 4.7.2	Test signal used to build Test Configurations

The signal's channel bandwidth and subcarrier spacing used to build NR Test Configurations shall be selected according to tables 4.7.2-1 and 4.7.2-2.

Table 4.7.2-1: Signal to be used to build NR TCs for SAN type 1-H and SAN type 1-O

| Operating Band characteristics |  | FDL_high – FDL_low <100 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 5 MHz (Note 1, 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. NOTE 2: For NRTC1, if NB-IoT is not supported, the signal's channel bandwidth shall be the narrowest supported one. |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 4.7.3.1	NRTC1 generation

NRTC1 shall be constructed on a per band basis using the following method:

-	Declared maximum SAN RF Bandwidth supported for contiguous spectrum operation (D.16) shall be used;

-	Select the carrier to be tested according to 4.7.2 and place it adjacent to the lower SAN RF Bandwidth edge. Place same signal adjacent to the upper SAN RF Bandwidth edge.

-    If NB-IoT operation in NTN NR in-band is supported, place the power boosted NB-IoT RB at the lower outermost RB at the lower SAN RF Bandwidth edge eligible for NB-IoT operation in NTN NR in-band according to clause 5.7.3 of TS 36.108 [22] and the definition in clause 3.1. If more than one NB-IoT carrier is supported with NB-IoT operation in NTN NR in-band, place the power boosted NB-IoT RB at the upper outermost RB at the upper SAN RF Bandwidth edge eligible for NB-IoT operation in NTN NR in-band.

-	For transmitter tests, select as many carriers (according to 4.7.2) that the SAN supports within an operating band and fit in the rest of the declared maximum SAN RF Bandwidth (D.16). Place the carriers adjacent to each other starting from the upper SAN RF Bandwidth edge. The nominal channel spacing defined in TS 38.108 [2], clause 5.4.1 shall apply.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 4.8.3	Applicability of test configurations for single-band operation

The applicable test configurations are specified in the tables below for each the supported RF configuration, which shall be declared according to clause 4.6. The generation and power allocation for each test configuration is defined in clause 4.7. This clause contains the test configurations for a SAN capable of single carrier, multi-carrier operation in contiguous spectrum in single band.

For a SAN declared to be capable of single carrier operation only (D.39), a single carrier (SC) shall be used for testing.

For conducted test, for a SAN declared to support multi-carrier operation in contiguous spectrum within a single band (D.39) with single band connector, the test configurations in the second column of table 4.8.3-1 shall be used for testing.

For OTA test, for a SAN declared to support multi-carrier operation in contiguous spectrum within a single band (D.39) with single band RIB, the test configurations in the second column of table 4.8.3-2 shall be used for testing.

Unless otherwise stated, single carrier configuration (SC) tests shall be performed using signal with narrowest supported channel bandwidth and the smallest supported sub-carrier spacing.

Table 4.8.3-1: Test configurations for a single-band connector

| SAN test case | Contiguous spectrum capable SAN |
| --- | --- |
| Base station output power | NRTC1 |
| RE Power control dynamic range | Tested with Error Vector Magnitude |
| Total power dynamic range (Note 2) | SC |
| Frequency error | Tested with Error Vector Magnitude |
| Error Vector Magnitude (Note 2) | NRTC1 |
| Occupied bandwidth | SC |
| Adjacent Channel Leakage power Ratio (ACLR) | NRTC1 |
| Out-of-band emissions | NRTC1, SC (Note 1) |
| Transmitter spurious emissions | NRTC1 |
| Reference sensitivity level | SC |
| Dynamic range | SC |
| Adjacent Channel Selectivity (ACS) | NRTC1 |
| Out-of-band blocking | NRTC1 |
| In-channel selectivity | SC |
| Note 1: OOBE SC shall be tested using the widest supported channel bandwidth and the highest supported sub-carrier spacing. Note 2: There is no specific test for NB-IoT operation in NTN NR in-band for these requirements, tests could be performed using NR signal only, without NB-IoT. |  |

Table 4.8.3-2: Test configurations for a single-band RIB

| SAN test case | Contiguous spectrum capable SAN |
| --- | --- |
| Radiated transmit power | NRTC1 |
| OTA base station maximum output power | NRTC1 |
| OTA RE Power control dynamic range | Tested with Error Vector Magnitude |
| OTA total power dynamic range (Note 2) | SC |
| OTA frequency error | Tested with Error Vector Magnitude |
| OTA error Vector Magnitude (Note 2) | NRTC1 |
| OTA Occupied bandwidth | SC |
| OTA ACLR | NRTC1 |
| OTA out-of-band emissions | NRTC1, SC (Note 1) |
| OTA transmitter spurious emissions | NRTC1 |
| OTA sensitivity | SC |
| OTA reference sensitivity level | SC |
| OTA dynamic range | SC |
| OTA adjacent channel selectivity | NRTC1 |
| OTA out-of-band blocking | NRTC1 |
| OTA in-channel selectivity | SC |
| Note 1: OOBE SC shall be tested using the widest supported channel bandwidth and the highest supported subcarrier spacing. Note 2: There is no specific test for NB-IoT operation in NTN NR in-band for these requirements, tests could be performed using NTN NR signal only, without NB-IoT. |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

##### 4.9.2.2.7	NB-IoT operation in NTN NR in-band test model (NR-N-TM)

The NB-IoT operation in NTN NR in-band test shall be performed by puncturing one NR RB at the eligible (according to clause 5.7.3 of TS 36.108 [22] and the definition in clause 3.1) in-band position closest to NR minimum guard band.

The set-up of physical channels for transmitter tests shall be according to the FR1 test model 1.1 (NR-SAN-FR1-TM1.1) on all NR carriers and the NB-IoT Test Model (N-TM) defined in TS 36.181 [23] on all NB-IoT carriers.

The power for NR RE and NB-IoT RE is set by following procedures:

-	The average power per RE over all RBs (both NR and NB-IoT) is calculated according to manufacturer's declared rated carrier output power (Prated,c, TABC in D.49. D.51);

Average power per RE (Pavg) = Prated,c,TABC NRB×NSCRBW

-	The power of boosted NB-IoT RE (PNB-IoT) for the SAN is calculated according to manufacturer's declared rated NB-IoT maximum power dynamic range (X dB >= 6 dB or 3 dB), with the power boosting only applies on the NNB_IoT REs containing NB-IoT signal.

Power of boosted NB-IoT RE (PNB-IoT) = Pavg*10(X/10) W

-	The remaining power is allocated to NNR NR REs.

Power of per NR RE =Prated,c,TABC-PNB-IoT×NNB-IoTNNR W

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 4.9.2.6	Data content of Physical channels and Signals for NR-N-TM

Detailed configuration for the transmitter characteristic tests are defined in TS 36.181 [23],

### 4.9.3	NB-IoT testing

Unless otherwise stated, the NB-IoT operation in NTN NR in-band test shall be performed by puncturing one NR RB at the eligible (according to clause 5.7.3 of TS 36.108 [22] and the definition in clause 3.1) in-band position closest to NR minimum guard band; those are denoted LNB-IoT (Left) and RNB-IoT (Right).

Unless otherwise stated, the NB-IoT operation in NTN NR in-band receiver tests shall be performed by using the tone located on the NB-IoT RB's edge, which is closest to NR minimum guard band; those are denoted BNB-IoT for LNB-IoT and TNB-IoT for RNB-IoT.

Unless otherwise stated, a SAN declared to be capable of NB-IoT operation in NTN NR in-band is only required to pass the transmitter tests for NR with NB-IoT operation in NTN NR in-band; it is not required to perform the transmitter tests again for NR only.

Unless otherwise stated, a SAN declared to be capable of NB-IoT operation in NTN NR in-band is only required to pass the receiver tests for NR with NB-IoT operation in NTN NR in-band; it is not required to perform the receiver tests again for NR only.

NOTE:	The SAN should be configured (RRC signalling) for the NB-IoT E-UTRA guardband mode during the tests.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

# 5	Operating bands and channel arrangement

For the NR and NB-IoT operation in NTN NR in-band operating bands specification, their channel bandwidth configurations, channel spacing and raster, as well as synchronization raster specification, refer to TS 38.108 [2], clause 5 and its relevant clauses.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 6.2.5	Test requirement

For each single-band connector under test, the power measured in clause 6.2.4.2 in step 3 shall remain within the values provided in table 6.2.5-1 for normal test environments, relative to the manufacturer's declared Prated,c,TABC for SAN type 1-H (D.34):

Table 6.2.5-1: Test requirement for conducted SAN output power

| SAN type | Normal test environment |
| --- | --- |
| SAN type 1-H | f ≤ 3 GHz: ± 2.7 dB |

NOTE:	For NB-IoT operation in NTN NR in-band, the NR carrier and NB-IoT carrier shall be seen as a single carrier occupied NR channel bandwidth, the output power over this carrier is shared between NR and NB-IoT. This note shall apply for Pmax,c,TABC and Prated,c,TABC.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 6.3.4	NB-IoT RB power dynamic range for NB-IoT operation in NTN NR in-band

#### 6.3.4.1	Definition and applicability

The NB-IoT RB power dynamic range (or NB-IoT power boosting) for NB-IoT operation in NTN NR in-band is the difference between the average power of NB-IoT REs (which occupy certain REs within a NR transmission bandwidth configuration plus 15 kHz at each edge but not within the NR minimum guard band GBChannel) and the average power over all REs (from both NB-IoT and the NR carrier containing the NB-IoT REs).

#### 6.3.4.2	Minimum requirement

The minimum requirement applies per single-band connector, or per multi-band connector.

The minimum requirement for SAN type 1-H is in TS 38.108 [2], clause 6.3.4.2.

#### 6.3.4.3	Test purpose

The test purpose is to verify that the NB-IoT RB power dynamic range for NB-IoT operation in NTN NR in-band is met as specified by the minimum requirement.

#### 6.3.4.4	Method of test

Requirement is tested together with operating band unwanted emissions test, as described in clause 6.6.4.

#### 6.3.4.5	Test requirements

NB-IoT RB power dynamic range for NB-IoT operation in NTN NR in-band shall be larger than or equal to the level specified in Table 6.3.4.5-1 or the NB-IoT power dynamic range declared by the SAN manufacturer (D63). This power dynamic range level is only required for one NB-IoT RB.

Table 6.3.4.5-1: NB-IoT RB power dynamic range for NB-IoT operation in NTN NR in-band

| SAN channel bandwidth (MHz) | NB-IoT RB frequency position | NB-IoT RB power dynamic range (dB) |
| --- | --- | --- |
| 5, 10 | Any | +5.6 |
| 15 | Within center 77*180kHz+15kHz at each edge | +5.6 |
|  | Other | +2.6 |
| 20 | Within center 102*180kHz+15kHz at each edge | +5.6 |
|  | Other | +2.6 |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 6.5.2.5	Test Requirements

The modulated carrier frequency of each NR carrier configured by the SAN shall be accurate to within 0.05 ppm + 12 Hz (tolerance) observed over 1 ms.

The frequency error requirement for NB-IoT is specified in TS 36.181 [23] clause 6.5.2.5.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 6.5.3.5	Test requirements

The EVM of each NR carrier for different modulation schemes on PDSCH shall be less than the limits in table 6.5.3.5-1.

Table 6.5.3.5-1 EVM requirements for SAN type 1-H carrier

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 18.5 % |
| 16QAM | 13.5 % |
| 64QAM (NOTE) | 9 % |
| NOTE: EVM requirement for 64QAM is optional. |  |

EVM shall be evaluated for each NR carrier over all allocated resource blocks and downlink slots. Different modulation schemes listed in table 6.5.3.5-1 shall be considered for rank 1.

For all bandwidths, the EVM measurement shall be performed for each NR carrier over all allocated resource blocks and downlink slots within 10 ms measurement periods. The boundaries of the EVM measurement periods need not be aligned with radio frame boundaries.

Table 6.5.3.5-2, 6.5.3.5-3, 6.5.3.5-4 below specify the EVM window length (W) for normal CP for SAN type 1-H.

Table 6.5.3.5-2 EVM window length for normal CP for NR, FR1, 15 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 5 | 512 | 36 | 14 | 40 |
| 10 | 1024 | 72 | 28 | 40 |
| 15 | 1536 | 108 | 44 | 40 |
| 20 | 2048 | 144 | 58 | 40 |
| NOTE: These percentages are informative and apply to a slot's symbols 1 to 6 and 8 to 13. Symbols 0 and 7 have a longer CP and therefore a lower percentage. |  |  |  |  |

Table 6.5.3.5-3 EVM window length for normal CP for NR, FR1, 30 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 5 | 256 | 18 | 8 | 40 |
| 10 | 512 | 36 | 14 | 40 |
| 15 | 768 | 54 | 22 | 40 |
| 20 | 1024 | 72 | 28 | 40 |
| NOTE: These percentages are informative and apply to a slot's symbols 1 through 13. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |

Table 6.5.3.5-4 EVM window length for normal CP for NR, FR1, 60 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (Note) (%) |
| --- | --- | --- | --- | --- |
| 10 | 256 | 18 | 8 | 40 |
| 15 | 384 | 27 | 11 | 40 |
| 20 | 512 | 36 | 14 | 40 |
| NOTE: These percentages are informative and apply to all OFDM symbols within subframe except for symbol 0 of slot 0 and slot 2. Symbol 0 of slot 0 and slot 2 may have a longer CP and therefore a lower percentage. |  |  |  |  |

The modulation quality requirements for NB-IoT are specified in TS 36.181 [23] clause 6.5.3.5.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

##### 6.6.2.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1.

1)	Connect the measurement device to the TAB connector as shown in annex D.1.1 for SAN type 1-H.

2)	For a SAN declared to be capable of single carrier operation (D.39), start transmission according to the applicable test configuration in clause 4.8 using the corresponding test model NR-SAN-FR1-TM1.1 at manufacturer's declared rated carrier output power per TAB connector (Prated,c,TABC, D.34).

For a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60), test shall be performed using NR-N-TM according to clause 4.9.2.2.7.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 6.6.3	Adjacent Channel Leakage Power Ratio (ACLR)

#### 6.6.3.1	Definition and applicability

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

The requirements shall apply outside the Satellite Access Node RF Bandwidth or Radio Bandwidth whatever the type of transmitter considered (e.g. single carrier or multi-carrier) and for all transmission modes foreseen by the manufacturer's specification.

The requirements shall also apply if the SAN supports NB-IoT operation in NTN NR in-band.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

##### 6.6.3.4.2	Procedure

For SAN type 1-H where there may be multiple TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.1.1 for SAN type 1-H. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect the single-band connector or multi-band connector under test to measurement equipment as shown in annex D.1.1 for SAN type 1-H. All connectors not under test shall be terminated.

The measurement device characteristics shall be:

-	Measurement filter bandwidth: defined in clause 6.6.3.5.

-	Detection mode: true RMS voltage or true average power.

2)	For a connectors declared to be capable of single carrier operation only (D.39), set the representative connectors under test to transmit according to the applicable test configuration in clause 4.8 using the corresponding test models NR-SAN-FR1-TM 1.1 in clause 4.9.2 at rated carrier output power Prated,c,TABC for SAN type 1-H (D.34).

For a connector under test declared to be capable of multi-carrier operation set the connector under test to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in clauses 4.7 and 4.8 using the corresponding test models or set of physical channels in clause 4.9.2.

For a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60), test shall be performed using N-TM according to clause 4.9.2.2.7.

3)	Measure ACLR for the frequency offsets both side of channel frequency as specified in table 6.6.3.5.2-1. In multiple carrier case only offset frequencies below the lowest and above the highest carrier frequency used shall be measured.

4)	Repeat the test with the channel set-up according to NR-SAN-FR1-TM 1.2 in clause 4.9.2.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 6.6.4	Out-of-band emissions

#### 6.6.4.1	Definition and applicability

Unless otherwise stated, the out-of-band emission (OOBE) limits for SAN in FR1 are defined from BWSAN channel edge up to frequencies separated from the BWSAN channel edge by 200% of the necessary bandwidth, where the necessary bandwidth is BWSAN.

The requirements shall apply whatever the type of transmitter considered and for all transmission modes foreseen by the manufacturer’s specification.

Basic limits are specified in the tables below, where:

-	f is the separation between the BWSAN channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the channel edge frequency and the centre of the measuring filter.

-	ΔSat_ClassdB is the SAN class parameter in dB identified to characterize different SAN classes.

The requirements shall also apply if the SAN supports NB-IoT operation in NTN NR in-band.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

##### 6.6.4.4.2	Procedure

For SAN type 1-H where there may be multiple TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect the single-band connector under test to measurement equipment as shown in annex D.1.1. All connectors not under test shall be terminated.

As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity, efficiency and avoiding e.g. carrier leakage, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The measurement device characteristics shall be: Detection mode: True RMS.

The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in table 4.1.2.2-1.

2)	For a connectors declared to be capable of single carrier operation only, set the representative connectors under test to transmit according to the applicable test configuration in clause 4.8 at manufacturer's declared rated carrier output power per TAB connector (Prated,c,TABC, D.34). Channel set-up shall be according to NR-SAN-FR1-TM 1.1.

For a connector under test declared to be capable of multi-carrier operation set the connector under test to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in clauses 4.7 and 4.8 using the corresponding test models or set of physical channels in clause 4.9.2.

For a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60), test shall be performed using NR-N-TM according to clause 4.9.2.2.7.

3)	Step the centre frequency of the measurement filter in contiguous steps and measure the emission within the specified frequency ranges with the specified measurement bandwidth.

4)	Repeat the test for the remaining test cases, with the channel set-up according to NR-SAN-FR1-TM 1.2.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 6.6.5.1	Definition and applicability

The transmitter spurious emission limits shall apply from 30 MHz to 11 GHz, excluding the SAN transponder bandwidth BWSAN and the frequency range where the out-of-band emissions apply.

The lower limit and upper limit are as per ITU-R recommendation SM.329, Table 1: For systems operating within 600 MHz and 5.2 GHz, the lower limit is 30 MHz and the upper limit is the 5th harmonic of the higher frequency.

The lower limit of 30 MHz can be replaced as per ITU-R SM.329-12: Systems having an integral antenna incorporating a waveguide section, or with an antenna connection in such form, and of unperturbed length equal to at least twice the cut-off wavelength, do not require spurious domain emission measurements below 0.7 times the waveguide cut-off frequency.

The requirements shall apply whatever the type of transmitter considered (e.g. single carrier or multi-carrier). It applies for all transmission modes foreseen by the manufacturer's specification.

The requirements shall apply to SAN that support NR or NR with NB-IoT operation in NR in-band.

The requirements shall apply to SAN that support NR or NR with NB-IoT operation in NR in-band.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

##### 6.6.5.4.2	Procedure

For SAN type 1-H where there may be multiple TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect the single-band connector under test to measurement equipment as shown in annex D.1.1 for SAN type 1-H. All connectors not under test shall be terminated with a matched load.

2)	Measurements shall use a measurement bandwidth in accordance to the conditions in clause 6.6.5.5.

The measurement device characteristics shall be:

-	Detection mode: True RMS.

The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table 4.1.2.2-1.

3)	For a connectors declared to be capable of single carrier operation only (D.39), set the representative connectors under test to transmit according to the applicable test configuration in clause 4.8 at rated carrier output power (Prated,c,TABC, D.34). Channel set-up shall be according to NR-FR1-TM 1.1.

For a connector under test declared to be capable of multi-carrier operation (D.39) set the connector under test to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in clauses 4.7 and 4.8 using the corresponding test models or set of physical channels in clause 4.9.2.

For a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60), test shall be performed using NR-N-TM according to clause 4.9.2.2.7.

4)	Measure the emission at the specified frequencies with specified measurement bandwidth.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

## 7.1	General

Conducted receiver characteristics are specified at the TAB connector for SAN type 1-H, with full complement of transceivers for the configuration in normal operating condition.

Unless otherwise stated, the following arrangements apply for conducted receiver characteristics requirements in clause 7:

-	Requirements shall be met for any transmitter setting.

-	The requirements shall be met with the transmitter unit(s) ON.

-	Throughput requirements do not assume HARQ retransmissions.

-	When SAN is configured to receive multiple carriers, all the throughput requirements are applicable for each received carrier.

-	For ACS and blocking characteristics, the negative offsets of the interfering signal apply relative to the lower SAN RF Bandwidth edge, and the positive offsets of the interfering signal apply relative to the upper SAN RF Bandwidth edge.

-	Requirements shall also apply for SAN supporting NB-IoT operation in NTN NR in-band. The corresponding NB-IoT requirements are specified in clause 7 of TS 36.181 [23].

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 7.2.4.2	Procedure

The minimum requirement is applied to all connectors under test.

For SAN type 1-H the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested; see clause 7.1.

1)	Connect the connector under test to measurement equipment as shown in annex D.2.1 for SAN type 1-H.

2)	Set the SAN to transmit a signal using the applicable test configuration and corresponding power setting specified in clauses 4.7 and 4.8 using the corresponding test models or set of physical channels in clause 4.9.2, for SAN type 1-H set the TAB connector to the manufacturers declared rated carrier output power (Prated,c,TABC, D.34).

3)	Start the signal generator for the wanted signal to transmit the Fixed Reference Channels for reference sensitivity according to annex A.1.

4)	Set the signal generator for the wanted signal power as specified in clause 7.2.5.

5)	Measure the throughput according to annex A.1, as well as annex A.14 of TS 36.181 [23] for a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60).

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 7.2.5	Test requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.2.5-1 and 7.2.5-2 for SAN type 1-H in all operating band in FR1-NTN.

The reference sensitivity level requirements for NB-IoT are specified in clause 7.2.5 of TS 36.181 [23].

Table 7.2.5-1: SAN GEO class reference sensitivity levels

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-NTN-A1-1 (Note 1) | -98.6 |
|  |  | G-FR1-NTN-A1-10 (Note 3) | -98.6 (Note 2) |
| 10, 15 | 30 | G-FR1-NTN-A1-2 (Note 1) | -98.7 |
| 10, 15 | 60 | G-FR1-NTN-A1-3 (Note 1) | -95.8 |
| 20 | 15 | G-FR1-NTN-A1-4 (Note 1) | -92.2 |
|  |  | G-FR1-NTN-A1-11 (Note 4) | -92.2 (Note 2) |
| 20 | 30 | G-FR1-NTN-A1-5 (Note 1) | -92.5 |
| 20 | 60 | G-FR1-NTN-A1-6 (Note 1) | -92.6 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. NOTE 2: The requirements apply to SAN that supports NB-IoT operation in NTN NR in-band. NOTE 3: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. NOTE 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. |  |  |  |

Table 7.2.5-2: SAN LEO class reference sensitivity levels

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-NTN-A1-1 (Note 1) | -101.7 |
|  |  | G-FR1-NTN-A1-10 (Note 3) | -101.7 (Note 2) |
| 10, 15 | 30 | G-FR1-NTN-A1-2 (Note 1) | -101.8 |
| 10, 15 | 60 | G-FR1-NTN-A1-3 (Note 1) | -98.9 |
| 20 | 15 | G-FR1-NTN-A1-4 (Note 1) | -95.3 |
|  |  | G-FR1-NTN-A1-11 (Note 4) | -95.3 (Note 2) |
| 20 | 30 | G-FR1-NTN-A1-5 (Note 1) | -95.6 |
| 20 | 60 | G-FR1-NTN-A1-6 (Note 1) | -95.7 |
| NOTE 1: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. NOTE 2: The requirements apply to SAN that supports NB-IoT operation in NTN NR in-band. NOTE 3: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. NOTE 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. |  |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 7.3.4.2	Procedure

The minimum requirement is applied to all connectors under test.

For SAN type 1-H the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested; see clause 7.1.

1)	Connect the connector under test to measurement equipment as shown in annex D.2.2 for SAN type 1-H.

2)	Set the signal generator for the wanted signal to transmit as specified in table 7.3.5-1 according to the appropriate SAN class as well as table 7.3.5-1a for a SAN declared to be capable of NB-IoT operation in NTN  in-band (D.60).

3)	Set the Signal generator for the AWGN interfering signal at the same frequency as the wanted signal to transmit as specified in table 7.3.5-1 according to the appropriate SAN class as well as table 7.3.5-1a for a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60).

4)	Measure the throughput according to annex A.2 as well as annex A.15 of TS 36.181 [23] for a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60).

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 7.3.5	Test requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.2 with parameters specified in table 7.3.5-1 for SAN LEO class.

For NB-IoT operation in NTN NR in-band, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A of TS 36.181 [23] with parameters specified in table 7.3.5-1a for SAN LEO class.

Table 7.3.5-1: SAN LEO class dynamic range

| SAN channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-NTN-A2-1 | -76.1 | -88.2 | AWGN |
|  | 30 | G-FR1-NTN-A2-2 | -76.8 |  |  |
| 10 | 15 | G-FR1-NTN-A2-1 | -76.1 | -85.0 | AWGN |
|  | 30 | G-FR1-NTN-A2-2 | -76.8 |  |  |
|  | 60 | G-FR1-NTN-A2-3 | -73.8 |  |  |
| 15 | 15 | G-FR1-NTN-A2-1 | -76.1 | -83.2 | AWGN |
|  | 30 | G-FR1-NTN-A2-2 | -76.8 |  |  |
|  | 60 | G-FR1-NTN-A2-3 | -73.8 |  |  |
| 20 | 15 | G-FR1-NTN-A2-4 | -69.9 | -81.9 | AWGN |
|  | 30 | G-FR1-NTN-A2-5 | -69.9 |  |  |
|  | 60 | G-FR1-NTN-A2-6 | -70.2 |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. |  |  |  |  |  |

Table 7.3.5-1a: SAN LEO class dynamic range for NB-IoT operation in NTN NR in-band

| SAN channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 | FRC A15-1 in |  | -88.2 |  |
| 10 | Annex A.15 in | -89.1 | -85.0 | AWGN |
| 15 | TS 36.181 [23] |  | -83.2 |  |
| 20 |  |  | -81.9 |  |
| 5 | FRC A15-2 in |  | -82.5 |  |
| 10 | Annex A.15 in | -95.0 | -79.3 | AWGN |
| 15 | TS 36.181 [23] |  | -77.5 |  |
| 20 |  |  | -76.2 |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

##### 7.4.1.4.2	Procedure

The minimum requirement is applied to all connectors under test.

For SAN type 1-H the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested; see clause 7.1.

1)	Connect the connector under test to measurement equipment as shown in annex D.2.3 for SAN type 1-H.

2)	For FDD operation, set the SAN to transmit:

-	For single carrier operation set the connector under test to transmit at manufacturers declared rated carrier output power (Prated,c,TABC, D.34).

-	For a connector under test declared to be capable of multi-carrier  operation (D.39) set the connector under test to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in clauses 4.7 and 4.8 using the corresponding test models or set of physical channels in clause 4.9.2

3)	Set the signal generator for the wanted signal to transmit as specified in table 7.4.1.5-1.

4)	Set the signal generator for the interfering signal to transmit at the frequency offset and as specified in table 7.4.1.5-1 and 7.4.1.5-2.

5)	Measure the throughput according to annex A.2, as well as annex A.14 of TS 36.181 [23] for a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60).

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 7.4.1.5	Test requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For SAN, the wanted and the interfering signal coupled to the SAN type 1-H TAB connector are specified in table 7.4.1.5-1 and the frequency offset between the wanted and interfering signal in table 7.4.1.5-2 for ACS. The characteristics of the interfering signal is further specified in annex E.

For SAN supporting NB-IoT operation in NTN NR in-band, the wanted and the interfering signal coupled to the SAN type 1-H antenna connector are specified in table 7.4.1.5-1 and the frequency offset between the wanted and interfering signal in table 7.4.1.5-2 for ACS. The reference measurement channel for the NB-IoT wanted signal is identified in clause 7.2.5 of TS 36.181 [23]. The characteristics of the interfering signal is further specified in annex E.

The ACS requirement is applicable outside the SAN RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the SAN RF Bandwidth edges or Radio Bandwidth edges.

Conducted requirement is defined at the TAB connector for SAN type 1-H.

Table 7.4.1.5-1: Satellite Access Node ACS requirement

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20 (NOTE 1) | PREFSENS + 6 dB (NOTE 2) | GEO SAN class: -57 LEO SAN class: -60 |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the SAN for that bandwidth. NOTE 2: PREFSENS depends on the SAN channel bandwidth as specified in table 7.2.2-1 and 7.2.2-2 in TS 36.108 [22]. For NB-IoT, PREFSENS depends also on the sub-carrier spacing as specified in tables 7.2.2-3 and 7.2.2-4 of TS 36.108 [22]. |  |  |

Table 7.4.1.5-2: Satellite Access Node ACS interferer frequency offset values

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal center frequency offset from the lower/upper SAN RF Bandwidth edge (MHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±2.5025 | 5 MHz CP-OFDM NR signal 15 kHz SCS, 25 RBs |
| 10 | ±2.5075 |  |
| 15 | ±2.5125 |  |
| 20 | ±2.5025 |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 7.5.4.2	Procedure

The minimum requirement is applied to all TAB connectors under test. For SAN type 1-H the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested; see clause 7.1.

1)	Connect the connector under test to measurement equipment as shown in annex D.2.3.

2)	For FDD operation, set the SAN to transmit a signal according to clause 4.9.2, connector under test to transmit on all carriers configured using the applicable test configuration and corresponding power setting specified in clauses 4.7 and 4.8.

The transmitter may be turned OFF for the out-of-band blocker tests when the frequency of the blocker is such that no IM2 or IM3 products fall inside the bandwidth of the wanted signal.

3)	Set the signal generator for the wanted signal as defined in clause 7.5.5 to transmit as specified in table 7.5.5-1.

4)	Set the signal generator for the interfering signal to transmit at the frequency offset and as specified in table 7.5.5-1. The CW interfering signal shall be swept with a step size of 1 MHz over than range 1 MHz to (FUL_low - ΔfOOB) MHz and (FUL_high + ΔfOOB) MHz to 12750 MHz.

5) Measure the throughput according to annex A.1 as well as annex A.14 of TS 36.181 [23] for a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60).

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 7.5.5	Test requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to SAN type 1-H TAB connector using the parameters in table 7.5.5-1.

The reference measurement channel for the wanted signal is identified in clause 7.2.2 for each SAN channel bandwidth and further specified in annex A.1.

For NB-IoT operation in NTN NR in-band, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to SAN type 1-H TAB connector using the parameters in table 7.5.5.1-1. The reference measurement channel for the NB-IoT wanted signal is identified in clause 7.2.5 of TS 36.181 [23]. The characteristics of the interfering signal is further specified in annex E.

The out-of-band blocking requirement apply from 1 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB up to 12750 MHz, including the downlink frequency range of the FDD operating band for SAN. The ΔfOOB for SAN type 1-H is defined in table 7.5.5-2.

Minimum conducted requirement is defined at the TAB connector for SAN type 1-H.

Table 7.5.5-1: Out-of-band blocking requirement for NR

| Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- |
| PREFSENS + 6 dB (NOTE 1) | -44 | CW carrier |
| NOTE 1: For SAN, PREFSENS depends on the SAN channel bandwidth. For NB-IoT, PREFSENS depends also on the sub- carrier spacing as specified in tables 7.2.1-5, 7.2.1-5a and 7.2.1-5c of TS 36.108 [21]. NOTE 2: For NB-IoT, up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |

Table 7.5.5-2: ΔfOOB offset for NR operating bands

| SAN type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| SAN type 1-H | FUL,high – FUL,low < 100 MHz | 20 |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 7.8.4.2	Procedure

The minimum requirement is applied to all connectors under test.

For SAN type 1-H the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested; see clause 7.1.

1)	Set the signal generator for the wanted signal to transmit as specified from table 7.8.5-1 and 7.8.5-2 as well as table 7.8.5-1a and 7.8.5-2a for a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60).

2)	Set the signal generator for the interfering signal to transmit at the frequency offset and as specified from table 7.8.5-1 and 7.8.5-2 as well as table 7.8.5-2a and 7.8.5-2b for a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60).

3)	Measure the throughput according to annex A.1 as well as annex A.14 of TS 36.181 [23] for a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60).

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 7.8.5	Test requirements

For SAN type 1-H, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 7.8.5-1 for SAN GEO class, in table 7.8.5-2 for SAN LEO class. The characteristics of the interfering signal is further specified in annex E.

For NB-IoT operation in NTN NR in-band, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A of TS 36.181 [23] with parameters specified in table 7.8.5-1a for SAN LEO class, in table 7.8.5-2a for SAN LEO class.

Table 7.8.5-1: SAN GEO class ICS requirement

| SAN channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A1-7 | -96.8 | -92.0 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10,15,20 | 15 | G-FR1-A1-1 | -94.9 | -88.1 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 5 | 30 | G-FR1-A1-8 | -97.5 | -92.0 | DFT-s-OFDM NR signal, 30 kHz SCS, 5 RBs |
| 10,15,20 | 30 | G-FR1-A1-2 | -95 | -89.0 | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 10,15,20 | 60 | G-FR1-A1-9 | -94.4 | -89.0 | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for SAN channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the SAN channel bandwidth of the wanted signal. |  |  |  |  |  |

Table 7.8.5-1a: Wide Area SAN GEO class ICS requirement for NB-IoT operation in NTN NR in-band

| SAN channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 | FRC A14-1 in Annex A.14 in TS 36.181 [23] | -120.5 | -92.0 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20 |  |  | -88.1 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 5 | FRC A14-2 in Annex A.14 in TS 36.181 [23] | -126.5 | -92.0 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10, 15, 20 |  |  | -88.1 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| NOTE: Interfering signal is placed in one side of the Fc, while the NB-IoT PRB is placed on the other side. Both interfering signal and NB-IoT PRB are placed at the middle of the available PRB locations. The wanted NB-IoT tone is placed at the centre of this NB-IoT PRB. |  |  |  |  |

Table 7.8.5-2: SAN LEO class ICS requirement

| SAN channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A1-7 | -99.9 | -83.1 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10,15,20 | 15 | G-FR1-A1-1 | -98 | -79.2 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 5 | 30 | G-FR1-A1-8 | -100.6 | -83.1 | DFT-s-OFDM NR signal, 30 kHz SCS, 5 RBs |
| 10,15,20 | 30 | G-FR1-A1-2 | -98.1 | -80.1 | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 10,15,20 | 60 | G-FR1-A1-9 | -97.5 | -80.1 | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for SAN channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the SAN channel bandwidth of the wanted signal. |  |  |  |  |  |

Table 7.8.5-2a: SAN LEO class ICS requirement for NB-IoT operation in NTN NR in-band

| SAN channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 | FRC A14-1 in Annex A.14 in TS 36.181 [23] | -123.6 | -83.1 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10,15,20 |  |  | -79.2 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 5 | FRC A14-2 in Annex A.14 in TS 36.181 [23] | -129.6 | -83.1 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10,15,20 |  |  | -79.2 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for SAN channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the SAN channel bandwidth of the wanted signal. |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 9.2.5	Test requirement

For each declared conformance beam direction pair, the EIRP measurement results in clause 9.2.4.2 shall remain within the values provided in table 9.2.5-1, relative to the manufacturer's declared rated beam EIRP (D.11) value:

Table 9.2.5-1: Test requirement for radiated transmit power

|  | Normal test environment |
| --- | --- |
| SAN type 1-H, SAN type 1-O | f ≤ 3 GHz: ± 3.3 dB |
| SAN type 2-O | 17.3 GHz < f ≤ 20.2 GHz: ± 5.1 dB |

NOTE:	For NB-IoT operation in NTN NR in-band, the NR carrier and NB-IoT carrier shall be seen as a single carrier occupied NR channel bandwidth, the total radiated power is shared between NR and NB-IoT. This note shall apply for Pmax,c,TRP and Prated,c,TRP.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 9.3.5.1	SAN type 1-O

The final TRP measurement result in clause 9.3.4.2 shall remain:

-	within +3.4 dB and -3.4 dB of the manufacturer's declared rated carrier TRP Prated,c,TRP carrier frequency f ≤ 3.0 GHz;

NOTE:	For NB-IoT operation in NTN NR in-band, the NR carrier and NB-IoT carrier shall be seen as a single carrier occupied NR channel bandwidth, the total radiated power is shared between NR and NB-IoT.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 9.4.4	OTA NB-IoT RB power dynamic range for in-band operation

#### 9.4.4.1	Definition and applicability

The OTA NB-IoT RB power dynamic range (or NB-IoT power boosting) for in-band operation is the difference between the average power of NB-IoT REs (which occupy certain REs within a NR transmission bandwidth configuration plus 15 kHz at each edge but not within the NR minimum guard band GBChannel) and the average power over all REs (from both NB-IoT and the NR carrier containing the NB-IoT REs).

#### 9.4.4.2	Minimum requirement

The minimum requirement for SAN supporting NB-IoT operation in NTN NR in-band is in TS 38.108 [2], clause 9.4.4.2.

#### 9.4.4.3	Test purpose

The test purpose is to verify that the NB-IoT RB power dynamic range for NB-IoT operation in NTN NR in-band is met as specified by the minimum requirement.

#### 9.4.4.4	Method of test

Requirement is tested together with operating band unwanted emissions test, as described in clause 9.7.4.

#### 9.4.4.5	Test requirements

#### 9.4.4.5.1	SAN type 1-O

NB-IoT RB power dynamic range for NB-IoT operation in NTN NR in-band or guard band shall be larger than or equal to the level specified in Table 9.4.4.5-1 or the NB-IoT power dynamic range declared by the SAN manufacturer (D63). This power dynamic range level is only required for one NB-IoT RB.

Table 9.4.4.5-1: NB-IoT RB power dynamic range for NB-IoT operation in NTN NR in-band

| SAN channel bandwidth (MHz) | NB-IoT RB frequency position | NB-IoT RB power dynamic range (dB) |
| --- | --- | --- |
| 5, 10 | Any | +5.6 |
| 15 | Within center 77*180kHz+15kHz at each edge | +5.6 |
|  | Other | +2.6 |
| 20 | Within center 102*180kHz+15kHz at each edge | +5.6 |
|  | Other | +2.6 |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 9.6.2.5	Test Requirements

The modulated carrier frequency of each NR carrier configured by the SAN shall be accurate to within 0.05 ppm + 12 Hz (tolerance) observed over 1 ms.

The frequency error requirement for NB-IoT is specified in TS 36.181 [23] clause 9.6.2.5.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

##### 9.6.3.5.1	SAN type 1-O

For SAN type 1-O, the EVM of each NR carrier for different modulation schemes on PDSCH shall be less than the limits in table 9.6.3.5.1-1 for FR1-NTN.

Table 9.6.3.5.1-1: EVM requirements for SAN type 1-O for FR1-NTN

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 18.5 % |
| 16QAM | 13.5 % |
| 64QAM (NOTE) | 9 % |
| NOTE: EVM requirement for 64QAM is optional. |  |

EVM shall be evaluated for each NR carrier over all allocated resource blocks and downlink slots. Different modulation schemes listed in table 9.6.3.5.1-1 shall be considered for rank 1.

For NR, for all bandwidths, the EVM measurement shall be performed for each NR carrier over all allocated resource blocks and downlink slots within 10 ms measurement periods. The boundaries of the EVM measurement periods need not be aligned with radio frame boundaries.

Table 9.6.3.5.1-2, 9.6.3.5.1-3, 9.6.3.5.1-4 below specify the EVM window length (W) for normal CP for SAN type 1-O.

Table 9.6.3.5.1-2: EVM window length for normal CP, FR1-NTN, 15 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 5 | 512 | 36 | 14 | 40 |
| 10 | 1024 | 72 | 28 | 40 |
| 15 | 1536 | 108 | 44 | 40 |
| 20 | 2048 | 144 | 58 | 40 |
| NOTE: These percentages are informative and apply to a slot's symbols 1 to 6 and 8 to 13. Symbols 0 and 7 have a longer CP and therefore a lower percentage. |  |  |  |  |

Table 9.6.3.5.1-3: EVM window length for normal CP, FR1-NTN, 30 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 5 | 256 | 18 | 8 | 40 |
| 10 | 512 | 36 | 14 | 40 |
| 15 | 768 | 54 | 22 | 40 |
| 20 | 1024 | 72 | 28 | 40 |
| NOTE: These percentages are informative and apply to a slot's symbols 1 through 13. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |

Table 9.6.3.5.1-4: EVM window length for normal CP for NR, FR1-NTN, 60 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (Note) (%) |
| --- | --- | --- | --- | --- |
| 10 | 256 | 18 | 8 | 40 |
| 15 | 384 | 27 | 11 | 40 |
| 20 | 512 | 36 | 14 | 40 |
| NOTE: These percentages are informative and apply to all OFDM symbols within subframe except for symbol 0 of slot 0 and slot 2. Symbol 0 of slot 0 and slot 2 may have a longer CP and therefore a lower percentage. |  |  |  |  |

The modulation quality requirement for NB-IoT are specified in TS 36.181 [23] clause 9.6.3.5.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

##### 9.7.2.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1.

Directions to be tested: OTA coverage range reference direction (D.30).

Beams to be tested: Declared beam with the highest intended EIRP for the narrowest intended beam corresponding to the smallest BeWθ, or for the narrowest intended beam corresponding to the smallest BeWϕ (D.3, D.11).

For SAN declared to be capable of single carrier operation, start transmission according to the applicable test configuration in clause 4.8 using the corresponding test model NR-SAN-FR1-TM1.1 for SAN type 1-O or NR-SAN-FR2-TM1.1 for SAN type 2-O in clause 4.9.2 at manufacturers declared rated carrier EIRP (Prated,c,EIRP, D.11).

For a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60), test shall be using NR-N-TM according to clause 4.9.2.2.7.

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------

#### 9.7.3.1	Definition and applicability

OTA Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The measured power is TRP.

The requirement shall be applied per RIB.

For a RIB operating in multi-carrier, the OTA ACLR requirements in clause 9.7.3.2 apply to SAN channel bandwidths of the outermost carrier.

The requirements shall also apply if the SAN supports NB-IoT operation in NTN NR in-band.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

##### 9.7.3.4.2	Procedure

The following procedure for measuring TRP is based on the directional power measurements as described in TS 38.141-2 [18], annex I. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber if so follow steps 1, 3, 4, 6, 8, 9, 10, and 11.

1)	Place the SAN at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the SAN with the test system.

3)	The measurement devices characteristics shall be:

-	Measurement filter bandwidth: defined in clause 9.7.3.5.

-	Detection mode: true RMS voltage or true power averaging.

-	The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in table 4.1.2.2-2.

4)	For single carrier operation, set the SAN to transmit according to the applicable test configuration in clause 4.8 using the corresponding test model(s) in clause 4.9.2 at manufacturers declared rated carrier TRP output power declared per RIB (Prated,c,TRP).

For SAN declared to be capable of multi-carrier operation use the applicable test signal configuration and corresponding power setting specified in clauses 4.7.2 and 4.8 using the corresponding test model(s) in clause 4.9.2 on all carriers configured.

For a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60), test shall be performed using NR-N-TM according to clause 4.9.2.2.7.

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------

#### 9.7.4.1	Definition and applicability

The OTA limits for out-of-band emissions are specified as TRP per RIB unless otherwise stated.

For a RIB operating in multi-carrier, the requirements apply to SAN channel bandwidths of the outermost carrier.

The requirements shall also apply if the SAN supports NB-IoT operation in NTN NR in-band.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

##### 9.7.4.4.2	Procedure

The following procedure for measuring TRP is based on the directional power measurements as described in TS 38.141-2 [18], annex I. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber if so follow steps 1, 3, 4, 6.

1)	Place the SAN at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the SAN with the test system.

3)	The measurement devices characteristics shall be:

-	Measurement filter bandwidth: as defined in clause 6.7.4.5.

-	Detection mode: true RMS voltage or true power averaging.

As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity, efficiency and avoiding e.g. carrier leakage, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in table 4.1.2.2-2.

4)	For single carrier operation, set the SAN to transmit according to the applicable test configuration in clause 4.8 using the corresponding test model(s) in clause [4.9.2] at manufacturers declared rated carrier TRP output power declared per RIB (Prated,c,TRP).

For a SAN declared to be capable of multi-carrier operation, use the applicable test signal configuration and corresponding power setting specified in clause [4.7.2] and 4.8 using the corresponding test model(s) in clause 4.9.2 on all carriers configured.

For a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60), test shall be performed using NR-N-TM according to clause 4.9.2.2.7.

------------------------------------------------------------- NEXT CHANGE ----------------------------------------------

##### 9.7.5.2.1	Definition and applicability

The general OTA transmitter spurious emissions requirements are specified as TRP per RIB, per cell, unless otherwise specified.

The requirements shall apply to SAN that support NR or NR with NB-IoT operation in NTN NR in-band.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

###### 9.7.5.2.4.2	Procedure

The following procedure for measuring TRP is based on directional power measurements as described in TS 38.141-2 [18], annex I. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber if so follow steps 1, 3, 4, 5, 7.

1)	Place the SAN at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the SAN with the test system.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in clause 9.7.5.2.5.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

-	The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table 4.1.2.2-2 for FR1-NTN and Table 4.1.2.2-3 for FR2-NTN.

5)	Set the SAN to transmit:

-	For RIB declared to be capable of single carrier operation only, set the RIB to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test model in clause 4.9.2 (i.e. NR-SAN-FR1-TM1.1 for SAN type 1-O and NR-SAN-FR2-TM1.1 for SAN type 2-O), at manufacturer's declared rated output power Prated,c,TRP.

-	For a RIB declared to be capable of multi-carrier operation, set the RIB to transmit according to the corresponding test model in clause 4.9.2 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 4.7.2 and 4.8.

- 	For a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60), test shall be performed using NR-N-TM according to clause 4.9.2.2.7.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

## 10.1	General

General test conditions for receiver tests are given in clause 4, including interpretation of measurement results and configurations for testing. SAN configurations for the tests are defined in clause 4.5.

Radiated receiver characteristics are specified at RIB, with full complement of transceivers for the configuration in normal operating condition.

Unless otherwise stated, the following arrangements apply for the radiated receiver characteristics requirements in clause 10:

-	Requirements shall be met for any transmitter setting.

-	The requirements shall be met with the transmitter unit(s) ON.

-	Throughput requirements defined for the radiated receiver characteristics do not assume HARQ retransmissions.

-	When SAN is configured to receive multiple carriers, all the throughput requirements are applicable for each received carrier.

-	For ACS and blocking characteristics, the negative offsets of the interfering signal apply relative to the lower SAN RF Bandwidth edge, and the positive offsets of the interfering signal apply relative to the upper SAN RF Bandwidth edge.

-	Each requirement shall be met over the RoAoA specified.

-	Requirements shall also apply for SAN supporting NB-IoT operation in NTN NR in-band. The corresponding NB-IoT requirements are specified in clause 10 of TS 36.181 [23].

NOTE 1:	In normal operating condition the SAN in FDD operation is configured to transmit and receive at the same time.

For FR1-NTN requirements which are to be met over the OTA REFSENS RoAoA absolute requirement values are offset by the following term:

ΔOTAREFSENS = 44.1 - 10*log10(BeWθ,REFSENS*BeWφ,REFSENS) dB for the reference direction

and

ΔOTAREFSENS = 41.1 - 10*log10(BeWθ,REFSENS*BeWφ,REFSENS) dB for all other directions

For requirements which are to be met over the minSENS RoAoA absolute requirement values are offset by the following term:

ΔminSENS = PREFSENS – EISminSENS (dB)

For FR2-NTN requirements which are to be met over the OTA REFSENS RoAoA absolute requirement values are offset by the following term:

ΔFR2_REFSENS = -3 dB for the reference direction

and

ΔFR2_REFSENS = 0 dB for all other directions

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 10.2.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.4.1.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Align the SAN with the test antenna in the declared direction to be tested.

4)	Ensure the polarization is accounted for such that all the power from the test antenna is captured by the SAN under test.

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	For FDD operation, set the SAN to transmit beam(s) of the same operational band as the OSDD being tested according to the appropriate test configuration in clauses 4.7 and 4.8.

7)	Start the signal generator for the wanted signal to transmit:

-	The test signal as specified in clause 10.2.5.

8)	Set the test signal mean power so the calibrated radiated power at the SAN Antenna Array coordinate system reference point is as specified in clause 10.2.5.

9)	Measure the throughput according to annex A.1, as well as annex A.14 of TS 36.181 [23] for a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60) for each supported polarization.

10)	Repeat steps 3 to 9 for all OSDD(s) declared for the SAN (D.19), and supported polarizations.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 10.2.5.2	Test requirements for SAN type 1-H and SAN type 1-O

For each measured carrier, the throughput measured in step 9 of clause 10.2.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 10.2.5.2-1.

Table 10.2.5.2-1: SAN GEO and LEO class EIS levels

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA sensitivity level, EIS (dBm) (Note 2) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-NTN-A1-1 (Note 1) | Declared minimum EIS + 1.3 |
|  |  | G-FR1-NTN-A1-10 (Note 2, 3) |  |
| 10, 15 | 30 | G-FR1-NTN-A1-2 (Note 1) |  |
| 10, 15 | 60 | G-FR1-NTN-A1-3 (Note 1) |  |
| 20 | 15 | G-FR1-NTN-A1-4 (Note 1) |  |
|  |  | G-FR1-NTN-A1-11 (Note 2, 4) |  |
| 20 | 30 | G-FR1-NTN-A1-5 (Note 1) |  |
| 20 | 60 | G-FR1-NTN-A1-6 (Note 1) |  |
| NOTE: EIS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. NOTE 2: The requirements also apply to SAN that supports NB-IoT operation in NTN NR in-band. NOTE 3: EIS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. NOTE 4: EIS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. |  |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 10.3.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.4.1.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Align the SAN with the test antenna in the declared direction to be tested.

4)	Ensure the polarization is accounted for such that all the power from the test antenna is captured by the SAN under test.

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	Set the SAN to transmit beam(s) of the same operational band as the OTA REFSENS RoAoA being tested according to the appropriate test configuration in clauses 4.7 and 4.8.

7)	Start the signal generator for the wanted signal to transmit:

-	The test signal as specified in clause 10.3.5.

8)	Set the test signal mean power so the calibrated radiated power at the SAN Antenna Array coordinate system reference point is as specified in clause 10.3.5.

9)	Measure the throughput according to annex A.1 for each supported polarization.

10) For SAN type 1-O declared to be capable of NB-IoT operation in NTN NR in-band (D.60), measure throughput according to annex A.14 of TS 36.181 [23].

11)	 Repeat steps 3 to 9 for all OTA REFSENS conformance test directions of the SAN (D.45) and supported polarizations.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 10.3.5.2	Test requirements for SAN type 1-O

For each measured carrier, the throughput measured in step 9 of clause 10.3.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in tables 10.3.5.2-1 and 10.3.5.2-2.

The reference sensitivity level requirements for NB-IoT are specified in clause 10.3.2 of TS 36.181 [23].

Table 10.3.5-1: SAN GEO class reference sensitivity levels

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-NTN-A1-1 (Note 1) | -98.0 - ΔOTAREFSENS |
|  |  | G-FR1-NTN-A1-10 (Note 3) | -98.0 - ΔOTAREFSENS (Note 2) |
| 10, 15 | 30 | G-FR1-NTN-A1-2 (Note 1) | -98.1 - ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-NTN-A1-3 (Note 1) | -95.2 - ΔOTAREFSENS |
| 20 | 15 | G-FR1-NTN-A1-4 (Note 1) | -91.6 - ΔOTAREFSENS |
|  |  | G-FR1-NTN-A1-11 (Note 4) | -91.6 - ΔOTAREFSENS (Note 2) |
| 20 | 30 | G-FR1-NTN-A1-5 (Note 1) | -91.9 - ΔOTAREFSENS |
| 20 | 60 | G-FR1-NTN-A1-6 (Note 1) | -92.0 - ΔOTAREFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. NOTE 2: The requirements apply to SAN that supports NB-IoT operation in NTN NR in-band. NOTE 3: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. NOTE 4: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. |  |  |  |

Table 10.3.5-2: SAN LEO class reference sensitivity levels

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-NTN-A1-1 (Note 1) | -101.1 - ΔOTAREFSENS |
|  |  | G-FR1-NTN-A1-10 (Note 3) | -101.1 - ΔOTAREFSENS (Note 2) |
| 10, 15 | 30 | G-FR1-NTN-A1-2 (Note 1) | -101.2 - ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-NTN-A1-3 (Note 1) | -98.3 - ΔOTAREFSENS |
| 20 | 15 | G-FR1-NTN-A1-4 (Note 1) | -94.7 - ΔOTAREFSENS |
|  |  | G-FR1-NTN-A1-11 (Note 4) | -94.7 - ΔOTAREFSENS (Note 2) |
| 20 | 30 | G-FR1-NTN-A1-5 (Note 1) | -95.0 - ΔOTAREFSENS |
| 20 | 60 | G-FR1-NTN-A1-6 (Note 1) | -95.1 - ΔOTAREFSENS |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. NOTE 2: The requirements apply to SAN that supports NB-IoT operation in NTN NR in-band. NOTE 3: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-10 mapped to the 24 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-1 mapped to disjoint frequency ranges with a width of 25 resource blocks each. NOTE 4: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for a single instance of G-FR1-NTN-A1-11 mapped to the 105 NR resource blocks adjacent to the NB-IoT PRB, and for each consecutive application of a single instance of G-FR1-NTN-A1-4 mapped to disjoint frequency ranges with a width of 106 resource blocks each. |  |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 10.4.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.4.2.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Align the SAN with the test antenna in the declared direction to be tested.

4)	Ensure the polarization is accounted for such that all the power from the test antenna is captured by the SAN under test.

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	For FDD operation, set the SAN to transmit beam(s) of the same operational band as the OTA REFSENS RoAoA being tested according to the appropriate test configuration in clauses 4.7 and 4.8.

7)	Set the test signal mean power so that the calibrated radiated power at the SAN Antenna Array coordinate system reference point is as follows:

a)	Set the signal generator for the wanted signal to transmit as specified in table 10.4.5.2-1 as well as table 10.4.5.2-1a for a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60).

b)	Set the signal generator for the AWGN interfering signal at the same frequency as the wanted signal to transmit as specified in table 10.4.5.2-1 as well as table 10.4.5.2-1a for a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60).

8)	Measure the throughput according to annex A.2 as well as annex A.15 of TS 36.181 [23] for a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60) for each supported polarization.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 10.4.5.2	Test requirements for SAN type 1-O

For each measured carrier, the throughput measured in step 6 of clause 10.4.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in annex A.2 with parameters specified in tables 10.4.5.2-1.

Table 10.4.5.2-1: SAN LEO class dynamic range

| SAN channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-NTN-A2-1 | -76.1 - ΔOTAREFSENS | -88.2 - ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-NTN-A2-2 | -76.8 - ΔOTAREFSENS |  |  |
| 10 | 15 | G-FR1-NTN-A2-1 | -76.1 - ΔOTAREFSENS | -85.0 - ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-NTN-A2-2 | -76.8 - ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-NTN-A2-3 | -73.8 - ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-NTN-A2-1 | -76.1 - ΔOTAREFSENS | -83.2 - ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-NTN-A2-2 | -76.8 - ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-NTN-A2-3 | -73.8 - ΔOTAREFSENS |  |  |
| 20 | 15 | G-FR1-NTN-A2-4 | -69.9 - ΔOTAREFSENS | -81.9 - ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-NTN-A2-5 | -69.9 - ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-NTN-A2-6 | -70.2 - ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. |  |  |  |  |  |

Table 10.4.5.2-1a: SAN LEO class dynamic range for NB-IoT operation in NTN NR in-band

| SAN channel bandwidth (MHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 5 | FRC A15-1 in Annex A.15 in TS 36.181 [23] | -89.1 - ΔOTAREFSENS | -88.2 - ΔOTAREFSENS | AWGN |
| 10 |  |  | -85.0 - ΔOTAREFSENS | AWGN |
| 15 |  |  | -83.2 - ΔOTAREFSENS | AWGN |
| 20 |  |  | -81.9 - ΔOTAREFSENS | AWGN |
| 5 | FRC A15-2 in Annex A.15 in TS 36.181 [23] | -95.0 - ΔOTAREFSENS | -88.2 - ΔOTAREFSENS | AWGN |
| 10 |  |  | -85.0 - ΔOTAREFSENS | AWGN |
| 15 |  |  | -83.2 - ΔOTAREFSENS | AWGN |
| 20 |  |  | -81.9 - ΔOTAREFSENS | AWGN |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

##### 10.5.1.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.4.2.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Align the SAN with the test antenna in the declared direction to be tested.

4)	Align the SAN so that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	For FDD operation, set the SAN to transmit beam(s) of the same operational band as the OSDD or OTA REFSENS RoAoA being tested according to the appropriate test configuration in clauses 4.7 and 4.8.

7)	Set the test signal mean power so that the calibrated radiated power at the SAN Antenna Array coordinate system reference point is as follows:

a)	Set the signal generator for the wanted signal to transmit as specified in table 10.5.1.5.2-1 for SAN type 1-O and table 10.5.1.5.3-1 for SAN type 2-O.

b)	Set the signal generator for the interfering signal at the adjacent channel frequency of the wanted signal to transmit as specified in table 10.5.1.5.2-1 for SAN type 1-O and table 10.5.1.5.3-1 for SAN type 2-O.

8)	Measure throughput according to annex A.1 as well as annex A.14 of TS 36.181 [23] for a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60) for each supported polarization, for multi-carrier operation the throughput shall be measured for relevant carriers specified by the test configuration specified in clauses 4.7.2 and 4.8.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

##### 10.5.1.5.2	Test requirements for SAN type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal and the interfering signal are from the same direction, and the AoA of the incident wave of a received signal and the interfering signal are within the minSENS RoAoA.

The wanted and interfering signals apply to each supported polarization, under the assumption of polarization match.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For FR1, the OTA wanted and the interfering signal are specified in table 10.5.1.5.2-1 and table 10.5.1.5.2-2 for ACS. The reference measurement channel for the OTA wanted signal is identified in clause 10.5.1.2 and is further specified in annex A.1. The characteristics of the interfering signal is further specified in TS 38.108 [2] annex C.

For SAN supporting NB-IoT operation in NTN NR in-band, the wanted and the interfering signal coupled to the SAN type 1-H antenna connector are specified in table 10.5.1.5.2-1 and the frequency offset between the wanted and interfering signal in table 10.5.1.5.2-2 for ACS. The reference measurement channel for the NB-IoT wanted signal is identified in clause 7.2.5 of TS 36.181 [23]. The characteristics of the interfering signal is further specified in annex E.

The OTA ACS requirement is applicable outside the SAN RF Bandwidth or Radio Bandwidth. The OTA interfering signal offset is defined relative to the SAN RF Bandwidth edges or Radio Bandwidth edges.

Table 10.5.1.5.2-1: OTA ACS requirement for SAN type 1-O

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) (NOTE 2) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20 (NOTE 1) | EISminSENS + 6 dB | LEO SAN class: -60 – ΔminSENS GEO SAN class: -57 – ΔminSENS |
| NOTE 1: The SCS for the lowest/highest carrier received is the lowest SCS supported by the SAN for that bandwidth NOTE 2: EISminSENS depends on the SAN channel bandwidth |  |  |

Table 10.5.1.5.2-2: OTA ACS interferer frequency offset for SAN type 1-O

| SAN channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±2.5025 | 5 MHz CP-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 10 | ±2.5075 |  |
| 15 | ±2.5125 |  |
| 20 | ±2.5025 |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 10.6.5	Test requirements for SAN type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of the received signal and the interfering signal are from the same direction and are within the minSENS RoAoA.

The wanted signal applies to each supported polarization, under the assumption of polarization match.

For OTA wanted and OTA interfering signals provided at the RIB using the parameters in table 10.6.5-1, the following requirements shall be met:

-	The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel. The reference measurement channel for the OTA wanted signal is identified in clause 10.3.2 for each SAN channel bandwidth and further specified in annex A.1.

-	For NB-IoT operation in NR in-band, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel. The reference measurement channel for the OTA NB-IoT wanted signal is identified in clause 10.3.2 for each SAN channel bandwidth and further specified in annex A.1.

For SAN type 1-O the OTA out-of-band blocking requirement apply from 30 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB up to 12750 MHz, including the downlink frequency range of the SAN operating band. The ΔfOOB for SAN type 1-O is defined in table 10.6.5-2.

Table 10.6.5-1: OTA out-of-band blocking performance requirement

| Wanted signal mean power (dBm) | Interfering signal RMS field-strength (V/m) | Type of interfering signal |
| --- | --- | --- |
| EISminSENS + 6 dB (NOTE 1) | 0.0029 (NOTE 2) | CW carrier |
| NOTE 1: EISminSENS depends on the channel bandwidth as specified in clause 10.2. NOTE 2: The RMS field-strength level in V/m is related to the interferer EIRP level at a distance described as , where EIRP is in W and r is in m. NOTE 3: For SAN supporting in-band NB-IoT operation, up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of 0.0103 V/m for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1 MHz step size. |  |  |

Table 10.6.5-2: ΔfOOB offset for satellite operating bands

| SAN type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| SAN type 1-O | FUL,high – FUL,low < 100 MHz | 20 |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 10.9.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.4.2.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Align the SAN with the test antenna in the declared direction to be tested.

4)	Align the SAN to that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)	For FDD operation, set the SAN to transmit beam(s) of the same operational band as the OTA REFSENS RoAoA or OSDD being tested according to the appropriate test configuration in clauses 4.7 and 4.8.

7)	Set the test signal mean power so the calibrated radiated power at the SAN Antenna Array coordinate system reference point is as specified as follows:

a)	Adjust the signal generator for the wanted signal as specified in:

For SAN type 1-O, table 10.9.5.1-1 for SAN GEO class, in table 10.9.5.1-2 for SAN LEO class on one side of the FC.

For SAN type 1-O declared to be capable of NB-IoT operation in NTN NR in-band (D.60), table 10.9.5.1-1a for SAN GEO class, in table 10.9.5.1-2a for SAN LEO class on one side of the FC.

For SAN type 2-O, table 10.9.5.2-1 on one side of the FC.

b)	Adjust the signal generator for the interfering signal as specified in:

For SAN type 1-O, table 10.9.5.1-1 for SAN GEO class, in table 10.9.5.1-2 for SAN LEO class at opposite side of the FC and adjacent to the wanted signal.

For SAN type 1-O declared to be capable of NB-IoT operation in NTN NR in-band (D.60), table 10.9.5.1-1a for SAN GEO class, in table 10.9.5.1-2a for SAN LEO class at opposite side of the FC and adjacent to the wanted signal.

For SAN type 2-O, table 10.9.5.2-1 at opposite side of the FC and adjacent to the wanted signal.

8)	Measure throughput according to annex A.1 as well as annex A.14 of TS 36.181 [23] for a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60) for each supported polarization.

9)	Repeat the measurement with the wanted signal on the other side of the FC, and the interfering signal at opposite side of the FC and adjacent to the wanted signal.

10)	Repeat for all the specified measurement directions and supported polarizations.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 10.9.5.1	SAN type 1-O

The requirement shall apply at the RIB when the AoA of the incident wave of the received signal and the interfering signal are the same direction and are within the minSENS RoAoA

The wanted and interfering signals applies to each supported polarization, under the assumption of polarization match.

For a wanted and an interfering signal coupled to the RIB, the following requirements shall be met:

-	For SAN type 1-O, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A.1 with parameters specified in table 10.9.5.1-1 for SAN GEO class, in table 10.9.5.1-2 for SAN LEO class.

-	For SAN type 1-O supporting NB-IoT operation in NTN NR in-band, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A of TS 36.181 [23] with parameters specified in table 10.9.5.1-1a for SAN GEO class, in table 10.9.5.1-2a for SAN LEO class.

Table 10.9.5.1-1: SAN GEO class ICS requirement

| SAN channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A1-7 | -96.5 - ΔminSENS | -92.0 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10,15,20 | 15 | G-FR1-A1-1 | -94.6 - ΔminSENS | -88.1 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 5 | 30 | G-FR1-A1-8 | -97.2 - ΔminSENS | -92.0 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 5 RBs |
| 10,15,20 | 30 | G-FR1-A1-2 | -94.7 - ΔminSENS | -89.0 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 10,15,20 | 60 | G-FR1-A1-9 | -94.1 - ΔminSENS | -89.0 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for SAN channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the SAN channel bandwidth of the wanted signal. |  |  |  |  |  |

Table 10.9.5.1-1a: SAN GEO class ICS requirement for NB-IoT operation in NR in-band

| SAN channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | FRC A14-1 in Annex A.14 in TS 36.181 [23] | -120.5 - ΔminSENS | -92.0 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10,15,20 |  |  |  | -88.1 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 5 | 15 | FRC A14-2 in Annex A.14 in TS 36.181 [23] | -126.5 - ΔminSENS | -92.0 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10,15,20 |  |  |  | -88.1 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| NOTE: Interfering signal is placed in one side of the Fc, while the NB-IoT PRB is placed on the other side. Both interfering signal and NB-IoT PRB are placed at the middle of the available PRB locations. The wanted NB-IoT tone is placed at the centre of this NB-IoT PRB. |  |  |  |  |  |  |

Table 10.9.5.1-2: SAN LEO class ICS requirement

| SAN channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A1-7 | -99.6 - ΔminSENS | -83.1 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10,15,20 | 15 | G-FR1-A1-1 | -97.7 - ΔminSENS | -79.2 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 5 | 30 | G-FR1-A1-8 | -100.3 - ΔminSENS | -83.1 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 5 RBs |
| 10,15,20 | 30 | G-FR1-A1-2 | -97.8 - ΔminSENS | -80.1 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 RBs |
| 10,15,20 | 60 | G-FR1-A1-9 | -97.2 - ΔminSENS | -80.1 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 RBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for SAN channel bandwidth of the wanted signal according to the table 5.4.2.2-1. The aggregated wanted and interferer signal shall be centred in the SAN channel bandwidth of the wanted signal. |  |  |  |  |  |

Table 10.9.5.1-2a: SAN LEO class ICS requirement for NB-IoT operation in NR in-band

| SAN channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | FRC A14-1 in Annex A.14 in TS 36.181 [23] | -123.6 - ΔminSENS | -83.1 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10,15,20 |  |  |  | -79.2 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 5 | 15 | FRC A14-2 in Annex A.14 in TS 36.181 [23] | -129.6 - ΔminSENS | -83.1 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 10,15,20 |  |  |  | -79.2 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| NOTE: Interfering signal is placed in one side of the Fc, while the NB-IoT PRB is placed on the other side. Both interfering signal and NB-IoT PRB are placed at the middle of the available PRB locations. The wanted NB-IoT tone is placed at the centre of this NB-IoT PRB. |  |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

# A.1	Fixed Reference Channels for RF Rx requirement (QPSK, R=1/3)

The parameters for the reference measurement channels are specified in table A.1-1 for FR1-NTN reference sensitivity level, ACS, out-of-band blocking, in-channel selectivity, OTA sensitivity, OTA reference sensitivity level, OTA ACS, OTA out-of-band blocking and OTA in-channel selectivity.

The reference measurement channels for the dynamic range requirement are captured in annex A.2.

FR2-NTN parameters for the reference measurement channels are specified in table A.1-2.

Table A.1-1: Fixed Reference Channels for SAN Rx requirements, FR1-NTN

| Reference channel | G-FR1-NTN-A1-1 | G-FR1-NTN-A1-2 | G-FR1-NTN-A1-3 | G-FR1-NTN-A1-4 | G-FR1-NTN-A1-5 | G-FR1-NTN-A1-6 | G-FR1-NTN-A1-7 | G-FR1-NTN-A1-8 | G-FR1-NTN-A1-9 | G-FR1-NTN-A1-10 | G-FR1-NTN-A1-11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 30 | 60 | 15 | 30 | 60 | 15 | 30 | 60 | 15 | 15 |
| Allocated resource blocks | 25 | 11 | 11 | 106 | 51 | 24 | 15 | 6 | 6 | 24 | 105 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Payload size (bits) | 2152 | 984 | 984 | 9224 | 4352 | 2088 | 1320 | 528 | 528 | 2088 | 8968 |
| Transport block CRC (bits) | 16 | 16 | 16 | 24 | 24 | 16 | 16 | 16 | 16 | 16 | 24 |
| Code block CRC size (bits) | - | - | - | 24 | - | - | - | - | - | - | 24 |
| Number of code blocks – C | 1 | 1 | 1 | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
| Code block size including CRC (bits) (Note 3) | 2168 | 1000 | 1000 | 4648 | 4376 | 2104 | 1336 | 544 | 544 | 2104 | 4520 |
| Total number of bits per slot | 7200 | 3168 | 3168 | 30528 | 14688 | 6912 | 4320 | 1728 | 1728 | 6912 | 30240 |
| Total symbols per slot | 3600 | 1584 | 1584 | 15264 | 7344 | 3456 | 2160 | 864 | 864 | 3456 | 15120 |
| NOTE 1: UL-DMRS-config-type = 1 with UL-DMRS-max-len = 1, UL-DMRS-add-pos = 1 with l0= 2, l = 11 as per table 6.4.1.1.3-3 of TS 38.211 [5]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity and in-channel selectivity. NOTE 3: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [7]. |  |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------