3GPP TSG RAN Meeting #107	RP-250645

Incheon, KR, March 12-14, 2025

Agenda item:	15.2.6

Source:	Tejas Networks

Title:	Scope of AI/ML for NG-RAN in 5G-Adv Rel-20

Document for:	Discussion

# 1	Introduction

As per the RAN chair in [1] ,it is expected to continue evolving AI/ML for NG-RAN in Release-20.

1)	Rel-19 leftovers;

2)	Additional use cases? If so, very limited additional use case

3)	Consider a study first, followed by WI conversion

1)	Rel-19 leftovers;

2)	Additional use cases? If so, very limited additional use case

3)	Consider a study first, followed by WI conversion

A baseline set of objectives for further discussion listed in [1] are:

In this paper, we provide our views on the Rel-19 leftovers for Release-20.

# 2	Discussion

## 2.1	Release 19 leftovers

2.1.1 Federated learning support for Release -19 use case (AI/ML enabled slicing.

AI/ML enabled slicing is one of the use cases in Release 19 under AI/ML for NG-RAN. This use case is an inference-based, meaning that the AI/ML models already exist within the gNBs (or gNB-CU in a split architecture). However, it is also important to conduct a study on the model training aspect, including how these models are created, updated, and maintained.

4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based network slicing:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

As per the TR 38.743[2]:

The drawbacks of the above-mentioned solutions in TR 38.743 are:

- In solution 1, raw data needs to be sent to OAM from gNBs for training, which causes data privacy issues.

- In solution 2, the absence of insights from other gNBs will hinder the overall performance of the models within the gNBs.

Introducing Federated Learning is a promising solution for this. Various types of Federated Learning, such as Horizontal Federated Learning and Vertical Federated Learning, have already been implemented among NWDAFs and AFs in SA2.

Fig 1. Federated Learning Model

In the Federated Learning approach, the OAM (Operation, Administration, and Maintenance) functions as the server, while the gNBs act as the clients. Each gNB collects its own local data and trains its model based on that data. Instead of sending the raw data to the OAM, the gNBs send only the model weights (i.e., the learned parameters) to the OAM. The OAM then aggregates these model weights from all participating gNBs to create an aggregated model. This aggregated model parameters are sent back to the gNBs, which continue the training process on their local data. The iterative process continues until the global model in the OAM converges.

This approach eliminates the need for transmitting raw data from the gNBs to the OAM, ensuring that sensitive user data remains local and enhancing both privacy and efficiency. We propose that RAN3 studies Federated Learning for the use case of slicing, allowing gNBs to collaboratively create a global model, tailored to the unique data and conditions of each individual gNB.

Proposal 1: RAN3 to study and undertake normative work on Federated Learning for the AI/ML enabled slicing, where the OAM acts as the FL server and the gNBs serve as FL clients, the model training will occur in both the OAM and the gNBs, while the inference will take place in the gNBs.

# 3	Conclusions

Proposal 1: RAN3 to study and undertake normative work on Federated Learning for the AI/ML enabled slicing, where the OAM acts as the FL server and the gNBs serve as FL clients, the model training will occur in both the OAM and the gNBs, while the inference will take place in the gNBs.

# References

- RP-243292 RAN’s Chair Summary for 5G advanced for Release-20.

- TR 38.743