3GPP TSG-RAN WG Meeting #107	                                          RP-250652

Incheon, Korea, March 12-14, 2025

Source: 	CEWiT

Title:  	Discussion on work item scope for AIML mobility

Agenda Item:	15.2.4

Document for:	Discussion and Decision

# Introduction

Rel-20 being a pivotal release of 5G, it is important to ensure that we use the gains we observed in Rel-19 study item. The below are the agreements made in RAN#106:

A WG-level work item is expected in Rel-20, with details depending on the outcome of the study in Rel-19. Need to keep the workload reasonable, especially considering the various aspects of the study (RRM measurement prediction, measurement event prediction, RLF/HOF, etc.). Do we need to prioritize the above different aspects in Rel-20?

In this document, we will discuss how we would like to take the work item phase in Rel-20 ensuring a tighter scope of work.

# Discussion

As a part of the study item, the below scenarios were covered:

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

High priority cases are highlighted. Below are the sub-cases which were considered :

| Prediction cases | Measurement result for input | Prediction result for output |
| --- | --- | --- |
| Cell-level (Case 1) | L1-RSRP per beam | L1-RSRP per beam |
| Cell-level (Case 2) | L1/L3-RSRP per cell | L3-RSRP per cell |
| Cell-level (Case 3) | L1-RSRP per beam | L3-RSRP per cell |
| Beam-level | L1-RSRP per beam | L1-RSRP per beam |

Considering the outcomes of the study, we propose the below scope for Rel-20 5GA WI.

The gains observed in temporal domain predictions (specifically temporal domain case B, subcase 2) were considerable. This can be utilized and made functional in Rel-20 WI.

Proposal 1 : RRM prediction of cell-level measurements in the temporal domain should be a part of Rel-20 5G WI

Another major area of work is measurment event prediction. Here, both direct as well as indirect predictions are possible. The agreements from the previous meeting states that RAN2 would prioritise indirect prediction, and aims at improving HOF rate or save UE power by reducing the measurements. To start with, A3 event could be predicted. However, it can be expanded to A1 to A5 later.

Proposal 2 : Measurement event prediction should be a part of Rel-20 5G WI

Spatial domain predictions were down-prioritised in Rel-19 study. However in the upcoming releases of 5G and 6G it will be crucial to do this. This can further help with reduced measurements and/or lesser interruptions to reduce the measurement gaps.

Proposal 3 : Further study of spatial prediction of measurements to be carried out

# Conclusion

Proposal 1 : RRM prediction of cell-level measurements in the temporal domain should be a part of Rel-20 5G WI

Proposal 2 : Measurement event prediction should be a part of Rel-20 5G WI

Proposal 3 : Further study of spatial prediction of measurements to be carried out