3GPP TSG-RAN WG2 #129		                                        R2-2500447

Athens, Greece, February 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1228 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the mandatory features for IAB-MT and NCR-MT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips, Qualcomm, Ericsson, Xiaomi, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core, NR_IAB_enh-Core, NR_netcon_repeater-Core |  |  |  |  |  | Date: |  |  | 2025-2-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During initial access, IAB-MT performs cell barring based on the IAB-Support IE in SIB1, i.e. if this field is absent, the cell is barred for IAB-node. Besides, an IAB-MT shall include the iab-NodeIndication in the RRCSetupComplete message if connecting as an IAB-node. In our understanding, these two features are mandatory features for IAB-MT, i.e. cell barring based on IAB-Support, and inclusion of iab-NodeIndication. The same situation exists in mobile IAB. And in mobile IAB, it is captured in TS 38.306 that it is mandatory for mobile IAB-MT to support cell barring based on mobileIAB-Support and to support inclusion of mobileIAB-NodeIndication. However, these two mandatory features for IAB-MT are missing in current TS 38.306. The same issue exists for NCR-MT. These two mandatory features (i.e. cell barring based on ncr-Support, and inclusion of ncr-NodeIndication) for NCR-MT are missing in TS 38.306. Besides, as captured in current TS 38.306, a mobile IAB-MT shall apply the same capabilities as IAB-MT unless indicated otherwise. However, the IAB-MT features of "Cell barring based on IAB-Support" and "Inclusion of iab-NodeIndication" shall not be applied to mobile IAB-MT. So it needs to be clarified that these two features are not applicable to mobile IAB-MT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that it’s mandatory to support cell barring based on IAB-Support and inclusion of iab-NodeIndication for IAB-MT. - Clarify that it’s mandatory to support cell barring based on ncr-Support and inclusion of ncr-NodeIndication for NCR-MT. Impact analysis: Impacted functionality: IAB-MT, NCR-MT Inter-operability: No inter-operability issue is identified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - In case the IAB-MT does not implement cell barring based on IAB-Support, it may connect to gNBs that do not support IAB, and consequently, it may not be able to support IAB. In case the IAB-MT does not implement inclusion of iab-NodeIndication, the IAB-node cannot be authorized for IAB operation since the IAB-donor won’t select an AMF supporting IAB for the IAB-node. - In case the NCR-MT does not implement cell barring based on ncr-Support, it may connect to gNBs that do not support NCR. In case the NCR-MT does not implement inclusion of ncr-NodeIndication, the NCR-MT cannot be authorized for NCR operation since the gNB won’t select an AMF supporting NCR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.15.1, 4.2.23.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

FIRST CHANGE

4.2.15.1	Mandatory IAB-MT features

Table 4.2.15.1-1, Table 4.2.15.1-2 and Table 4.2.15.1-3 capture feature groups, which are mandatory for an IAB-MT. In addition, it is mandatory for an IAB-MT which is not a mobile IAB-MT to support the following features:

-	Cell barring based on IAB-Support, as specified in TS 38.331 [9].

-	Inclusion of iab-NodeIndication, as specified in TS 38.331 [9].

All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification are optional for an IAB-MT, unless indicated otherwise.

Next Change

4.2.23.1	Mandatory NCR-MT features

Table 4.2.23.1-1, Table 4.2.23.1-2 and Table 4.2.23.1-3 capture feature groups, which are mandatory for an NCR-MT. In addition, it is mandatory for an NCR-MT to support the following features:

-	Cell barring based on ncr-Support, as specified in TS 38.331 [9].

-	Inclusion of ncr-NodeIndication, as specified in TS 38.331 [9].

CA, MR-DC, handover (e.g. CHO, DAPS, CPAC, etc), unlicensed band, HPUE Duty cycle, MPR related UE features and corresponding capabilities are not supported by an NCR-MT. 7.5kHz UL raster shift is not applicable to NCR-MT. All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification are optional for an NCR-MT, unless indicated otherwise.

End of Changes