3GPP TSG-RAN WG2 Meeting #129	R2-2501571

# Athens, Greece, Feb 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2049 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction On SP CSI Reporting Activation/Deactivation MAC CE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-Core |  |  |  |  |  | Date: |  |  | 2025.2.21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The enhanced SP CSI Reporting on PUCCH MAC CE is introduced for NW energy saving in the MAC specification, however, the description of the Ni, x field description is really confusing - Ni,x: this field indicates the activation/deactivation status of the Semi-Persistent CSI report sub-configuration x within csi-ReportSubConfigToAddModList of CSI-ReportConfigId i, as specified in TS 38.331 [5]. If Si set to 1, the octet corresponding to Ni,0 to Ni,7 is present. If Si set to 0, the octet corresponding to Ni,0 to Ni,7 is not present. N0,0 refers to the report sub-configuration which has the lowest csi-ReportSubConfigID within the list, N0,1 to the report sub-configuration which has the second lowest csi-ReportSubConfigID and so on. If the number of report sub-configurations within the list with type set to csi-ReportSubConfigToAddModList in the indicated BWP is less than x + 1, the MAC entity shall ignore the Ni,x field. The Ni,x field is set to 1 to indicate that the corresponding Semi-Persistent CSI report sub-configuration x shall be activated. The Ni,x field is set to 0 to indicate that the corresponding Semi-Persistent CSI report sub-configuration x shall be deactivated; For example, Firstly, the term of ‘CSI-ReportConfigId i’ is not an accurate description here, since the i represents the i-th lowest CSI-ReportConfigId with the type set to the semiPersistentOnPUCCH in the csi-ReportConfigToAddModList not the value of the Id. Please see below: - Si: This field indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within csi-ReportConfigToAddModList, as specified in TS 38.331 [5]. S0 refers to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the lowest CSI-ReportConfigId within the list with type set to semiPersistentOnPUCCH, S1 to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the second lowest CSI-ReportConfigId and so on. If the number of report configurations within the list with type set to semiPersistentOnPUCCH in the indicated BWP is less than i + 1, MAC entity shall ignore the Si field. The Si field is set to 1 to indicate that the corresponding Semi-Persistent CSI report configuration shall be activated. The Si field is set to 0 to indicate that the corresponding Semi-Persistent CSI report configuration i shall be deactivated; Secondly, it is really confusing of the meaning of the ‘type set to csi-ReportSubConfigToAddModList in the indicated BWP’ |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify the i value of the Nx,i means the CSI-ReportConfigId indicated by Si - Clarify “If the number of report sub-configurations in the csi-ReportSubConfigToAddModList of CSI-ReportConfigId indicated by Si is less than x+1, the MAC entity shall ignore the Ni,x field” Impact Analysis Impacted 5G architecture options: NR SA, MR-DC Impacted functionality: Sub-configuration for CSI reporting activation/deactivation Inter-operability: - If the network is implemented according to the CR and the UE is not, UE may not decode the MAC CE correctly and wrongly activate or deactivate the subconfiguration of the report. - If the UE is implemented according to the CR and the NW is not, NW may send the MAC CE with a wrong information to UE, and UE may activate the unintended CSI Report sub configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If this CR is not approved, the MAC CE is not workable for SP CSI reporting with mulitple sub-reporting configuration.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.80 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.1.3.80	Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE

The enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size and consists of the following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	Si: This field indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within csi-ReportConfigToAddModList, as specified in TS 38.331 [5]. S0 refers to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the lowest CSI-ReportConfigId within the list with type set to semiPersistentOnPUCCH, S1 to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the second lowest CSI-ReportConfigId and so on. If the number of report configurations within the list with type set to semiPersistentOnPUCCH in the indicated BWP is less than i + 1, MAC entity shall ignore the Si field. The Si field is set to 1 to indicate that the corresponding Semi-Persistent CSI report configuration shall be activated. The Si field is set to 0 to indicate that the corresponding Semi-Persistent CSI report configuration i shall be deactivated;

-	Ni,x: This field indicates the activation/deactivation status of the Semi-Persistent CSI report sub-configuration x within csi-ReportSubConfigToAddModList, as specified in TS 38.331 [5]. If Si set to 1, the octet corresponding to Ni,0 to Ni,7 is present. If Si set to 0, the octet corresponding to Ni,0 to Ni,7 is not present. N0,0 refers to the report sub-configuration which has the lowest csi-ReportSubConfigID within the list, N0,1 to the report sub-configuration which has the second lowest csi-ReportSubConfigID and so on. If the number of report sub-configurations within the list in the indicatedCSI-ReportConfig is less than x + 1, the MAC entity shall ignore the Ni,x field. The Ni,x field is set to 1 to indicate that the corresponding Semi-Persistent CSI report sub-configuration x shall be activated. The Ni,x field is set to 0 to indicate that the corresponding Semi-Persistent CSI report sub-configuration x shall be deactivated;

-	R: Reserved bit, set to 0.

Figure 6.1.3.80-1: Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE