3GPP TSG-RAN5 Meeting #106	R5-251445

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0653 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of eCall test case 11.14 for CEN alignments |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New test case 11.14 needs to be added as part of eCallCEN-UEConTest WI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 11.14 eCall only mode / eCall to URI for test service / MSD transfer and 200 OK with ACK / SIP INFO request for MSD Update / PSAP Callback / Success |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eCallCEN-UEConTest WI will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.14 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0350 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-251445 is the final version R5-251405 |  |  |  |  |  |  |  |  |

<Start of modified section>

## 11.14	eCall only mode / eCall to URI for test service / MSD transfer and 200 OK with ACK / SIP INFO request for MSD Update / PSAP Callback / Success

11.14.1	Test Purpose (TP)

(1)

with { UE subjected to national regulations in CEN EN 17184 has an ongoing ecall to uri for test services }

ensure that {

when { SS sends SIP INFO message for MSD }

then { the UE sends SIP INFO message including MSD transfer }

}

(2)

with { UE subjected to national regulations in CEN EN 17184 having previously established eCall to URI for test services }

ensure that {

when { UE receives INVITE for an incoming IMS call for PSAP callback }

then { UE responds with 200 OK including correctly composed header fields }

}

(3)

with { UE subjected to national regulations in CEN EN 17184 having previously established eCall to URI for test services }

ensure that {

when { SS requests for tranfer of MSD during an incoming IMS call }

then { UE responds with 200 OK and performs MSD transfer }

}

11.14.2.	Conformance requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-19 requirements.

[Rel-19, TS 24.229, clause 5.1.6A]:

This clause specifies a feature which applies to UEs subject to national regulations in CEN EN 17184:2024 [300]. If the upper layers request establishment of an IMS call of the test eCall type while in limited service state, the request shall be rejected. Otherwise, the UE shall follow normal voice call setup to attempt the test eCall. If the CS domain is selected, the UE shall attempt a test eCall setup as a non-emergency services call using appropriate access technology specific procedures.

If the IM CN subsystem is selected and the UE is not currently registered, the UE shall perform an initial registration, as described in subclause 5.1.1.2. The UE shall then apply voice domain selection as specified in 3GPP TS 23.221 [6] and 3GPP TS 23.501 [257] and the procedures as specified in subclause 5.1.2A and 5.1.3 to send an INVITE with the following additions:

1)	the UE shall set the Request-URI to a non-emergency service URN (e.g. SIP, Service, Tel URIs) that is defined by a national regulator, PSAP or eCall test provider for testing purposes; and

NOTE 1:	The Request-URI can be set to "urn:service:test.sos.ecall" as specified in RFC 8147 [244] or the URI can be configured in USIM files; see 3GPP TS 31.102 [15C].

2)	the UE shall insert in the INVITE request:

a)	a To header field with the same service URN as in the Request-URI;

b)	a From header field that includes the public user identity or the tel URI associated with the public user identity, as described in subclause 4.2;

c)	a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

d)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

e)	 a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 2:	Further content for the INVITE such as set the test bit in the MSD is defined in CEN EN 15722:2020 [245] and others is defined in RFC 8147 [244].

Then the UE shall proceed as follows:

1)	if the UE receives a 200 (OK) response to the INVITE request not containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to either "true" or "false"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall send the MSD using audio media stream encoded as described in 3GPP TS 26.267 [9C];

2)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful;

3)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "false"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as unsuccessful;

4)	if the UE receives a 486 (Busy Here), 600 (Busy Everywhere) or 603 (Decline) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "true"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as successful and shall report the response and successful MSD transfer to upper layers; and

5)	in all other cases, the UE may perform a normal voice call redial to reattempt the test eCall as described above

While a test eCall remains established a UE shall support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD and the procedures specified in 3GPP TS 26.267 [9C] for transfer of an updated MSD using the audio media stream.

NOTE 3:	For the updated MSD the test bit is set in the MSD according to CEN EN 15722:2020 [245].

After a test eCall has been established and released, successfully or unsuccessfully, a UE shall support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD during an incoming IMS call when the following conditions are fulfilled:

1)	the incoming IMS call happens within an implementation dependent time period according to the national requirements within the region of eCall service (i.e. for EU region the timer value is 1 hour as defined by the T9 timer in CEN EN 17184:2024 [300]);

2)	the INVITE for the incoming IMS call contains a P-Asserted-Identity header field containing the same non-emergency service URN as included in the Request-URI for the test eCall or as included in a P-Asserted-Identity header field received in a response to the INVITE for the test eCall;

3)	the INVITE for the incoming IMS call contains an Accept header field that includes the value "application/EmergencyCallData.Control+xml"; and

4)	the INVITE for the incoming IMS call contains a Recv-Info header field with header field value EmergencyCallData.eCall.MSD.

When conditions 1) to 4) are fulfilled, the UE shall include in the 200 OK:

a)	a Recv-Info header field with header field value EmergencyCallData.eCall.MSD; and

b)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244].

The UE shall also support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD.

NOTE 4:	As defined in CEN EN 17184:2024 [300], the recipient of a test eCall can instigate a callback to the UE to verify UE callback capability and ability to transfer updated MSD.

NOTE 5:	The recipient of a test eCall is encouraged to call back the UE after a shorter period in order to complete the test eCall without a long wait by a test eCall user.

When conditions 1) and 2) are fulfilled, the UE shall support request and transfer of an updated MSD using audio media stream as described in 3GPP TS 26.267 [9C].

NOTE 6:	A test eCall recipient can request transfer of an updated MSD using audio media stream if the procedures specified in subclause 5.1.6.11.3 cannot be supported by the recipient, the serving PLMN, the HPLMN or by some other intermediate network.

NOTE 7:	For a trustable and secure identification of an eCall callback and the related MSD transfer between UE and the PSAP (or test center), the PSAP (or test center) is part of the trust domain of the IMS home operator network where the UE under test belongs to.

[TS 24.229, clause 5.1.6.11.3]:

During an emergency session established for eCall type of emergency service as described in subclause 5.1.6.11.2, if the UE receives an INFO request with:

1)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

2)	a multipart/mixed body including:

a)	an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] containing a "request" element with an "action" attribute set to "send-data" and a "datatype" attribute set to "eCall.MSD"; and

b)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.Control+xml" MIME body part; and

3)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body;

the UE shall proceed as follows:

1)	if the UE is able to provide an updated MSD, the UE shall send an INFO request containing:

a)	an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244];

b)	a multipart/mixed body including:

i)	an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244] containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 as specified in CEN EN 15722:2015 [245]; and

ii)	a Content-Disposition header field set to "By-Reference" associated with the "application/EmergencyCallData.eCall.MSD" MIME body part; and

c)	a Content-Disposition header field set to "Info-Package" associated with the multipart/mixed body; and

…

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.3 and 5.1.6A.

11.14.3	Test description

11.14.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	the eCall capable UE is equipped with ‘eCall only’ enabled USIM configured as per TS 38.508-1 [4] Table 6.4.1-24.

Preamble:

-	 The UE is in test state 0-A (Switched Off) as defined in TS 38.508-1 [21], subclause 4.4A.2.

11.14.3.2	Test procedure sequence

Table 11.14.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is triggered to initiate test eCall to uri for test services. | - | - | - | - |
| 2-16 | Steps 1-15 of generic procedure specified in Table 4.5.2.2-2 of TS 38.508-1 [21] are performed to register the UE to 5GC. | - | - | - | - |
| 17 | Step 2a1 of generic procedure specified in Table 4.5A.2.2.2-2 of TS 38.508-1 [21] is performed. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST | - | - |
| 18-20 | Steps 3-5a1 of generic procedure specified in Table 4.5A.2.2.2-1 of TS 38.508-1 [21] are performed to establish the IMS PDU session. | - | - | - | - |
| 21-24 | Steps 1-4 as defined in Annex A.2 are performed (IMS registration). | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 25-33, steps 5-8 as defined in Annex A.2 are performed and the steps specified in Annex A.10 on PUBLISH may happen. | - | - | - | - |
| 25 | Step 1 as defined in A.23 is performed | --> | INVITE | - | - |
| 26-28 | Steps 11-13 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed to establish the QoS flow for the test call. |  |  |  |  |
| 29-30 | Steps 2-3 as defined in Annex A.23 are performed to complete the establishment of the test eCall. |  |  |  |  |
| 31-32 | Steps 4-5 as defined in Annex A.23 are performed to request MSD update. | - | - | - | - |
| 33 | Check: Does UE send SIP INFO message with MSD update? | --> | SIP INFO | 1 | P |
| 34 | SS responds SIP INFO with 200 OK. | <-- | 200 OK | - | - |
| 35 | Release eCall for test service using the generic procedure described in TS 38.508-1 [21] subclause 4.9.18. | - |  | - | - |
| 36 | SS waits for 10 seconds | - | - | - | - |
| 37-49 | Steps 1-12 of the generic test procedure for IMS MT speech call establishment in 5GC specified in TS 38.508-1 [21] subclause 4.9.16.2.2 are performed. | - | - | - | - |
| - | EXCEPTION: Steps 50a1-50b6 describe behaviour that depends on UE configuration; the “lower case letter” identifies a step sequence that takes place if such configuration was conducted. | - | - | - | - |
| 50a1-50a6 | IF the UE is configured to use preconditions THEN steps 6-10A of Annex A.5.1a take place. | - | - | - | - |
| 50a7 | Check: Does the UE send a correctly composed 200 OK for INVITE request? (Step 11 of annex A.5.1a) | --> | 200 OK | 2 | P |
| 50a8 | SS sends ACK | <-- | ACK | - | - |
| 50b1-50b4 | ELSE steps 6-8A of Annex A.5.2a of take place | - | - | - | - |
| 50b5 | Check: Does the UE send a correctly composed 200 OK for INVITE request? (Step 9 of annex A.5.2a) | --> | 200 OK | 2 | P |
| 50b6 | SS send ACK | <-- | ACK | - | - |
| 51-52 | Steps 4-5, as defined in Annex A.23 of TS 34.229-5 [47] are performed to request MSD update. | - | - | - | - |
| 53 | Check: Does UE send SIP INFO message with MSD update? | --> | SIP INFO | 3 | P |
| 54 | SS responds SIP INFO with 200 OK. | <-- | 200 OK | - | - |
| 55 | Generic test procedure for IMS MT call release in 5GS described in TS 38.508-1 [21] subclause 4.9.18 takes place. | - | - | - | - |

11.14.3.3	Specific message contents

Table 11.14.3.3-1: INVITE (Step 25, table 11.14.3.2-1)

Derivation path: Step 1 in Annex A.23 with condition A25 instead of A20/A21 and without condition A7

Table 11.14.3.3-2: 200 OK for INVITE (Step 29, Table 11.14.3.2-1)

Derivation path: Step 2 in Annex A.23 with Conditions A12 and A13 and without condition A6

Table 11.14.3.3-3: INVITE (Step 46, Table 11.14.3.2-1; step 9a1 or 9b1 of TS 38.508-1 [21] Table 4.9.16.2.2-1)

Derivation path: Step 1 in Annex A.5.1a or A.5.2a with condition A13

Table 11.14.3.3-4: 200 OK (Steps 50a7,50b5, Table 11.14.3.2-1)

Derivation path: Step 2 in Annex A.23 with condition A24

Table 11.14.3.3-5: INFO (Steps 31,51, Table 11.14.3.2-1)

Derivation path: Step 4 in Annex A.23 with condition A1

<End of modified section>