3GPP TSG RAN #107			RP-250802

Incheon, Korea, March 12th – 14th, 2025

Title:	Moderator's summary for RAN1 led REL-20 topics

Source:	RAN1 Chair (Samsung)

# Introduction

This document summaries the discussion of following RAN1 topics:

- AI/ML Air Interface

- NR-MIMO Evolution

- Ambient IoT

- Additional RAN1-led Candidate Topics: Coverage Enhancement (incl. FR2)

- Additional RAN1-led Candidate Topics: Network Energy Savings

The summaries provided in sections 2.2, 3.2, 4.2, 5.2, and 6.2 were made based on submitted contributions in agenda items 15.1, 15.2.1, 15.2.2, 15.2.3, 15.2.7.1 and 15.2.7.2.

# AI/ML Air Interface

## Submitted Contributions for AI/ML Air Interface

RP-250072	View on Rel-20 AI/ML for air interface	Samsung

RP-250079	AIML for air interface	Ericsson

RP-250088	AI and ML for NR air interface in Rel-20 5G-Advanced	NVIDIA

RP-250094	Views on Rel-20 AI/ML Enhancement	Google

RP-250118	AIML for air interface Phase2	NEC

RP-250134	View on Rel-20 AI/ML for air interface for 5G-Advanced	SK Telecom

RP-250162	Views on AI/ML for air interface in 5G-Advanced Rel-20	Fujitsu Limited

RP-250174	Potential work scope for AIML Air Interface in Rel-20	OPPO

RP-250177	Views on AI/ML air interface for Rel-20 5G-Advanced	Spreadtrum, UNISOC

RP-250184	On Rel-20 AI/ML for Air Interface	Apple

RP-250200	Scope of AI/ML based physical layer enhancements in 5G-A Rel-20	vivo,

RP-250217	Views on AI/ML air interface in 5G-A Rel-20	Xiaomi

RP-250255	Views on Rel-20 AI/ML for Air interface	CMCC

RP-250292	AIML for air interface	Tejas Network Limited

RP-250295	Post-Deployment Views on AI/ML for the Air Interface	VIAVI Solutions

RP-250306	Qualcomm Views on AI/ML for Air Interface WI in Rel-20	Qualcomm Innovation Center Inc

RP-250325	MU MIMO – Enhance CSI feedback Type 1 with AI/ML to achieve Type 2 performance	T-Mobile USA Inc.

RP-250335	View on Rel-20 AI/ML for air interface for 5G-Advanced	SEU

RP-250337	Proposals for AIML Air Interface in 5G-A R20	CAICT.

RP-250347	Scope of Rel-20 AI/ML for 5G-Advanced	InterDigital, Inc.

RP-250352	AI/ML Radio Technology in 6G and beyond	DeepSig Inc

RP-250359	Views on Rel-20 5G-A AI/ML for air interface enhancements	NTT DOCOMO, INC.

RP-250399	Discussion on Rel-20 AI/ML air-interface	LG Electronics

RP-250493	Rel-20 AI/ML for NR Air Interface	Huawei, HiSilicon

RP-250499	Views on AI/ML enhancement for Rel-20	Pengcheng laboratory

RP-250510	Views on 5G-A Rel-20 AI/ML for air interface	CATT

RP-250529	AI/ML for air interface Rel-20 WI scope	Futurewei

RP-250548	AI/ML air interface for 5G-Advanced in Rel-20	ZTE Corporation, Pengcheng Laboratory

RP-250643	Views on Rel-20 5G-Adv AI/ML Air-Interface WI	Intel Corporation

## Summary of AI/ML Air Interface contributions

The following table captures some of the proposed objectives with good level of support (at least 5 supporting companies).

|  | Proposed objectives and supporting companies | Motivation and additional details |
| --- | --- | --- |
| 1 | CSI compression for two-sided model Support: Samsung, NVIDIA, Google, NEC, Fujitsu, OPPO, vivo, Qualcomm, CAICT, CATT, Futurewei, Xiaomi, Spreadtrum, AT&T Against: Ericsson, InterDigital, LGE | Proponents argue that there has been sufficient study on CSI compression in Rel-18 and Rel-19 to justify this feature, hence, the item is ready for normative work. Opponents argue that the trade-off between cost and benefit of this feature is not justified due to the following: - Limited and non-critical gain - High complexity compared to that of legacy CSI computation - Possible complexity issues with inter-vendor training collaboration, e.g., dataset, model parameters sharing |
| 2 | Expansion of AI/ML to new use cases and features Support: Google, NEC, Apple, LGE, ZTE, Intel, Pengcheng Lab, TEJAS Networks, Huawei/HiSi, Vodafone, Spreadtrum, NVIDIA, T-Mobile, InterDigital, SKT Against: Samsung, CATT, VIAVI Solutions, Qualcomm, Xiaomi | Beam management enhancement - Mechanisms to reduce the TCI activation delay for the TCI state based on the predicted beam - Candidate beam detection for BFR based on predicted beam - Tx-Rx beam pair prediction - AI based BM in RACH procedure - UE initiated beam reporting for AI based beam management CSI enhancement - Extension of CSI compression for TDD - NW-side model CSI prediction - Enhance CSI feedback Type 1 with AI/ML to achieve Type 2 for MU-MIMO performance - CSI prediction using both periodic CSI-RS and aperiodic CSI-RS RS overhead reduction - SRS overhead reduction - CSI-RS overhead reduction - Overlaid DMRS and data with one-sided AI/ML channel estimation Joint channel and source coding Positioning enhancement based on angle measurement |
| 3 | Leftover from Rel-19 AI/ML positioning (2nd priority cases) Support: NEC, LGE, ZTE, Intel, Pengcheng Lab Against: Samsung, CATT, Spreadtrum | 2nd priority cases for positioning in Rel-19 - Direct AI/ML positioning – Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning – Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning |
| 4 | Additional LCM specification support Support: CMCC, CATT, China Unicom, Spreadtrum, vivo, ZTE, Ericsson, Qualcomm, AT&T | Specify field channel information collection for AI/ML Support for UE-side models data collection and model transfer/delivery |

Based on the contributions submitted to agenda item 15.2.1, the following observations are made.

- Overall, the proposals on Rel-20 AI/ML for Air Interface can be categorized into the following four areas

  - Transition of the Rel-19 study on CSI compression to normative work

  - Expansion of AI/ML to new use cases and features

  - Leftover from Rel-19 AI/ML positioning (2nd priority cases)

  - Additional LCM specification support

- For normative work on CSI compression, there was good level of support (14 companies) although 3 companies showed negative views

  - Additional discussion necessary on aspects such as inter-vendor training collaboration, sub-use cases if CSI compression is made part of Rel-20 5G-Adv

- For expansion of AI/ML to new use cases and features, 15 companies were supportive and had proposals. However, 5 companies had negative view on introducing any new use cases or features for 5G-Adv in Rel-20. Also, proposals from the proponent of new use cases covered wide range of topics as shown below.

  - Beam management enhancement

    - Relatively high level of support compared to the other three areas. Some of the proposals were submitted to Rel-19 but were not specified due to various reasons.

  - CSI enhancement

  - RS overhead reduction

  - Positioning enhancement based on angle measurement

- For leftover from Rel-19 AI/ML positioning (2nd priority cases), the level of support was lower compared to other areas (e.g. compared CSI compression).

- A number of companies proposed additional LCM specification support for data collection and model transfer/delivery

## Moderator observations and recommendations

Based on the Wednesday offline discussions, the following observations have been made:

- CSI compression for two-sided model

  - There was good level of support for specifying CSI compression for two-sided model as part of Rel-20 5G-Adv

  - However, the final decision to support CSI compression should be dependent on the outcome of the ongoing Release 19 AI/ML air interface study item

- New use case / features in Release 20 5G-Adv

  - There was no consensus on including new use cases / features in Release 20

    - New use case / features in Release 20 (18): ZTE, Google, Huawei, NEC, vivo, Tejas NW, Fujitsu, IDC, IIT Madras, SKT, KDDI, Nokia, Orange, IIT Kanpur, DeepSig, KT, Vodafone, T-Mobile

    - No new use case / features in Release 20 (17): DOCOMO, CMCC, CATT, Samsung, Intel, Lenovo, Panasonic, OPPO, Ericsson, MTK, AT&T, Apple, Kyocera, Xiaomi, Verizon, FAI, Spreadtrum

- 2nd priority cases from Rel-19 AI/ML positioning

  - Clear majority of companies were in negative of including 2nd priority cases from Rel-19 AI/ML positioning in Rel-20

    - Include 2nd priority cases from Rel-19 AI/ML positioning as part of Release 20 (3): NEC, CeWiT, Intel

    - Do not include 2nd priority cases from Rel-19 AI/ML positioning as part of Release 20 (12): Samsung, Google, Spreadtrum, CATT, Huawei, Futurewei, DOCOMO, Xiaomi, Verizon, OPPO, T-Mobile USA, Fujitsu

Moderator assumes the following for the purpose of TU planning:

- Include CSI compression for two-sided model as part of Release 20 5G-Adv

  - Final decision on whether to support CSI compressions for two-sided model as part of Release 20 5G-Adv will be made considering the outcome of Release 19 AI/ML air interface study item

  - If final decision is to not include CSI compression, TU for Release 20 5G-Adv AI/ML air interface will be revised accordingly

- No new use case / features in Release 20 5G-Adv AI/ML air interface

- No 2nd priority cases from Rel-19 AI/ML positioning in Release 20 5G-Adv AI/ML air interface

- Further discuss additional LCM specification support (e.g. data collection and model transfer/delivery) in RAN#108

- [2] TUs in RAN1 and [2] TUs in RAN2 for Release 20 5G-Adv AI/ML air interface

# NR-MIMO Evolution

## Submitted Contributions for NR-MIMO Evolution

RP-250054	Scope for Rel-20 NR MIMO Evolution	Samsung

RP-250080	MIMO Phase 6	Ericsson

RP-250095	Views on Rel-20 MIMO Enhancement	Google

RP-250119	Views on MIMO enhancement for NR	NEC

RP-250173	Discussion on NR MIMO Enhancement in Rel-20	OPPO

RP-250178	Views on Rel-20 MIMO evolution	Spreadtrum, UNISOC

RP-250185	On Rel-20 5G-Advanced MIMO	Apple

RP-250201	Scope of MIMO phase 6 in 5G-A Rel-20	vivo

RP-250218	Views on NR MIMO Phase 6	Xiaomi

RP-250221	On Rel-20 NR MIMO enhancements	Nokia

RP-250303	Scope for MIMO phase 6 in 5G-A Rel-20	Tejas Network Limited

RP-250316	Views on MIMO WI in Rel-20	Qualcomm Incorporated

RP-250336	Views on Rel-20 5G Advanced proposals	Verizon Sweden

RP-250348	Scope of Rel-20 NR MIMO 5G-Advanced	InterDigital, Inc.

RP-250360	Views on Rel-20 5G-A MIMO enhancements	NTT DOCOMO, INC.

RP-250400	Discussion on Rel-20 MIMO	LG Electronics

RP-250419	MIMO enhancement in Rel-20 for 5G-A	Lenovo

RP-250479	On Rel-20 MIMO Scope	MediaTek Inc.

RP-250491	Rel-20 MIMO scope	Huawei, HiSilicon

RP-250506	Discussion on 5G-A Rel-20 MIMO	CATT

RP-250528	Proposal for MIMO Rel-20 WI	Futurewei, Lenovo, Spreadtrum

RP-250549	Views on MIMO evolution for 5G-A in Rel-20	ZTE Corporation, Sanechips

RP-250642	Views on Rel-20 5G-Adv MIMO WI	Intel Corporation

RP-250650	Views on MIMO for Release 20	CEWiT

RP-250774	WF for Long Term Covariance Feedback in Rel-20 MIMO	Huawei, HiSilicon, China Unicom, Google, LGU+, China Telecom

Submitted after RAN submission deadline and not captured in section 3.2.

## Summary of NR-MIMO contributions

Wide range of proposals were proposed in the contributions in agenda item 15.2.2. The following table captures some of the proposed objectives with good level of support (at least 5 supporting companies).

|  | Proposed objectives and supporting companies | Motivation and additional details |
| --- | --- | --- |
| 1 | UL Enh: UL frequency-selective (open-loop) precoder cycling Support: Ericsson (conditional), Google, IDC, Nokia, Huawei/HiSi (conditional), Nokia, DOCOMO, Samsung, Spreadtrum, vivo, Xiaomi, ZTE Against: LG, OPPO | Open-loop diversity via precoder cycling across UL PRGs, targeting 2Tx PUSCH transmission, based on legacy UL 2Tx codebook. Proponents argue, from simulation results, performance gain especially at high speed from transmit diversity gain. Opponents argue that it causes channel estimation issue (arguing for non-coherent precoders only), interference fluctuation (for small PRG size), and significant impact on implementations (gNB/UE). There are also results showing lack of performance gain. |
| 2 | UL Enh: 2Tx full-coherent UL codebook with 8/16PSK alphabets Support: CEWiT, IDC, DOCOMO, Qualcomm, Samsung, vivo, ZTE Against: Ericsson, MediaTek, Nokia, OPPO | Extending the legacy UL 2Tx codebook (QPSK-based) by adding additional precoders with higher resolution. The number of additional precoders may or may not require extension to the legacy TPMI design. Proponents argue, from simulation results, significant performance gain. Opponents argue no urgency since the current UL deployment is based on non-coherent 2Tx precoders – requesting clarification on the urgent need. |
| 3 | UL Enh: Cross-slot SRS, SRS frequency hopping for partial sounding Support: Ericsson, Huawei/HiSi, IDC, Intel, Nokia, DOCOMO, Samsung, OPPO, vivo, ZTE | Proponents argue that cross-slot SRS overcomes the inefficiency of, e.g. configuring 2 uplink symbols in one S slot, increased sounding latency in using 2 S slots (especially for antenna switching), or utilizing 2 U slots. SRS frequency hopping for partial sounding increases robustness against interference for partial sounding by allowing interpolation, At least one company opines that cross-slot SRS will introduce tighter phase coherence requirement, which needs to be discussed and addressed. Opponent argues the urgent commercial needs are unclear. |
| 4 | UL Enh: Asymmetric DL sTRP/UL mTRP beam management enhancements Support: Ericsson, IDC, Intel, Nokia (conditional), DOCOMO, OPPO, Samsung, vivo, ZTE Against: MediaTek(conditional) | PDCCH-order-triggered PRACH beam indication, and same Tx beam constraint across SRS occasions (per given SRS resource(s)) for beam sweeping, targeting FR2 Proponents argue these components are needed yet couldn’t be completed in Rel-19 due to scope issues. Opponents argue that the need is unclear. |
| 5 | UL Enh: UL frequency-selective (closed-loop) TPMI-based precoding Support: CATT, CEWiT, Huawei/HiSi, Lenovo, NEC, Nokia, DOCOMO, Samsung, Spreadtrum, ZTE Against: Ericsson, IDC, Intel, LG, MediaTek, OPPO | “Sub-band” precoding for UL based on legacy UL codebooks, targeting 4/8Tx transmission. Proponents argue based on potential performance gain which is beneficial for FWA/CPE scenarios. Opponents argue that this would result in channel estimation issue, large overhead (small # TPMI fields may diminish gain), significant impact on implementations (gNB/UE). There are also results showing lack of performance gain. |
| 6 | DL CSI: Early SRS/CSI/CSI-RS triggering for UEs in IDLE/INACTIVE to CONNECTED; and SCell activation in CONNECTED Support: CATT, Ericsson, IDC, MediaTek, NEC, Nokia, DOCOMO, OPPO, Samsung, Verizon, vivo, Xiaomi, ZTE | Early triggering for IDLE/INACTIVE CONNECTED is done via RACH MSG2/MSG4, with capability and configuration alignment via MSG1/MSG3 and SI, respectively. Early triggering for SCell activation in CONNECTED is done concurrently/in parallel with the activation message. Proponents argue that this avoids large performance loss (e.g. up to 80-90% in DL throughput for SRS-based TDD) during the transient time caused by stale CSI – detrimental in the current commercial deployments. |
| 7 | DL CSI: CSI-RS overhead reduction for larger # ports Support: CEWiT, Ericsson (conditional), IDC, Intel, MediaTek (conditional), DOCOMO, OPPO, Samsung, Tejas, vivo, ZTE, Tejas Networks, CEWiT, Reliance Jio, WiSIG Networks, IISC Bangalore, IIT Madras, IIT Hyderabad, IIT Kanpur, IIT Bhilai Against: CATT, Nokia | Additional support for 1/4 and 1/8 RE/RB/port for 48, 64, 128 ports (frequency-domain); and possibly sub-sampling in port-domain, without impacting legacy rate matching and CSI-RS resource aggregation. Proponents argue for energy saving benefit (e.g. for 7-8GHz hybrid BF). Good performance can be obtained by using advanced measurement algorithms at the UE side. Opponents argue that this would require AI/ML, especially for port-domain sub-sampling, which would be a topic for 6G. |
| 8 | DL CSI: Sub-band scalar IMR measurement report (targeting reciprocity-based operation) Support: Ericsson, IDC, Nokia, DOCOMO, Samsung, Verizon, vivo Against: MediaTek, ZTE | Sub-band (including wideband) scalar interference-plus-noise (IpN) measurement and report with larger dynamic range and higher resolution than the current 4-bit CQI. Rel-18/19 standalone aperiodic report on PUSCH can be reused to minimize spec impact. Proponents argue that interference measurement is tied with CQI and SU-MIMO assumption in legacy – introducing inaccuracy and latency (due to the use of open-loop link adaptation) for MU-MIMO. The benefit comes from higher accuracy and absence of SU-MIMO assumption in the legacy CSI framework. Opponents argue that how to use this report together with CSI (including channel measurement) is unclear. |
| 9 | Multi-beam: UEI/ED beam switching (left over from Rel-19 MIMO) Support: ASUSTek, Apple, Ericsson, Futurewei, IDC, LG, KDDI, Lenovo, DOCOMO, Spreadtrum, Transsion, vivo Against: CATT, Intel, Nokia, MediaTek, OPPO, Samsung, ZTE | Beam switching based on Rel-19 UEI/ED beam reporting using gNB confirmation (i.e. NW-controlled). This is currently being discussed in Rel-19 MIMO. Proponents argue from beam application latency reduction perspective. Opponents argue that FR2 deployment is limited and no urgency and/or the latency reduction benefit compared to UEI/ED beam reporting is unclear. Some prefer to wait until Rel-19 MIMO is complete, else for 6G. The applicability of this topic is contingent on the conclusion of Rel-19 MIMO which makes it difficult. |
| 10 | Multi-beam: Dynamic QCL update via MAC CE for periodic TRS Support: Apple, Ericsson, IDC, Verizon, vivo Against: Huawei/HiSi, Nokia, DOCOMO, Samsung, ZTE | Support for MAC CE based QCL update for common signals especially periodic TRS Proponents argue that this is to avoid frequent RRC reconfiguration (legacy spec uses RRC to update QCL for common signals). Opponents argue that since periodic TRS is cell-specific (UE-common), there is no need for updating this in a UE-specific manner. This topic has been discussed a few times and is controversial. |
| 11 | Enhanced DMRS port allocation Support: Tejas Networks, CEWiT, Reliance Jio, WiSIG Networks, IISC Bangalore, IIT Madras, IIT Hyderabad, IIT Kanpur, IIT Bhilai | Enhanced Multiplexing requirements due to Enhanced CSI ports to 128 or greater. To support a higher number of transmit-receive points (TRP) in mTRP (multiple-TRP) cases. - Introduce DMRS enhancement to support beyond 24 orthogonal DMRS ports Performance enhancements in high MCS, high delay spread to cater to FWA, CPE use cases. Improvement in the performance for deployments using higher SCS (larger impact of delay spread). Enhance mTRP deployment to manage uneven layer performance from issues like time misalignment. Address DMRS port allocation for increased DMRS ports. - Enhance number of DM-RS ports by increasing the OCC length. |

RP-250647 indicated general support for enhancements on UL capacity/coverage enhancements and DL CSI acquisition enhancements.

Based on the contributions submitted to agenda item 15.2.2, the following observations are made.

- Overall, the proposals on Rel-20 MIMO can be categorized into the following three areas

  - Uplink enhancement (items #1, #2, #3, #4, #5)

  - Downlink CSI enhancement (items #6, #7, #8)

  - Multi-beam enhancement (items #9, #10)

  - DMRS enhancement (item #11)

- Good level of support on uplink enhancement, downlink CSI enhancement, and DMRS enhancement

  - However, compared to other proposals, item #5 (UL frequency-selective closed-loop precoding) has relatively large number (6) of companies with negative views

- Compared to uplink enhancement, downlink CSI enhancement, and DMRS enhancement, the level of support on multi-beam enhancement was lower or had significant number of companies who had negative views.

## Moderator observations and recommendations

MIMO has been part of every NR release with at least 2~2.5 TUs per release. While the support for another MIMO enhancement in Release 20 was clearly high, there were multiple companies who voiced that there was no commercial need for it. Also, clear majority of companies were in favor of a compact scope.

Moderator assumes the following for the purpose of TU planning:

- For Rel-20 5G-Adv MIMO, the scope should be limited and well justified by commercial needs

- [1] TU in RAN1 and [0.5] TU in RAN2 for Release 20 5G-Adv MIMO

# Ambient IoT

## Submitted Contributions for Ambient IoT

RP-250073	Views on Rel-20 Ambient IoT	Samsung

RP-250120	Views on Rel-20 Ambient IoT	NEC

RP-250150	Views on Rel-20 Ambient IoT	Ericsson

RP-250160	Proposed scope of Rel-20 A-IoT	LG Electronics Polska

RP-250171	Discussion on potential scoping for Ambient IoT in Rel-20	OPPO

RP-250179	WID scope consideration on Rel-20 Ambient IoT	Spreadtrum, UNISOC

RP-250186	On Rel-20 Ambient IoT	Apple

RP-250202	Scope of Ambient IoT in 5G-A Rel-20	vivo

RP-250219	Consideration on Rel-20 Ambient IoT	Xiaomi

RP-250256	Consideration on Ambient IoT for Rel-20	CMCC

RP-250301	Views on Rel-20 AIoT	Sony Corporation

RP-250317	Views on Ambient IoT WI in Rel-20	Qualcomm Incorporated

RP-250361	Views on Ambient IoT in Rel-20	NTT DOCOMO, INC.

RP-250381	Views on Rel-20 Ambient IoT	China Telecom

RP-250459	Views on Rel-20 Ambient IoT	IIT Kanpur

RP-250507	Views on Ambient IoT works in Rel-20	CATT

RP-250522	Rel-20 Ambient IoT scope	Huawei, HiSilicon

RP-250527	Ambient-IoT Rel-20 WI scope	Futurewei

RP-250550	Views on Ambient IOT enhancement in 5G-A Rel-20	ZTE Corporation, Sanechips

RP-250651	Views on A-IoT for Rel. 20	CEWiT

RP-250664	Views on Scope for 5G-A R20 Ambient IoT	Tejas Network Limited

RP-250331	AIoT - Need for Positioning and	T-Mobile USA Inc.

From agenda item 15.2.7

## Summary of Ambient IoT contributions

For Rel-20 Ambient IoT, RAN had some preliminary discussions and agreements as part of the Rel-19 Ambient IoT discussions in RAN#106.

Recap of workplan for Rel-20 Ambient IoT in RP-243280 (endorsed in RAN#106):

Workplan for Rel-20: - Includes D2T2 - Includes Device 2 - A limited number of additional use cases, deployments, connectivity topologies, devices, traffic types in TR38.848 as a starting point - Note: work on D2T2, device 2 shall follow the Rel-19 study conclusions, and further down-selection of architecture options for topology 2 may be needed. For indoor use cases, deviations from Rel-19 study conclusion shall be well justified and agreed. - Details to be discussed at a later RAN plenary meeting, including the need for study phase.

Following table captures some of the proposed objectives with good level of support (at least 5 supporting companies).

|  | Proposed objectives and supporting companies | Motivation and additional details |
| --- | --- | --- |
| 1 | Support A-IoT Device 2 General consensus | Device 2 is necessary in order to achieve wider coverage for ambient IoT. Device 2 was part of the endorsed Rel-20 Workplan on A-IoT. One necessary decision point is which device sub-type to include in Rel-20. Alternatives are - Support both Device 2a and 2b Supported by Samsung, LGE, xiaomi, China Telecom, Sony - Support only Device 2b or prioritize Device 2b Supported by CMCC, Qualcomm, DOCOMO, ZTE, OPPO, Apple, Spreadtrum/UNISOC, vivo - Support another device type (e.g. device C) other than 2a, 2b Supported by Huawei/HiSi, ZTE, OPPO, vivo |
| 2 | Support additional topologies and deployment scenarios General consensus | The intermediate node in topology 2 can help extend coverage of indoor A-IoT deployment. Note that D2T2 was included as part of the endorsed Rel-20 A-IoT workplan. - Support D2T2 for indoor ambient IoT scenarios. Rel-19 Ambient IoT only supports indoor use cases. Outdoor use cases will expanding Ambient IoT applicability. - Support D4T1 and D4T2 for outdoor ambient IoT scenarios. |
| 3 | Support for DO-A traffic General consensus | Rel-19 A-IoT only allows for DO-DTT, DT traffic types which are not applicable for uses cases where the device need to send data periodically or event-based without being triggered by the reader, such as in sensor use cases. |
| 4 | Support for sensor and positioning use cases Support: CMCC, DOCOMO, Huawei/HiSi, ZTE, xiaomi, China Telecom, CATT, IIT Kanpur, Futurewei, T-Mobile | In addition to the inventory and command use cases, positioning and sensor use cases also account for a significant portion of the use cases identified in the SA1 study. Rel-19 A-IoT proximity determination does not provide sufficient positioning accuracy for A-IoT location and tracking use cases. |
| 5 | Support for unavailability direction 2 Support: Samsung, CMCC, Qualcomm, DOCOMO, ZTE, OPPO, Apple, CEWiT | Since device 2 has higher peak power consumption than device 1, a more advanced solution is required to extend the sustainable time. - Support unavailability direction 2 at least for Device 2 in order to extend device operation time |
| 6 | Carrier wave inside of topology Support: NEC, Apple, Sony, Qualcomm, IIT Kanpur, CEWiT, Tejas | In topology 2, the intermediate node acts as a reader, and this reader UE can also transmit a CW signal to the device. - Support CW signal transmission by an intermediate node in topology 2 |

In order to complete the normative work on the first release of Ambient IoT, the scope in Rel-19 was limited in terms of device type, topology, traffic type, and deployment scenario. Key motivation of Rel-20 Ambient IoT is to expand the applicability of this technology to additional commercially relevant use cases.

In high level, companies were aligned in the objectives for Rel-20 Ambient IoT. However, companies did have different views on the details of what to support within each high-level objective. For example, there were different views on which device sub-type to support within Device 2 as well as proposals to support device type other than Device 2. Also, a number of companies indicated that a study item or study phase would be necessary for some of the aspects being proposed for Rel-20. These aspects were not part of the Rel-19 study. Such aspects include

- Additional topologies / deployment scenarios (e.g. D4T1, D4T2)

- Applicable device types for topology / deployment scenario

- Support for A-IoT and NR coexistence in outdoor scenarios (including whether it is needed)

- Support for sensor and positioning use cases

- Carrier wave inside of topology

## Moderator observations and recommendations

Based on the Wednesday offline discussions, the following observations have been made:

- There was general consensus on the support of device 1 in D2T2 in Release 20 A-IoT

- Clear majority of companies were supportive of having only device 2b in D1T1 and D2T2 in Release 20 A-IoT

  - Only support device 2b in D1T1 and D2T2

    - Panasonic, Apple, DOCOMO, ZTE, Qualcomm, LGE, Lenovo, Spreadtrum, Futurewei, Honor, CMCC, OPPO, Nokia, NEC, CATT (but prefer C if only one is supported), Samsung, CEWiT, IDC, Xiaomi, Semtech, MediaTek

  - Support both device 2a and 2b in D1T1 and D2T2

    - LGE, NEC, CEWiT, IDC, Tejas, Xiaomi, China Telecom

- Multiple companies voiced support for outdoor scenarios based on commercial needs (supported by vivo, Huawei, Futurewei, ZTE, China Unicom, Honor, NEC, CATT, China Telecom, IIT).

  - However, there were concerns on the required workload from Sony, Apple, Lenovo, Spreadtrum, Qualcomm, Ericsson, OPPO (even for item 1), Nokia, MediaTek, IDC.

Moderator assumes the following for the purpose of TU planning:

- Device 1 in D2T2

- Device 2b in D1T1 and D2T2

- Study on A-IoT for outdoor scenarios

- Further discuss other aspects of Rel-20 A-IoT (e.g. DO-A traffic, unavailability direction 2) in RAN#108

- [3~3.5] TUs in RAN1 and [2] TUs in RAN2 for Release 20 5G-Adv A-IoT

# Additional RAN1-led Candidate Topics: Coverage Enhancement (incl. FR2)

## Submitted Contributions for Coverage Enhancement (incl. FR2)

RP-250074	View on Rel-20 coverage enhancement	Samsung

RP-250096	Views on Rel-20 Coverage Enhancement	Google

RP-250124	Views on Rel-20 Coverage Enhancement for 5G-Adv	NEC

RP-250205	Uplink PA non-linearity handling for coverage enhancement in 5G-A Rel-20	vivo

RP-250299	NR coverage enhancements for Rel-20	Nokia

RP-250350	New WI proposal for RIS in NR	Reliance Jio

RP-250363	Views on UL Coverage Enhancements in Rel-20 5G-A	NTT DOCOMO, INC.

RP-250383	WF on NR coverage enhancements for Rel-20	China Telecom

RP-250492	PUSCH coverage enhancement for 5G-Advanced in Rel-20	Huawei, HiSilicon

RP-250511	Views on 5G-A Rel-20 coverage enhancement	CATT

RP-250541	Views on Rel-20 Coverage Enhancement for 5G-Advanced	Ericsson

RP-250554	Views on UL coverage enhancements for 5G-A in Rel-20	ZTE Corporation, Sanechips

## Summary of Coverage Enhancement (incl. FR2) contributions

Wide range of proposals were proposed in the contributions in agenda item 15.2.7.1. The following 3 proposed objectives had very good level of support from most of the companies who were positive towards Rel-20 coverage enhancement.

|  | Proposed objectives and supporting companies | Motivation and additional details |
| --- | --- | --- |
| 1 | Multiple PRACH TX with different beams (Msg3) Support: NEC, Nokia, DOCOMO, China Telecom, China Unicom, CHTTL, KDDI, KT, Orange, SKT, Telecom Italia, Telstra, Verizon, CATT, ZTE | Beam correspondence accuracy has large impact on the performance of PRACH transmission and the followed msg3 transmission. The following objectives are proposed: - Multiple PRACH TX over different beams including beam selection for Msg3 |
| 2 | Msg5 PUSCH repetition Support: NEC, DOCOMO, China Telecom, China Unicom, CHTTL, KDDI, KT, Orange, SKT, Telecom Italia, Telstra, Verizon, CATT, ZTE | Field tests have shown that Msg5 is the bottleneck for initial access, due to the large payload size of Msg5 when compared to other messages. The following objectives are proposed - PUSCH repetition scheduled by DCI 0_0 with C-RNTI |
| 3 | Improvement of PUSCH coverage for high UL data rate Support: Huawei/HiSi, China Telecom, SKT, TIM, Deutsche Telekom, DOCOMO, Vodafone, KDDI, CEWiT, FutureWei, TCL, CAICT, Google | Specify enhancements to improve PUSCH coverage for high uplink data rate, including extending pi/2-BPSK to more MCS entries in MCS tables |
| 4 | Open Reconfigurable Intelligent Surface in 5G NR Support: Reliance Jio, WiSig IIT-H, IIT-M, CEWiT | - Definition of an RIS controller, types of RISs. - Standardize the interface between the Base station and the RIS controller and the RIS. - UE assistance extensions for RIS - Standardize the interface between the OAM and the RIS Controller with extension to the CU, DU messages |

2 companies (in RP-250074 and RP-250541) explicitly indicated that they were not supportive of any coverage enhancement in Rel-20.

Based on the contributions submitted to agenda item 15.2.7.1, the following observations are made.

- Companies were generally supportive of having coverage enhancement in Rel-20. However, there were two companies who explicitly indicated that they were not supportive of any coverage enhancement in Rel-20.

- Multiple PRACH TX with different beams and Msg5 PUSCH repetition have been discussed as candidate enhancements since Rel-18. In the latest discussion on these two proposals in RAN#104, RAN1 chair’s assessment on required TU was 0.5 TUs for 3 quarters.

- For improvement of PUSCH coverage for high UL data rate, if the scope is restricted to extending pi/2-BPSK to more MCS entries in MCS tables, the required effort in RAN1 could be small.

## Moderator observations and recommendations

Based on the Thursday offline discussions, the following proposals seem acceptable for inclusion as part of Release 20 5G-Adv:

- Multiple PRACH TX over different beams including beam selection for Msg3

- PUSCH repetition scheduled by DCI 0_0 with C-RNTI

- Specify enhancements to improve PUSCH coverage for high uplink data rate by extending pi/2-BPSK to more MCS entries in MCS tables

Expected TU allocation: 0.5 TU per meeting (for only part of Rel-20)

# Additional RAN1-led Candidate Topics: Network Energy Savings (NES)

## Submitted Contributions for Network Energy Savings

RP-250075	Way forward on NES in Rel-20	Samsung, Intel, LGE, Xiaomi

RP-250097	Views on Rel-20 NES Enhancement	Google

RP-250125	Support of On-demand SSB for Idle/Inactive UEs in Rel-20	NEC, CEWiT

RP-250151	Views on Rel-20 Network Energy Saving	Ericsson

RP-250222	On Rel-20 NR NES enhancements	Nokia

RP-250260	Considerations on Rel-20 NES	CMCC

RP-250285	Views on Release-20 5G-Advanced NES WI	KT Corp.

RP-250309	Network Energy Savings (NES) - 5G-Advanced in Rel-20	Fraunhofer IIS, Fraunhofer HHI

RP-250346	Network energy saving enhancements	Tejas Network Limited

RP-250494	Network energy saving enhancements for 5G-Advanced in Rel-20	Huawei, HiSilicon

RP-250555	Views on further network energy saving enhancement in 5G-A Rel-20	ZTE Corporation, Sanechips

RP-250653	Views on Network Energy Saving for Rel. 20	CEWiT

## Summary of Network Energy Savings contributions

Wide range of proposals were proposed in the contributions in agenda item 15.2.7.2. Proposals with good level of support (at least 5 companies) are captured in the following table.

|  | Proposed objectives and supporting companies | Motivation and additional details |
| --- | --- | --- |
| 1 | Enhancements on SSB Support: CEWiT, ZTE, Tejas Networks, CMCC, Nokia, NEC, Huawei/HiSi, TIM, Orange, FutureWei, TCL, CAICT, Google, Fraunhofer IIS/HHI, KT | Common signals/channels transmission reduces the network sleep opportunity, especially in the case of empty/low load. Specifically, the following proposals were made under the broad objective of enhancements on SSB - On-demand SSB for UEs in idle/inactive mode - SSB periodicity extension - SSB adaptation within a burst - UE-initiated on-demand SSB |
| 2 | Enhancements for DTX/DRX operation Support: CEWiT, Fraunhofer IIS/HHI, Nokia, Huawei/HiSi, KT, TIM, Orange, FutureWei, TCL, CAICT, Google, Nokia | Expand the applicability of DTX/DRX operation to obtain further network energy saving. Specifically, the following proposals were made under the broad objective of enhancements on DTX/DRX - Extending Cell DTX/DRX operation to common signals - Cell DTX/DRX enhancements for RRC connected - Cell DTX/DRX extension for RRC idle/inactive |
| 3 | Enhancements on SIB1 Support: Fraunhofer IIS/HHI, Tejas Networks, ZTE, Huawei/HiSi, KT, CEWiT, TIM, Orange, FutureWei, TCL, CAICT, Google | Common signals/channels transmission reduces the network sleep opportunity, especially in the case of empty/low load. Specifically, the following proposals were made under the broad objective of enhancements on SIB1 - On-demand SIB1 for UEs in idle/inactive mode - SIB-less NES cell for UE in idle/inactive - Standalone On-demand SIB1 |
| 4 | On-demand CSI-RS/TRS Support: Huawei/HiSi, KT, CEWiT, TIM, Orange, FutureWei, TCL, CAICT, Google, Fraunhofer IIS/HHI, ZTE | In addition to the common signal/channel, periodic UE specific reference signals also reduce network sleep opportunity. Specifically, the following proposal was made by the proponents - Dynamic adaptation of UE specific signal/channel, e.g., CSI-RS/TRS adaptation |

17 companies were supportive of having some sort of NES enhancement in Rel-20 while 5 companies (in RP-250075 and RP-250151) explicitly indicated that they were not supportive of any kind of NES enhancement in Rel-20. The 5 companies who had negative views on Rel-20 NES reasoned that Rel-20 NES is not likely to be commercialized.

Based on the contributions submitted to agenda item 15.2.7.2, the following observations are made.

- Overall, the proposals on Rel-20 NES can be categorized into the following four areas

  - Enhancements on SSB (multiple proposals within this category)

  - Enhancements for DTX/DRX operation (multiple proposals within this category)

  - Enhancements on SIB1 (multiple proposals within this category)

  - On-demand CSI-RS/TRS

- There was good level of support for having some form of NES enhancement in Rel-20. However, 5 companies had negative views on any NES enhancement in Rel-20 due to very low likelihood of any of the proposed objectives being commercialized. Additionally, although the level of support is strong, the proposals are widely spread out.

## Moderator observations and recommendations

During the Thursday offline session, the moderator checked company views on the need for any NES enhancement on Release 20 5G-Adv

- Positive (NES enhancement in Release 20, 16 companies): ZTE, Telecom Italia, Huawei, CEWiT, ITRI, Futurewei, Google, China Telecom, Honor, KT, Nokia, NEC, Fraunhofer, Sharp, TCL, Tejas

- Negative (No NES enhancement in Release 20, 15 companies): LGE, IDC, Spreadtrum, Qualcomm, Panasonic, Xiaomi, Lenovo, Ericsson, Samsung, Fujitsu, ETRI, Apple, OPPO, AT&T, Intel

In RAN#107, there is no consensus on the support of NES enhancement in Release 20 5G-Adv.