3GPP TSG RAN Meeting #108	RP-250884
Prague, Czech Republic, June 9-13, 2025

Agenda item:	16.2

Source:	IITH, WiSig Networks

Title:	6G Requirements for 6.425 to 7.125 GHz frequency range

Document for:	Discussion

# 1	Introduction and Discussion

India plans to deploy 6G in the 6.425–7.125 GHz band, where operators could receive up to 200 MHz of contiguous spectrum. Supporting such wide allocations will require suitable sub-carrier spacing (SCS).

Because propagation loss in this band exceeds that of the current 5G mid-band, both downlink and uplink must compensate with higher antenna gain. In today’s mid-band deployments, a 26 dBm UE power class is typical; TDD duty-cycle relief allows per-TTI power boosting. With a low duty cycle—e.g., a single-symbol TTI—a UE should be able to raise its instantaneous transmit power to 30 dBm or more while keeping the long-term average within regulatory limits, subject to regional rules.

To close the uplink budget, the network should combine:

Larger antenna arrays at the BTS and, where feasible, at the UE

Higher UE transmit power that pushes the PA closer to saturation

Although coverage is the first priority, spectral-efficiency (SE) gains are equally important—especially for fixed-wireless access (FWA), where user density and throughput demands are high. FWA terminals are typically roof- or pole-mounted, so uplink coverage is less restrictive, but SE improvements are critical. As a design target, achieving a 2–3× SE increase over a 32-TRx massive-MIMO mid-band system would make the 6.425–7.125 GHz band highly attractive for FWA and other high-capacity use cases.

# 2	Proposals

Proposal 1

Mandate (uplink + downlink) in the 6.425–7.125 GHz band for all 6G deployments. DL only operation is optional.

Proposal 2 – Higher-Power UE Mode

Leverage low-duty-cycle TDD to permit instantaneous UE transmit powers of ≥ 30 dBm while keeping time-averaged emissions within regulatory limits.

Proposal 3 – Enhanced Antenna Arrays

Require scalable, higher-element antenna arrays at both BTS and UE to offset the additional path loss in this band and to maintain balanced downlink and uplink link budgets.

Proposal 4 – Spectrum Efficiency Targets

As a design target, achieving a 2–3× SE increase over a 32-TRx massive-MIMO mid-band system.