3GPP TSG RAN Meeting #108	RP-250886
Prague, Czech Republic, June 9-13, 2025

Agenda item:	16.2

Source:	IITH, WiSig Networks

Title:	Waveform Considerations for 6G

Document for:	Discussion

# 1	Introduction and Discussion

In 5G, both DFT-S-OFDM and CP-OFDM are mandatory for the user equipment (UE). Because DFT-S-OFDM has inherently low peak-to-average power ratio (PAPR), the power amplifier (PA) can operate closer to saturation—especially when π/2-BPSK with spectrum shaping is employed. Although successive 5G releases have exploited the additional headroom afforded by the low-PAPR waveform, the UE PA is still dimensioned for CP-OFDM, so the amplifier is not driven fully to saturation. A ground-up 6G design, particularly in new frequency bands, can remove this limitation.

5G also introduces phase-tracking reference signals (PT-RS) for DFT-S-OFDM, in which the reference signal is time-multiplexed with data before the DFT stage. PT-RS tracks intra-symbol frequency offset and Doppler and is predominantly used in FR2.

For uplink control channels, format 0 relies on sequence-based transmissions, whereas the short-duration format 2 employs OFDM waveform spanning one or two symbols. DFT-S-OFDM control channels, in contrast, cover the entire transmission time interval (TTI) and are not specified for short bursts.

At recent 6G workshop, several members recommended adopting DFT-S-OFDM and its variants. Building on PT-RS DFT-S-OFDM and π/2-BPSK with spectrum shaping, we propose a flexible uplink waveform that preserves low PAPR while enabling short-duration control or data bursts (down to a single symbol). By embedding a demodulation reference signal (DMRS) within every OFDM symbol, the waveform supports self-contained channel estimation, equalization, and robust operation under Doppler and frequency-offset impairments. These goals can be achieved through modest extensions of the current PT-RS DFT-S-OFDM framework, yielding significant gains in energy efficiency, transmit power, and coverage—especially in new spectrum.

For frequencies above 6.425 GHz, the 6G study item should evaluate time-multiplexing DMRS, data or control, and PT-RS before the DFT. A suitable spectrum-shaping filter, possibly with excess bandwidth, allows full channel estimation and equalization within a single symbol. Detailed analysis of this approach, termed Orthogonal Time-Frequency Division Multiplexing (OTFDM), is presented in arXiv:2409.01114. The study shows that in the 6.425-7.125 GHz band, per-symbol DMRS equalization can support user speeds up to 500 km/h.

Key benefits of the proposed waveform

- Single-symbol transmission enables one-symbol TTIs or single-symbol control, reducing latency and saving energy.

- Built-in resilience to Doppler shifts and frequency offsets.

- Low PAPR, especially with π/2-BPSK and spectrum shaping.

- Lower reference-signal overhead because DMRS can appear in any OFDM symbol.

- Full support for uplink MIMO with the requisite number of layers to raise data rates.

# 2	Proposals

Proposal 1

Adopt π/2-BPSK DFT-S-OFDM with spectrum shaping as the reference uplink waveform for new 6G frequency bands. Use this waveform to specify both the UE power-amplifier design targets and the modulation-specific maximum power reduction (MPR).

Proposal 2

In the 6G study item, evaluate a waveform in which DMRS, PT-RS, and data/control symbols are time-multiplexed prior to the DFT and shaped with modulation-dependent excess bandwidth. Apply this framework to uplink control and data channels so that the UE can fully leverage low PAPR, high Doppler resilience, reduced latency, and improved energy efficiency.