3GPP TSG-RAN Meeting #108	RP-250901

Prague, Czech Republic, 9th – 13th June 2025

Title: 	Other cross domain topics for Rel-20 5G-A

Agenda Item:	15.4.4

Source: 	Nokia

Document for:	Discussion

# 1				Introduction

In this contribution we discuss some cross domain topics for Rel-20 5G-A.

# 2	Discussion

Fragmented Carriers. If a WI on Fragmented Carriers is created in Rel-20, it should be clear from the discussions during the SI phase that TU allocations for both RF and RRM related work will be necessary, and the WI objectives need to be aligned to actual operator deployment scenarios. This implies that the fragmented carrier intra-band CA operation needs to be considered as part of inter-band CA configuration.

LB-LB CA with switching. It seems that this type of operation is a relatively limited case. If this were to be included in Rel-20, dynamic switching should be considered. If Rel-20 will include dynamic switching in addition to the semi-static switching defined in Rel-19, there will be additional RRM work which would need a dedicated TU allocation. If additional switching gaps are needed to support this feature, then their impact needs to be considered when the overall gain is assessed.

Irregular CBW. As agreed in RAN#106 [1], the 6 MHz channel bandwith was deferred to Release 20. The expected work is similar as for the ongoing WI on 7 MHz channel bandwidth (WI as started in RAN4#114 and is intended to be completed in RAN4#116) and it can be considered to be included for Release 20. It should be noted it was also agreed that no other (than 6 and 7 MHz) new irregular channel bandwidths will be created for FR1 in the future for 5G.

Non-collocated intraband CA/EN-DC UE type 3. Quite a lot of RAN4 effort has been put into developing UE type 2 and type 4 requirements. It is anticipated that work for type 3 UE requirements would be very controversial as BS TA requirement 3 us and needed MRTD/MTTD requirements of 260 ns clash and UEs would not work properly in all circumstances. Type 2 (2CC/layer) and type 4 (4CC/layer) did not have this contradiction as all CCs (and layers) have dedicated recievers whereas type 3 has shared recivers. Both type 3 and type 4 support 4CC/layers but from network operation and system performance point of view type 4 is superior compared to type 3 as it has separate recievers instead of shared recivers. Consequently we see it being challenging to reach any conclusions on type 3, and it seems unlikely to be a productive use of time in RAN4 in Rel-20.

# 3	Conclusion

In this contribution we have shared our views on potential Rel-20 cross domain topics. Out of the four topics with highest visibility, the 6 MHz channel bandwidth topic is the one that has clear objectives and could be progressed, while the others would need more discussion.

# 4	References

[1] RP-243322, Way Forward on Irregular Channel Bandwidths, T-Mobile USA