3GPP TSG RAN Meeting #108		RP- 250905
Prague, Czech Republic, 9th – 13th June, 2025

Agenda item:	15.4.3

Source:	Nokia

Title:	RRM topics for 5G-Advanced Rel-20

Document for:	Discussion

# 1	Introduction

In RAN plenary #107 companies proposed their initial views regarding possible content for the RAN4 Rel-20 package. [1] summarized the discussion the proposals made for RAN4 Rel-20 5G-Adavanced. In this paper we discuss, based on the outcome from RAN#107, our view on the possible RAN4 RRM Rel-20 content.

# 2	Discussion

## 2.1	RRM Enhancements

The conclusions drafted in the moderator summary [1] lists the possible candidate objectives for Rel-20 package, in a non-prioritized manner, where the blue highlight indicates a candidate with good common support among companies:

RRM enhancement

- RLM/BFD relaxation for RedCap UE

  - Consider to reuse Rel-17 solution as baseline

  - Consider both 1Rx and 2Rx.

- Fast SCell activation [and PSCell addition] enhancement

  - Consider the reduction of the delay of measurement and/or procedure

    - FFS on the detailed solutions

  - Whether to have this item depends on the detailed proposals of solution

    - One unified solution is expected

- Uplink only SCell (NES SSB-less SCell enhancement)

- FR2 multi-Tx and multi-Rx

- Enhanced RRM requirements for NES

- Enhanced RedCap with LTM

- R18 leftover on LTM RRM

- Interruption at NR SRS antenna port switching

- Parallel measurement

- Inter-frequency measurement enhancement for LTM

- On-demand deactivated SCell measurement

- A-TRS based RRM enhancement

- Dynamic RTD/TTD status update

- AL/ML beam prediction testing

- Handover interruption minimization

- Interruption-free DCI operation

- Extend OD-SSB to PSCell in NR-DC

In addition to the large number of proposals above, it must be assumed that during Rel-20 there will be incoming work from the other WGs related to the RAN1/2-led items. This workload should be estimated by RAN4 and in fact the workload should be discussed in RAN4 meeting such that there is a common understanding of the expected overall workload and estimated number of TUs.

Observation 1: The expected workload from other WGs Rel-20 WIs should be discussed to reach a common understanding on the amount of TUs needed in RAN4.

Such discussion and common understanding will help ensuring reasonable workload in RAN4 and help in deciding the number of TU’s are available for RAN4 RRM and hence, how much work should be agreed.

### 2.1.1	RLM/BFD relaxation for 1Rx and 2Rx RedCap UE

Considering the fact that UE power saving for RedCap devices is of importance, we believe that enabling RLM and BFD relaxation for RedCap devices is beneficial and important.

If Rel-20 will include this feature, RAN4 must consider both 1Rx and 2Rx RedCap devices. Additionally, a solution for RedCap devices must use the RLM and BFD relaxation which was defined for non-RedCap devices in Rel-17. Such assumption will ensure that the Rel-20 work load related to such WI can be limited. We assume that both 2Rx and 1Rx can be based directly on Rel-17 work while some simulations may be needed for the 1Rx Redcap device.

Observation 2: Create a WI addressing RLM/BFD relaxation for 1Rx and 2Rx RedCap UE’s in Rel-20.

Other work for (e)Redcap devices has been proposed as part of the Rel-20 package. This work has been proposed as part of the Cross RF and baseband topics.

Due to the synergies related to the two RedCap proposals in RAN4, we suggest defining one single WI including objectives related to RedCap, covering both proposals:

- RLM/BFD relaxation for RedCap UE

- 3MHz channel bandwidth for (e)Redcap devices.

Observation 3: Create one single RedCap WI including objectives for RLM/BFD relaxation and 3MHz channel bandwidth for (e)Redcap devices.

We discuss this in more detail in [3].

### 2.1.2	SCell activation enhancement for Intra-band continuous CA (Fast SCell activation)

Depending on whether there are TUs available for further RAN4 RRM Rel-20 work, RAN4 RRM could potentially work on at most one well-scoped WI in addition to the work on Redcap as discussed in section 2.1.1.

In Rel-19, RAN4 has worked on RRM_enh_ph5 where RAN4 have defined serving cell relaxation for serving cell measurement for intra-band contiguous CA. However, based on the current status in RAN4 it is still not decided whether Rel-19 will only address FR2 or whether also FR1 will be covered [2]. We see the feature attractive for both network and UE. Hence, if Rel-19 will not be able to define relaxed serving cell measurements for intra-band contiguous CA, we propose to continue this work in Rel-20 timeframe.

Observation 4: Define serving cell measurement relaxation for intra-band contiguous CA in FR1.

As RAN4 has already discussed the issue intensively during Rel-19, we believe the remaining discussion time needed would be minimal.

In addition to including FR1, the issue of the SCell activation time and CSI reporting delay for such intra-band contiguous SCell, which has not been measured prior to being activated, should be clarified in Rel-20.

The current total SCell activation delay is currently defined as (38.133, 8.3.2):

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where:

- THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

- Tactivation_time is the SCell activation delay in milliseconds.

- TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

The Tactivation_time depends on various conditions. One condition is whether the SCell being activated has been measured and reported. This condition cannot be fulfilled in the scenario under discussion where the UE is allowed to measure only one serving cell within the same band. To make the feature robust and deployment-friendly it needs to be clear what SCell activation delay the network can assume. Otherwise, there is a risk of having undefined UE SCell activation delay which would have significant negative impact on the success of the feature. Therefore, we propose to address this aspect in Rel-20.

Observation 5: Define UE SCell activation delay for intra-band CA by defining SCell Tactivation_time for a non-measured SCell.

Depending on the Rel-19 progress, Rel-20 should define such delay for FR1 and FR2.

Based on the scenario under discussion (intra-band contiguous CA) we suggest enhancing the total SCell activation delay by analysing the CSI reporting related to the SCell activation can be optimized. The current total SCell activation delay states when the the UE at latest shall be able to send a valid CSI report to the network. This is to ensure a maximum delay before network can assume having a valid CSI report to be used scheduling data on the SCell. However, as the scenario under discussion is contiguous SCells there may be possibility to enhance the delay related to TCSI_reporting.

Observation 6: Analyse SCell activation delay reduction for intra-band CA by optimizing TCSI_reporting for a non-measured SCell.

All work shall be limited to the particular scenario where the network has allowed UE relaxed measurements for intra-band contiguous serving cells.

Hence, we suggest a continuation of the RRM_enh_ph5 work on relaxed serving cell measurements for intra-band contiguous CA, including following two objectives:

Objectives:

- Define serving cell measurement relaxation for intra-band contiguous CA in FR1.

- SCell activation delay reduction for intra-band CA by defining SCell activation time (Tactivation_time) and optimizing the CSI reporting (TCSI_reporting) for a non-measured SCell.

Observation 7: Create a Rel-20 WI addressing one specific case of SCell activation enhancements, namely: UE requirements for a UE allowed to not perform serving cell measurements (continuation of Rel-19 work), which was left over from Rel-19 WI on serving cell measurement relaxation for intra-band contiguous CA.

Proposal 1 Objectives to be included are:

- Define SCell activation delay for intra-band contiguous CA by defining SCell activation time (Tactivation_time) and the CSI reporting (TCSI_reporting) for a non-measured SCell:

  - Define the requirements provided network allows serving cell measurement relaxation for intra-band contiguous CA using Rel-19 solution as baseline.

  - Include both FR1 and FR2.

# 4	Conclusion

In the paper we discussed, based on the outcome from 107 meeting and latest RAN4 status, our view on the possible RAN4 RRM Rel-20 content. Based on this discussion we propose that for RAN4 RRM at most two Rel-20 WI’s are agreed. In general, we propose:

Observation 1: The expected workload from other WGs Rel-20 WIs should be discussed to reach a common understanding on the amount of TUs needed in RAN4.

For Rel-20 RAN4 RRM WI’s we propose (based on the available TU’s):

RLM/BFD relaxation for 1Rx and 2Rx RedCap UE:

Observation 2: Create a WI addressing RLM/BFD relaxation for 1Rx and 2Rx RedCap UE’s in Rel-20.

Observation 3: Create one single RedCap WI including objectives for RLM/BFD relaxation and 3MHz channel bandwidth for (e)Redcap devices.

SCell activation enhancement for Intra-band continuous CA (Fast SCell activation):

Observation 4: Define serving cell measurement relaxation for intra-band contiguous CA in FR1.

Observation 5: Define UE SCell activation delay for intra-band CA by defining SCell Tactivation_time for a non-measured SCell.

Observation 6: Analyse SCell activation delay reduction for intra-band CA by optimizing TCSI_reporting for a non-measured SCell.

Observation 7: Create a Rel-20 WI addressing one specific case of SCell activation enhancements, namely: UE requirements for a UE allowed to not perform serving cell measurements (continuation of Rel-19 work), which was left over from Rel-19 WI on serving cell measurement relaxation for intra-band contiguous CA.

Proposal 1: Objectives to be included are:

- Define SCell activation delay for intra-band contiguous CA by defining SCell activation time (Tactivation_time) and the CSI reporting (TCSI_reporting) for a non-measured SCell:

  - Define the requirements provided network allows serving cell measurement relaxation for intra-band contiguous CA using Rel-19 solution as baseline.

  - Include both FR1 and FR2.

All work shall be limited to the particular scenario where network has allowed UE relaxed measurements for intra-band contiguous serving cells.

# References

- [1] RP-250817, Moderator's summary for RAN4 led REL-20 topics, RAN4 Chair (Huawei)

- [2] R4-2508264, WF on RRM requirements for NR_RRM_Ph5_Part1, Moderator (Apple)

- [3] RP-250907, “RedCap evolution in 5G-Advanced Rel-20”, Nokia