3GPP TSG-RAN4 Meeting #115	R4-2508734

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-2 | CR | 0037 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_repeaters-Perf) CR to TS 38.115-2: Correction of the OTA input intermodulation test requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_repeaters-Perf |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | OTA input intermodulation requirement for the repeater type 2-O is corrected to define explicit limit values (instead of "Core requirement plus TT"), and the MU/TT values are corrected (to replace erroneous FR1 numbers, with the reuse of OTA ACLR/OBUE TRP values). |  |  |  |  |  |  |  |  |
| Summary of change: |  | MU and TT values correction. Input intermodulation test requirement limits correction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Erroneous MU/TT values would remain. Input intermodulation test requirement limit would remain unspecified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2,2, 6.7, Annex C.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

#### 4.1.2.2	Radiated characteristics measurements

The maximum OTA Test System uncertainty for radiated characteristics measurements are given in tables 4.1.2.2-1. Details for derivation of OTA Test System uncertainty are given in corresponding clauses in TR 37.941 [13].

Table 4.1.2.2-1: Maximum Test System uncertainty for radiated characteristics tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2 Radiated transmit power (EIRP) | Normal condition: ±1.7 dB (24.25 – 29.5 GHz) ±2.0 dB (37 – 43.5 GHz) ±2.2 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
|  | Extreme condition: ±3.1 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.5 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.3 OTA repeater output power (TRP) | ±2.1 dB (24.25 – 29.5 GHz) ±2.4 dB (37 – 43.5 GHz) ±2.6 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.4 OTA frequency stability | ±1] Hz Measurement results of [5000] Hz |  |
| 6.5 OTA out of band gain | ±2.1 dB, 24.25GHz < f ≦ 29.5GHz ±2.4 dB, 37GHz < f ≦ 43,5GHz ±2.6 dB, 43.5GHz < f ≦ 48.2GHz |  |
| 6.6.2 OTA ACLR | Absolute ACLR: ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.6.3 OTA operating band unwanted emission | ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.6.4 OTA spurious emissions | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 60 GHz |  |
| 6.7 OTA EVM | 1.25% signal analyser 2% stimulus signal |  |
| 6.8 OTA input intermodulation | ±3.9 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.9 dB, 37 GHz < f ≤ 43.5 GHz ±5.4 dB, 43.5 GHz < f ≤ 48.2 GHz |  |
| 6.9 OTA ACRR | ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.10.1 OTA transmitter OFF power | ±2.9 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.6 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.10.2 OTA transient period | N/A |  |

------------------------------ Next modified section ------------------------------

## 6.7	OTA input intermodulation

### 6.7.1	Definition and applicability

#### 6.7.1.1	General

The input intermodulation is a measure of the capability of the repeater to inhibit the generation of interference in the passband, in the presence of interfering signals on frequencies other than the passband.

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the repeater to maintain the wanted frequency free of internally created interference.

The measurements shall apply to both uplink and downlink paths of the repeater, during the transmitter ON state at maximum gain.

#### 6.7.1.2	Minimum requirements

The minimum requirement is in TS 38.106 [2] clause 7.7.2.

#### 6.7.1.3	Test purpose

The purpose of this test is to verify that the repeater meets the intermodulation characteristics requirements as specified by the minimum requirements.

#### 6.7.1.4	Method of test

##### 6.7.1.4.1	Initial conditions

Test environment: normal; see Annex B

A measurement system set-up is shown in Annex E.

##### 6.7.1.4.2	Procedure

1a)	Verify measurement impact from feeding test signal by generating a signal for repeater input with repeater to be turned off.  Verify measured result is enough below requirement limit.

1b)	Set the repeater to maximum gain.

2)	Place the repeater with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in Annex E.

3)	Align the test antennas and repeater both in input and output directions of the repeater.

4)	Align the repeater and test antennas so that repeater antennas are polarization matched with the test antenna(s)

5)	Configure the beam peak direction of the repeater according to declared reference beam direction pair for the appropriate beam identifier.

6)	Adjust the frequency of the input signals, either below or above the passband, so that one carrier, f1, is 1 MHz outside the channel edge frequency of the first or last channel in the passband, and the lowest order intermodulation product from the two carriers is positioned in the centre of the passband.

7)	Measure the increase in output power in the passband when the interferer is applied

8)	Repeat the measurement for all supported polarizations and all specified measurement directions.

9)	Repeat the measurement for the opposite path of the repeater.

#### 6.7.1.5	Test requirements

For the parameters specified in table 6.7.1.5-1, the power in the passband shall not increase with more than:

- 13.9 dB, 24.25 GHz < f ≤ 29.5 GHz

- 13.9 dB, 37 GHz < f ≤ 43.5 GHz

- 15.4 dB, 43.5 GHz < f ≤ 48.2 GHz

at the output of the repeater as measured with 1 MHz measurement bandwidth, compared to the level obtained without interfering signals applied.

Table 6.7.1.5-1 specifies the parameters for two interfering signals, where:

-	f1 offset is the offset from the channel edge frequency of the first or last channel in the passband of the closer carrier.

-	GRX_ANT is the gain of the receive side antennas and is calculated from EIRP and TRP declaration.

Table 6.7.1.5-1: Input intermodulation requirement parameters

| f1 offset | Interfering signal levels | Type of signals | Measurement bandwidth |
| --- | --- | --- | --- |
| 1 MHz | -53dBm – G_RX_ANT | 2 CW carriers | 1 MHz |

------------------------------ Next modified section ------------------------------

Annex C (informative):
Test tolerances and derivation of test requirements

# C.1	Measurement of radiated characteristics

Table C.1-1: Derivation of test requirements (radiated tests)

| Test | Minimum requirement in TS 38.106 [2] | Test Tolerance (TT) | Test requirement in the present document |
| --- | --- | --- | --- |
| 6.2 Radiated transmit power (EIRP) | See TS 38.106 [2], clause 7.2 | Normal conditions: 1.7 dB, 24.25GHz < f ≦ 29.5GHz 2.0 dB, 37GHz < f ≦ 43.5GHz 2.2 dB, 43.5GHz < f ≦ 48.2GHz Extreme conditions: 3.1 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz 3.5 dB, 43.5GHz < f ≦ 48.2GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.3 OTA repeater output power (TRP) |  | 2.1 dB, 24.25GHz < f ≦ 29.5GHz 2.4 dB, 37GHz < f ≦ 43,5GHz 2.6 dB, 43.5GHz < f ≦ 48.2GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.4 OTA frequency stability | See TS 38.106 [2], clause 7.3 | 12 Hz | Formula: Frequency Error limit + TT |
| 6.5 OTA out of band gain | See TS 38.106 [2], clause 7.4 | ±2.1 dB, 24.25GHz < f ≦ 29.5GHz ±2.4 dB, 37GHz < f ≦ 43,5GHz ±2.6 dB, 43.5GHz < f ≦ 48.2GHz | Formula: Minimum Requirement + TT |
| 6.6.2 OTA ACLR | See TS 38.106 [2], clause 7.5.2 | Relative: 2.3 dB, 24.25GHz < f ≦ 29.5GHz 2.6 dB, 37GHz < f ≦43.5GHz 2.8 dB, 43.5GHz < f ≦ 48.2GHz Absolute: 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦43.5GHz 2.9 dB, 43.5GHz < f ≦48.2GHz | Formula: Relative limit - TT Absolute limit +TT |
| 6.6.3 OTA operating band unwanted emission | See TS 38.106 [2], clause 7.5.3 | 0 MHz f < 0.1*BWcontiguous 2.7 dB, 24.25GHz < f ≦ 29.5GHz 2.7 dB, 37GHz < f ≦43.5GHz 2.9 dB, 43.5GHz < f ≦48.2GHz 0.1*BWcontiguous f < fmax 0 dB | Formula: Minimum Requirement + TT |
| 6.6.4 OTA spurious emissions | See TS 38.106 [2], clause 7.5.4 | 0 dB | Formula: Minimum Requirement + TT |
| 6.7 OTA EVM | See TS 38.106 [2], clause 7.6 | 1.4%, Up to 16QAM 1.5%, 64QAM NOTE 2 1.8%, 256QAM Note 3 | Test requirement limit shifted by RSS of minimum requirement and stimulus signal EVM. Analyser error added to requirement limit. Formula: EVM limit + TT |
| 6.7 OTA input intermodulation | See TS 38.106 [2], clause 7.7 | ±3.9 dB, 24.25 GHz < f ≤ 29.5 GHz ±3.9 dB, 37 GHz < f ≤ 43.5 GHz ±5.4 dB, 43.5 GHz < f ≤ 48.2 GHz | Formula: Minimum Requirement + TT |
| 6.9 OTA ACRR | See TS 38.106 [2], clause 7.8 | ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) | Formula: Minimum Requirement – TT |
| 6.10.1 OTA transmitter OFF power | See TS 38.106 [2], clause 7.9.2 | 2.9 dB, 24.25GHz < f ≦ 29.5GHz 3.3 dB, 37GHz < f ≦ 43.5GHz 3.6 dB, 43.5GHz < f ≦ 48.2GHz | Formula: Minimum Requirement + TT |
| 6.10.2 OTA transient period | See TS 38.106 [2], clause 7.9.3 | N/A |  |
| NOTE 1: TT values are applicable for normal condition unless otherwise stated. NOTE 2: Support of 64QAM is based on the declaration NOTE 3: Support of 256QAM is based on the declaration. |  |  |  |

----------------------------- End of modified section -------------------------