3GPP TSG-RAN4 Meeting #115	R4-2507450

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 6107 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (LTE_terr_bcast_bands_part2-Core) CR to TS 36.101: terminology alignment for the "LTE based 5G terrestrial broadcast" feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_bands_part2-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It was observed, that there is terminology misalignment between multiple MS Word document parts of this specification. In couple of places the terrestrial broadcast feature is called as "5G terrestrial broadcast", which incorrect as this is 36-series specification. Those incorrect occurrences shall be re-named to "LTE based 5G terrestrial broadcast". |  |  |  |  |  |  |  |  |
| Summary of change: |  | Feature terminology alignments to remove inconsistencies. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature terminology mis-alignments would remain causing ambiguities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5H, 5.6H, 10.4, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

## 5.5H	Operating bands for LTE based 5G terrestrial broadcast

LTE based 5G terrestrial broadcast is designed to operate in the the operating bands defined in Table 5.5H-1.

Table 5.5H-1 LTE based 5G terrestrial broadcast operating bands

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 107 |  | N/A |  | 612 MHz | – | 652 MHz | SDO |
| 108 |  | N/A |  | 470 MHz | – | 698 MHz | SDO |

------------------------------ Next modified section ------------------------------

## 5.6H	PMCH bandwidth for LTE based 5G terrestrial broadcast

Requirements in the present document are specified for the bandwidths listed in Table 5.6H-1.

Table 5.6H-1: Transmission bandwidth configuration NRB for LTE based 5G terrestrial broadcast

| Transmission bandwidth configuration NRBPMCH | 30 | 35 | 40 |
| --- | --- | --- | --- |
| PMCH bandwidth [MHz] | 6 | 7 | 8 |

The LTE based 5G terrestrial broadcast network operates on 6, 7, and 8 MHz channels and the requirements in this specification apply according to configuration by the higher layer parameter pmch-Bandwidth (see TS 36.213 [6]) in the MBSFN area (see TS 36.331 [7]).

Note:  Upon configuration of the PMCH bandwidth, the UE is assumed to configure its baseband filtering to 10 MHz.  This assumption is not intended to restrict the UE implementation.

### 5.6H.1	PMCH bandwidths per operating band for LTE based 5G terrestrial broadcast

LTE based 5G terrestrial broadcast bandwidths and operating bands are shown in Table 5.6H.1-1

Table 5.6H.1-1: LTE based 5G terrestrial broadcast bandwidths per operating band

| Operating Band | PMCH bandwidth |  |  |
| --- | --- | --- | --- |
|  | 6 MHz | 7 MHz | 8 MHz |
| 107 | Yes | Yes | Yes |
| 108 | Yes | Yes | Yes |

------------------------------ Next modified section ------------------------------

## 10.4 FDD with LTE based 5G terrestrial broadcast

For all tests in section 10.4 the applicability rules are defined in Table 10.4-1 depending on the capabilities of the UE. The requirements in clause 10.4 are applicable to UEs that support LTE based 5G terrestrial broadcast.

Table 10.4-1: LTE based 5G terrestrial broadcast tests applicability

| Tests | Applicable if UE supports at least the following capability |
| --- | --- |
| Table 10.4.1.1-2 test 1 | LTE based 5G terrestrial broadcast support of mbms-ScalingFactor0dot37-r16 and timeSeparationSlot4-r16 |
| Table 10.4.1.1-2 test 2 | LTE based 5G terrestrial broadcast support of mbms-ScalingFactor0dot37-r16 and timeSeparationSlot2-r16 |
| Table 10.4.1.2-2 test 1 | LTE based 5G terrestrial broadcast support of mbms-ScalingFactor2dot5-r16 |
| Table 10.4.2.1-1 test 1 | LTE based 5G terrestrial broadcast support of mbms-ScalingFactor0dot37-r16 and LTE based 5G terrestrial broadcast support of PBCH repetition in CAS |
| Table 10.4.2.1-1 test 2 | LTE based 5G terrestrial broadcast support of mbms-ScalingFactor2dot5-r16 and LTE based 5G terrestrial broadcast support of PBCH repetition in CAS |

For the requirements defined in this section, the difference between CRS EPRE and the MBSFN RS EPRE should be set to 16dB for subcarrier spacing as 0.37kHz, 7.8 dB for subcarrier spacing as 2.5kHz because the UE demodulation performance might be different when this condition is not met (e.g. in scenarios where power offsets are present, such as scenarios when reserved cells are present).

------------------------------ Next modified section ------------------------------

Table A.3.8.1-9: Fixed Reference Channel for subcarrier spacing 0.37 kHz with LTE based 5G terrestrial broadcast MBMS dedicated cell

| Parameter | PMCH |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unit | Value |  |  |  |  |  |  |  |
| Reference channel |  | R.106-1 FDD | R.106-2 FDD | R.106-3 FDD | R.106-4 FDD | R.106-5 FDD | R.106-6 FDD | R.106-7 FDD | R.106-8 FDD |
| Channel bandwidth | MHz | 10 | 10 | 6 | 7 | 8 | 6 | 7 | 8 |
| Allocated resource blocks |  | 50 | 50 | 30 | 35 | 40 | 30 | 35 | 40 |
| Allocated slots per 40ms(Note1) |  | 13 | 13 | 13 | 13 | 13 | 13 | 13 | 13 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.48 | 0.52 | 0.49 | 0.48 | 0.48 | 0.54 | 0.53 | 0.52 |
| Information Bit Payload (Note 2) |  |  |  |  |  |  |  |  |  |
| For each slot | Bits | 63776 | 63776 | 39232 | 45532 | 51024 | 39232 | 45532 | 51024 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 11 | 11 | 7 | 8 | 9 | 7 | 8 | 9 |
| Binary Channel Bits Per Subframe |  |  |  |  |  |  |  |  |  |
| For each slot | Bits | 133650 | 121500 | 80190 | 93555 | 106920 | 72900 | 85050 | 97200 |
| MBMS UE Category |  | ≥ 2 | ≥ 2 | ≥ 2 | ≥ 2 | ≥ 2 | ≥ 2 | ≥ 2 | ≥ 2 |
| Note 1: First subframe of every 40ms is allocated for non-MBMS transmission. Note 2: Zero OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |  |  |

Table A.3.8.1-10: Fixed Reference Channel for subcarrier spacing 2.5 kHz with LTE based 5G terrestrial broadcast MBMS dedicated cell

| Parameter | PMCH |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unit | Value |  |  |  |  |  |
| Reference channel |  |  | R.107 FDD | R.107-1 FDD | R.107-2 FDD | R.107-3 FDD |  |
| Channel bandwidth | MHz |  | 10 | 6 | 7 | 8 |  |
| Allocated resource blocks |  |  | 50 | 30 | 35 | 40 |  |
| Allocated subframes per 40ms(Note1) |  |  | 39 | 39 | 39 | 39 |  |
| Modulation |  |  | 16QAM | 16QAM | 16QAM | 16QAM |  |
| Target Coding Rate |  |  | 0.46 | 0.46 | 0.46 | 0.46 |  |
| Information Bit Payload (Note 2) |  |  |  |  |  |  |  |
| For each subframes | Bits |  | 9912 | 5992 | 6968 | 7992 |  |
| Number of Code Blocks per Sub-Frame (Note 3) |  |  | 2 | 1 | 2 | 2 |  |
| Binary Channel Bits Per Subframe |  |  |  |  |  |  |  |
| For each subframes | Bits |  | 21600 | 12960 | 15120 | 172800 |  |
| MBMS UE Category |  |  | ≥ 2 | ≥ 2 | ≥ 2 | ≥ 2 |  |
| Note 1: First subframe of every 40ms is allocated for non-MBMS transmission. Note 2: Zero OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |

------------------------------ End of modified section -------------------------