3GPP TSG RAN Meeting #108			RP-250973

Prague, Czech Republic, June 9-13, 2025

Agenda item:	15.2.4

Title:	Discussion on normative working scope of AIML mobility in Rel-20

Source:	OPPO, ZTE, Ericsson, Interdigital, vivo, CAICT, China Telecom, Xiaomi, Samsung, China Unicom, CATT, Huawei, NTT Docomo

Document for:	Discussion

- Introduction

At RAN plenary#107 there is an extensive discussion on R20 5G-A package. As one of the important conclusions, AI mobility is listed as one of the potential WIs in Rel20. The summary from RAN Chair and WG Chairs [1] further capture potential working scope of AI Mobility listed as below:

After RAN#107 plenary meeting, RAN2/RAN4 further made progress in April RAN2#129bis/RAN4#114bis meeting and May RAN2#130/RAN4#115 meeting. And more specifically RAN2 focuses on spec impact aspects after concluding evaluation exercise based on simulation. RAN2 concluded majority important LCM procedures e.g. applicability procedure, inference configuration, performance monitoring for UE sided model. The progress for network sided model is mainly focusing on data collection since the spec impact on inference input is rather minor. The agreements achieved in two meetings are captured in the status report in [3].

This contribution further updates RAN2’s evaluation result to justify the motivation for normative work in Rel20 based on joint contribution [2]. Working scope is also updated based on latest RAN2/RAN4 progress and guidance from RAN#107 plenary. The initial version of potential normative working scope is listed in the Annex.

- Discussion

There are mainly 4 use cases in Rel19 study item on AI Mobility and RAN2 focus on RRM prediction and measurement event prediction. HOF and RLF prediction are down prioritized and hence not in the scope anymore.

Table 5.2.1-1: Prioritization of evaluation scenarios [4]

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

The RRM prediction use case is the core part of the study since other use cases are built on it basically e.g. the measurement event prediction with indirect methodology. Extensive simulations are conducted on high priority scenarios as indicated in Table 5.2.1-1 [4]. Before evaluating two goals of the study are captured as following in the TR [4]:

The use cases in this study focus on RRC_CONNECTED mode and cover RRM measurement prediction, measurement event prediction and RLF/HOF prediction for PCell change procedure in standalone NR scenario. The study of the use cases is driven mainly by two study goals. The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements. The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption).

The study on temporal domain case A is to verify the 2nd goal i.e. whether a proactive handover mechanism is feasible and if yes, how is the performance. For purely RRM prediction, RAN2 has observed that the performance i.e. the L3 RSRP prediction accuracy is around 1dB in average. And obviously AI algorithm outperforms non-AI algorithm e.g. sample and hold. The performance gain against non-AI increases in more challenging scenarios e.g. with higher UE speed and longer prediction window etc.

Figure 2-1 (Figure 5.2.2.1.3-1 in TR contribution [4])

Figure 2-1 illustrates the gain in terms of prediction accuracy by assuming 60Km/h with prediction window length between 40ms and 1600ms.

The further simulation on measurement event prediction based on temporal domain case A shows high F1 score, on top of which SLS indicates that HOF rate decreased quite many compared to legacy handover procedure.

Figure 2-2 CDF for HOF rate difference based on FR2 intra-frequency temporal domain case A

In Figure 2-2, both CDF curves show the HOF rate difference between AI and baseline (legacy L3 HO scheme). Note the negative value means the performance gain while positive value on the opposite. The handover model option 1 (blue curve) shows there is performance gain in terms of handover failure rate but not significant. Furthermore, majority companies show handover model option 2 (brown curve) to have much more promising performance of HOF rate.

Observation 1: The evaluation on intra-frequency temporal domain case A shows that proactive handovers are feasible with good performance gain.

The study on temporal domain case B is to verify the 1st goal i.e. whether the degradation of handover performance is acceptable while partial measurement is skipped. For purely RRM prediction, RAN2 has observed that prediction accuracy e.g. with 50% MRRT (measurement reduction rate in temporal domain) is less than 1 dB among majority company’s simulation result as illustrated in Figure 2-3:

Figure 2-3 CDF for FR1 intra-frequency temporal domain case B with sliding filtering

RAN2 also observed high F1 score for the measurement event prediction on top of temporal domain case B and concluded in relevant SLS that minor or even no handover performance degradation is observed. The detailed SLS simulation result can be found in Table 5.5.2.2-1 in [4]. RAN2 also confirmed that spatial domain prediction is feasible and hence will study its spec impact.

Observation 2: The evaluation on intra-frequency temporal domain case B shows that reducing measurement in temporal domain is feasible with acceptable or even no performance degradation.

The study on frequency domain prediction experience similar prediction accuracy situation and measurement event prediction. It means network could get measurement result of one frequency layer without any measurement if another frequency with correlation is measured and reported. And thus, the demand on measurement gap can be relaxed and user throughput can be improved. The detailed simulation result in Table 5.2.2.1.2-1 in [4] also shows that AI algorithm outperformed non-AI for both single cell and cluster approach.

Observation 3: The evaluation on inter-frequency domain shows that it is feasible to relax measurement gap configuration to improve user throughput and reduce measurement efforts.

RAN2 also studies generalization aspect of the AI mobility model for RRM measurement prediction and observed that the generalization issue is minor across UE speeds as shown in Table 5.2.2.2.1-1[4] (FR1 intra-frequency temporal domain case B), Table 5.2.2.2.3-1 [4] (FR2 intra-frequency temporal domain case A). The study on difference cell configuration also shows there is minor generalization issue since the predication accuracy loss is marginal as shown in Table 5.2.2.2.1-2 [4] (FR1 intra-frequency temporal domain case B) and Table 5.2.2.2.3-1[4] (FR2 intra-frequency temporal domain case A)

The inter-frequency prediction has generalization problems as depicted by the brown curve in Figure 2-4. However, the model trained with mixed dataset i.e. GC#2 (blue curve) bears close performance to baseline. It means generalization problems can be resolved in the proper way.

Figure 2-4 CDF for prediction accuracy loss of FR1 inter-frequency prediction

Observation 4: The evaluation on generalization aspects shows that model for AI mobility could be well generalized

RAN2 made some progress on spec impact on high level. First, RAN2 confirmed study scope as listed in following table:

| Scenarios | UE sided model | Network sided model |
| --- | --- | --- |
| Measurement event prediction | Yes, | No (up to NW implementation) |
| Temporal domain case A | Yes | Yes* |
| Temporal domain case B | Yes | Yes* |
| Frequency domain prediction | Yes | Yes* |
| Spatial domain prediction | Yes | Yes* |
| L3 beam level measurement prediction | Yes | Yes |

Table 2-study scope of spec impact (* sub case1 and sub case 3 are pending)

Some progress made based on the discussion on the scenarios listed in Table 2-1. Our understanding of the agreements could be summarized as follows:

- Only functionality-based LCM is considered for AI/ML mobility (i.e. model-based LCM is not supported)

- The general LCM framework for beam management can be the baseline (where applicable) for AI mobility

  - For UE-sided model, RAN2 concluded on applicability procedure, inference configuration, performance monitoring etc.

  - For network-sided model, RAN2 concluded on data collection. The spec impact on inference input is rather minor and no spec impact for performance monitoring is foreseen.

- RRM measurement configuration/report framework will be widely used as baseline for inference, report, data collection etc. procedures

- AI mobility will not touch FFS of AI/ML PHY, especially those controversial issues like Data transfer (of UE sided model) and model delivery

In the coming RAN2 meeting in August, left open issues will be discussed e.g. Measurement event prediction specific LCM procedure details and maybe also further scenario specific LCM details etc.

The key progress of performance requirement on RRM measurement prediction is concluded in RAN4. And the main open issue is the testability aspect. The endorsed RAN4 TP [5] is also incorporated in TR [4] for information to this plenary meeting.

Based on all collected information in this paper, the objectives in Annex reflects what could be potential working scope of normative work of AI mobility for NR.

- Conclusion

Observation 1: The evaluation on intra-frequency temporal domain case A shows that proactive handover is feasible with good performance gain.

Observation 2: The evaluation on intra-frequency temporal domain case B shows that reducing measurement in temporal domain is feasible with acceptable or even no performance degradation.

Observation 3: The evaluation on inter-frequency domain shows that it is feasible to relax measurement gap configuration to improve user throughput and reduce measurement efforts.

Observation 4: The evaluation on generalization aspects shows that model for AI mobility could be well generalized

Based on listed observations, we propose:

Proposal 1: Agree on working scope in Annex

- References

[1] RP-250812 Rel-20 5G-Advanced.pptx RAN Vice Chairs, RAN1, RAN2, RAN3, RAN4 Chairs

[2] RP-250167 RP-Discussion on normative working scope of AI mobility for NR, OPPO, ZTE, MediaTek, Nokia, CATT, CAICT

[3] RP-250970 Status report for SID AI mobility in NR

[4] RP-250971 TR 38.744 for AI Mobility in NR

[5] R4-2508419	TP on RAN4 aspects for TR 38.744

- Annex: Draft WID objectives

Objective of SI or Core part WI or Testing part WI

This WID focuses on AI/ML-assisted mobility in RRC_CONNECTED mode wherein RRM measurement prediction and measurement event prediction could be applied for e.g. L3 handover procedure and SCell addition or change. Only functionality-based LCM procedures are supported. In general, RRM measurement configuration and report framework in RRC layer will be used as baseline.

Specify support for RRM measurement prediction for both UE and network sided models [RAN2]:

- Scenarios including intra-frequency temporal domain case A and case B, inter-frequency prediction and spatial domain prediction are in the scope. Both FR1 and FR2 are applicable for all the listed scenarios.

- L3 Cell level and L3 beam level prediction are in the scope

- Specify signalling and protocol aspects to enable LCM functionality management for UE sided functionality including:

  - UE capability request and response procedure

  - Applicability report for both full and partial configuration approach (considering further progress for AI/ML PHY)

  - Inference configuration and report

  - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality

  - UE sided data collection, for which AI/ML PHY request/configuration framework is taken as baseline.

- Note1: Data transfer and model delivery for UE sided model are out of scope of this WID. And model related choices can be up to UE’s implementation

- Specify signalling and protocol aspects to enable LCM functionality management for network sided model including:

  - Network sided data collection, at least

    - UE is configured to log RRM measurement results.

    - UE supports periodical logging and L3 event triggered logging.

    - Logged data can be fetched once UE indicate availability due to predefined triggered

  - Support of multiple time instances of RRM measurement results per cell in one measurement report.

Specify support for measurement event prediction for UE sided models [RAN2]:

- Support same scenarios as RRM measurement prediction.

- In R20 only measurement event A1~A6 are in the scope

- Specify signalling and protocol for measurement event prediction specific aspect of LCM procedures including capability/applicability reporting, inference configuration and report, performance monitoring and UE sided data collection

For both RRM measurement prediction and measurement event prediction: (RAN4)

	Specify core requirement

	Specify LCM-related requirements, if any

Objective of Performance part WI

For both RRM measurement prediction and measurement event prediction: (RAN4)

	Specify RRM performance requirements and tests

	Specify necessary test cases and performance requirements for LCM procedure, if any

Note2: RAN2 related objectives reflect what is under study so far. In RAN#109 plenary meeting, RAN2 related objectives including their prioritization will be revisited based on RAN2’s further progress in August 2025

Note3: In RAN#109 plenary meeting, RAN4 related objectives will be revisited based on RAN4’s further progress in August 2025