3GPP TSG RAN    Meeting #108	RP-250984

Prague, Czechia, June 9-13, 2025

Agenda item:	16.2

Source:	Nokia

Title:	Considerations for 6G Working Group Study Item

Document for:	Endorsement

# 1	Introduction

TSG RAN#108 is to approve the 6G Study Item for the Working Groups. This contribution presents some key points to consider for the content of this Study Item.

# 2	Proposal for Objectives

It is proposed to capture at least the following points amongst the Objectives of the 6G Working Group Study Item:

General (A):

A1) The study shall target one non-backward compatible radio access technology to meet the 6G use cases and requirements as defined during the RAN requirements study.

A2) Frequency ranges to be addressed up to 52.6GHz. This shall incorporate the entire range of FR1 (including 6.425-7.125GHz), the range of 7.125 – 15.35GHz, and FR2-1 (24.25 GHz – 52.6GHz).

A3) Single stand-alone architecture to support the RAN requirements.

A4) Enhanced spectral efficiency.

A5) Enhanced overall coverage, focusing on cell-edge performance and UL coverage.

A6) Much improved energy efficiency and savings, both for network and device.

A7) Wider channel bandwidth (at least 200MHz, and including 400MHz) support for 6G deployments at least above 2 GHz.

A8) Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 6.425-7.125 and 7.125-8.4GHz, whilst achieving comparable coverage to 5G mid-band.

A9) Scalable and forward compatible design for diverse device types.

A10) System simplification, including reducing configuration complexity.

Physical Layer (B):

B1) Waveform (OFDM-based) and modulations, using 5G NR as a baseline.

B2) Compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS).

B3) Enable BS evolution and hardware reuse for network upgrade from 5G RAT to 6G RAT at least for the baseline 6G radio functionalities

B4) Channel coding: use LDPC and Polar Code as baseline, with LDPC used for data channels.

B5) Determine Channel Bandwidth: at least minimum  and maximum (at least 200 MHz but preferably up to 400 MHz), with support for flexible channel bandwidths (not only multiples of 5 MHz).

B6) Numerology: avoid multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range)

Radio Protocols (C):

C1) Single stand-alone streamlined, lean, modular and implementation-friendly radio protocol design enabling easy evolution of the existing services and introduction of new services

C2) Enable flexible uplink and downlink band pairing

C3) Utilize device type for simplifying UE capabilities and related signalling framework compared to 5G.

RAN architecture (D):

D1) RAN-CN functional split, interface, protocol stack and procedures

D2) RAN internal functional split, interfaces, protocol stacks and procedures

Migration and Mobility (E):

E1) 6G standalone system with 5G-6G Multi-RAT Spectrum Sharing as baseline for migration

E2) Study whether any additional aggregation mechanism is necessary, as triggered by TSG-RAN.

E3) Mobility between 5G NR RAT and 6G RAT.

6G Radio core and performance requirements (F):

F1) Intra-3GPP co-existence studies

F2) BS RF and UE RF requirement framework improvements and simplification

F3) Improve 6G BS core, performance, conformance specifications

F4) Improved 6G UE RF requirements considering different device types and implementations, including single carrier Tx/Rx requirements (including power classes and default power class, MPR and A-MPR framework and assumptions, REFSENS definition, ACS and blocking requirements, re-assessment of validity of requirements from 5G), CA configuration naming convention (i.e. CA_1C, CA_n1C, ?) and use of database to capture combinations into specification, necessity of current band combination specific requirements such as dTib, dRin and MSD, and testability aspects of RF and performance requirements.

F5) BS RF requirements, including Tx/Rx requirements for different BS classes, BS types and frequency ranges, and how to address introduction of 6G in MSR and AAS specifications.

F6) RRM improvements, including measurement framework targeting low latency and UE and network energy efficiency, measurements with and without gaps, and relation between measurements and RRC states; carrier activation and bandwidth adaptation aspects, targeting simplicity and low latency; simplified RRM specification structure.

F7) Demod requirements, including foundational assumptions for 6G demodulation requirements (with spatial channel models as the baseline for all MIMO requirements, focus on performance requirements rather than functional requirements, demodulation requirements not being limited by TxEVM assumptions, testability of L1 signals and broadcast channel, and need for test applicability rules and BS manufacturer declarations and non-signalled UE capabilities for demod requirements), study to what extent outer loop link adaptation shall be assumed for 6G requirements, and study to what extent system level throughput requirements shall be defined for 6G.

F8) Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity

F9) Handling channel bandwidths which are not multiple of 5MHz, and asymmetric DL/UL channel bandwidths.

F10) Definition of frequency range(s)

AI/ML enablers (G):

G1) AI/ML enablers for 6G Radio and Radio Access Network, leveraging 5G AI/ML framework as baseline for 6G AI/ML management, including life-cycle-management (LCM), data collection, testing, verification and validation), augmented for the new 6G use cases.

G2) As the first step of the study, identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

G3) AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

- LCM procedures, e.g.  configuration, (de-)activation, fallback mode, verification, testing, validation, etc…

- Data collection and data management, based on the 5G NR AI/ML data collection requirements.

Sensing (H):

H1) PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…)

H2) Evaluate sensing performance for the selected use case

H3) Associated architecture, higher layer procedures and protocol aspects

H4) RAN4 aspects of sensing including RF, coexistence, and testability.

# 3	Proposal for Interim Milestones

To drive swift progress of the study work it is important to set interim milestones and targets. We see the need for at least the following interim milestones and targets:

TSG#110 (December/2025)

Interim assessment on waveform, numerologies, modulation, channel coding for Communication: identify any extensions areas beyond 5G NR baseline.

TSG#112 (June/2026):

RAN1 to provide interim assessment on the following areas for communication:

-	Waveform, modulation, channel coding: scope of enhancements beyond NR baseline

-	Channel bandwidth (min and max), frame structure, numerology

-	Basic sync signal structure and associated periodicity(ies)

For objectives where RAN4 may be impacted, RAN1 shall coordinate with RAN4 early to enable the above assessment by June 2026.

RAN3 to provide interim study results to allow TSGs to make a decision on:

-	RAN-CN interface: P2P vs SBI

-	RAN internal functional split

RAN plenary to make a decision on additional 6G-6G aggregation beyond 6G CA: 6G-6G DC. RAN plenary will task relevant RAN WGs for any specific technical analysis, as needed.

NOTE: It is planned to decide on Release-21 timeline in June/2026.