3GPP TSG RAN Meeting #108	RP-251038

# Prague, Czech Republic, June 9-13, 2025

Agenda item:	16.2

Source:	Samsung

Title:	Proposed objectives for RAN WG 6G SID

Document for:	Discussion and endorsement

1	Introduction

3GPP 6G workshop facilitated presentations and discussions on overall vision, radio technologies, and system architectures of 6G. Taking into account the outcome of 3GPP 6G workshop, RAN#107 approved the revised SID “Study on 6G Scenarios and requirements” in RP-250810 to include more detailed objectives on 3GPP’s deployment scenarios, requirements, time plan of 6G Radio beyond IMT-2030 TPRs.

It is planned to approve RAN WG SI on 6G Radio in RAN#108. In this contribution, we propose a set of elements for inclusion in the objective section of RAN WG 6G SID to ensure coordination and consistency with the RAN plenary 6G study.

2	Discussion

2.1	Target frequency ranges

6G Radio should be developed to utilize all available frequency bands, including the ones that may be resulted from refarming from previous generations or identified from the on-going discussion toward WRC-27 as well as regional discussions. Sub-THz range doesn’t show the possibility of commercial deployments yet and hence should not be considered.

Hence, the study should address frequency ranges up to 52.6 GHz, including at least the full range of FR1 (up to 7.125 GHz), the range between FR1 and FR2-1 (including the bands around 7 GHz), and FR2-1 (24.25 – 52.6 GHz).

2.2	General design principle for 6G Radio

The study should aim to develop a single technology framework for 6G Radio to support both existing and new services, fulfilling the deployment scenarios and requirements identified in the RAN study on 6G scenarios and requirements. This framework should be built on a stand-alone architecture that ensures an acceptable performance and complexity trade-off.

6G Radio should be developed for the identified practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least the following.

- Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Such streamlined approach simplifies design and implementation, and ensures that the system remains manageable and cost-effective. For example, by standardizing key functionalities across diverse deployment scenarios, the framework reduces the need for bespoke solutions, thereby lowering development and operational costs. In addition, reducing configuration complexity and enabling more efficient management and implementation are key objectives to enhance operational efficiency and reduce deployment challenges.

- Energy efficiency and energy saving: both for network and device

  - Energy efficiency and energy saving for both network and device are prioritized to reduce operational costs, enhance sustainability, and to improve user satisfaction. This includes optimizing power consumption for base stations and user equipment (UE), leveraging energy-oriented initial access, designing advanced sleep modes, and implementing energy-efficient algorithms.

- Enhanced spectral efficiency

  - Enhanced spectral efficiency is targeted to maximize resource utilization and support high-capacity services. This involves optimizing modulation schemes, channel coding schemes (LDPC for data and Polar coding for control), and spectrum allocation strategies to ensure that available bandwidth is used effectively while improving data reliability, energy efficiency, and throughput.

- Enhanced overall coverage, focus on cell-edge performance and UL coverage

  - This is to ensure reliable connectivity in challenging environments. Techniques such as SBFD, advanced MIMO operations, and enhanced multi-carrier operations across different bands can improve signal strength in cell edge areas. UL coverage can be improved by enhancing power control mechanisms and optimizing resource allocation for UE transmissions.

- Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz

  - This is to enable higher data rates and capacity that would be essential for support of e.g., immersive communications and real-time services.

- Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

  - This ensures comparable coverage to 5G mid-band and reduces deployment costs by leveraging the extensive infrastructure of 5G, while minimizing the need for new site installations.

- Target scalable and forward compatible design for diverse device types

  - This includes a modular architecture that allows for easy integration of new technologies and features as they emerge. For example, the framework supports a range of device capabilities, from IoT devices (e.g., LPWA, (e)RedCap) to high-performance eMBB UEs like FWA devices, ensuring that the network can adapt to evolving use cases without requiring significant redesigns of hardware.

- Improved spectrum utilization and operations taking into account diverse spectrum allocations

  - This involves multi-RAT spectrum sharing mechanisms that enable 6G to coexist with 5G, as well as advanced implementations for supporting multiple band combinations, different duplex aggregation, and carrier aggregation with MIMO at base station and UE.

- Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs

  - This common design ensures that the network can support a wide range of applications without requiring separate solutions for each use case, thereby reducing complexity and enhancing interoperability.

- Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - This is to provide seamless global coverage and involves developing an integrated operation across TN and NTN.

- System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

  - This is to enhance operational efficiency and reduce deployment challenges, including streamlining network configurations, support for automating routine tasks, and realizing intelligent management systems that optimize resource allocation and network performance in real time.

2.3	Physical layer

While 6G offers an opportunity to have a clean slate design of physical layer, a pragmatic approach would be desirable to balance new designs with implementation complexity. Although reinventing the physical layer may unlock disruptive capabilities, any design must be weighed against real-world constraints in cost, complexity, and realistic deployment scenarios. Therefore, introducing necessary enhancements on top of the proven foundations of 5G such as numerology, frame structure, OFDM-based waveforms, LDPC and Polar coding, would be a desirable path forward. This approach would enable accelerated standardization and ecosystem readiness, while still achieving substantial gain.

- Waveforms. 5G NR waveforms (OFDM for downlink and uplink, DFT-s-OFDM for uplink) with necessary enhancements and should be considered.

  - This is to ensure a smooth transition from 5G to 6G with necessary enhancements such as DFT-s-OFDM with PAPR reduction technology for UL coverage improvement.

- Modulation. 5G NR modulation with necessary enhancements should be considered.

  - Non-uniform constellations can be considered for enhancement of performance and energy efficiency of high order modulation.

- Channel coding. LDPC with enhancements for data and Polar coding with enhancements for control should be considered.

  - This is to improve the energy efficiency of LDPC and the support of larger control information size for Polar coding.

- Frame structure with compatibility with 5G NR for efficient support of 5G-6G multi-RAT spectrum sharing.

- Numerology (SCS, CBW, etc.), avoiding multiple numerologies for the same band and the same subrange.

- Initial access including related signals and channels (e.g., synchronization signals, raster configurations, PBCH, PRACH) and procedures (e.g., random access, system information acquisition, paging)

  - Initial access mechanism should be optimized to prioritize energy efficiency and to ensure fast and reliable connection establishment.

- Physical layer control signalling (e.g., DCI/UCI, data scheduling, and HARQ operation).

  - It should be intended to simplify physical layer control mechanisms, feedback framework, data scheduling, and HARQ operations while enhancing throughput, latency, and reliability. Techniques for consideration include simplified timeline, simplified UCI multiplexing/dropping rules for overlapping channels, simplified HARQ-ACK codebooks compared to 5G NR, simplified BWP operation, and minimal formats.

- Intra-6G spectrum utilization and aggregation.

  - This is to address spectrum utilization and aggregation of multiple carriers in wide range of bands. This includes enabling flexible carrier aggregation mechanisms that support diverse deployment scenarios, supporting scalable way of multi-band combinations based on RF implementation with acceptable level of UE complexity.

- MIMO operation.

  - Although MIMO is an essential technology for 6G, increasing the number of antennas raises power consumption and complexity, thereby making an efficient and balanced design critical for 6G. MIMO in 6G should support up to 256 antenna ports, while still enabling scalable and energy-efficient operations. Non-AI-based MIMO uses conventional methods for CSI feedback and beam management, while AI-based one integrates machine learning (ML) to optimize CSI reconstruction, feedback compression, and beam management.

- Duplexing.

  - Exploring improvements in duplexing schemes should be considered for 6G Radio, such as SBFD in a single carrier and over multiple carriers (from the same or multiple bands) with possible extension to multiple TRPs (i.e., transmission and reception on different points), to enhance UL coverage and resource efficiency for at least C-band, upper 6 GHz band, FR2 and new FR3.

2.4	Radio interface protocol architecture and procedure

In the development of 6G communication systems, the protocol specifications must comprehensively support all the essential functionalities and features of earlier generations to support existing services. At the same time, they must introduce significant enhancements to address the growing demands of next-generation applications as well as the lessons learned from 5G. The study on radio interface protocol should include at the least the following.

- User Plane architecture and protocol.

  - The user plane protocol stack, comprising sub-layers such as Service Data Adaptation Protocol (SDAP), Packet Data Convergence Protocol (PDCP), Radio Link Control (RLC), and Medium Access Control (MAC), has evolved from 4G to 5G, supporting higher throughput and lower latency. However, there remain opportunities for further optimization to meet 6G’s aggressive performance targets. One key area of improvement is header format optimization. Current protocols often include variable-length headers that introduce processing delays, particularly in high-speed data transmission. Transitioning to fixed-length or more compact header structures can reduce overhead and enhance processing efficiency. Furthermore, cross-layer optimizations, where information is shared between protocol layers to improve decision-making, can lead to significant performance gains.

- Control Plane architecture and protocol.

  - The control plane in 4G and 5G has grown increasingly complex due to the incremental addition of features across multiple releases. While this evolution has enabled advanced capabilities like carrier aggregation, MIMO, etc., it has also led to bloated specifications that are difficult to interpret, implement, and test. For 6G, a fundamental redesign is necessary to streamline the control plane while maintaining backward compatibility where needed.

- Access stratum security aspects, in alignment with requirements from SA3.

  - As 6G networks would become more dynamic and distributed, the security of the access stratum (AS)—particularly Layer 2 signalling—becomes important. The increasing use of MAC Control Elements (MAC CEs) and broadcast system information (e.g., SIBs) introduces new attack vectors that can be exploited to disrupt network operations or compromise user privacy.

- UE capability signalling framework with improvements and simplification compared to 5G NR.

  - In designing 6G Radio, how to transfer UE capability remains a critical focus, particularly in terms of size optimization. In 5G NR, efforts to reduce UE capability were discussed under RACS (UE Radio Capability Signalling Optimization), but these did not lead to actual commercialization. Therefore, alternative optimization directions need to be explored. For instance, methods such as delta signalling based on the reference capability defined in standards could be considered to achieve this goal.

- Mobility, including related RRM.

  - Mobility management is a cornerstone of cellular networks, and while 5G has introduced numerous advanced mobility features—such as conditional handover, dual active protocol stack (DAPS), and L1/L2-triggered mobility (LTM)—only a subset of these features have been widely deployed in commercial networks. This discrepancy arises from the complexity of implementation and the marginal performance gains in real-world scenarios relative to the added overhead. For 6G, it is imperative to conduct a rigorous evaluation of existing mobility mechanisms to identify and retain only the most essential and practical features that provide substantial benefits, such as reduced latency and seamless connectivity. At the same time, 6G must push the boundaries of mobility management for improved reliability. This necessitates optimizing handover procedures, potentially leveraging predictive algorithms powered by artificial intelligence and machine learning (AI/ML) to anticipate user equipment (UE) movement patterns.

2.5	RAN architecture, interface protocols and procedures

The study on 6G RAN architecture, interface protocols and procedures should take the current 5G standard as the baseline and consider necessary enhancements to ensure smooth migration from 5G to 6G while meeting the requirements of 6G RAN. Its study area should include at least the following.

- Overall RAN architecture.

  - This requires careful consideration to support emerging technologies like AI-native operation and integrated terrestrial-satellite networks, necessitating close collaboration between RAN2 and RAN3 working groups to align air interface protocols with architectural aspects.

- RAN-CN functional split, interface, protocol and procedures.

  - RAN-CN functional split and interface protocols need to accommodate 6G's key requirements, where aspects like protocol stack refinement, signalling efficiency and data transmission reliability, high scalability and flexibility, AI/ML native design, energy efficiency and security-by-design principles become paramount.

  - The RAN-CN interface must ensure that latency levels are maintained at the same standard as those of previous-generation communication systems. This is essential for seamless connectivity and a consistent user experience across different network generations.

- RAN internal functional split, interfaces, protocol and procedures.

  - RAN functional partitioning and interfaces must be flexible, supporting CU-DU split and CP-UP separation that enable network optimizations while maintaining clear functional boundaries.

2.6	RF, RRM, and demodulation

The RAN4 study on RF, RRM and demodulation aspects is crucial to ensure the functionality and performance of 6G Radio deployments. The study area should include the following.

- System parameters including channel and synchronization rasters.

  - Restriction on system parameters should be considered depending on different frequency ranges and bands.

- Coexistence evaluation for the harmonized operation across existing and new frequency bands/ranges, and for new scenario(s), e.g., SBFD-SBFD system co-existence in the adjacent channel.

  - Frequency bands for the coexistence evaluation should include FR3 (and reassessment on upper 6 GHz if necessary).

- For BS RF aspects, study the Tx/Rx conducted/radiated requirements for different BS classes, BS types and frequency ranges.

- For UE RF aspects, study UE RF Tx/Rx requirements for single carrier and multi-carrier operation.

- For both BS and UE RF, study specification structure aiming at simplification of maintenance.

- RRM measurement procedure and performance

- RRM procedure and performance for efficient support of spectrum utilization and multi-carrier operation

- RRM specification structure aiming at simplification of maintenance

- Foundational assumptions for 6G demodulation requirements (inheriting the outcome of SCM study in 5G-Advanced if appropriate)

- Testability aspects aiming for conductive and/or OTA testing with reasonable complexity

2.7	5G-6G migration and interworking/mobility

Smooth transition from 5G to 6G should be ensured for successful introduction of 6G networks while not causing degradation of end use satisfaction with 5G networks. Deployment of 6G Radio in the existing 5G spectrum, i.e., multi-RAT spectrum sharing, is expected to be supported from the beginning of 6G. Considering that MRSS would need to change the existing 5G implementation and that it may not be possible to deploy 6G in the existing 5G band from 6G Day 1, additional mechanism would need to be considered. An example is dual stack, where the UE implements both 5G and 6G protocol stacks and simultaneously connects to both 5G RAN and 6G RAN without radio-level aggregation. Interworking/mobility between 5G NR and 6G Radio also needs to be studied.

Considering the above, the study area should include the following.

- 5G-6G Multi-RAT Spectrum Sharing for migration. [RAN1-led]

- Study on additional necessary migration mechanism. RAN will make decision on additional migration mechanism based on the study of RAN WGs, in coordination with SA2 (if necessary).  [RAN, RAN2, RAN1, RAN3, RAN4]

- Interworking/mobility between 5G NR and 6G radio. [RAN2-led]

2.8	AI/ML for 6G

The integration of AI/ML into 6G Radio is a critical enabler for achieving improvements in performance, efficiency, and adaptability across phys. Use cases for AI/ML span various layers such as MIMO, mobility, and transceiver enhancements. For example, AI/ML can enhance MIMO precoding by compressing channel information, thereby improving spectral efficiency. Similarly, in mobility management, AI/ML can optimize handover procedures, reducing latency and ensuring seamless connectivity.

It would be important to take a holistic approach in designing AI/ML support to ensure efficient and coordinated operations across all related layers, leveraging the 5G AI/ML framework wherever possible. A key aspect of advanced AI/ML framework is the model transfer allowing standardized AI models to be deployed without vendor-specific collaboration, ensuring interoperability. These models can be transferred between network and UE, for tasks such as CSI (Channel State Information) compression where explicit feedback is optimized using AI-trained encoders and/or codebooks (e.g., W1), to significantly reduce overhead and optimize site-specific performance. AI/ML models can be dynamically activated or deactivated based on network conditions, ensuring optimal performance without unnecessary resource consumption. In addition, data collection and management framework should be integrated in 6G RAN to collect required data and ensure accurate training of network-side model, thereby necessitating robust data pipelines that ensure data integrity, privacy, and security.

While AI/ML is a powerful tool for enhancing 6G networks, the design must ensure that the 6G system can operate effectively without AI/ML. This flexibility is critical for scenarios where AI/ML may not be feasible or necessary, e.g., in low-complexity deployments or regions with limited computational resources.

Considering the above, the AI/ML study for 6G should include the following.

- Identify and study use cases to be supported in 6G, taking into account the trade-off between performance, complexity, etc.

  - WGs relevant for the identified use cases should ensure coordinated discussion between AI-specific aspects and the application area (e.g., MIMO, mobility, transceiver enhancements).

- Extensible AI/ML framework based on the identified use cases, leveraging 5G AI/ML framework as appropriate, ensuring that 6G can operate without AI/ML.

  - LCM procedures, including configuration, (de-)activation, fallback mode, training, verification, testing, validation.

  - Data collection and management in coordination with SA WGs.

2.9	Integrated sensing and communication (ISAC)

The integration of sensing in 6G networks represents a new approach to network design, enabling the convergence of communication and sensing functionalities across all layers of the RAN. This integration is particularly important as ISAC technologies are poised to unlock new use cases and enhance network capabilities. To develop commercially meaningful technologies, the study on ISAC should focus on the following.

- Identify detailed requirements based on uses cases and associated requirements of TR 38.914. [RAN1]

  - These requirements must be tailored to specific use cases, such as autonomous driving, industrial automation, and smart cities.

- Study physical layer functions and procedures for sensing (e.g., waveform, reference signals, measurements, etc.) necessary to meet the identified requirements, including the aspects of integration with communications. [RAN1, RAN4]

  - For instance, waveforms optimized for both communication and sensing can enhance spectral efficiency while maintaining high performance. Reference signals must be designed to support accurate sensing measurements, enabling the network to detect and interpret environmental data effectively.

- Evaluate sensing performance [RAN1]

  - The sensing performance, e.g., sensing accuracy, must be rigorously evaluated for the selected use cases to ensure meeting the associated requirements.

- Radio interface and RAN architecture, protocol, and procedures necessary for support of sensing [RAN2, RAN3]

  - The design should support both sensing and communication functionalities across all layers of the RAN. This includes defining higher-layer procedures and protocols that enable efficient data processing, decision-making, and resource allocation.

- RF/performance requirements, including the study on coexistence of sensing and communications as necessary [RAN4]

  - For the integration of sensing in 6G Radio, the study on coexistence including spectrum management and interference mitigation should be performed in RAN4. This would ensure that ISAC can be effectively deployed in diverse environments, from urban areas to industrial sites.

2.10	Time plan and interim milestone

In relation to 6G study in RAN WGs, the following time plan was endorsed in RAN#103 (RP-240823).

- RAN1 starts in 3Q/25 until 1Q/27; RAN2/3/4 start in 4Q/25 until 2Q/27 - Rel-21 timeline is to be decided no later than June 2026

RAN plenary SID in RP-250810 has the following objective regarding the time plan for RAN WG 6G study.

- Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework

Defining a time plan could be helpful to draw early decisions on important topics so that 6G study in RAN WGs can proceed in a structured manner with timely decisions avoiding endless iterations. However, rushing toward agreements may result in suboptimal technical choices, which could degrade the quality of 6G standard. In order to have a good balance between the speed of progress and the technical quality of standard, a set of technical components for early decisions should be carefully selected. It would be reasonable to define an interim milestone for early decisions only for the features that have the critical impact on 6G product development process and/or discussion of the other features could have an interim milestone.

Considering the overall time of RAN WG 6G study (see above), it would be natural to take RAN#112 (June 2026) as the interim milestone for early decisions. We propose the following as the interim milestone.

- RAN WGs to provide their study results on additional necessary migration mechanism other than 5G-6G MRSS to RAN#112, aiming for making a decision in RAN#112.

- RAN1 to provide the following to RAN#112

  - Waveform, modulation, channel coding: agreed scope of enhancements beyond 5G NR baseline

  - Decisions on channel bandwidth, frame structure, numerology, synchronization signal structure

- RAN3 to provide the following to RAN#112

  - RAN-CN interface (e.g., P2P vs SBI)

  - RAN internal functional split and interface (e.g., CU-DU split, CP-UP split)

For the other features, we don’t see the need for defining an interim milestone as they would not be a bottleneck for early start of the 6G product development process. In case of AI/ML framework, while the AI/ML framework established in 5G offers a foundation, the unique demands of 6G, such as model transfer, online training, and real-time adaptability, necessitate the design of enhanced or entirely new framework. Research into the upgraded AI/ML framework will be critical to unlocking 6G’s full potential and must be treated as a core stream of the whole study phase.

3	Proposal

To ensure consistency with the RAN plenary 6G SID in RP-250810 and to define clear work scope for each WG as well as necessary interim milestones, we propose the following elements for the objective section of RAN WG 6G SID.

Element 1: target frequency ranges

The study should address frequency ranges up to 52.6 GHz, including at least the full range of FR1 (up to 7.125 GHz), the range between FR1 and FR2-1 (including the bands around 7 GHz), and FR2-1 (24.25 – 52.6 GHz).

Element 2: overarching objectives from RAN plenary 6G SID

- Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting

- Energy efficiency and energy saving: both for network and device

- Enhanced spectral efficiency

- Enhanced overall coverage, focus on cell-edge performance and UL coverage

- Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz

- Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

- Target scalable and forward compatible design for diverse device types

- Improved spectrum utilization and operations taking into account diverse spectrum allocations

- Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs

- Aim at a harmonized 6G Radio design for TN and NTN, including their integration

- System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Element 3: physical layer [RAN1-led]

- Waveforms. 5G NR waveforms (OFDM for downlink and uplink, DFT-s-OFDM for uplink) with necessary enhancements should be considered.

- Modulation. 5G NR modulation with necessary enhancements should be considered.

- Channel coding. LDPC with enhancements for data and Polar coding with enhancements for control should be considered.

- Frame structure with compatibility with 5G NR for efficient support of 5G-6G multi-RAT spectrum sharing.

- Numerology (SCS, CBW, etc.), avoiding multiple numerologies for the same band and the same subrange.

- Initial access including related signals and channels (e.g., PSC, SSC, PBCH, PRACH) and procedures (e.g., random access, system information acquisition, paging).

- Physical layer control signalling (e.g., DCI/UCI, data scheduling, and HARQ operation).

- Intra-6G spectrum utilization and aggregation.

- MIMO operation.

- Duplexing.

- Other physical layer signals, channels, and procedures.

Element 4: radio interface protocol architecture and procedure [RAN2-led]

- User Plane architecture and protocol.

- Control Plane architecture and protocol.

- Access stratum security aspects, in alignment with requirements from SA3.

- UE capability signalling framework with improvements and simplification compared to 5G NR.

- Mobility, including related RRM.

Element 5: RAN architecture, interface protocols and procedures [RAN3-led]

- Overall RAN architecture.

- RAN-CN functional split, interface, protocol and procedures.

- RAN internal functional split, interfaces, protocol and procedures.

Element 6: RF, RRM, and demodulation [RAN4-led]

- Study system parameters including channel and synchronization rasters.

- Coexistence evaluation for the harmonized operation across existing and new frequency bands/range, and for new scenario(s), e.g., SBFD-SBFD system co-existence in the adjacent channel.

- Study BS and UE RF aspects:

  - For BS RF aspects, study the Tx/Rx conducted/radiated requirements for different BS classes, BS types and frequency ranges.

  - For UE RF aspects, study UE RF Tx/Rx requirements for single carrier and multi-carrier operation.

  - For both BS and UE RF, study specification structure aiming at simplification of maintenance.

- Study RRM aspects:

  - RRM measurement procedure and performance

  - RRM procedure and performance for efficient support of spectrum utilization and multi-carrier operation

  - RRM specification structure aiming at simplification of maintenance

- Foundational assumptions for 6G demodulation requirements (inheriting the outcome of SCM study in 5G-Advanced if appropriate)

- Testability aspects aiming for conductive and/or OTA testing with reasonable complexity

Element 7: 5G-6G migration and interworking/mobility

- 5G-6G Multi-RAT Spectrum Sharing for migration. [RAN1-led]

- Study on the additional necessary migration mechanism. RAN will make decision on the additional migration mechanism based on the study of RAN WGs, in coordination with SA2 (if necessary). [RAN, RAN2, RAN1, RAN3, RAN4]

- Interworking/mobility between 5G NR and 6G radio. [RAN2-led]

Element 8: AI/ML for 6G [RAN1/2/3/4]

- Identify and study use cases to be supported in 6G, taking into account the trade-off between performance, complexity, etc.

  - WGs relevant for the identified use cases should ensure coordinated discussion between AI-specific aspects and the application area (e.g., MIMO, mobility, transceiver enhancements).

- Extensible AI/ML framework based on the identified use cases, leveraging 5G AI/ML framework as appropriate, ensuring that 6G can operate without AI/ML.

  - LCM procedures, e.g., configuration, (de-)activation, fallback mode, training, verification, testing, validation.

  - Data collection and management in coordination with SA WGs.

Element 9: Integrated sensing and communications (ISAC)

- Identify detailed requirements based on uses cases and associated requirements of TR 38.914. [RAN1]

- Study physical layer functions and procedures for sensing (e.g., waveform, reference signals, measurements, etc.) necessary to meet the identified requirements, including the aspects of integration with communications. [RAN1, RAN4]

- Evaluate sensing performance [RAN1]

- Radio interface and RAN architecture, protocol, and procedures necessary for support of sensing [RAN2, RAN3]

- RF/performance requirements, including the study on coexistence of sensing and communications as necessary [RAN4]

Element 10: Time plan and interim milestone

- RAN1 starts in 3Q/25 until 1Q/27; RAN2/3/4 start in 4Q/25 until 2Q/27

- RAN WGs to provide their study results on additional necessary migration mechanism other than 5G-6G MRSS to RAN#112, aiming for making a decision in RAN#112.

- RAN1 to provide the following to RAN#112

  - Waveform, modulation, channel coding: agreed scope of enhancements beyond 5G NR baseline

  - Decisions on channel bandwidth, frame structure, numerology, synchronization signal structure

- RAN3 to provide their study results on the following to RAN#112

  - RAN-CN interface (e.g., P2P vs SBI)

  - RAN internal functional split and interface (e.g., CU-DU split, CP-UP split)

4	Conclusion

We propose to agree inclusion of the proposed elements in section 3 in the objective section of RAN WG 6G SID.