3GPP TSG-RAN Meeting #108	RP-251044

Prague, Czech Republic, 9 – 13 June 2025

Agenda item:		15.2.2	IoT NTN (RAN2)

Title:		Views on NTN for IoT Phase 4

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

Following the Rel-20 5G-Advanced workshop at RAN#106, the endorsed RAN Chair’s Summary [1] identifies IMS Voice over GEO in IoT NTN for FDD as a candidate work item for Rel-20 5G-Advanced (see Annex). In RAN#107, the moderator summary for RAN2 led Rel-20 topics conclude the baseline for voice over NB-IoT-NTN via GEO enhancement in Rel-20[2].

Conclusions on IoT NTN The following enhancements will be considered as baseline for IoT NTN: - Voice over NB-IoT-NTN via GEO - Study phase for 1 or 2 Quarters to downselect between CP or UP by RAN#1xy (TBD) - Work on detailed objectives for RAN#108 ~max TU estimate for IoT NTN : RAN2 [1], RAN1 [0.5], RAN3 [?]

In this contribution, we further discuss the potential required work for voice over NB-IoT-NTN via GEO based on [3].

# 2. Discussion

2.1	Motivation

GEO satellites offer global coverage with minimal constellation complexity, enabling accelerated commercialization of 5G IoT NTN. NB-IoT devices, with their inherent ultra-coverage capabilities, are particularly suited for GEO scenarios with constrained link budgets. The low-cost NB-IoT devices and relatively easy GEO network construction make the NB-IoT NTN commercially attractive.

IMS Voice is both an attractive and important service for any telecommunication network. Supporting IMS Voice over GEO in IoT NTN would enable critical commercial use cases, particularly for emergency communications without available TN.

3GPP has been working on IoT NTN FDD since Rel-17. However, due to the NB-IoT standard restriction and limited link budget under GEO, IoT NTN IMS voice is not supported in Rel-17, 18, and 19.

Given Rel-20’s limited timeline, prioritizing emergency IMS Voice over GEO (i.e., voice communication between IoT NTN terminals and Public Safety Answering Point) is critical for early commercialization.

Proposal 1: The objectives of the voice over NB-IoT-NTN via GEO should prioritize supporting emergency calls.

2.2	Works on WGs and Cooperation with SA

To support IMS Voice over GEO, there are several challenges from the perspective of the RAN.

VoIP Packetisation Challenges

While the AMR codec is supported in IoT standards, its 20 ms packetization intervals (multiples of 20 ms) introduce frequent packet arrivals, increasing L1/L2 control overhead.

Observation 1: The frequent arrival/transmission of VoIP packets means a large control overhead for lower layers (L1/L2) in the radio protocol stack.

Latency Considerations

- Regenerative payload GEO: RTT ≥ 200 ms (depending on elevation, marked as green in the above figure)

- Transparent payload GEO: RTT ≥ 400 ms (depending on elevation, marked as orange in the above figure)

Observation 2: Despite the data transmission delay, the RTT of the schedule signaling impacts the IMS voice user experience. Efficient VoIP packet scheduling enhancement is necessary.

Type-B half-duplex FDD constraints

For NB-IoT NTN, only type-B half-duplex FDD operation is supported[3]. In this type of operation, guard periods (GP), each referred to as a half-duplex guard subframe, are created by the UE by

- not receiving a downlink subframe immediately preceding an uplink subframe from the same UE, and

- not receiving a downlink subframe immediately following an uplink subframe from the same UE.

In emergency communication, 1) using voice may not be the only effective method for ensuring accurate information exchange; 2) IMS voice service relaxation could help minimize potential collisions among the online parties.

Observation 3: Preventing UL and DL voice data collision on the terminal side requires both scheduling enhancements and IMS voice service relaxation (RAN1, RAN2, and SA2).

Observation 4: To ensure the integrity and effectiveness of the communication among the online parties, RAN requires SA's assistance on the IMS voice service layer enhancement.

Based on the discussions above, we consider the following objectives for voice over NB-IoT-NTN via GEO in Rel-20.

Proposal 2: Support RAN1 investigating potential constraints from the physical layer to support voice over NB-IoT-NTN via GEO (e.g., coverage, scheduling and etc.) after the study phase for downselecting between CP or UP.

Proposal 3: Support efficient scheduling enhancement (e.g., Semi-Persistent Scheduling, SPS) for NB-IoT for UL and DL VoIP packets (RAN1/RAN2).

Proposal 4: Support jointly resolving the UL and DL VoIP packet collisions among the online parties between RAN and SA (RAN1/RAN2/SA2).

# 3. Conclusion

In this contribution, we further discussed the potential required works for IoT NTN in RAN WGs and cooperation with SA and have the following observations and proposals.

Working Directions for voice over NB-IoT-NTN via GEO in Rel-20 5G-Advanced:

Proposal 1: The objectives of the voice over NB-IoT-NTN via GEO should prioritize supporting emergency calls.

Objectives for voice over NB-IoT-NTN via GEO in Rel-20 5G-Advanced:

Observation 1: The frequent arrival/transmission of VoIP packets means a large control overhead for lower layers (L1/L2) in the radio protocol stack.

Observation 2: Despite the data transmission delay, the RTT of the schedule signaling impacts the IMS voice user experience. Efficient VoIP packet scheduling enhancement is necessary.

Observation 3: Preventing UL and DL voice data collision on the terminal side requires both scheduling enhancements and IMS voice service relaxation (RAN1, RAN2, and SA2).

Observation 4: To ensure the integrity and effectiveness of the communication among the online parties, RAN requires SA's assistance on the IMS voice service layer enhancement.

Proposal 2: Support RAN1 investigating potential constraints from the physical layer to support voice over NB-IoT-NTN via GEO (e.g., coverage, scheduling and etc.) after the study phase for downselecting between CP or UP.

Proposal 3: Support efficient scheduling enhancement (e.g., Semi-Persistent Scheduling, SPS) for NB-IoT for UL and DL VoIP packets (RAN1/RAN2).

Proposal 4: Support jointly resolving the UL and DL VoIP packet collisions among the online parties between RAN and SA (RAN1/RAN2/SA2).

# References

[1] RP-243292, RAN Chair’s Summary of 5G-Advanced in Rel-20, RAN Chair

[2] RP-250803, Moderator's summary for RAN2 led REL-20 topics, RAN2 Chair (Interdigital)

[3] RP-250122, NTN for IoT Phase 4, NEC

# Annex for reference [1]