3GPP TSG-RAN Meeting #108	RP-251046

Prague, Czech Republic, 9 – 13 June 2025

Agenda Item:		15.3.1

Source:		NEC

Title:			Views on Rel-20 AI/ML for NG-RAN Phase 3

Document for:		Discussion and decision

# Introduction

In RAN#106 and RAN#107 meetings, some discussion has taken place on Rel-20 AI/ML for NG-RAN and the following WF was endorsed as in [1].

In this contribution, we further provide our views on Rel-20 AI/ML for NG-RAN study, including the detailed study scopes.

# Discussion

## 2.1	Justification

AI/ML is an advanced tool to help enhance network performance and maintain operation cost-effectiveness. For AI/ML for NG-RAN topic, in Rel-18 and Rel-19, RAN3 covered the following use cases for both non-split and split architecture:

- Load Balancing

- Energy Saving

- Mobility Optimization

- Network slicing

- Capacity and Coverage Optimization

A Rel-20 study item is needed to finalize Rel-19 leftovers. Besides, on the basis of the existing 5G network architecture, it is preferred to investigate more new AI/ML based use cases to enrich the 5G AI/ML feature.

## 2.2	Rel-20 priority

Considering Rel-20 is likely to be the last release for 5G, in this release, we should entirely finalize the ongoing specification tasks for 5G if possible. Therefore, for the Rel-20 study, we suggest to consider Rel-19 leftovers (i.e., continuous MDT collection as discussed in 2.3), which have already been studied in Rel-19 to some extent but still need further discussion on solution down selection, as 1st priority items, and then we can discuss whether or what new use cases can be included.

Proposal 1: For Rel-20 AI/ML for NG-RAN Phase 3, Rel-19 leftovers should be considered as 1st priority items.

## 2.3	SI objectives

Based on our contribution [2] [3] and also taking into account the abovementioned WF and the latest progress of the ongoing Rel-19 AI/ML for NG-RAN WI, below lists our preferred Rel-20 use cases in order of priority.

- Continuous MDT collection (normative work)

For Rel-19 AI/ML for NG-RAN WI, all the sub-topics progressed well until now. However, for continuous MDT collection, even though RAN3 already identified the potential solutions and coordinated with SA5 on it in the previous meeting, SA5 cannot finalize the solution without RAN3’s clarification on some details and thus one reply LS [4] was sent out to SA5 in last RAN3#128 meeting. Considering RAN3 may still need to discuss the RAN3 impact based on SA5’s reply, for example, how to keep measurement continuity after handover, if RAN3 cannot receive the SA5 reply LS in time and finalize RAN3 part in next RAN3#129 meeting, we propose to continue the normative work based on SA5’s reply in Rel-20.

Proposal 2: Continue the normative work for continuous MDT collection based on SA5 solution in Rel-20 SI.

- AI/ML based RVQoE prediction

5G QoE measurement collection is a mechanism targeting at collecting application layer measurements from UEs. QoE metrics include average throughput, initial playout delay, buffer level.

In order to optimize resource allocation per UE to improve user experience, QoE prediction by AI/ML model can be used in gNB for better resource reallocation. In case of not impacting legacy QoE mechanism, only the QoE metrics that are visible to NG-RAN can be considered as input data for AI/ML model in gNB, and therefore we suggest to focus on RVQoE prediction in this use case. The study can include e.g. RVQoE measurement collection for AI/ML model training and inference, future RVQoE prediction for specific UE(s), QoE prediction exchange over Xn interface for better handover decision or whole network optimization and actual RVQoE measurements feedback for model performance monitoring.

Proposal 3: Study on RVQoE predication is suggested to be conducted in Rel-20 SI.

- Finer granularity energy saving

Nowadays operators rank sustainability as the most important area for their network development. Energy saving is an important feature for operators contributing to operational cost savings and commitments to net zero. With the continuous growth of 5G builds, high energy consumption is an urgent issue to resolve. To address this issue, AI/ML is a powerful tool to help network achieve energy optimization.

In Rel-19 Enhancements for AI/ML for NG-RAN SI, a conclusion was drawn after study [3]:

“Enhancements to the node-level EC to a finer granularity could be beneficial, assuming that such enhancements enable network-level energy saving”.

However, due to the limited study time, it was difficult to reach a consensus on specific work tasks of this topic for the following specification development, and the ES topic was excluded from Rel-19.

Considering the high interest of this use case, in Rel-20, we suggest to continue the study on AI/ML based finer granularity energy saving, taking into account the discussion and achieved agreements in Rel-19. For example, several NES technologies have already been developed in previous releases and AI/ML can facilitate these mechanisms to achieve best performance. To be specific, talking about cell-level energy saving, cell DTX/DRX was introduced to facilitate reducing gNB transmission/reception activity time, and for this mechanism, using AI/ML, an optimum cell DTX and cell DRX pattern can be obtained based on the future resource needs.

Proposal 4: Study on AI/ML based finer granularity energy saving, including cell-level, is suggested to be continued in Rel-20 SI.

Based on the analysis above, the objectives of the new Rel-20 SI are summarized as below:

The aim is to complete Rel-19 leftover and investigate new AI/ML based use cases and identify enhancements to support more AI/ML functionality with existing NG-RAN interfaces and architecture. The detailed objectives are listed as follows: - Continue the normative work for continuous MDT collection based on SA5 solution - Study AI/ML based RVQoE - Study finer granularity energy saving, e.g. cell-level Note: RAN3 should take the Rel-18 and Rel-19 discussions into account. Note: Coordination with RAN2, SA2, SA5 if needed.

Proposal 5: RAN to approve a new Rel-20 SI on AI/ML for NG-RAN Phase 3 with objectives captured as above.

## 2.4	TU request

Regarding the requested TU, we support the 6-month SI and 12-month WI plan as in WF. And we think 1.5 TU is appropriate if, as proposed above, three sub-topics are included.

Proposal 6: For Rel-20 AI/ML for NG-RAN Phase 3, RAN to agree the 6-month SI and 12-month WI time plan with 1.5 TU allocated.

# Proposals

In this contribution, we shared our further considerations on Rel-20 AI/ML for NG-RAN study and the following proposals were drawn from the analysis.

Proposal 1: For Rel-20 AI/ML for NG-RAN Phase 3, Rel-19 leftover should be considered as 1st priority items.

Proposal 2: Continue the normative work for continuous MDT collection based on SA5 solution in Rel-20 SI.

Proposal 3: Study on RVQoE predication is suggested to be conducted in Rel-20 SI.

Proposal 4: Study on AI/ML based finer granularity energy saving, including cell-level, is suggested to be continued in Rel-20 SI.

Proposal 5: RAN to approve a new Rel-20 SI on AI/ML for NG-RAN Phase 3 with objectives captured as above.

The aim is to complete Rel-19 leftover and investigate new AI/ML based use cases and identify enhancements to support more AI/ML functionality with existing NG-RAN interfaces and architecture. The detailed objectives are listed as follows: - Continue the normative work for continuous MDT collection based on SA5 solution - Study AI/ML based RVQoE - Study finer granularity energy saving, e.g. cell-level Note: RAN3 should take the Rel-18 and Rel-19 discussions into account. Note: Coordination with RAN2, SA2, SA5 if needed.

Proposal 6: For Rel-20 AI/ML for NG-RAN Phase 3, RAN to agree the 6-month SI and 12-month WI time plan with 1.5 TU allocated.

# Reference

[1] RP-250812, Summary for RAN Release 20 5G-Adv

[2] RP‑242507, AIML for NG-RAN Phase 3, NEC

[3] RP-250123, Views on Rel-20 AI/ML for NG-RAN Phase 3, NEC

[4] R3-253958, Reply LS to LS on Continuous MDT, RAN3