3GPP TSG-RAN Meeting #108	RP-251047

Prague, Czech Republic, 9 – 13 June 2025

Agenda item:		15.3.2

Title:		View on Rel-20 ISAC study

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

During the RAN#107 meeting, the Integrated Sensing And Communication (ISAC) for NR was discussed and the following summary was achieved in the RP-250787 Moderator's summary for RAN3 led Rel-20 topics [1]:

The discussion on sensing in 5GA and 6G are independent. The objectives of Sensing/ISAC in Rel-20 are listed below: - Study on gNB mono-static mode for UAV use case. [RAN3, RAN1] - Study the procedures and signaling support between RAN and CN, and the corresponding sensing measurements. [RAN3, RAN1] - Performance evaluation for gNB mono-static sensing. [RAN1] Note: No new waveform/RS to be specified in RAN1 and no UE impact is expected. Note: Coordination with SA2 when needed. It’s assumed 9 months for the Rel-20 Study Item on Sensing/ISAC, followed by 9 months for the subsequent Work Item based on the conclusion of SI.

And the following statement is captured in the chair summary RP-250812 [2]:

The following are discussed and to be further checked in RAN#108: The discussion on sensing/ISAC in 5GA and 6G are independent. A set of possible objectives of Sensing/ISAC in Rel-20 are listed below: - Study on gNB mono-static mode and possibly bi-static mode for UAV use case. [RAN3, RAN1] - Study the procedures and signaling support between RAN and CN, and the corresponding sensing measurements. [RAN3, RAN1] - Performance evaluation for gNB sensing. [RAN1] - Note: No new waveform/RS to be specified in RAN1 and no UE impact is expected. - Note: Coordination with SA2 when needed.

This contribution would like to discuss further on Rel-20 ISAC based on the progresses above.

# 2. Discussion

2.1 The gNB sensing mode for UAV

The demonstration structures of the gNB mono-static and bi-static sensing modes are shown in Figures 1 and 2. In the mono-static mode, the gNB both transmits the sensing signal and receives the reflected signal, whereas in the bi-static mode, gNB1 transmits the sensing signal while gNB2 receives the reflected signal.

Figure 1: Demonstration structure of gNB mono-static sensing mode

Figure 2: Demonstration structure of gNB bi-static sensing mode

In the RAN#107 meeting, the gNB mono-static mode was more supported than the gNB bi-static sensing mode. Some companies think, compared to the bi-static mode, the mono-static sensing mode has no sensing resource coordination required between gNBs.

However, real-time sensing signal transmission and reception by the gNB itself has high requirements on the gNB's transmitter/receiver (e.g., full-duplex capability). Additionally, even in mono-static mode, sensing resource exchange between gNBs remains essential for interference management. Thus, the bi-static sensing mode can offer greater deployment flexibility without introducing significant additional standard impact or workload compared to the mono-static mode.

Observation 1: The mono-static sensing mode requires full-duplex capability for the gNB, and information exchange between gNBs is still needed for interference management. In contrast , the bi-static sensing mode provides more deployment flexibility without providing significant additional standard impact and workload compared to the mono-static sensing mode.

Proposal 1: It is proposed that both the mono-static and bi-static sensing mode should be included in the 5G-Adv study.

2.2	Architecture, sensing measurement signalling and procedures

Network architecture to support ISAC

There are two potential architectural options to support the ISAC service:

- Option 1: The RAN node are controlled by the 5GC, with sensing data transmitted to the application server via the 5GC.

- Option 2: RAN sensing nodes are controlled by OAM, with sensing data sent to the application server via OAM, bypassing the 5GC.

Figure 3: Option 1: via 5G Core Network

Figure 4: Option 2: via OAM and bypass 5G Core Network

Option 1 may introduce standardization impacts on the 5GC and the 5GC-RAN interface, whereas Option 2 is more related with interaction between RAN and OAM which has minimal impact on RAN3 and no impact to Core Network.

Proposal 2: The ISAC 5G-Adv study should evaluate the following two architectural options for sensing control and data collection:

- Option 1: RAN sensing nodes are controlled by the 5GC, with sensing data transmitted to the application server via the 5GC.

- Option 2: RAN sensing nodes are controlled by OAM, with sensing data sent to the application server via OAM, bypassing the 5GC.

The necessary signaling to support ISAC

For both Option 1 and Option 2, firstly, the exchange of sensing assistance information between the 5GC/OAM and RAN node is required to enable the sensing process. In addition, if the sensing service is triggered by the 5GC/OAM, a sensing measurement request message sent from the 5GC/OAM to the RAN node is also required. Furthermore, in response to either the sensing measurement request or RAN node initiated sensing measurement, the RAN node shall report the collected sensing data to the 5GC/OAM by the measurement reporting message.

Proposal 3: For both Option 1&2, the sensing measurement signaling may include:

- Sensing assistance information exchange between the 5GC/OAM and RAN

- Sensing measurement request message from the 5GC/OAM to the RAN node

- Sensing data reporting message from the RAN node to the 5GC/OAM.

2.3	Sensing measurements and performance evaluation

In RAN#107, it was suggested to study the necessary sensing measurements in order to ensure the ISAC framework works as expected from operator perspective. We assume this can be done in RAN1, along with the architecture and signalling considered in RAN3.

On the other hand, for this performance evaluation for gNB sensing, unlike the legacy UE or the gNB requirements, the sensing itself will not directly impact the communication at least within the potential scope of Rel-20. Having understood its need as a study, it would be necessary for RANP to give a clear guidance for that before starting the SI in WGs.

Proposal 4: RAN should give a clear guidance on sensing measurements and performance evaluation for gNB sensing to WGs.

2.4	Time line

In the previous meeting, multiple companies supported allocating a 9-month study period followed by 9 months of normative work for 5G-Adv ISAC in Rel-20 with 1 TU. We believe this 9-month study period with 1 TU is an appropriate timeframe, enabling RAN to properly evaluate whether the study outcomes are sufficient to transition to normative work within Rel-20's remaining timeline.

Proposal 5: It is proposed to allocate:

- A 9-month study period with 1 TU for 5G-Adv ISAC in Rel-20

- RAN should evaluate the study outcomes after the 9 months study to determine if they sufficiently support transitioning to normative work within Rel-20's remaining timeframe.

# 3. Conclusion

Observation 1: The mono-static sensing mode requires full-duplex capability for the gNB, and information exchange between gNBs is still needed for interference management. In contrast , the bi-static sensing mode provides more deployment flexibility without providing significant additional standard impact and workload compared to the mono-static sensing mode.

Proposal 1: It is proposed that both the mono-static and bi-static sensing mode should be included in the 5G-Adv study.

Proposal 2: The ISAC 5G-Adv study should evaluate the following two architectural options for sensing control and data collection:

- Option 1: RAN sensing nodes are controlled by the 5GC, with sensing data transmitted to the application server via the 5GC.

- Option 2: RAN sensing nodes are controlled by OAM, with sensing data sent to the application server via OAM, bypassing the 5GC.

Proposal 3: If Option 1 is adopted, the sensing measurement signaling may include :

- Sensing assistance information exchange between the 5GC/OAM and RAN

- Sensing measurement request message from the 5GC/OAM to the RAN node

- Sensing data reporting message from the RAN node to the 5GC/OAM.

Proposal 4: RAN should give a clear guidance on sensing measurements and performance evaluation for gNB sensing to WGs.

Proposal 5: It is proposed to allocate:

- A 9-month study period with 1 TU for 5G-Adv ISAC in Rel-20

- RAN should evaluate the study outcomes after the 9 months study to determine if they sufficiently support transitioning to normative work within Rel-20's remaining timeframe.

# References

[1] RAN3-led: RP-250787, Moderator's summary for RAN3 led REL-20 topics, RAN3 Chair (ZTE)

[2] RP-250812, Summary for RAN Release 20 5G-Adv