3GPP TSG RAN Meeting #108	RP-251051
Prague, Czech Republic, June 9-13, 2025

Agenda Item:	15.3.3

Source:	Ericsson

Title:	Discussion on Rel-20 Data Collection for SON/MDT

Document for:	Discussion, Decision

# 1	Introduction

The objectives of SON/MDT in Rel-20 including and to be down-selected on the following (1 or 2 use cases):

- MRO enhancement for R19 LTM (the detail scope should be limited and needs to be futher checked based on the progress in R19 LTM). [RAN3, RAN2]

- Support of SON/MDT enhancements for UAV or LP-WUS/WUR [RAN3, RAN2]

Note:  Coordination with other working groups when needed.

SON features were first introduced in LTE to support self-configuration and self-optimization of the network. In NR, SON/MDT was introduced in Rel-16, and has in the following releases been enhanced to support more use cases and accommodate new features in the network. Also, in Rel-20, we see the need for enhancements of SON/MDT to match new features introduced in Rel-19. In RAN#107, the following was agreed on a Rel-20 SON/MDT Work Item:

In the following we discuss our view on what to focus on the Work Item.

# 2	Discussion

## 2.1	MRO Enhancements for Rel-19 Mobility

SON/MDT includes support for Mobility Robustness Optimization (MRO), which has over the NR development been enhanced to follow the development of NR mobility. The MRO solutions typically focus on reporting of Radio Link Failures (RLF) related to mobility and Near Failures in order to optimize mobility configurations. Based on RLF reports, failures can be classified into too late handover, too early handover, and handover to wrong cell. Sometimes also the (UE collected) Mobility History Information (MHI) and the (network collected) UE History information (UHI) tracking the history of cells the UE has visited is considered as MRO features.

In release-19, NR mobility enhancements Phase 4 [1] is introduced. The objective of the work item includes

- Inter-CU LTM support

- Measurement related enhancements supporting LTM

- Conditional LTM

With the enhancements of the mobility support, the MRO framework needs to be updated to also account for the enhanced mobility features, as well as to ensure that legacy MRO solutions are not in the risk of breaking, for example by “pollution” of the statistics from UEs with other prerequisites due to usage of the enhanced mobility solutions. Besides, Rel-19 MRO only focused on MCG LTM. In Rel-20, MRO for LTM in NR-DC can also be considered.

In the possible objectives agreed in RAN#107, the scope of MRO enhancements for R19 LTM is given with the disclaimer that the detailed scope should be limited and needs to be further checked based on the progress in R19 LTM. Based on the discussions at the last RAN plenary meeting, it appears that the majority of companies are interested in MRO for inter-CU LTM, while opinions remain divided on the other topics.

By enabling inter-CU LTM, the network will be able to gain the advantages of LTM for a far greater number of handovers, which makes inter-CU LTM as one of the most attractive features. Currently, MRO only supports intra-CU LTM, which may significantly limit its potential for optimizing LTM operation. In order for the network to maximize the benefits of MRO for LTM, MRO enhancement for inter-CU LTM must be considered.

Conditional LTM enables shorter interruptions due to early DL/UL synchronization and RACH-less access, and it also increases robustness and accelerates the triggering of cell switch execution compared to normal LTM. With MRO enhancement for CLTM, the failure and near failure CLTM can be detected and the related configurations and procedures regarding, e.g. the execution conditions, early UL/DL sync, can be optimized to fully leverage the benefits of conditional LTM. Therefore, we propose to incorporate MRO for conditional LTM into the scope of Rel-20.

LTM with dual connectivity in Rel-19 has some limitations, e.g. the case that inter-CU LTM is configured in both MCG and SCG is not supported. Moreover, RAN#107 has decided not to support conditional LTM with dual connectivity. Considering these limitations on LTM with dual connectivity and the limited time, we think that the MRO for LTM with dual connectivity should have a lower priority.

Include MRO enhancement for inter-CU LTM and conditional LTM in Rel-20.

## 2.2	SON/MDT for Wake-Up Signal Observability

In Release-18, a study [2] was performed on low-power wake-up signal and receiver for NR, which in Rel-19 is followed by a work item [3] specifying the wake-up signal for both RRC Idle/Inactive and RRC connected states. The wake-up receiver (WUR) is a low power receiver in UEs that can detect a wake-up signal (WUS) and, if needed, wake up the main receiver (MR) to detect an incoming message, typically a paging message.

MDT logging and reporting, performed by the UE, is used to observe the network to understand coverage and performance. With the introduction of WUS and WUR, MDT needs to be updated both to provide WUS observability, and to ensure that the legacy MDT solution is not breaking by “pollution” of the statistics from UEs using WUS. The wake-up receiver and the main receiver have different characteristics, and to understand the measurements collected by MDT it needs to be clear whether the UE has measured using the wake-up receiver or the main receiver. This is not possible to be understood using the existing MDT framework.

Further, one usage of MDT measurements is optimization aiming at avoiding or decreasing coverage holes. Since the wake-up receiver link performance can be expected to be worse than the legacy main receiver performance (Rel-19 WUR work item description states the LP-WUS reception should match Msg3 PUSCH performance, which is several dB worse than paging PDCCH performance), the coverage of DL WUS needs careful consideration. Hence, it is beneficial to enable the network to configure MDT measurements for the wake-up signal.

In addition, Rel-19 “Enhancements of network energy savings for NR” WI [4] considers on-demand (OD) SIB1 acquisition. The purpose of this objective is to help the network to conserve energy by avoiding SIB1 transmission in certain cells instead of, as in legacy, periodically transmitting SIB1 in all NR cells. Rather, SIB1 is to be transmitted on-demand in certain cells when a UE needs it. If a UE needs SIB1, it will transmit an UL wake-up signal (WUS) based on the configuration received from an assisting cell which, in turn, triggers the OD-SIB1 transmission by the network.

Such coordination between the NES cells (i.e., cells with OD-SIB1) and the assisting cells which broadcast the UL WUS configuration requires, at least, alignment in terms of coverage as otherwise the mismatch of coverage between such cells leads to sub-optimal performance in the OD-SIB1 acquisition.

Include SON/MDT for WUS observability as an objective for SON/MDT Rel-20.

# 3	Proposal

Based on the discussion in the previous sections we propose the following:

Proposal 1	Include MRO enhancement for inter-CU LTM and conditional LTM in Rel-20.

Proposal 2	Include SON/MDT for WUS observability as an objective for SON/MDT Rel-20.

# 4	References

RP-250339, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN Meeting #107, Mar 12-14, 2025

RP-232672, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, vivo, 3GPP TSG RAN Meeting #101, Sep 11-15, 2023

RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, 3GPP TSG RAN Meeting #105, Sep 9-12, 2024

RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson, 3GPP TSG RAN Meeting #105, Sep 9-12, 2024