3GPP TSG-RAN5 Meeting #107	R5-252010

# St Julian’s, Malta, 19 – 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1924 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to 6.2E V2X test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case of inter-band V2X MOP is not completed. The minimum requirement is updated as apply to each operating band, and no exception applies. There is no need to repeat the test if V2X and FR1 band are tested. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Aligning the minimum requirement. Completing the inter-band V2X test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.1.0.3, 6.2E.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 1010 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2E	Transmitter power for V2X in FR1

6.2E.1	UE Maximum Output Power for V2X

6.2E.1.0	Minimum conformance requirements

6.2E.1.0.1	UE maximum output power for Intra-band contiguous V2X

For intra-band contiguous V2X operating UE, the allowed UE maximum output power shall be applied in Table 6.2.2-1 [5] for E-UTRA SL transmission or applied in Table 6.2.1-1 [2] for NR SL transmission, respectively.

Table 6.2E.1.0.1-1: Maximum output power for V2X combination (continuous sub-blocks)

| V2X configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| V2X_(n)47AA |  |  | 23 | +2/-31 |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power Class 3 is the default power class unless otherwise stated. NOTE 3: Only single switched UL is supported |  |  |  |  |

6.2E.1.0.2	UE maximum output power for Intra-band non-contiguous V2X

For intra-band non-contiguous V2X operating UE, the allowed UE maximum output power shall be applied in Table 6.2.2-1 [5] for E-UTRA SL transmission or applied in Table 6.2.1-1 [2] for NR SL transmission, respectively.

Table 6.2E.1.0.2-1: Maximum output power for V2X combination (non-contiguous sub-blocks)

| V2X configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| V2X_47A_n47A |  |  | 23 | +2/-31 |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power Class 3 is the default power class unless otherwise stated. NOTE 3: Only single switched UL is supported |  |  |  |  |

6.2E.1.0.3	UE maximum output power for Inter-band V2X

For the NR V2X inter-band concurrent operation, the maximum output power is specified in Table 6.2E.1.0.3-1 for each operating band. The period of measurement shall be at least one sub frame (1ms).

Table 6.2E.1.0.3-1: Con-current V2X UE Power Class

| V2X con-current operating band Configuration | NR or E-UTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_20A_n38A | 20 |  |  |  |  | 23 | ±24 |  |  |
|  | n38 |  |  |  |  | 23 | ±2 |  |  |
| V2X_n71A_47A | n71 |  |  |  |  | 23 | +2/-2.5 |  |  |
|  | 47 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 1: For the con-current band combinations, the simultaneous transmission and reception of sidelink and Uu interfaces can be supported while operation is agnostic of the service used on each interface. NOTE 2: PPowerClass is the maximum output power specified without taking into account the tolerance for each operation band. NOTE 3: For inter-band con-current operation, the aggregation power apply to the total transmitted power over all component carriers (per UE). NOTE 4: 4 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2E.1.

6.2E.1.1	UE maximum output power for Intra-band contiguous V2X

6.2E.1.1.1	Test purpose

To verify that UE’s transmit power doesn’t exceed the range prescribed by the specified power value.

6.2E.1.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. Given only single switched SL is supported as per clause 6.2E.1.0.1. The requirements in this test case can be well covered in clause 6.2.2G of TS 36.521-1 [10] and clause 6.2E.1 of TS 38.521-1 [8] and don’t need to be tested again.

6.2E.1.2	UE maximum output power for Intra-band non-contiguous V2X

6.2E.1.1.1	Test purpose

To verify that UE’s transmit power doesn’t exceed the range prescribed by the specified power value.

6.2E.1.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. Given only single switched SL is supported as per clause 6.2E.1.0.2. The requirements in this test case can be well covered in clause 6.2.2G of TS 36.521-1 [10] and clause 6.2E.1 of TS 38.521-1 [8] and don’t need to be tested again.

6.2E.1.3	UE maximum output power for Inter-band V2X

6.2E.1.3.1	Test purpose

To verify that UE’s transmit power doesn’t exceed the range prescribed by the specified power value.

6.2E.1.3.2	Test applicability

The maximum output power requirements are specified for each operating band. No exception requirements applicable to NR/E-UTRA Uu operation or NR/E-UTRA V2X operation. The requirements in this test case can be well covered in clause 6.2.2 of TS 36.521-1 [10] and clause 6.2.1 of TS 38.521-1 [8] for Uu operation, and clause 6.2.2G of TS 36.521-1 [10] and clause 6.2E.1 of TS 38.521-1 [8] for V2X operation.

6.2E.2	UE maximum output power reduction for V2X

6.2E.2.0	Minimum conformance requirements

6.2E.2.0.1	UE maximum output power reduction for Intra-band V2X

For intra-band V2X operating UE, maximum output power reduction specified in clause 6.2.3G [5] and in clause 6.2E.2 [2] apply, respectively.

6.2E.2.0.2	UE maximum output power reduction for Inter-band V2X

For the inter-band con-current NR V2X operation, the allowed maximum power reduction (MPR) for the maximum output power shall be applied per each component carrier. The MPR requirements in subclause 6.2.3 of TS 36.101 [5] apply for E-UTRA Uu operation in licensed band, and the MPR requirements in subclause 6.2E.2 of TS 38.101-1 [2] apply for NR sidelink operation. The MPR requirements in subclause 6.2.3G of TS 36.101 [5] apply for E-UTRA V2X operation, and the MPR requirements in subclause 6.2.2 of TS 38.101-1 [2] apply for NR Uu operation.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2E.2.

6.2E.2.1	UE maximum output power reduction for Intra-band V2X

6.2E.2.1.1	Test purpose

To verify that the backoff of UE’s transmit power doesn’t exceed the range prescribed by the specified MPR value and tolerance specified in clause of configured output power.

6.2E.2.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. Given only single switched SL is supported as per clause 6.2E.1.1 and 6.2E.1.2. The requirements in this test case can be well covered in clause 6.2.3G of TS 36.521-1 [10] and clause 6.2E.2 of TS 38.521-1 [8] and don’t need to be tested again.

6.2E.2.2	UE maximum output power reduction for Inter-band V2X

6.2E.2.2.1	Test purpose

To verify that the backoff of UE’s transmit power doesn’t exceed the range prescribed by the specified MPR value and tolerance specified in clause of configured output power.

6.2E.2.2.2	Test applicability

The maximum output power reduction requirements are specified for each component carrier. No exception requirements applicable to NR/E-UTRA Uu operation or NR/E-UTRA V2X operation. The requirements in this test case can be well covered in clause 6.2.3 of TS 36.521-1 [10] and clause 6.2.2 of TS 38.521-1 [8] for Uu operation, and clause 6.2.3G of TS 36.521-1 [10] and clause 6.2E.2 of TS 38.521-1 [8] for V2X operation.

6.2E.3	UE additional maximum output power reduction for V2X

6.2E.3.0	Minimum conformance requirements

6.2E.3.0.1	UE additional maximum output power reduction for Intra-band V2X

For intra-band V2X operating UE, additional maximum output power reduction specified in clause 6.2.4G [5] and in clause 6.2E.3 [2] apply, respectively.

6.2E.3.0.2	UE additional maximum output power reduction for Inter-band V2X

For the inter-band con-current NR V2X operation, the allowed additional maximum power reduction (A-MPR) for the maximum output power shall be applied per each component carrier. The A-MPR requirements in subclause 6.2.4 of TS 36.101 [5] apply for E-UTRA Uu operation in licensed band, and the A-MPR requirements in in subclause 6.2E.3 of TS 38.101-1 [2] apply for NR sidelink operation in Band n47. The A-MPR requirements in subclause 6.2.4G of TS 36.101 [5] apply for E-UTRA V2X operation, and the A-MPR requirements in subclause 6.2.3 of TS 38.101-1 [2] apply for NR Uu operation.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2E.3.

6.2E.3.1	UE additional maximum output power reduction for Intra-band V2X

6.2E.3.1.1	Test purpose

To verify that the additional backoff of UE’s transmit power doesn’t exceed the range prescribed by the specified A-MPR value and tolerance specified in clause of configured output power when the network signalling (NS) is indicated in RRC signalling.

6.2E.3.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. The requirements in this test case can be well covered in clause 6.2.4G of TS 36.521-1 [10] and clause 6.2E.3 of TS 38.521-1 [8] and don’t need to be tested again.

6.2E.3.2	UE additional maximum output power reduction for Inter-band V2X

6.2E.3.2.1	Test purpose

To verify that the additional backoff of UE’s transmit power doesn’t exceed the range prescribed by the specified A-MPR value and tolerance specified in clause of configured output power when the network signalling (NS) is indicated in RRC signalling.

6.2E.3.2.2	Test applicability

The maximum output power reduction requirements are specified for each component carrier. No exception requirements applicable to NR/E-UTRA Uu operation or NR/E-UTRA V2X operation. The requirements in this test case can be well covered in clause 6.2.4 of TS 36.521-1 [10] and clause 6.2.3 of TS 38.521-1 [8] for Uu operation, and clause 6.2.4G of TS 36.521-1 [10] and clause 6.2E.3 of TS 38.521-1 [8] for V2X operation.

6.2E.4	Configured output power for V2X

6.2E.4.0	Minimum conformance requirements

6.2E.4.0.1	UE configured output power for Intra-band V2X

For intra-band V2X operating UE, each UE configured output power specified in clause 6.2.5G [5] and in clause 6.2E.4 [2] apply, respectively.

6.2E.4.0.2	UE configured output power for Inter-band V2X

When a UE is configured for simultaneous V2X sidelink transmission and uplink transmissions for inter-band con-current operation, the UE is allowed to set its configured maximum output power PCMAX,c,Uu and PCMAX,c,V2X for the configured E-UTRA or NR uplink carrier and the configured NR V2X sidelink or E-UTRA V2X sidelink carrier, respectively, and its total configured maximum output power PCMAX,c. The TIB,V2X of PCMAX,c,Uu is specified in Table 6.2E.4.2-1.

The configured maximum output power PCMAX c,Uu(p) in subframe p for the configured E-UTRA or NR uplink carrier shall be set within the bounds:

PCMAX_L,c, Uu (p) ≤  PCMAX,c, Uu (p) ≤  PCMAX_H,c, Uu (p)

where PCMAX_L,c,Uu and PCMAX_H,c, Uu are the limits for a serving cell c as specified in subclause 6.2.5 TS 36.101 [5] or 6.2.4 TS 38.101-1 [2].

The configured maximum output power PCMAX c,V2X (q) in slot q for the configured NR or E-UTRA V2X SL carrier shall be set within the bounds:

PCMAX,c,V2X (q) ≤  PCMAX_H,c,V2X (q)

where PCMAX_H,c,V2X is the limit as specified in subclause 6.2E.4 of TS 38.101-1 [2] or 6.2.5G of TS 36.101 [5].

The total UE configured maximum output power PCMAX (p,q) in a subframe p of E-UTRA or NR uplink carrier and a slot q of NR or E-UTRA V2X sidelink carrier that overlap in time shall be set within the following bounds for synchronous and asynchronous operation unless stated otherwise:

PCMAX_L (p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

with

PCMAX_L (p,q) =  PCMAX_L,c,Uu (p)

PCMAX_H (p,q) = 10 log10 [pCMAX_H,c, Uu (p) + pCMAX_H,c,V2X (q)]

where pCMAX_H,c,V2X and pCMAX_H,c,Uu are the limits PCMAX_H,c,V2X (q) and PCMAX_H,c,Uu (p) expressed in linear scale.

The measured total maximum output power PUMAX over both the E-UTRA or NR uplink and NR or E-UTRA V2X carriers is

PUMAX = 10 log10 [pUMAX,c,Uu + pUMAX,c,V2X],

where pUMAX,c,Uu  denotes the measured output power of serving cell c for the configured E-UTRA uplink carrier or NR uplink carrier, and pUMAX,c,V2X  denotes the measured output power for the configured NR V2X sidelink carrier or E-UTRA V2X sidelink carrier expressed in linear scale.

When a UE is configured for synchronous V2X sidelink and uplink transmissions,

PCMAX_L(p, q)   –  TLOW (PCMAX_L(p, q))  ≤  PUMAX  ≤  PCMAX_H(p, q)  + THIGH (PCMAX_H(p, q))

where PCMAX_L (p,q) and PCMAX_H (p,q) are the limits for the pair (p,q) and with the tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX specified in Table 6.2E.4.1-1 of TS 38.101-1 [2]. PCMAX_L may be modified for any overlapping portion of slots (p, q) and (p +1, q+1).

Table 6.2E.4.0.2-1: ΔTIB,V2X for inter-band con-current V2X operation (two bands)

| V2X con-current operating band Configuration | Operating Band | ΔTIB,V2X [dB] |
| --- | --- | --- |
| V2X_20A_n38A | 20 | 0.01 |
| Note 1: The ΔTIB,V2X is applied on top of ΔTIB,c of DC_20A_n38A UE that is considered harmonic trap filter to reduce 3rd harmonic impact from Band 20. |  |  |

The normative reference for this requirement is TS 38.101-3 [4] clause 6.2E.4.

6.2E.4.1	Configured output power for Intra-band V2X

6.2E.4.1.1	Test purpose

To verify the measured UE configured maximum output power PUMAX is within the specified bounds.

6.2E.4.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. The requirements in this test case can be well covered in clause 6.2.5G of TS 36.521-1 [10] and clause 6.2E.4 of TS 38.521-1 [8] and don’t need to be tested again.

6.2E.4.2	Configured output power for Inter-band V2X

6.2E.4.2.1	Test purpose

To verify the measured UE configured maximum output power PUMAX is within the specified bounds.

6.2E.4.2.2	Test applicability

The configured output power for the inter-band V2X is derived based on the requirement of each component carrier. No exception requirements applicable to NR/E-UTRA Uu operation or NR/E-UTRA V2X operation. The requirements in this test case can be well covered in clause 6.2.5 of TS 36.521-1 [10] and clause 6.2.4 of TS 38.521-1 [8] for Uu operation, and clause 6.2.5G of TS 36.521-1 [10] and clause 6.2E.4 of TS 38.521-1 [8] for V2X operation.

< End of Changes >