3GPP TSG-RAN5 Meeting #107	R5-252013

# St Julian’s, Malta, 19 – 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1926 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to 6.5E V2X test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test cases of output RF spectrum emissions of V2X in FR1 are not completed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For spurious emission co-existence, test case is completed. For other test cases, general principle applies due to no exceptional requirements.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5E.1.0, 6.5E.1.1, 6.5E.2, 6.5E.3.1.0, 6.5E.3.1.1, 6.5E.3.1.2, 6.5E.3.2(new), 6.5E.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 1011 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5E	Output RF spectrum emissions for V2X operation in FR1

6.5E.1	Occupied bandwidth for V2X

6.5E.1.0	Minimum conformance requirements

For intra-band V2X, the occupied bandwidth specified in clause 6.6.1G in TS 36.101 [5] and specified in clause 6.5E.1 in TS 38.101-1 [2] apply for each frequency range respectively.

For the inter-band con-current NR V2X operation, the occupied bandwidth shall be applied per each component carrier. The requirements specified in subclause 6.6.1 of TS 36.101 [5] shall apply for the E-UTRA uplink in licensed band and the requirements specified in subclause 6.5E.1 of TS 38.101-1 [2] shall apply for the NR sidelink. The requirements specified in subclause 6.6.1G of TS 36.101 [5] shall apply for the E-UTRA sidelink and the requirements specified in subclause 6.5.1 of TS 38.101-1 [2] shall apply for the NR Uu.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5E.1.

6.5E.1.1	Occupied bandwidth for intra-band V2X

6.5E.1.1.1	Test purpose

Same test purpose as in clause 6.5E.1 in TS 38.521-1 [8].

6.5E.1.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. The requirements in this test case can be well covered in clause 6.6.1G of TS 36.521-1 [10] and clause 6.5E.1 of TS 38.521-1 [8] and don’t need to be tested again.

6.5E.1.2	Occupied bandwidth for inter-band V2X

6.5E.1.2.1	Test purpose

Same test purpose as in clause 6.5E.1 in TS 38.521-1 [8].

6.5E.1.2.2	Test applicability

For inter-band V2X operation, no exception requirements are specified for the E-UTRA/NR Uu carrier and E-UTRA/NR SL carrier.

For inter-band V2X configuration with E-UTRA Uu and NR sidelink, the requirements in this test case can be well covered in clause 6.6.1 of TS 36.521-1 [10] for E-UTRA Uu carrier and clause 6.5E.1 of TS 38.521-1 [8] for NR sidelink carrier and don’t need to be tested again.

For inter-band V2X configuration with E-UTRA sidelink and NR uplink, the requirements in this test case can be well covered in clause 6.6.1G of TS 36.521-1 [10] for E-UTRA sidelink carrier and clause 6.5.1 of TS 38.521-1 [8] for NR uplink carrier and don’t need to be tested again.

6.5E.2	Out of band emission for V2X

6.5E.2.0	Minimum conformance requirements

For intra-band V2X, out-of-band emissions specified in clause 6.6.2G in TS 36.101 [5] and specified in clause 6.5E.2 in TS 38.101-1 [2] apply for each frequency range respectively.

For the inter-band con-current NR V2X operation, the general SEM/additional SEM requirements and ACLR shall be applied per each component carrier. The general SEM/additional SEM requirements and ACLR specified in subclause 6.6.2 of TS 36.101 [5] shall apply for the E-UTRA uplink in licensed band and the general SEM/additional SEM and ACLR requirements specified in subclause 6.5E.2 of TS 38.101-1 [2] shall apply for the NR sidelink. The requirements specified in subclause 6.6.2G of TS 36.101 [5] shall apply for the E-UTRA sidelink and the requirements specified in subclause 6.5.1 of TS 38.101-1 [2] shall apply for the NR Uu.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5E.2.

6.5E.2.1	Out of band emission for intra-band V2X

6.5E.2.1.1	Test purpose

Same test purpose as in clause 6.5E.2 in TS 38.521-1 [8].

6.5E.2.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. Given only single switched SL is supported as per clause 6.5E.2.0. The requirements in this test case can be well covered in clause 6.6.2G of TS 36.521-1 [10] and clause 6.5E.2 of TS 38.521-1 [8] and don’t need to be tested again.

6.5E.2.2	Out of band emission for inter-band V2X

6.5E.2.2.1	Test purpose

Same test purpose as in clause 6.5E.2 in TS 38.521-1 [8].

6.5E.2.2.2	Test applicability

For inter-band V2X operation, no exception requirements are specified for the E-UTRA/NR Uu carrier and E-UTRA/NR SL carrier.

For inter-band V2X configuration with E-UTRA Uu and NR sidelink, the requirements in this test case can be well covered in clause 6.6.2 of TS 36.521-1 [10] for E-UTRA Uu carrier and clause 6.5E.2 of TS 38.521-1 [8] for NR sidelink carrier and don’t need to be tested again.

For inter-band V2X configuration with E-UTRA sidelink and NR uplink, the requirements in this test case can be well covered in clause 6.6.2.1G, 6.6.2.2G and 6.6.2.3G of TS 36.521-1 [10] for E-UTRA sidelink carrier and clause 6.5.2 of TS 38.521-1 [8] for NR uplink carrier and don’t need to be tested again.

6.5E.3	Spurious emissions for V2X

6.5E.3.1	General spurious emissions for V2X

6.5E.3.1.0	Minimum conformance requirements

For intra-band V2X, the general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [5] and clause 6.5E.3.1 of TS 38.101-1 [2] apply for each frequency range respectively.

For inter-band V2X, the general spurious emissions requirements shall be applied per each component carrier. The requirements specified in clause 6.6.3.1 of TS 36.101 [5] shall apply for the E-UTRA uplink in license band and the requirements specified in clause 6.5E.3.1 of TS 38.101-1 [2] shall apply for the NR sidelink. The requirements specified in subclause 6.6.3G of TS 36.101 [5] shall apply for the E-UTRA sidelink and the requirements specified in subclause 6.5.3.1 of TS 38.101-1 [2] shall apply for the NR Uu.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5E.3.

6.5E.3.1.1	General spurious emissions for intra-band V2X

6.5E.3.1.1.1	Test purpose

Same test purpose as in clause 6.5E.3.1 in TS 38.521-1 [8].

6.5E.3.1.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. The requirements in this test case can be well covered in clause 6.6.3.1 of TS 36.521-1 [10] and clause 6.5E.3.1 of TS 38.521-1 [8] and don’t need to be tested again.

6.5E.3.1.2	General spurious emissions for inter-band V2X

6.5E.3.1.2.1	Test purpose

Same test purpose as in clause 6.5E.3.1 in TS 38.521-1 [8].

6.5E.3.1.2.2	Test applicability

For inter-band V2X operation, no exception requirements are specified for the E-UTRA/NR Uu carrier and E-UTRA/NR SL carrier.

For inter-band V2X configuration with E-UTRA Uu and NR sidelink, the requirements in this test case can be well covered in clause 6.6.3.1 of TS 36.521-1 [10] for E-UTRA Uu carrier and clause 6.5E.3.1 of TS 38.521-1 [8] for NR sidelink carrier and don’t need to be tested again.

For inter-band V2X configuration with E-UTRA sidelink and NR uplink, the requirements in this test case can be well covered in clause 6.6.3G.1 of TS 36.521-1 [10] for E-UTRA sidelink carrier and clause 6.5.3.1 of TS 38.521-1 [8] for NR uplink carrier and don’t need to be tested again.

6.5E.3.2	Spurious emission band UE co-existence for V2X

6.5E.3.2.0	Minimum conformance requirements

For intra-band V2X, the spurious emissions band UE co-existence requirements specified in clause 6.6.3.2 of TS 36.101 [5] and clause 6.5E.3.2 of TS 38.101-1 [2] apply for each frequency range respectively.

For inter-band con-current V2X operation with the single CC uplink assigned to E-UTRA or NR band for coexistence with protected bands for the specified simultaneous transmission of the inter-band con-current V2X configurations in Table 6.5E.3.2.2-1. The intersection of the requirements for the individual bands specified in clause 6.5.3.2 shall also apply for the specified simultaneous transmission of the inter-band con-current V2X. Intersection of a requirement means that both UL or sidelink transmission constituent bands have the same protected band requirement specified and if one or both protected bands have note(s) associated those note(s) also apply.

For the inter-band con-current NR V2X operation, the UE-coexistence requirements in Table 6.5E.3.2.2-1 shall apply.

NOTE:	For inter-band con-current V2X operation with uplink assigned to E-UTRA/NR band and slidelink transmission assigned to E-UTRA/NR V2X operating band, the requirements in Table 6.5E.3.2.2-1 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5E.3.2.2-1 and in clause 6.5.3.2 would be considered to be verified by the measurements verifying the one uplink inter-band con-current UE to UE co-existence requirements.

Table 6.5E.3.2.0-1: Requirements for inter-band con-current V2X operation

| V2X con-current operating band cofiguration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| V2X_n71_47 | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: Applicable when NS_33 is configured by the pre-configured radio parameters for power class 3 V2X UE. NOTE 4: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5E.3.

6.5E.3.2.1	Spurious emission band UE co-existence for intra-band V2X

6.5E.3.2.1.1	Test purpose

Same test purpose as in clause 6.5E.3.2 in TS 38.521-1 [8].

6.5E.3.2.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. The requirements in this test case can be well covered in clause 6.6.3.2 of TS 36.521-1 [10] and clause 6.5E.3.2 of TS 38.521-1 [8] and don’t need to be tested again.

6.5E.3.2.2	Spurious emission band UE co-existence for inter-band V2X

6.5E.3.2.2.1	Test purpose

Same test purpose as in clause 6.5E.3.2 in TS 38.521-1 [8].

6.5E.3.2.2.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support inter-band concurrent V2X configuration.

6.5E.3.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5E.3.2.0.

6.5E.3.2.2.4	Test description

6.5E.3.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on inter-band concurrent V2X configuration specified in clause 5.5E.4. All of these configurations shall be tested with applicable test parameters for each V2X configuration, are shown in Table 6.5E.3.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2 of TS 36.521-1 [10] and Annexes A.2 of TS 38.521-1 [8]. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2 of TS 38.521-1 [8].

Table 6.5E.3.2.2.4.1-1: Inter-band concurrent V2X Test Configuration Table

FFS

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, FFS for TE diagram and clause FFS for UE diagram.

2.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3. The parameter settings for the V2X sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] clause 4.10. Message content exceptions are defined in clause 6.5E.3.2.2.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [10] Annex C.0, and uplink signals according to TS 36.521-1 [10] Annex H for the E-UTRA cell.

4.	The E-UTRA UL Reference Measurement channels and the V2X Reference Measurement Channel are set according to Table 6.5E.3.2.2.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1 [10] Annex B.0 and TS 38.521-1 [8] Annex B.0 for E-UTRA CG and NR CG respectively.

6.	Ensure the UE is in State FFS according to TS 36.508 [7] clause FFS. Message content exceptions are defined in clause 6.5E.3.2.2.4.3.

6.5E.3.2.2.4.2	Test procedure

1.	The SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the E-UTRA UL RMC according to Table 6.5E.3.2.2.4.1-1. The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the E-UTRA UL and NR sidelink RMCs.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure that the UE transmits at PUMAX level. Configure the UE to transmit PSCCH and PSSCH with the PUMAX level of each test points.

3.	Measure the mean power over all component carriers in the inter-band concurrent V2X configuration. The period of measurement shall be at least continuous duration of one sub-frame (1ms).

6.5E.3.2.2.4.3	Message contents

Message contents are according to TS 36.508 [11] clause 4.6 for E-UTRA Uu carrier and TS 38.508-1 [6] clause 4.6.3 for NR sidelink carrier.

6.5E.3.2.2.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence for inter-band concurrent V2X configuration are release specific and can be found in Table 6.5E.3.2.2.5-1. If the UE support a band combination from a later release, the requirements for this band combination are taken from the table of earliest release where requirements for this band combination are defined.

Table 6.5E.3.2.2.5-1: Requirements for inter-band concurrent V2X according to UE NR release (two bands)

| V2X con-current operating band cofiguration | Release | Spurious emission |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected Band | Frequency range (Mhz) | Maximum Level (dBm) | MBW (MHz) | NOTE |
| V2X_n71_47 | Rel-16 | N/A |  |  |  |  |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: Applicable when NS_33 is configured by the pre-configured radio parameters for power class 3 V2X UE. NOTE 4: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. |  |  |  |  |  |  |

6.5E.4	Transmit intermodulation for V2X

6.5E.4.0	Minimum conformance requirements

For intra-band V2X, transmit intermodulation requirements specified in clause 6.7G.1 of TS 36.101 [5] and clause 6.5E.4 of TS 38.101-1 [2] apply for each frequency range respectively.

For inter-band con-current NR V2X operation, the transmit intermodulation requirements shall be applied per each component carrier. The requirements specified in subclause 6.7.1 of TS 36.101 [5] shall apply for the E-UTRA uplink in licensed band and the requirements specified in subclause 6.5E.4 of TS 38.101-1 [2] shall apply for the NR sidelink. The requirements specified in subclause 6.7.1G of TS 36.101 [5] shall apply for the E-UTRA sidelink and the requirements specified in subclause 6.5.4 of TS 38.101-1 [2] shall apply for the NR Uu.

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5E.4.

6.5E.4.1	Transmit intermodulation for intra-band V2X

6.5E.4.1.1	Test purpose

Same test purpose as in clause 6.5E.4.1 in TS 38.521-1 [8].

6.5E.4.1.2	Test applicability

No exception requirements applicable to NR V2X operation or E-UTRA V2X operation. The requirements in this test case can be well covered in clause 6.7G.1 of TS 36.521-1 [10] and clause 6.5E.4.1 of TS 38.521-1 [8] and don’t need to be tested again.

6.5E.4.2	Transmit intermodulation for inter-band V2X

6.5E.4.2.1	Test purpose

Same test purpose as in clause 6.5E.4.1 in TS 38.521-1 [8].

6.5E.4.2.2	Test applicability

For inter-band V2X operation, no exception requirements are specified for the E-UTRA/NR Uu carrier and E-UTRA/NR SL carrier.

For inter-band V2X configuration with E-UTRA Uu and NR sidelink, the requirements in this test case can be well covered in clause 6.7.1 of TS 36.521-1 [10] for E-UTRA Uu carrier and clause 6.5E.4.1 of TS 38.521-1 [8] for NR sidelink carrier and don’t need to be tested again.

For inter-band V2X configuration with E-UTRA sidelink and NR uplink, the requirements in this test case can be well covered in clause 6.7.1G of TS 36.521-1 [10] for E-UTRA sidelink carrier and clause 6.5.4 of TS 38.521-1 [8] for NR uplink carrier and don’t need to be tested again.

<Unchanged Text Skipped>

F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2B.1.1 UE Maximum Output Power for Intra-Band Contiguous EN-DC | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |
| 6.2B.1.2 UE Maximum Output Power for Intra-Band Non-Contiguous EN-DC | MAX (MULTE, MUSA) MULTE ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz MUSA f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz | MULTE is MU of LTE specified in clause 6.2.2 in TS 36.521-1 [10]. MUSA is MU of FR1 SA specified in clause 6.2.1 in TS 38.521-1 [8]. |
| < Unchanged Lines Skipped > |  |  |
| 6.5B.3.1.1 General spurious emissions for intra-band contiguous EN-DC | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |
| 6.5B.3.1.2 Spurious emission band UE co-existence for intra-band contiguous EN-DC | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |
| 6.5B.3.2.1 General spurious emissions for Intra-band non-contiguous EN-DC | Same as clause 6.5.3.1 in TS 38.521-1 [8] |  |
| 6.5B.3.2.2 Spurious Emission band UE co-existence for intra-band non-contiguous EN-DC | Same as clause 6.5.3.2 in TS 38.521-1 [8] |  |
| 6.5B.3.3.1 General spurious emissions for Inter-band EN-DC within FR1 | MAX (MULTE, MUSA) MULTE 9kHz < f ≤ 4GHz: ± 2.0 dB 4GHz < f ≤ 19GHz: ± 4.0 dB MUSA for results > -60 dBm: ±2.0 dB, 9kHz < f ≤ 3GHz ±2.5 dB, 3GHz < f ≤ 4GHz ±4.0 dB, 4GHz < f ≤ 19GHz ±6.0 dB, 19GHz < f ≤ 26GHz | MULTE is MU of LTE specified in clause 6.6.3.1 in TS 36.521-1 [10]. MUSA is MU of FR1 SA specified in clause 6.5.3.1 in TS 38.521-1 [8]. |
| 6.5B.3.3.2 Spurious emission band UE co-existence for Inter-band within FR1 | MAX (MULTE, MUSA) MULTE ±2.0 dB for results > -60 dBm, f ≤ 3.0GHz ±2.5 dB, 3.0GHz < f ≤ 4.2GHz ±3.0 dB for results ≤ -60 dBm, f ≤ 3.0GHz ±3.6 dB, 3.0GHz < f ≤ 4.2GHz MUSA for results > -60 dBm: ±2.0 dB, 9kHz < f ≤ 3GHz ±2.5 dB, 3GHz < f ≤ 4GHz ±4.0 dB, 4GHz < f ≤ 19GHz ±6.0 dB, 19GHz < f ≤ 26GHz | MULTE is MU of LTE specified in clause 6.6.3.2 in TS 36.521-1 [10]. MUSA is MU of FR1 SA specified in clause 6.5.3.2 in TS 38.521-1 [8]. |
| 6.5B.3.4.1 General Spurious Emissions for Inter-band including FR2 (2 CCs) | Same as clause 6.5.3.1 in TS 38.521-2 [9] |  |
| 6.5B.3.4.1a General Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) | Same as clause 6.5.3.1_1 in TS 38.521-2 [9] |  |
| 6.5B.3.4.1_1.1 General Spurious Emissions for Inter-band including FR2 (2 NR CCs) | Same as clause 6.5A.3.1.1 in TS 38.521-2 [9] |  |
| 6.5B.3.4.1_1.2 General Spurious Emissions for Inter-band including FR2 (3 NR CCs) | Same as clause 6.5A.3.1.2 in TS 38.521-2 [9] |  |
| 6.5B.3.4.1_1.3 General Spurious Emissions for Inter-band including FR2 (4 NR CCs) | Same as clause 6.5A.3.1.3 in TS 38.521-2 [9] |  |
| 6.5B.3.4.1_1.4 General Spurious Emissions for Inter-band including FR2 (5 NR CCs) | Same as clause 6.5A.3.1.4 in TS 38.521-2 [9] |  |
| 6.5B.3.4.1_1.5 General Spurious Emissions for Inter-band including FR2 (6 NR CCs) | Same as clause 6.5A.3.1.5 in TS 38.521-2 [9] |  |
| 6.5B.3.4.1_1.6 General Spurious Emissions for Inter-band including FR2 (7 NR CCs) | Same as clause 6.5A.3.1.6 in TS 38.521-2 [9] |  |
| 6.5B.3.4.1_1.7 General Spurious Emissions for Inter-band including FR2 (8 NR CCs) | Same as clause 6.5A.3.1.7 in TS 38.521-2 [9] |  |
| 6.5B.3.4.1D General Spurious Emissions for inter-band EN-DC including FR2 for UL MIMO | Same as clause 6.5D.3.1 in TS 38.521-2 [9] |  |
| 6.5B.3.4.2 Spurious emission band UE co-existence for Inter-band including FR2 | Same as clause 6.5.3.2 in TS 38.521-2 [9] |  |
| 6.5B.3.4.2a Spurious emission band UE co-existence with Power Boost for Inter-band including FR2 (1 NR CC) | Same as clause 6.5.3.2_1 in TS 38.521-2 [9] |  |
| 6.5B.3.4.2_1.1 Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (2 NR CCs) | Same as clause 6.5A.3.2.1 in TS 38.521-2 [9] |  |
| 6.5B.3.4.2_1.2 Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (3 NR CCs) | Same as clause 6.5A.3.2.2 in TS 38.521-2 [9] |  |
| 6.5B.3.4.2_1.3 Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (4 NR CCs) | Same as clause 6.5A.3.2.3 in TS 38.521-2 [9] |  |
| 6.5B.3.4.2D Spurious emission band UE co-existence for inter-band EN-DC including FR2 for UL MIMO | Same as clause 6.5D.3.2 in TS 38.521-2 [9] |  |
| 6.5B.4.1 Additional Spurious Emissions for Intra-band contiguous EN-DC | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |
| 6.5B.4.2 Additional Spurious Emissions for Intra-band non-contiguous EN-DC | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |
| 6.5B.4.3 Additional Spurious Emissions for Inter-band EN-DC within FR1 | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |
| 6.5B.4.4 Additional Spurious Emissions for Inter-band including FR2 (1 NR CC) | Same as clause 6.5.3.3 in TS 38.521-2 [9] |  |
| 6.5B.4.4a Additional Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) | Same as clause 6.5.3.3_1 in TS 38.521-2 [9] |  |
| 6.5B.4.4_1.1 Additional Spurious Emissions for Inter-band including FR2 (2 NR CC) | Same as clause 6.5A.3.3.1 in TS 38.521-2 [9] |  |
| 6.5B.4.4_1.2 Additional Spurious Emissions for Inter-band including FR2 (3 NR CC) | Same as clause 6.5A.3.3.2 in TS 38.521-2 [9] |  |
| 6.5B.4.4_1.3 Additional Spurious Emissions for Inter-band including FR2 (4 NR CC) | Same as clause 6.5A.3.3.3 in TS 38.521-2 [9] |  |
| 6.5B.4.4D Additional Spurious Emissions for Inter-band including FR2 for UL MIMO | Same as clause 6.5D.3.3 in TS 38.521-2 [9] |  |
| 6.5B.5.3 Transmit intermodulation for Inter-band EN-DC within FR1 | Same as clause 6.5.4.3 in TS 38.521-1 [8] |  |
| 6.5E.3.2.2 Spurious emission band UE co-existence for inter-band V2X | Same as 6.5B.3.3.2 |  |
| 6.5H.1.3 Occupied bandwidth for inter-band EN-DC with UL MIMO within FR1 | Same as clause 6.5D.1 in TS 38.521-1 [8] |  |
| 6.5H.2.3.1 Spectrum emission mask for inter-band EN-DC with UL MIMO within FR1 | Same as clause 6.5D.2.2 in TS 38.521-1 [8] |  |
| 6.5H.2.3.2 Additional spectrum emission mask for inter-band EN-DC with UL MIMO within FR1 | Same as clause 6.5D.2.3 in TS 38.521-1 [8] |  |
| 6.5H.3.3.1 General spurious emissions for inter-band EN-DC with UL MIMO within FR1 | Same as clause 6.6.3.1 in TS 36.521-1 [10] for E-UTRA CC Same as clause 6.5D.3.1 in TS 38.521-1 [8] for NR CC |  |

<Unchanged Text Skipped>

F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.2B.1.1 UE Maximum Output Power for Intra-Band Contiguous EN-DC | Same as 6.2.1 in TS 38.521-1 [8] |  |
| 6.2B.1.2 UE Maximum Output Power for Intra-Band Non-Contiguous EN-DC | MAX (TTLTE, TTSA) TTLTE 0.7 dB, f ≤ 3.0GHz 1.0 dB, 3.0GHz < f ≤ 4.2GHz TTSA f ≤ 3.0GHz 0.7 dB, BW ≤ 40MHz 1.0 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.0 dB, BW ≤ 100MHz | TTLTE is TT of LTE specified in 6.2.2 in TS 36.521-1 [10]. TTSA is TT of FR1 SA specified in 6.2.1 in TS 38.521-1 [8]. |
| 6.2B.1.3 UE Maximum Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC) | MAX (TTLTE, TTSA) TTLTE 0.7 dB, f ≤ 3.0GHz 1.0 dB, 3.0GHz < f ≤ 4.2GHz TTSA f ≤ 3.0GHz 0.7 dB, BW ≤ 40MHz 1.0 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.0 dB, BW ≤ 100MHz | TTLTE is TT of LTE specified in 6.2.3 in TS 36.521-1 [10]. TTSA is TT of FR1 SA specified in 6.2.2 in TS 38.521-1 [8]. |
| 6.2B.1.3_1 UE Maximum Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC) | Same as 6.2B.1.3 |  |
| 6.2B.1.4 UE Maximum Output Power for Inter-Band EN-DC including FR2 | Same as 6.2.1 in TS 38.521-2 [9] |  |
| < Unchanged Lines Skipped > |  |  |
| 6.5B.3.1.1 General spurious emissions for intra-band contiguous EN-DC | Same as 6.5.3.1 in TS 38.521-1 [8] |  |
| 6.5B.3.1.2 Spurious emission band UE co-existence for intra-band contiguous EN-DC | Same as 6.5.3.2 in TS 38.521-1 [8] |  |
| 6.5B.3.2.1 General spurious emissions for Intra-band non-contiguous EN-DC | Same as 6.5.3.1 in TS 38.521-1 [8] |  |
| 6.5B.3.2.2 Spurious Emission band UE co-existence for intra-band non-contiguous EN-DC | Same as 6.5.3.2 in TS 38.521-1 [8] |  |
| 6.5B.3.3.1 General spurious emissions for Inter-band EN-DC within FR1 | Same as 6.5.3.1 in TS 38.521-1 [8] for NR CC Same as 6.6.3.1 in TS 36.521-1 [10] for E-UTRA CC |  |
| 6.5B.3.3.2 Spurious emission band UE co-existence for Inter-band within FR1 | Same as 6.5.3.2 in TS 38.521-1 [8] for NR CC Same as 6.6.3.2 in TS 36.521-1 [10] for E-UTRA CC |  |
| 6.5B.3.4.1 General Spurious Emissions for Inter-band including FR2 (1 NR CC) | Same as 6.5.3.1 in TS 38.521-2 [9] |  |
| 6.5B.3.4.1a General Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) | Same as clause 6.5.3.1_1 in TS 38.521-2 [9] |  |
| 6.5B.3.4.1_1.1 General Spurious Emissions for Inter-band including FR2 (2 NR CCs) | Same as clause 6.5A.3.1.1 in TS 38.521-2 [9] |  |
| 6.5B.3.4.1_1.2 General Spurious Emissions for Inter-band including FR2 (3 NR CCs) | Same as clause 6.5A.3.1.2 in TS 38.521-2 [9] |  |
| 6.5B.3.4.1_1.3 General Spurious Emissions for Inter-band including FR2 (4 NR CCs) | Same as clause 6.5A.3.1.3 in TS 38.521-2 [9] |  |
| 6.5B.3.4.1_1.4 General Spurious Emissions for Inter-band including FR2 (5 NR CCs) | Same as clause 6.5A.3.1.4 in TS 38.521-2 [9] |  |
| 6.5B.3.4.1_1.5 General Spurious Emissions for Inter-band including FR2 (6 NR CCs) | Same as clause 6.5A.3.1.5 in TS 38.521-2 [9] |  |
| 6.5B.3.4.1_1.6 General Spurious Emissions for Inter-band including FR2 (7 NR CCs) | Same as clause 6.5A.3.1.6 in TS 38.521-2 [9] |  |
| 6.5B.3.4.1_1.7 General Spurious Emissions for Inter-band including FR2 (8 NR CCs) | Same as clause 6.5A.3.1.7 in TS 38.521-2 [9] |  |
| 6.5B.3.4.1D General Spurious Emissions for inter-band EN-DC including FR2 for UL MIMO | Same as clause 6.5D.3.1 in TS 38.521-2 [9] |  |
| 6.5B.3.4.2 Spurious emission band UE co-existence for Inter-band including FR2 | Same as 6.5.3.2 in TS 38.521-2 [9] |  |
| 6.5B.3.4.2a Spurious emission band UE co-existence with Power Boost for Inter-band including FR2 (1 NR CC) | Same as clause 6.5.3.2_1 in TS 38.521-2 [9] |  |
| 6.5B.3.4.2_1.1 Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (2 NR CCs) | Same as clause 6.5A.3.2.1 in TS 38.521-2 [9] |  |
| 6.5B.3.4.2_1.2 Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (3 NR CCs) | Same as clause 6.5A.3.2.2 in TS 38.521-2 [9] |  |
| 6.5B.3.4.2_1.3 Spurious emission band UE co-existence for Inter-band EN-DC including FR2 (4 NR CCs) | Same as clause 6.5A.3.2.3 in TS 38.521-2 [9] |  |
| 6.5B.3.4.2D Spurious emission band UE co-existence for inter-band EN-DC including FR2 for UL MIMO | Same as clause 6.5D.3.2 in TS 38.521-2 [9] |  |
| 6.5B.4.1 Additional Spurious Emissions for Intra-band contiguous EN-DC | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |
| 6.5B.4.2 Additional Spurious Emissions for Intra-band non-contiguous EN-DC | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |
| 6.5B.4.3 Additional Spurious Emissions for Inter-band EN-DC within FR1 | Same as clause 6.5.3.3 in TS 38.521-1 [8] |  |
| 6.5B.4.4 Additional Spurious Emissions for Inter-band including FR2 (1 NR CC) | Same as clause 6.5.3.3 in TS 38.521-2 [9] |  |
| 6.5B.4.4a Additional Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC) | Same as clause 6.5.3.3_1 in TS 38.521-2 [9] |  |
| 6.5B.4.4_1.1 Additional Spurious Emissions for Inter-band including FR2 (2 NR CC) | Same as clause 6.5A.3.3.1 in TS 38.521-2 [9] |  |
| 6.5B.4.4_1.2 Additional Spurious Emissions for Inter-band including FR2 (3 NR CC) | Same as clause 6.5A.3.3.2 in TS 38.521-2 [9] |  |
| 6.5B.4.4_1.3 Additional Spurious Emissions for Inter-band including FR2 (4 NR CC) | Same as clause 6.5A.3.3.3 in TS 38.521-2 [9] |  |
| 6.5B.4.4D Additional Spurious Emissions for Inter-band including FR2 for UL MIMO | Same as clause 6.5D.3.3 in TS 38.521-2 [9] |  |
| 6.5B.5.3 Transmit intermodulation for Inter-band EN-DC within FR1 | Same as 6.5.4 in TS 38.521-1 [8] |  |
| 6.5E.3.2.2 Spurious emission band UE co-existence for inter-band V2X | Same as 6.5B.3.3.2 |  |
| 6.5H.1.3 Occupied bandwidth for inter-band EN-DC with UL MIMO within FR1 | Same as 6.5D.1 in TS 38.521-1 [8] |  |
| 6.5H.2.3.1 Spectrum emission mask for inter-band EN-DC with UL MIMO within FR1 | Same as clause 6.5D.2.2 in TS 38.521-1 [8] |  |
| 6.5H.2.3.2 Additional spectrum emission mask for inter-band EN-DC with UL MIMO within FR1 | Same as clause 6.5D.2.3 in TS 38.521-1 [8] |  |

< End of Changes >